3GPP TSG-RAN WG2 Meeting#127      					           R2-2406998

Maastricht , NL, Aug 19th – 23th, 2024

Source: 			ZTE corporation, Sanechips

Title: 	Consideration on QoE configuration release during inter-RAT mobility

Agenda item:		7.14.2

Document for: 	Discussion and Decision

# Introduction

This contribution intends to discuss the issues on QoE configuration release during inter-RAT mobility and the description of AppLayerIdleInactiveConfig. Proposed solution is also included in conclusion part.

# Discussion

Clarify the behavior of QoE configuration release during inter-RAT mobility

In RAN2-126 meeting, RAN2 agreed UE should release all NR QoE configurations and reports upon successful completion of the mobility from NR with the intention to allow keeping idle/inactive QoE configuration in case UE reestablishes to NR after mobility from NR failure.

RAN2-126 Agreement: =>In Rel-18, we correct the QoE config release procedure in the following way: UE releases all NR QoE configurations and reports upon successful completion of the mobility from NR.

However, it is noticed that after mobility from NR failure, the UE will initiate reestablishment procedure, during which UE may reselect to either NR or LTE cells. There is no issue when UE establishes connection towards NR cells, while for LTE case further consideration is needed. According to current specs, if the UE reselects to an LTE cell, it will firstly perform the NR going to RRC_IDLE procedure and then perform the LTE connection establishment procedure. It can be observed from highlighted description below, during the going to RRC_IDLE procedure, only QoE configurations unsuitable for non-connected states and RVQoE configuration are released, which would result in the UE retaining IDLE/INACTIVE QoE configurations/reports when establishing a connection in LTE. In such case, UE behavior is undefined on how to deal with the stored IDLE/INACTIVE QoE configuration/reports, which could potentially cause issues with subsequent QoE configurations and/or reports. Thus, the handling of QoE configurations/reports in cases of mobility from NR failure requires further clarification within the specs.

------------------------------------------------------------------- From 38331 ----------------------------------------------------------

    - 5.3.11	UE actions upon going to RRC_IDLE

[...]

1>	indicate the release of the RRC connection to upper layers together with the release cause;

1>	for each application layer measurement configuration without appLayerIdleInactiveConfig configured:

2>	inform upper layers about the release of the application layer measurement configuration;

2>	release the application layer measurement configuration;

2>	discard any application layer measurement reports which were not yet fully submitted to lower layers for transmission;

2>	consider itself not to be configured to send application layer measurement reports for the measConfigAppLayerId;

1>	for each application layer measurement configuration with appLayerIdleInactiveConfig configured:

2>	inform upper layers about the release of the RAN visible application layer measurement configuration;

2>	discard any RAN visible application layer measurement reports received from upper layers;

2>	initiate the procedure in 5.5b.1.2;

------------------------------------------------------------------- From 38331 ----------------------------------------------------------

Observation 1: When UE reselects to EUTRA cell after mobility from NR failure, the UE will perform going to RRC_IDLE procedure and initiate RRC connection set up in EUTRA, during which UE will release legacy QoE configurations and RVQoE except the IDLE/INACTIVE QoE configurations.

Observation 2: If the UE retains the IDLE/INACTIVE QoE configurations and reports when it successfully establishes a connection on LTE, the stored QoE configuration/reports may lead to issues for subsequent QoE configurations/reports.

As discussed above, it is necessary to define UE behavior to release the stored IDLE/INACTIVE QoE configurations and reports when the UE reselects to an E-UTRA cell. Potential resolutions could be as below:

- Option 1: UE release all stored QoE configurations and reports when performing NR going to RRC_IDLE procedure. (this requires specs update in NR RRC)

- Option 2: UE releases QoE configurations and reports applied to IDLE/INACTIVE states after successful RRC connection establishment in E-UTRA. (this requires specs update in LTE RRC)

Observation 3: Below options can be considered to release IDLE/INACTIVE QoE configurations/reports after mobility from NR failure:

- Option 1: UE release all stored QoE configurations and reports when performing NR going to RRC_IDLE procedure.

- Option 2: UE releases QoE configurations and reports applied to IDLE/INACTIVE states after successful RRC connection establishment in E-UTRA.

For Option 1, directly releasing all QoE configurations during the NR going to RRC_IDLE procedure may lead to certain issues. For example, the UE may fail to setup connection to an E-UTRA cell and return to NR, in such case it is still useful to keep the idle/inactive QoE configuration/reports. Moreover, the NR going to RRC_IDLE procedure is not solely triggered by mobility from NR failure, and it can be initiated in other intra-NR mobility scenarios as well. In such cases, UE still needs to retain the idle/inactive QoE configurations if the UE remains in the NR cell. Option 2 allows UE to release the IDLE/INACTIVE states QoE configurations and report only after successfully establishes a connection in LTE, which can avoids unnecessary release of QoE configurations and reports.

Observation 4: Option 1 might lead to the UE unnecessarily releasing QoE configurations and reports in case UE reselects to NR cells, while option 2 can avoid this issue by only having the UE release QoE configurations and reports upon successful connection establishment in LTE.

Based on the above analysis, it is proposed to go for option 2 that UE releases IDLE/INACTIVE QoE stored in AS layer when successfully establishing a connection in LTE. Upon releasing the stored IDLE/INACTIVE QoE UE also needs to inform application layer about releasing the corresponding QoE configuration as well.

Proposal 1: Specify in LTE RRC specs that the UE should release the IDLE/INACTIVE states QoE configurations/reports stored in the AS layer, if any, upon successful RRC connection establishment; and inform the application layer to release the corresponding QoE configuration stored.

Furthermore, since the UE behavior proposed requires interactions between AS and APP layers, which may need to be further checked by SA4 and CT1 that if existing specs can comply with this UE behavior. It is suggested to send an LS to inform them about the newly introduced UE behavior and inter-layer interactions, and check if any specs update is needed.

Proposal 2: Send LS to inform SA4 and CT1 that RAN2 has agreed to update LTE RRC specs that upon successful RRC Connection establishment procedure in LTE UE will release the NR QoE configurations/reports stored in both the AS layer and the application layer, and ask them to check if their specs requires any updates.

Clarification on AppLayerIdleInactiveConfig description

As highlighted in below text QoE measurement configurations applicable to the RRC_IDLE and RRC_INACTIVE states are equally pertinent for measurements in connected state.

- 38.300 - 21.2.4 QoE Measurement Handling in RRC_IDLE and RRC_INACTIVE States For application measurement configurations applicable in RRC_IDLE and RRC_INACTIVE states, the UE continues on-going QoE measurement collection when entering RRC_IDLE or RRC_INACTIVE state, and also when returning to RRC_CONNECTED state.

However the description of the IE (AppLayerIdleInactiveConfig) designated for QoE measurements in RRC_IDLE and RRC_INACTIVE states only indicate it is applicable for IDLE/INACTIVE, while it is also used in connected mode. Therefore, it is proposed to add RRC_connected mode in the description of AppLayerIdleInactiveConfig.

- 38.331 - – AppLayerIdleInactiveConfig The IE AppLayerIdleInactiveConfig indicates parameters specific to application layer measurements applicable to RRC_IDLE and RRC_INACTIVE.

Observation 5: AppLayerIdleInactiveConfig is applicable to all RRC states while in current RRC specs it is specified that it is specific to APP measurements applicable to RRC_IDLE and RRC_INACTIVE which is incorrect.

Proposal 3: Add in the description of AppLayerIdleInactiveConfig in RRC specs that this parameter is also applicable to connected states.

An TP have been provided in Annex 1 for P1 and P3, and a draft Ls has been provided in Annex 2 for P2. It is proposed that RAN2 discusses and agrees on the TP and the LS provided.

# Conclusion and proposals

Based on above analysis, we have the following proposals:

Observation 1: When UE reselects to EUTRA cell after mobility from NR failure, the UE will perform going to RRC_IDLE procedure and initiate RRC connection set up in EUTRA, during which UE will release legacy QoE configurations and RVQoE except the IDLE/INACTIVE QoE configurations.

Observation 2: If the UE retains the IDLE/INACTIVE QoE configurations and reports when it successfully establishes a connection on LTE, the stored QoE configuration/reports may lead to issues for subsequent QoE configurations/reports.

Observation 3: Below options can be considered to release IDLE/INACTIVE QoE configurations/reports after mobility from NR failure:

- Option 1: UE release all stored QoE configurations and reports when performing NR going to RRC_IDLE procedure.

- Option 2: UE releases QoE configurations and reports applied to IDLE/INACTIVE states after successful RRC connection establishment in E-UTRA.

Observation 4: Option 1 might lead to the UE unnecessarily releasing QoE configurations and reports in case UE reselects to NR cells, while option 2 can avoid this issue by only having the UE release QoE configurations and reports upon successful connection establishment in LTE.

Proposal 1: Specify in LTE RRC specs that the UE should release the IDLE/INACTIVE states QoE configurations/reports stored in the AS layer, if any, upon successful RRC connection establishment; and inform the application layer to release the corresponding QoE configuration stored.

Proposal 2: Send LS to inform SA4 and CT1 that RAN2 has agreed to update LTE RRC specs that upon successful RRC Connection establishment procedure in LTE UE will release the NR QoE configurations/reports stored in both the AS layer and the application layer, and ask them to check if their specs requires any updates.

Observation 5: AppLayerIdleInactiveConfig is applicable to all RRC states while in current RRC specs it is specified that it is specific to APP measurements applicable to RRC_IDLE and RRC_INACTIVE which is incorrect.

Proposal 3: Add in the description of AppLayerIdleInactiveConfig in RRC specs that this parameter is also applicable to connected states.

- Annex 1 TPs for P1 and P3

- ---------------------------------------   1st change to TS 36331 -----------------------------------------

5.3.3.4	Reception of the RRCConnectionSetup by the UE

NOTE 1:	Prior to this, lower layer signalling is used to allocate a C-RNTI. For further details see TS 36.321 [6];

The UE shall:

[...]

1>	enter RRC_CONNECTED;

1>	stop the cell re-selection procedure;

1>	consider the current cell to be the PCell;

1>	except for NB-IoT:

2>	if the UE supports RLF report for inter-RAT MRO EUTRA as defined in TS 38.306 [87], and if the UE has radio link failure or handover failure information available in VarRLF-Report of TS 38.331 [82] and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report of TS 38.331 [82]:

3>	if reconnectCellId in VarRLF-Report of TS 38.331 [82] is not set, and if the UE failed to perform reestablishment; or

3>	if reconnectCellId in VarRLF-Report of TS 38.331 [82] is not set, and if the UE selected the current PCell immediately after failure in performing MobilityFromNRCommand:

4>	if the selected PCell is an acceptable cell as defined in TS 36.304 [4]:

5>	set timeUntilReconnection in VarRLF-Report of TS 38.331 [82] to the time that elapsed since the MobilityFromNRCommand failure;

4>	if the selected PCell is a suitable cell as defined in TS 36.304 [4]:

5>	set timeUntilReconnection in VarRLF-Report of TS 38.331 [82] to the time that elapsed since the last radio link failure or handover failure;

5>	set eutraReconnectCellId in reconnectCellId in VarRLF-Report of TS 38.331 [82] to the global cell identity and the tracking area code of the PCell;

2>	if the UE radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report:

3>	if reconnectCellId in VarRLF-Report is not set, and if the UE failed to perform reestablishment:

4>	set timeUntilReconnection in VarRLF-Report to the time that elapsed since the last radio link failure or handover failure;

4>	set eutraReconnectCellId in reconnectCellId in VarRLF-Report to the global cell identity and the tracking area code of the PCell;

1>	If the UE has any stored application layer measurement configuration:

2>	release all application layer measurement configurations;

2>	inform upper layers to clear all stored application layer measurement configurations;

2>	discard the stored application layer measurement reports if any;

2>	consider itself not to be configured to send application layer measurement reports;

1>	set the content of RRCConnectionSetupComplete message as follows:

2>	if the RRCConnectionSetup is received in response to an RRCConnectionResumeRequest:

3>	if upper layers provide an S-TMSI:

4>	set the s-TMSI to the value received from upper layers;

3>	else if upper layers provide a 5G-S-TMSI:

4>	set the ng-5G-S-TMSI-Bits to ng-5G-S-TMSI with the value received from upper layers;

2>	else if upper layers provide a 5G-S-TMSI:

3>	set the ng-5G-S-TMSI-Bits to ng-5G-S-TMSI-Part2 to the leftmost 8 bits of 5G-S-TMSI received from upper layers;

---------------------------------------  1st change to TS 36331 -------------------------------------------

- ---------------------------------------  2nd change to TS 38331 -----------------------------------------

    - –	AppLayerIdleInactiveConfig

The IE AppLayerIdleInactiveConfig indicates parameters specific to application layer measurements applicable to RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED.

AppLayerIdleInactiveConfig information element

-- ASN1START

-- TAG-APPLAYERIDLEINACTIVECONFIG-START

AppLayerIdleInactiveConfig-r18 ::=   SEQUENCE {

measConfigAppLayerId-r18             MeasConfigAppLayerId-r17,

serviceType-r18                      ENUMERATED {streaming, mtsi, vr, spare5, spare4, spare3, spare2, spare1}  OPTIONAL, -- Need M

appLayerMeasPriority-r18             INTEGER (1..16)                                                           OPTIONAL, -- Need M

qoe-Reference-r18                    OCTET STRING (SIZE (6))                                                   OPTIONAL, -- Need M

qoe-MeasurementType-r18              ENUMERATED {sbased, mbased}                                               OPTIONAL, -- Need M

qoe-AreaScope-r18                    Qoe-AreaScope-r18                                                         OPTIONAL, -- Need M

mce-Id-r18                           OCTET STRING (SIZE (1))                                                   OPTIONAL, -- Need M

availableRAN-VisibleMetrics-r18      AvailableRAN-VisibleMetrics-r18                                           OPTIONAL, -- Need M

...

}

Qoe-AreaScope-r18 ::=                CHOICE {

cellGlobalIdList                     CellGlobalIdList-r16,

trackingAreaCodeList                 TrackingAreaCodeList-r16,

trackingAreaIdentityList             TrackingAreaIdentityList-r16,

plmn-IdentityList                    PLMN-IdentityList2-r16,

...

}

AvailableRAN-VisibleMetrics-r18 ::=  SEQUENCE {

appLayerBufferLevelList-r18          ENUMERATED {true}                                                         OPTIONAL, -- Need M

playoutDelayForMediaStartup-r18      ENUMERATED {true}                                                         OPTIONAL, -- Need M

...

}

-- TAG-APPLAYERIDLEINACTIVECONFIG-STOP

-- ASN1STOP

---------------------------------------  2nd change to TS 38331 -------------------------------------------

- Annex 2 LS for P2

Title:	LS on QoE configuration and reporting release during inter-RAT mobility

Release:	Release 18

Work Items:	NR_QoE_enh-Core

Source:	RAN2

To:	SA4,CT1

CC:

Contact Person:

Name:		Zhihong Qiu

E-mail Address:	qiu.zhihong@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

RAN2 has observed that after mobility from NR failure it is possible that UE could still have stored QoE configuration/reports applicable to IDLE/INACTIVE modes when reselects to an EUTRA cell, which could lead to issues for subsequent QoE configuration in EUTRA. To resolve this issue RAN2 has discussed and reached below agreements:

Agreements Specify in LTE RRC specs that the UE should release the IDLE/INACTIVE states QoE configurations and reports stored in the AS layer if any, after successful RRC connection establishment and inform the application layer to release to corresponding QoE configuration.

It is kindly asked that SA4 and CT1 take above agreements into account in their future work and check if any specs updates is needed.

# 2	Actions

To SA4 and CT1

ACTION: RAN2 kindly asks SA4 and CT1 to take the above into account, and check if any specs updates is needed.

# 3	Dates of next TSG-RAN WG2 meetings

Updated meeting schedule can be found at: https://portal.3gpp.org/?tbid=373&SubTB=380#/

TSG RAN WG2 Meeting #127-bis	 Oct  14th – Oct 18th, 2024		Hefei , CN

TSG RAN WG2 Meeting #128 Nov 18th – Nov 22th, 2024        Orlando , US