3GPP TSG-RAN WG2 Meeting #127                                                        R2-2406999

Maastricht, Netherlands, 19th - 23th August, 2024

Source:              CATT

Title:	             Discussion on the clarification of the SFN on Cell DTX/DRX

Agenda Item: 	7.3.2

Document for:	Discussion and Decision

- Introduction

In RAN2#126 meeting, the clarification of the SFN on Cell DTX/DRX was discussed but there is no clear conclusion on how to use the SFN.

Clarifications R2-2404945 Discussion on the unaligned SFN for Cell DTXDRX in MAC CATT discussion Rel-18 Netw_Energy_NR-Core Proposal: In case of unaligned SFN across carriers in a cell group, the SFN of the SpCell is used to calculate the Cell DTX/DRX duration. - LG doesn’t think we need that change as each serving cell would look for it’s own time, as DTX is executed per serving cell. Apple doesn’t think this is even in scope. - Nokia wonders if the UE knows the SFN of the SCells. LG thinks that the UE knows the SFN of the serving cell. Nokia explains that the SFN is in the MIB. LG and Lenovo explains that the UE has to read the MIB for synchornization purposes. Samsung thinks that there is no requirement to read the SFN in the MIB - Xiaomi aks if we can do CELL DTX/DRX with SSBless => Noted

In this contribution, we share our further consideration on the clarification of the SFN on Cell DTX/DRX.

- Discussion

According to the discussion at the last RAN2#126 meeting, some companies prefer to use the SFN of each serving cell for Cell DTX/DRX, but some companies have expressed some doubts about using the SFN of each serving cell for Cell DTX/DRX. Furthermore, we think that the SFN of Cell DTX/DRX should be clarified, whether using the SFN of each serving cell for Cell DTX/DRX or using the SFN of SpCell for Cell DTX/DRX.

In this section, we analyse the pros and cons of the following two cases, considering spec impact and UE power consumption, and provide the corresponding TP in Appendix.

- Case1: using the SFN of SpCell for Cell DTX/DRX

- Case2: using the SFN of each serving cell for Cell DTX/DRX

- Spec impact

Due to the SFN of different carriers may be different in Async CA, RAN2 agreed that the following clarifications [1] on SPS, configured grant and DRX are needed in RAN2#109e meeting [2].

5.3.1	 DL Assignment reception

…

NOTE:	In case of unaligned SFN across carriers in a cell group, the SFN of the concerned serving cell is used to calculate the HARQ Process ID used for configured downlink assignments.

5.4.1 	UL Grant reception

…

NOTE 4: In case of unaligned SFN across carriers in a cell group, the SFN of the concerned serving cell is used to calculate the HARQ Process ID used for configured uplink grants.

5.7 Discontinuous Reception (DRX)

…

NOTE 2:	In case of unaligned SFN across carriers in a cell group, the SFN of the SpCell is used to calculate the DRX duration.

5.8	 Transmission and reception without dynamic scheduling

5.8.1	 Downlink

…

NOTE:	In case of unaligned SFN across carriers in a cell group, the SFN of the concerned serving cell is used to calculate the occurences of configured downlink assignments.

5.8.2	 Uplink

…

NOTE:	In case of unaligned SFN across carriers in a cell group, the SFN of the concerned serving cell is used to calculate the occurences of configured uplink grants.

Observation1: Because SFN of different carriers may be unaligned in Async CA, RAN2 agreed to clarify that the UE use SFN of the SpCell for calculation of the DRX duration in DRX.

Observation 2: Because SFN of different carriers may be unaligned in Async CA, RAN2 agreed to clarify that the UE use SFN of concerned serving cell for calculation of HARQ process ID and downlink/uplink assignment occurrences in SPS/CG.

Similarly, in order to make it clear which SFN should be used in case of unaligned SFN across carriers in a cell group, the additional clarification is also needed for the calculation of the beginning of Cell DTX/DRX Active Period. And the corresponding TP of case 1 and case 2 for MAC spec are provided in Appendix 5.1 and 5.2, respectively.

Proposal 1: In order to make it clear which SFN should be used in case of unaligned SFN across carriers in a cell group, the additional clarification of SFN is needed for the calculation of the beginning of Cell DTX/DRX Active Period.

According to the field description of cellDTX-DTX-Config, a maximum of two Cell DTX/DRX patterns can be configured per MAC entity for different serving cells. In order to achieve the configured patterns are aligned, the restriction is added as yellow highlighted below [3].

cellDTX-DRX-Config Used to configure cell DTX/DRX for the serving cell, as specified in TS 38.321 [3]. A maximum of two cell DTX/DRX patterns can be configured per MAC entity for different serving cells. The two configured patterns are aligned, that the start and slot offset are common and the periodicity of one pattern is an integer multiple of the other. Cell DTX is configured only when connected mode DRX is configured.

However, if the beginning of the Cell DTX/DRX Active Period for each serving cell is calculated based on the SFN of this serving cell (case2), for Async CA, the beginning of Cell DTX/DRX Active Period may be different even if the start and slot offset are common for the two Cell DTX/DRX patterns, which against the purpose of aligning the two configured patterns.

Observation 3: For case 2, if the start and slot offset of two patterns are common, the two Cell DTX/DRX patterns may not be aligned in Async CA.

To achieve alignment of two Cell DTX/DRX patterns in case 2, for Async CA, the restrictions on the start and slot offset of two Cell DTX/DRX patterns can be removed, and through NW implementation, the alignment between two Cell DTX/DRX patterns can be achieved as much as possible. 	In this way, it is necessary to modify the field description of cellDTX-DTX-Config in RRC spec, and the corresponding TP of case 2 for RRC spec are provided in Appendix 5.3.

Observation 4: Compared to case1, Case 2 requires additional modifications to the start and slot offset of the two Cell DTX/DRX patterns in the RRC spec.

- UE power consumption

Figure 1 and Figure 2 show the beginning of the Cell DTX/DRX Active Period for two different cells of case 1 and case 2 in Async CA, respectively. When the start and slot offset of two Cell DTX/DRX patterns are common, the overlap of Cell DTX/DRX duration for cell 1 and cell 2 is duration1 for case1, and the overlap of Cell DTX/DRX durations for cell 1 and cell 2 is duration2 for case2. From UE perspective, the Cell DTX/DRX duration in one Cell DTX/DRX period in case2 is duration1+ (duration1- duration2), which is not friendly to the UE's power consumption.

Figure 1 The beginning of Cell DTX/DRX Active Period of case1 in Async CA

Figure 2 The beginning of Cell DTX/DRX Active Period of case 2 in Async CA

Based on the above analysis, the alignment between two Cell DTX/DRX patterns can be achieved as much as possible if the start and slot offset of two Cell DTX/DRX patterns are different, which is also applicable to the same Cell DTX/DRX patterns with different SFN, as shown in Figure2. Therefore, if the restrictions on the start and slot offset of two Cell DTX/DRX patterns are removed, the power consumption of UE in case 1 and case 2 is similar.

In addition, in current specification, UE is not required to read the SFN in MIB. However, for case2, UE must read the SFN of the serving cell to calculate the beginning of the Cell DTX/DRX Active Period, which may increase the power consumption of UE.

Observation 5: Compared to case1, the power consumption of UE in case2 is slightly higher.

- Cell DTX/DRX with SSB-less

Considering Cell DTX/DRX with SSB-less, there is no impact on SSB-less cell in case1 and case2. For case1, if Cell DTX/DRX is configured with a SSB-less cell, the SFN of the SSB-less cell can be obtained from the SpCell. For case2, if Cell DTX/DRX is configured with a SSB-less cell, the SFN of the SSB-less cell can be obtained from an intra-band serving cell for intra-band SSB-less SCell or the reference cell for inter-band SSB-less SCell.

Observation 6: Both case1 and case2 have no impact on Cell DTX/DRX with SSB-less.

Based on the discussion aforementioned, we propose that RAN2 discuss case1 and case2 and determine the corresponding specification modifications.

Proposal 2: RAN2 discuss which of the following cases can be used for Cell DTX/DRX:

-	Case1: using the SFN of SpCell for Cell DTX/DRX

-	Case2: using the SFN of each serving cell for Cell DTX/DRX

Proposal 2-1: If using the SFN of SpCell for Cell DTX/DRX is supported, add a note to clarify that the SFN of the SpCell is used to calculate the beginning of Cell DTX/DTX Active Period in case of unaligned SFN across carriers in a cell group.

Proposal 2-2: If using the SFN of each serving cell for Cell DTX/DRX is supported, add a note to clarify that the SFN of the concerned serving cell is used to calculate the beginning of Cell DTX/DTX Active Period in case of unaligned SFN across carriers in a cell group.

Proposal 2-3: If using the SFN of each serving cell for Cell DTX/DRX is supported, the restriction that the start and slot offset of two Cell DTX/DRX patterns are common is only applicable to sync CA case.

- Conclusion

According to the analysis in section 2, it is observed and proposed that:

Observation1: Because SFN of different carriers may be unaligned in Async CA, RAN2 agreed to clarify that the UE use SFN of the SpCell for calculation of the DRX duration in DRX.

Observation 2: Because SFN of different carriers may be unaligned in Async CA, RAN2 agreed to clarify that the UE use SFN of concerned serving cell for calculation of HARQ process ID and downlink/uplink assignment occurrences in SPS/CG.

Observation 3: For case 2, if the start and slot offset of two patterns are common, the two Cell DTX/DRX patterns may not be aligned in Async CA.

Observation 4: Compared to case1, Case 2 requires additional modifications to the start and slot offset of the two Cell DTX/DRX patterns in the RRC spec.

Observation 5: Compared to case1, the power consumption of UE in case2 is slightly higher.

Observation 6: Both case1 and case2 have no impact on Cell DTX/DRX with SSB-less.

Proposal 1: In order to make it clear which SFN should be used in case of unaligned SFN across carriers in a cell group, the additional clarification of SFN is needed for the calculation of the start of Cell DTX/DRX Active Period.

Proposal 2: RAN2 discuss which of the following cases can be used for Cell DTX/DRX:

-	Case1: using the SFN of SpCell for Cell DTX/DRX

-	Case2: using the SFN of each serving cell for Cell DTX/DRX

Proposal 2-1: If using the SFN of SpCell for Cell DTX/DRX is supported, add a note to clarify that the SFN of the SpCell is used to calculate the beginning of Cell DTX/DTX Active Period in case of unaligned SFN across carriers in a cell group.

Proposal 2-2: If using the SFN of each serving cell for Cell DTX/DRX is supported, add a note to clarify that the SFN of the concerned serving cell is used to calculate the beginning of Cell DTX/DTX Active Period in case of unaligned SFN across carriers in a cell group.

Proposal 2-3: If using the SFN of each serving cell for Cell DTX/DRX is supported, the restriction that the start and slot offset of two Cell DTX/DRX patterns are common is only applicable to sync CA case.

- Reference

- TS38.321, Medium Access Control (MAC) protocol specification (Release 18), 3GPP

- R2-2002382, Introduction of dormant BWP operation and Async CA, OPPO, RAN2#109e

- TS 38.331, "NR; Radio Resource Control (RRC); Protocol specification".

- Appendix

  - Text proposal for 38.321 in case1

<<<<<<<<<<<<<<<<<<<< Start of change >>>>>>>>>>>>>>>>>>>>

5.34.2	Cell Discontinuous Transmission

Cell DTX is configured if cellDTXDRXconfigType is set to dtx or dtxdrx. Cell DTX operation is activated and deactivated for each Serving Cell by:

-	receiving a cell DTX indication from lower layers indicating activation or deactivation of cell DTX operation, as specified in TS 38.213 [6];

-	configuring CellDTXDRX-Config by upper layers: if cell DTX is configured and cellDTXDRXactivationStatus is set to activated, cell DTX operation is activated upon cell DTX configuration; if cell DTX is configured and cellDTXDRXactivationStatus is set to deactivated, cell DTX operation is deactivated upon cell DTX configuration; if CellDTXDRX-Config is released, cell DTX operation is deactivated and all the corresponding configurations are released.

When cell DTX is configured and activated for a Serving Cell, the cell DTX Active Period includes the time while:

-	celldtxdrx-onDurationTimer is running for the associated Serving Cell.

For each Serving Cell configured with cell DTX, the MAC entity shall:

1>	if cell DTX is activated for this Serving Cell:

2>	if [(SFN × 10) + subframe number] modulo (celldtxdrx-Cycle) = (celldtxdrx-StartOffset):

3>	start celldtxdrx-onDurationTimer for this serving cell after celldtxdrx-SlotOffset from the beginning of the subframe.

1>	if cell DTX operation is deactivated for this Serving Cell; or

1>	if the Serving Cell is in the cell DTX Active Period:

2>	monitor PDCCH on this Serving Cell, as specified in TS 38.213 [6] and other clauses of this specification.

1>	if any drx-RetransmissionTimerDL, drx-RetransmissionTimerUL or drx-RetransmissionTimerSL (as described in clause 5.7) is running on any Serving Cell in the DRX group of this Serving Cell; or

1>	if ra-ContentionResolutionTimer (as described in clause 5.1.5) or msgB-ResponseWindow (as described in clause 5.1.4a) is running; or

1>	if a Scheduling Request is sent on PUCCH and is pending (as described in clause 5.4.4 or 5.22.1.5); or

1>	if a PDCCH indicating a new transmission addressed to the C-RNTI of the MAC entity has not been received after successful reception of a Random Access Response for the Random Access Preamble not selected by the MAC entity among the contention-based Random Access Preamble (as described in clauses 5.1.4 and 5.1.4a):

2>	monitor PDCCH on the Serving Cells in the DRX group of this Serving Cell, as specified in TS 38.213 [6] and other clauses of this specification.

1>	if ra-ResponseWindow (as described in clause 5.1.4) is running and this Serving Cell is the SpCell:

2>	monitor PDCCH on this Serving Cell (as described in clause 5.1.4).

NOTE: 	In case of unaligned SFN across carriers in a cell group, the SFN of the SpCell is used to calculate the beginning of the cell DTX Active Period.

For each Serving Cell configured with cell DTX, the MAC entity need not:

1>	if cell DTX operation is activated and the Serving Cell is not in the cell DTX Active Period:

2>	monitor PDCCH for the MAC entity's RNTIs listed in clauses 5.7 and 5.7b, irrespective of the requirements of clauses 5.7 and 5.7b, unless stated otherwise in this clause;

2>	instruct the physical layer to receive transport block on the DL-SCH of this Serving Cell according to a configured downlink assignment for SPS;

2>	indicate the presence of a configured downlink assignment and deliver the stored HARQ information to the HARQ entity;

2>	set the HARQ Process ID to the HARQ Process ID associated with the PDSCH duration of a configured downlink assignment;

2>	consider the NDI bit for the HARQ process corresponding to the PDSCH duration of a configured downlink assignment to have been toggled for the configured downlink assignment.

5.34.3	Cell Discontinuous Reception

Cell DRX is configured if cellDTXDRXconfigType is set to drx or dtxdrx. Cell DRX operation is activated and deactivated for each Serving Cell by:

-	receiving a cell DRX indication from lower layers indicating activation or deactivation of cell DRX operation, as specified in TS 38.213 [6];

-	configuring CellDTXDRX-Config by upper layers: if cell DRX is configured and cellDTXDRXactivationStatus is set to activated, cell DRX operation is activated upon cell DRX configuration; if cell DRX is configured and cellDTXDRXactivationStatus is set to deactivated, cell DRX operation is deactivated upon cell DRX configuration; if CellDTXDRX-Config is released, cell DRX operation is deactivated and all the corresponding configurations are released.

When cell DRX is configured and activated for a Serving Cell, the cell DRX Active Period includes the time while:

-	celldtxdrx-onDurationTimer is running for the associated Serving Cell.

For each Serving Cell configured with cell DRX, the MAC entity shall:

1>	if cell DRX is activated for this Serving Cell:

2>	if [(SFN × 10) + subframe number] modulo (celldtxdrx-Cycle) = (celldtxdrx-StartOffset):

3>	start celldtxdrx-onDurationTimer for this serving cell after celldtxdrx-SlotOffset from the beginning of the subframe.

1>	if cell DRX is activated and the Serving Cell is not in the cell DRX Active Period:

2>	not instruct the physical layer to signal a SR on a PUCCH resource for SR;

2>	not increment the SR_COUNTER for a SR;

2>	not start the sr-ProhibitTimer for a SR;

2>	not deliver any configured uplink grant and the associated HARQ information to the HARQ entity;

2>	not instruct a HARQ process associated with a configured uplink grant to trigger a new transmission or a retransmission;

2>	not report CSI on PUCCH and semi-persistent CSI configured on PUSCH;

2>	if an emergency service is initiated by upper layers and this Serving Cell is the SpCell:

3>	initiate a Random Access procedure (as specified in clause 5.1.1).

NOTE:	How the MAC layer in the UE is aware of an ongoing emergency service is up to UE implementation.

NOTE 2:	In case of unaligned SFN across carriers in a cell group, the SFN of the SpCell is used to calculate the beginning of the cell DRX Active Period.

<<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>>

  - Text proposal for 38.321 in case2

<<<<<<<<<<<<<<<<<<<< Start of change >>>>>>>>>>>>>>>>>>>>

5.34.2	Cell Discontinuous Transmission

Cell DTX is configured if cellDTXDRXconfigType is set to dtx or dtxdrx. Cell DTX operation is activated and deactivated for each Serving Cell by:

-	receiving a cell DTX indication from lower layers indicating activation or deactivation of cell DTX operation, as specified in TS 38.213 [6];

-	configuring CellDTXDRX-Config by upper layers: if cell DTX is configured and cellDTXDRXactivationStatus is set to activated, cell DTX operation is activated upon cell DTX configuration; if cell DTX is configured and cellDTXDRXactivationStatus is set to deactivated, cell DTX operation is deactivated upon cell DTX configuration; if CellDTXDRX-Config is released, cell DTX operation is deactivated and all the corresponding configurations are released.

When cell DTX is configured and activated for a Serving Cell, the cell DTX Active Period includes the time while:

-	celldtxdrx-onDurationTimer is running for the associated Serving Cell.

For each Serving Cell configured with cell DTX, the MAC entity shall:

1>	if cell DTX is activated for this Serving Cell:

2>	if [(SFN × 10) + subframe number] modulo (celldtxdrx-Cycle) = (celldtxdrx-StartOffset):

3>	start celldtxdrx-onDurationTimer for this serving cell after celldtxdrx-SlotOffset from the beginning of the subframe.

1>	if cell DTX operation is deactivated for this Serving Cell; or

1>	if the Serving Cell is in the cell DTX Active Period:

2>	monitor PDCCH on this Serving Cell, as specified in TS 38.213 [6] and other clauses of this specification.

1>	if any drx-RetransmissionTimerDL, drx-RetransmissionTimerUL or drx-RetransmissionTimerSL (as described in clause 5.7) is running on any Serving Cell in the DRX group of this Serving Cell; or

1>	if ra-ContentionResolutionTimer (as described in clause 5.1.5) or msgB-ResponseWindow (as described in clause 5.1.4a) is running; or

1>	if a Scheduling Request is sent on PUCCH and is pending (as described in clause 5.4.4 or 5.22.1.5); or

1>	if a PDCCH indicating a new transmission addressed to the C-RNTI of the MAC entity has not been received after successful reception of a Random Access Response for the Random Access Preamble not selected by the MAC entity among the contention-based Random Access Preamble (as described in clauses 5.1.4 and 5.1.4a):

2>	monitor PDCCH on the Serving Cells in the DRX group of this Serving Cell, as specified in TS 38.213 [6] and other clauses of this specification.

1>	if ra-ResponseWindow (as described in clause 5.1.4) is running and this Serving Cell is the SpCell:

2>	monitor PDCCH on this Serving Cell (as described in clause 5.1.4).

NOTE: 	In case of unaligned SFN across carriers in a cell group, the SFN of the concerned serving cell is used to calculate the beginning of the cell DTX Active Period.

For each Serving Cell configured with cell DTX, the MAC entity need not:

1>	if cell DTX operation is activated and the Serving Cell is not in the cell DTX Active Period:

2>	monitor PDCCH for the MAC entity's RNTIs listed in clauses 5.7 and 5.7b, irrespective of the requirements of clauses 5.7 and 5.7b, unless stated otherwise in this clause;

2>	instruct the physical layer to receive transport block on the DL-SCH of this Serving Cell according to a configured downlink assignment for SPS;

2>	indicate the presence of a configured downlink assignment and deliver the stored HARQ information to the HARQ entity;

2>	set the HARQ Process ID to the HARQ Process ID associated with the PDSCH duration of a configured downlink assignment;

2>	consider the NDI bit for the HARQ process corresponding to the PDSCH duration of a configured downlink assignment to have been toggled for the configured downlink assignment.

5.34.3	Cell Discontinuous Reception

Cell DRX is configured if cellDTXDRXconfigType is set to drx or dtxdrx. Cell DRX operation is activated and deactivated for each Serving Cell by:

-	receiving a cell DRX indication from lower layers indicating activation or deactivation of cell DRX operation, as specified in TS 38.213 [6];

-	configuring CellDTXDRX-Config by upper layers: if cell DRX is configured and cellDTXDRXactivationStatus is set to activated, cell DRX operation is activated upon cell DRX configuration; if cell DRX is configured and cellDTXDRXactivationStatus is set to deactivated, cell DRX operation is deactivated upon cell DRX configuration; if CellDTXDRX-Config is released, cell DRX operation is deactivated and all the corresponding configurations are released.

When cell DRX is configured and activated for a Serving Cell, the cell DRX Active Period includes the time while:

-	celldtxdrx-onDurationTimer is running for the associated Serving Cell.

For each Serving Cell configured with cell DRX, the MAC entity shall:

1>	if cell DRX is activated for this Serving Cell:

2>	if [(SFN × 10) + subframe number] modulo (celldtxdrx-Cycle) = (celldtxdrx-StartOffset):

3>	start celldtxdrx-onDurationTimer for this serving cell after celldtxdrx-SlotOffset from the beginning of the subframe.

1>	if cell DRX is activated and the Serving Cell is not in the cell DRX Active Period:

2>	not instruct the physical layer to signal a SR on a PUCCH resource for SR;

2>	not increment the SR_COUNTER for a SR;

2>	not start the sr-ProhibitTimer for a SR;

2>	not deliver any configured uplink grant and the associated HARQ information to the HARQ entity;

2>	not instruct a HARQ process associated with a configured uplink grant to trigger a new transmission or a retransmission;

2>	not report CSI on PUCCH and semi-persistent CSI configured on PUSCH;

2>	if an emergency service is initiated by upper layers and this Serving Cell is the SpCell:

3>	initiate a Random Access procedure (as specified in clause 5.1.1).

NOTE:	How the MAC layer in the UE is aware of an ongoing emergency service is up to UE implementation.

NOTE 2:	In case of unaligned SFN across carriers in a cell group, the SFN of the concerned serving cell is used to calculate the beginning of the cell DRX Active Period.

<<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>>

  - Text proposal for 38.331 in case2

<<<<<<<<<<<<<<<<<<<< Start of change >>>>>>>>>>>>>>>>>>>>

6.3.2	Radio resource control information elements

–	ServingCellConfig

<Irrelevant Texts Omitted>

cellDTX-DRX-Config Used to configure cell DTX/DRX for the serving cell, as specified in TS 38.321 [3]. A maximum of two cell DTX/DRX patterns can be configured per MAC entity for different serving cells. The two configured patterns are aligned, that the periodicity of one pattern is an integer multiple of the other, and the start and slot offset are common for sync CA case. Cell DTX is configured only when connected mode DRX is configured.

<<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>>