3GPP TSG-RAN WG2 Meeting #127                                                                    R2-2407003

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Source:	CATT

Title:	Consideration on on-demand SIB1 issues

Agenda Item:	8.5.3

Document for:	Discussion and Decision

Introduction

In this contribution, we will discuss the remaining essential issues and the potential impact if supporting case 3, our views will be given as well.

Discussion

Issues on on-demand SIB1

In this section, regarding the achieved agreements in [1], the further issues associated to RAN2 are listed and discussed for on-demand SIB1.

The issues after camping on NES Cell

Need of WUS configuration acquisition in NES Cell

RAN2#126 meeting agreements SIB1 acquisition upon SIB change notification in NES cell Once the NES UE camps on the NES cell, if the UE receives SIB change notification, the UE is expected to receive SIB1 from NES cell. Paging and SIB1 update in NES cell After Rel-19 NES UEs camp in NES cell, the UE behaviour is same as the one defined as legacy normal camped state, e.g. paging reception, SIB1 update, etc.

According to above agreement, after camping on NES cell, UE will monitor the paging in NES Cell. Besides, for SIB updating, UE will receive SI change indication by short message in the modification period, and read the updated SI message in the followed modification period.

RAN2 already agreed that WUS configuration could be obtained from Cell A and stored by NES UE. After UE camping on NES Cell, it is possible that NES cell update its WUS configuration for some reason. For example, the network decides to adjust the RA resource/preamble because RA resource becomes rare or conflicts with the WUS configuration from another NES Cell.

Observation 1: It is possible that WUS configuration for on-demand SIB1 in NES cell will be updated.

RAN2 defined that the UE needs to apply the system information acquisition procedure if the valid SI is stored for 3 hours, by legacy procedure as defined in clause 5.2.2.3, UE needs to read SIB1 firstly and then read the corresponding SI. In this case, UE has to apply the correct WUS configuration to trigger network transmit on-demand SIB1 information. So, it is necessary for UE in NES Cell to obtain the updated WUS configuration if needed.

TS 38.331 5.2.2.2.1 SIB validity The UE shall apply the SI acquisition procedure as defined in clause 5.2.2.3 upon cell selection (e.g. upon power on), cell-reselection, return from out of coverage, after reconfiguration with sync completion, after entering the network from another RAT, upon receiving an indication that the system information has changed, upon receiving a PWS notification, upon receiving request (e.g., a positioning request) from upper layers; and whenever the UE does not have a valid version of a stored SIB or posSIB or a valid version of a requested SIB. When the UE acquires a MIB or a SIB1 or an SI message in a serving cell as described in clause 5.2.2.3, and if the UE stores the acquired SIB, then the UE shall store the associated areaScope, if present, the first PLMN-Identity in the PLMN-IdentityInfoList for non-NPN-only cells or the first NPN identity (SNPN identity in case of SNPN, or PNI-NPN identity in case of PNI-NPN) in the NPN-IdentityInfoList for NPN-only cells, the cellIdentity, the systemInformationAreaID, if present, and the valueTag, if present, as indicated in the si-SchedulingInfo for the SIB. If the UE stores the acquired posSIB, then the UE shall store the associated areaScope, if present, the cellIdentity, the systemInformationAreaID, if present, the valueTag, if provided in assistanceDataSIB-Element, and the expirationTime if provided in assistanceDataSIB-Element. The UE may use a valid stored version of the SI except MIB, SIB1, SIB6, SIB7 or SIB8 e.g. after cell re-selection, upon return from out of coverage or after the reception of SI change indication. The valueTag and expirationTime for posSIB is optionally provided in assistanceDataSIB-Element, as specified in TS 37.355 [49]. <skip> The UE shall: 1> delete any stored version of a SIB after 3 hours from the moment it was successfully confirmed as valid; <skip> 5.2.2.3 Acquisition of System Information 5.2.2.3.1 Acquisition of MIB and SIB1 <skip>

Proposal 1: When a valid SIB is stored for more than 3 hours, UE needs to read the SIB1 and the corresponding SIB by legacy procedure, it is necessary for UE to apply the WUS configuration acquisition in NES Cell to obtain the on-demand SIB1.

How to configure or update the WUS configuration

To support the transmission of WUS configuration at NES Cell, the network needs to broadcast the updated WUS configuration in SI message, then, UE will obtain it by SI change indication procedure with two MPs. Also, it is feasible for network to include the WUS configuration in RRC release message for Cell A.

Proposal 2:  The NES Cell will broadcast the WUS configuration in SI message.

Need of SIB1 validity

For NES scenario, network will transmit SIB1 based on-demand mechanism. In RAN2#126 meeting, companies discussed the issue on SIB1 validity and postponed this issue. Here, the need of SIB1 validity is discussed based on two cases:

- Case 1: UE is in Cell A or other non-NES Cell, and UE obtain the on-demand SIB1 of NES Cell by cell reselection or mobility, such as, moving from NES Cell.

- Case 2: UE is in NES Cell.

For case 1, the UE is in non-NES Cell, on-demand SIB1 in NES Cell may be updated at an unpredictable time point, so the stored on-demand SIB1 will be invalid. But, it is not necessary for UE to restore or update this on-demand SIB1, as it is not sure when the UE will move back to NES Cell, the early on-demand SIB1 updating when UE camping on non-NES cell is not necessary. If UE decides to move back to NES cell based on cell reselection procedure at a moment, it could trigger the WUS procedure again, on-demand SIB1 validity should not be considered in this case.

Proposal 3: NES Cell’s SIB1 validity should not be considered when UE camps on non-NES Cell.

For case 2, UE camps on NES Cell and has stored a valid on-demand SIB1 message. By legacy procedure, if a valid stored or requested SIB other than SIB1 is stored for more than 3 hours, UE will read the SIB1 and then read other SIB, so the UE will read and restore SIB1 naturally when other valid SIB is stored for more than 3 hours. If the network updates the SIB, it will perform the SI change notification procedure, also, the UE will read and stored the SIB1 firstly and then acquire the changed SIB. So, SIB1 will be read and restored once the other SIB is updated, no matter which is introduced by the case of valid SIB is stored for more than 3hours at UE side or the network side notices the SI change, the valid SIB1 will be updated following the moment when the other SIB is updated. So, it is difficult to find a case that only SIB1 is valid for almost 3 hours but other SIB is valid for less than 3 hours, because the SIB1 will be updated together when other SIB is updated. Hence, it is not necessary to introduce the validity of S1B1 for the case that SIB1 stored time is more than 3 hours, the SIB1 validity could be guaranteed by other SIB validity.

Proposal 4: It is not needed to consider the additional mechanism for SIB1 validity at NES Cell.

In short message, the bit of systemInfoModification could indicate the modification of SIB other than SIB6/7/8 and posSIBs. If SIB1 is changed, the network could perform legacy SI change notification, by which, UE will read and restored SIB1. So the SI change notification procedure could be used for SIB1 change, no extra spec impact will be introduced.

Proposal 5: The SI change notification procedure could be used for SIB1 change in NES Cell, no extra spec impact will be introduced.

The condition to trigger WUS

The legacy intra-F/inter-F cell re-selection procedure will used by UE , if NES cell is evaluated as the best cell or the candidate cell meets the conditions of the cell reselection, the UE will  triggers WUS transmission. In order to avoid more UEs camping on NES cell, the NES cell should be not served as much as possible. Some enhancement may be needed to strength the condition to camp on NES cell.  Such as extra measurement conditions besides the current cell reselection rule will be applied to avoid triggering WUS to attempt camping to NES Cell.

Proposal 6: Besides the cell reselection rule, RAN2 to further discuss the enhancement, such as measurement restriction, to avoid easily triggering WUS to attempt camping on NES Cell.

Issues on Msg.1 based RA procedure

RAN2#126 meeting agreements UE behaviour on RACH failure of OD-SIB1 REQ As baseline, upon random access procedure failure of OD-SIB1 request, UE regards OD-SIB1 can’t be acquired in the NES cell and considers it as barred. It doesn’t exclude the option to leave the determination to the UE.

In last meeting, as baseline, RAN2 already agreed that, UE needs to monitor RAR as network feedback, and upon random access procedure failure of OD-SIB1 request, UE regards OD-SIB1 can’t be acquired in the NES cell and considers it as barred.  In legacy on-demand SIB request procedure, it is UE implementation to request SIB again after the failed RA procedure. So, for on-demand SIB1 request, it also be considered as UE implementation to trigger another RA procedure, the UE will consider the NES cell barred if it could not acquired the OD-SIB1.

Proposal 7: It is UE implementation whether to trigger another RA procedure after Msg.1-based RA procedure is failed for OD-SIB1.

Impact to legacy UE

RAN2#126 meeting agreements NW need to bar the legacy UE from accessing the on-demand SIB1 cell (e.g. based on the existing barring mechanism) How to avoid/deprioritize the legacy UE camping at the Cell A attempting to switch to the NES Cell (but allowing the R19 NES UE to do that).

According to above agreement, the legacy UE will not camp on the NES cell by barring mechanism. However, the legacy UE will still attempt to access to NES cell. For example, based on legacy procedure, legacy UE will perform measurement to the NES cell. The measurement result for NES cell may be the best, the legacy UE will try to reading the MIB and SIB1 of NES cell, which is power consumption for legacy UE. In order to avoid the unnecessary attempting procedure, it is suggested to move the NES cell in blacklist. Then, the legacy UE will not perform the measurement and cell reselection procedure to NES cell.

Proposal 8: In order to avoid the unnecessary attempting to switch to NES Cell by legacy UE, it is suggested to move the NES cell in blacklist.

As the legacy UE cannot camp on the on-demand SIB1 cell, the paging transmission for legacy UEs is also not needed at NES Cell from the view of power saving.

Proposal 9: For network energy saving, the paging for legacy UEs at NES Cell needs to be avoided.

Whether and how to support Case3

In RAN1#117 meeting, the following agreements on scenarios have been achieved.

RAN1#117 meeting agreement For SIB1 in idle/inactive mode, prioritize RAN1 discussions on Case 2 and Case 3 - Case 2 (Option 1+B+X) is feasible from RAN1 perspective. - Further study Case 3, focusing on the additional NES benefits over Case 2, feasibility, complexity, and spec impact. For further study of on-demand SIB1 in idle/inactive mode, it is assumed that always-on SSB is transmitted on the NES cell with on-demand SIB1.

In RAN2#126 meeting, RAN2 also discussed case 3 but postponed it.

Figure 1:   The flow chart for case 2 and case 3

In our view, there are two alternative to support case 3.

- Alternative 1, reuse the legacy on-demand SIB request procedure, all the NES Cell’s SIB1(s) are included in a legacy SIB or a new SIB, UE will read the SIB to find its interesting NES Cell’s SIB1.

- Alternative 2, enhance on-demand SIB request procedure as case 2, the WUS for a specific NES Cell should be introduced for UE requesting a specific NES Cell’s SIB1.

The alternative 1 will make Cell A broadcast all of the collected NES Cell’s SIB1(s) because it could not identify which NES Cell is requested by UE, it is not power saving. Alternative 2 is aligned to current understanding to WUS based on-demand SIB1 request procedure, and give Cell A more flexibility to determine which NES Cell’s SIB1 is broadcasted, it is friendly to power saving requirement. So alternative 2 is preferable to support Case 3 if supported.

In following, we will give more analysis on the details of alternative 2.

According above flow chart for alternative 2, UE will trigger the WUS(a new configured RA preamble) to Cell A when cell reselection condition for NES Cell is meet, if UE received the RAR successfully, UE will read the SIB1 information of NES Cell from the Cell A other than from NES cell.

For above procedure, it will have impact in following aspects:

- Define a condition to trigger WUS for NES Cell’s SIB1

  - The condition could be legacy cell reselection condition, which is the same as case 2.

- WUS configuration

  - Cell A’s RA resource will be separated for WUS Configuration of multiple NES Cells

  - RA resource needs to be reconfigured if NES Cell becomes in normal status.

- Xn interface

  - Enhanced Xn interface to transmit original and updated on-demand SIB1 message from NES Cell.

- NES Cell’s SIB1 transmission on Cell A

  - For NES Cell’s SIB1 design, the requested SIB1(s) for multiple NES cell could be put in one SIB of Cell A with list type IE or in several separate SIB(s) on Cell A, the UE should identify the requested NES Cell’s SIB1 from the associated Cell A’s SIB(s), the Cell A also needs to notice UE which NES Cell’s SIB1 is updated if needed, the granularity will be at NES Cell level, hence, the design seems complex.

Based on above analysis, for on-demand SIB1 request procedure, case 3 needs to introduce a condition to trigger the WUS, RA configuration should be enhanced to include the RA resource/preamble for WUS configuration, the new SIB needs to be designed to include one or multiple NES Cell’s SIB(s).

NES Cell’s SIB1 updating will be more frequently than WUS configuration updating at Cell A in case 2, also, the data size of NES Cell’s SIB is bigger than the WUS configuration in case 2, these give more SI overhead to Cell A. Besides, case 3 also needs to enhance the information exchange over Xn interface.

It can be seen that NES cell could be power saving since it will not transmit the SIB1 to UE, but, Cell A will suffers more power consumption to transmit SIB1 of multiple NES cells, so it is not clear how much the power saving will be achieved at system level.

Considering Case 2 has been supported in SI phase, there is no significant benefit to power saving at system level for case 3, so it is suggested to not to support case 3 in R19 NES.

Observation 2:  For WUS based on-demand SIB1 request procedure, case 3 will introduce following enhancement:

- The condition to trigger WUS ;

- New RA configuration for WUS configuration for each NES Cell’s SIB1;

- New SIB design to include multiple requested NES Cell’s SIB(s).

Observation 3:  Compared with WUS configuration and transmission in case 2, case 3 will lead to more signaling overhead at Uu interface and Xn interface.

Observation 4: Case 3 will have benefits to NES Cell power saving, but it increases the power consumption of Cell A, so it is not clear how much the power saving will be achieved at system level.

Proposal 10: The case 3 will be not supported in R19 NES.

On Next work

Regarding current progress on on-demand SIB1, there is no critical issue that could not be solved at least for case 2 and case 3, we have agreed a feasible framework for case 2. Hence, it is suggested to move this agenda to WID.

Proposal 11: It is suggested to turn on-demand SIB1 SI to WI.

Conclusion

According to the analysis in section 2, we propose:

Observation 1: It is possible that WUS configuration for on-demand SIB1 in NES cell will be updated.

Proposal 1: When a valid SIB is stored for more than 3 hours, UE needs to read the SIB1 and the corresponding SIB by legacy procedure, it is necessary for UE to apply the WUS configuration acquisition in NES Cell to obtain the on-demand SIB1.

Proposal 2: The NES Cell will broadcast the WUS configuration in SI message.

Proposal 3: NES Cell’s SIB1 validity should not be considered when UE camps on non-NES Cell.

Proposal 4: It is not needed to consider the additional mechanism for SIB1 validity at NES Cell.

Proposal 5: The SI change notification procedure could be used for SIB1 change in NES Cell, no extra spec impact will be introduced.

Proposal 6: Besides the cell reselection rule, RAN2 to further discuss the enhancement, such as measurement restriction, to avoid easily triggering WUS to attempt camping on NES Cell.

Proposal 7: It is UE implementation whether to trigger another RA procedure after Msg.1-based RA procedure is failed for OD-SIB1.

Proposal 8: In order to avoid the unnecessary attempting to switch to NES Cell by legacy UE, it is suggested to move the NES cell in blacklist.

Proposal 9: For network energy saving, the paging for legacy UEs at NES Cell needs to be avoided.

Observation 2:  For WUS based on-demand SIB1 request procedure, case 3 will introduce following enhancement:

- The condition to trigger WUS ;

- New RA configuration for WUS configuration for each NES Cell’s SIB1;

- New SIB design to include multiple requested NES Cell’s SIB(s).

Observation 3:  Compared with WUS configuration and transmission in case 2, case 3 will lead to more signaling overhead at Uu interface and Xn interface.

Observation 4: Case 3 will have benefits to NES Cell power saving, but it increases the power consumption of Cell A, so it is not clear how much the power saving will be achieved at system level.

Proposal 10: The case 3 will be not supported in R19 NES.

Proposal 11: It is suggested to turn on-demand SIB1 SI to WI.

Reference

- R2-2405701, Report from session on V2X/SL, R19 NES and MOB, Samsung.