3GPP TSG-RAN WG2 Meeting #127                                                                    R2-2407004

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Source:	CATT

Title:	Consideration on adaptation of common signal/channel transmissions

Agenda Item:	8.5.4

Document for:	Discussion and Decision

Introduction

In RAN2#125bis meeting [1], RAN2 discussed paging adaptation for adaptation of common signal channel, and made the following agreement.

Agreements on paging adaptation:

- From the UE point of view, UE will monitor one PEI/PO every paging DRX cycle, i.e. the UE doesn’t skip PO in paging DRX cycle.

- For adaptation of paging occasions in time domain, RAN2 to study a) bundle paging frames and b) extend the values of N to have increased interval between PFs (e.g. T/64, T/128 ...) and compensating decrease in number of PFs by increasing POs per PF.

- For Paging adaptation, R2 discusses the following options on compatibility of legacy RRC_IDLE/RRC_INACTIVE UE:

- Option 1: Prevent the access of legacy UE via barring;

- Option 2: Separate paging resources for legacy UEs and Rel-19 NES UEs (assuming there are legacy UEs)

In this contribution, we would like to discuss further considerations on adaptation of common signal/channel transmission.

Discussion

Adaptation of Paging

Option a) Bundle paging frames

In RAN2#126 meeting, modifying current PF formula to reduce interval of consecutive PFs for bundling paging frames were proposed [2][3][4][5]. The formula of PF for bundling PFs can be:

(SFN + PF_offset) mod T = G*(UE_ID mod N)

G: the frame gap between consecutive PFs.

Take G=1 as an example. PF_offset=0, N=T/16. The bundling paging frames for T=32rf and 512rf is shown in Figure 1.

Option b) Extend the values of N and Ns

Currently, N at most can be set to T/16 and the maximum value of Ns is 4. In this option, the values of N are extended, e.g. T/64, T/128, and so on, to increase the interval between PFs. Meanwhile, the values of Ns are increased, e.g. 8, 16 and so on. Then there can be one PF in T with increasing POs per PF. In addition, the parameter firstPDCCH-MonitoringOccasionOfPO which is used to indicate the first PDCCH monitoring occasion for paging of each PO of the PF needs to be updated as the maximum value of Ns is increased and the interval of consecutive PFs is increased.

Take N=T/32 and Ns=8 an example. PF_offset=0. The adaptation of paging with option b) for T=32rf and 512rf is shown in Figure 1, where 8 POs in one PF are spread in two frames.

Figure 1 Examples of paging adaptation

As shown in Figure 1, if a UE operates in eDRX while T is no longer than 1024rf, a lot of PFs would be clustered at the head of an eDRX paging cycle with option a). But with option b), the clustered PF for the eDRX paging cycle (no longer than 1024rf) is well aligned with the occasions for DRX paging cycle. No additional PF is introduced with option b) if both DRX UEs and eDRX UEs with no longer than 1024rf eDRX paging cycle exist in a cell simultaneously. Therefore, if both DRX UEs and eDRX UEs with no longer than 1024rf eDRX paging cycle exist in a cell simultaneously, the energy saving gain of option a) is less than that of option b).

The DRX cycle for paging of a UE may be shorter than the T broadcasting in system information. Then the actual value of N in the UE may be less than 1 radio frame. For example, T is 64 radio frames while N is T/64 is broadcasting system information. The T of the UE is 32 radio frames. Then N is 1/2 for the UE which means there may be no PF in T. But it was agreed the UE will monitor one PEI/PO every paging DRX cycle in RAN2#125 meeting. Hence, the UE cannot use the N in this case. The UE can return to legacy paging mechanism if suitable legacy paging parameters for network power saving are configured, e.g N is T/16 for legacy paging mechanism. Or N can be equal to 1 in this case.

Table 1 Pros and cons of paging adaptation

|  | Option a) Bundle paging frames | Option b) Extend the values of N and Ns |
| --- | --- | --- |
| Spec impact | Introduce new PF formula with new parameter(s) | Introduce new values for N, Ns and firstPDCCH-MonitoringOccasionOfPO without changing PF formula. |
| Applicable scenarios | All values of DRX paging cycle. | Not applicable to the case that N is less than 1 when DRX paging cycle in UE is small. |
| Energy saving gain | Less than option b) if both DRX UEs and eDRX UEs with no longer than 1024rf eDRX paging cycle exist in a cell simultaneously. | More than option a) if both DRX UEs and eDRX UEs with no longer than 1024rf eDRX paging cycle exist in a cell simultaneously. |

Pros and cons of the two paging adaptation mechanisms are summarized in table 1. We observe and propose:

Proposal 1: RAN2 to discuss the two paging adaptation mechanism with the following observations.

- Option a) Bundle paging frames;

- Option b) Extend the values of N and Ns.

Observation 1: Option a) introduces new PF formula with new parameter(s) while option b) introduces new values for N, Ns and firstPDCCH-MonitoringOccasionOfPO without changing PF formula.

Observation 2: Option b) is not applicable to the case that N is less than 1 when DRX paging cycle in UE is small.

Observation 3: If both DRX UEs and eDRX UEs with no longer than 1024rf eDRX paging cycle exist in a cell simultaneously, the energy saving gain of option a) is less than that of option b).

In RAN2#125 meeting, we agreed to discuss two options on compatibility of legacy RRC_IDLE/RRC_INACTIVE UEs.

3. For Paging adaptation, R2 discusses the following options on compatibility of legacy RRC_IDLE/RRC_INACTIVE UE: - Option 1: Prevent the access of legacy UE via barring; - Option 2: Separate paging resources for legacy UEs and Rel-19 NES UEs (assuming there are legacy UEs)

But it is a bit confusion of option 2. In our understanding, additional parameters for paging adaptation needs to be introduced. This may lead some PF(s)/PO(s) for paging adaptation are different from legacy paging mechanism while some PF(s)/PO(s) for paging adaptation maybe overlapped with legacy paging mechanism. And it is recommended from network energy saving aspect to achieve the overlap PF(s)/PO(s) between paging adaptation and legacy paging mechanism as much as possible. Therefore, we want to clarify that additional paging resources, not separate paging resources, for Rel-19 NES UEs are introduced if there are legacy UEs in the cell.

Proposal 2: For the option 2 on compatibility of legacy RRC_IDLE/RRC_INACTIVE UEs, RAN2 further clarify that additional paging resources for Rel-19 NES UEs, not separate paging resources, are introduced if there are legacy UEs in the cell which allows some overlapping paging resources between paging adaptation for Rel-19 NES UEs and legacy paging mechanism.

Adaptation of SSB

In RAN1#117, the following agreements were made for adaptation of SSB in time domain.

Agreement For adaptation of SSB in time-domain, Option 1 is supported - Option 1: Adaptation of SSB burst periodicity using one or more SSB burst periodicity value(s) - Note: Using Option 2 to realize Option 1 is not precluded - Option 2: Adaptation based on two SSB configurations [where up to two configurations can be active] - FFS: details of the differences between the two SSB configurations, e.g. two different periodicities - FFS: Details including applicable scenarios - FFS: Support of Cell DTX for connected mode UEs for SSB

RAN2 can wait for RAN1’s further agreements on this topic.

Proposal 3: For adaptation of SSB, RAN2 can wait for RAN1's further conclusion.

Adaptation of PRACH in time domain

In RAN1#117, the following agreements were made for adaptation of PRACH in time domain.

Agreement For adaptation of PRACH in time-domain, support at least the following case(s) - Case 1: no time-domain overlap between the additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs - Case 2: time-domain overlap but no overlap in frequency domain between the additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs - Case 3: additional PRACH resources for NES-capable UEs and legacy PRACH resources overlap neither in time nor frequency domains - FFS: whether additional conditions are needed to support the above cases - FFS: Additional case whether full/partial overlap in both time and frequency is allowed - Above does not preclude discussion for the case where the configuration for additional PRACH resources contains legacy PRACH resources Agreement At least for the case where legacy ROs and additional ROs overlap in neither time nor frequency domain, for adaptation of PRACH in time-domain, the SSB-RO mapping rule for additional PRACH resources follows the legacy SSB-RO mapping rule. - Mapping SS/PBCH block indexes to valid additional PRACH occasions provided by semi-static signalling follows the legacy mapping order for preamble/time resource/frequency/PRACH slot indexes. - Note: This mapping is not impacted by time domain PRACH adaptation - Validation rules for the additional PRACH resources follow the legacy validation rules for PRACH resources configured for legacy UEs. Agreement For adaptation of PRACH in time-domain, the additional PRACH resources are configured based on at least: - a PRACH configuration index - FFS: whether the PRACH configuration index is same and/or different from the PRACH configuration index for the legacy PRACH resources Study further the following - When the PRACH configuration index for the additional PRACH resources is same as the PRACH configuration index for the legacy resource, - Additional parameter(s) for determining the additional PRACH resources e.g. - Scaled/adjusted PRACH configuration period - Additional timing offset - Adjusting the parameters (e.g., (x, y) value and slot number) of the PRACH configuration - Muting/masking ROs - When the PRACH configuration index for the additional PRACH resources is different from the PRACH configuration index for the legacy resource - Additional mechanisms (if any) for determining the additional PRACH resources e.g. - Muting/masking ROs (e.g. for the case when the PRACH configuration index for the additional PRACH resources contains legacy resources) - Additional parameters to facilitate condensed/cluster RACH resources in time-domain (including whether needed) Agreement For the adaptation mechanism for additional PRACH resources, study further the following: - Option 1: Higher layer signalling (with potential enhancements) based PRACH resource adaptation - Option 2: L1-based adaptation to indicate whether the additional PRACH resources provided by semi-static signalling are available or not - FFS: details - Strive to re-use existing DCI format(s) - Option 3: Adaptation of PRACH transmission according to predefined condition(s) - FFS: details - Option 4-rev1: L1-based adaptation to indicate whether a subset of the additional PRACH resources provided by semi-static signalling are available or not - FFS: whether the subset of the additional PRACH resources is in RO level / SSB-to-RO mapping cycle level/PRACH association period level/PRACH association pattern period level for time-domain PRACH adaptation - Strive to re-use existing DCI format(s) - Option 5: Enhanced cell DRX

RAN2 can wait for RAN1’s further agreements on this topic.

Proposal 4: For adaptation of PRACH in time domain, RAN2 can wait for RAN1's further conclusion.

Conclusion

According to the analysis in section 2, we propose:

Proposal 1: RAN2 to discuss the two paging adaptation mechanism with the following observations.

- Option a) Bundle paging frames;

- Option b) Extend the values of N and Ns.

Observation 1: Option a) introduces new PF formula with new parameter(s) while option b) introduces new values for N, Ns and firstPDCCH-MonitoringOccasionOfPO without changing PF formula.

Observation 2: Option b) is not applicable to the case that N is less than 1 when DRX paging cycle in UE is small.

Observation 3: If both DRX UEs and eDRX UEs with no longer than 1024rf eDRX paging cycle exist in a cell simultaneously, the energy saving gain of option a) is less than that of option b).

Proposal 2: For the option 2 on compatibility of legacy RRC_IDLE/RRC_INACTIVE UEs, RAN2 further clarify that additional paging resources for Rel-19 NES UEs, not separate paging resources, are introduced if there are legacy UEs in the cell which allows some overlapping paging resources between paging adaptation for Rel-19 NES UEs and legacy paging mechanism.

Proposal 3: For adaptation of SSB, RAN2 can wait for RAN1's further conclusion.

Proposal 4: For adaptation of PRACH in time domain, RAN2 can wait for RAN1's further conclusion.

Reference

- Draft_R2_125bis_Meeting_Report_v3

- R2-2404635, Further discussion on RAN2 work of common signal transmission adaptation, Apple, RAN2#126

- R2-2404263	, RAN2 impacts to enable adaptation of paging and RACH in time, Intel Corporation, RAN2#126

- R2-2405576, Discussion on adaptation of paging signal/channel, III, RAN2#126

- R2-2404228, Adaptation of common signal/channel transmissions, Samsung, RAN2#126