3GPP TSG-RAN WG2 Meeting #127                                                     R2-2407005

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Source:	CATT

Title:	Discussion on MRO Enhancements for Mobility

Agenda Item:	8.10.2

Document for:	Discussion and Decision

1. Introduction

In RAN2#126 meeting, MRO enhancements for LTM, CHO with candidate SCGs and subsequent CPAC were discussed and achieved the following agreements [1].

LTM We correct the definition of scenario 3a as follows: Case 3a: the UE detects HOF/RLF in the LTM target cell and performs reestablishment procedure in a cell other than the source cell and the target cell. If available, log the L1 measurements for serving cell, target cell and other LTM candidate cells in RLF report, upon RLF or mobility failure. Reuse the existing approach of using timeConnFailure and the reconnectCellId in RLF-report also for LTM failures, details TBD. Log the LTM cell ID upon performing recovery an LTM candidate cell, details TBD e.g. which field. Extend lastHO-Type in RLF-Report to indicate the LTM cell switch as last executed mobility procedure. CHO with candidate SCGs RAN2 to enhance the RLF report with additional information regarding the state of the two execution conditions. We see later if we also can enhance the SCGFailureInformation report. Subsequent CPAC RAN2 will look into failure and near failure scenarios for subsequent execution of SCPAC and see if/what enhancements are needed.

In this contribution, we continue the discussion on MRO enhancements for LTM, CHO with candidate SCGs and subsequent CPAC, and show our views.

2. Discussion

2.1 MRO enhancements for LTM

RLF report should be enhanced to record the LTM failure information. When the UE is configured with LTM configuration, UE performs L1 measurement and reporting, based on which the network makes a handover decision and sends LTM cell switch command to UE to perform LTM execution. In RAN2#126 meeting, RAN2 agreed that L1 measurement of serving cell, target cell and other LTM candidate cell(s) are included in RLF report upon RLF or mobility failure. In addition, similar as CHO, the LTM recovery cell ID also was agreed to be recorded in RLF report for LTM MRO. Besides, UE can also record the LTM candidate cell(s) configured by network (explicit or implicit) for providing more information to network for mobility optimization.

Proposal 1: Enhance RLF report for LTM MRO to include the LTM candidate cell(s).

For successful LTM execution but near failure occurs, legacy SHR trigger conditions (i.e., T310 threshold, T312 threshold, T304 threshold) can be applied to LTM. RAN2 to further discuss whether there is new trigger condition for LTM needed.

Proposal 2: The following legacy SHR trigger conditions can also be applied to LTM. RAN2 further discusses whether new trigger condition is needed for LTM.

- T310 threshold;

- T312 threshold;

- T304 threshold.

SHR should be enhanced to record the successful LTM information. Similar as L1 measurement of serving cell, target cell and other LTM candidate cell(s) are included in RLF report upon RLF or mobility failure, L1 measurement of serving cell, target cell and other LTM candidate cell(s) should also be recorded in SHR for successful LTM execution. In addition, similar as CHO, the LTM candidate cell(s) can be recorded in SHR.

Proposal 3: Enhance SHR for LTM MRO to include at least the following information:

- L1 measurement of serving cell, target cell and other LTM candidate cell(s);

- LTM candidate cell(s).

The LTM configuration will not be released after successful LTM execution, which means subsequent LTM can continue to be performed. However, LTM execution is based on the L1 measurement results which vary very rapidly and cause the frequent LTM execution. Considering that the legacy RLF report/SHR only includes the latest failure/successful handover information, for LTM, there is high probability that the old RLF report/SHR will be overrode by new RLF report/SHR due to the frequent LTM execution, this will cause the information loss for network optimization. In order to address the issue, recording multiple RLF report/SHR can be considered as a candidate solution.

Proposal 4: RAN2 to discuss how to address the issue of frequent override report for subsequent LTM, recording multiple reports can be considered as a candidate solution.

2.2 CHO with candidate SCGs

In Rel-18, CHO with candidate SCGs was supported for accessing a best PSCell when the UE accesses the target PCell. It is the enhancement for CHO or CHO with target SCG. In Rel-18 SON/MDT, during the discussion of SPR, the scenario “HO with SN change” was proposed to postpone and didn’t discussed until Rel-18 SON/MDT WID is completed. In Rel-19 SON/MDT, it has been agreed to consider the enhancement for CHO with candidate SCGs, we think RAN2 should clarify whether the leftover scenario (i.e., CHO with target SCG) from R18 is in the scope of Rel-19 WID. From our perspective, this scenario (i.e., CHO with target SCG) can be supported with less effort if the CHO with candidate SCGs is supported. Therefore, we propose to consider both CHO with target SCG and CHO with candidate SCGs scenarios in R19 SON/MDT WID.

Proposal 5: For bullet “CHO with candidate SCG” in MRO enhancement for mobility, also consider the R18 leftover scenario of “CHO with target SCG”.

In case of CHO with target SCG or CHO with candidate SCGs, the PCell change and PSCell change are performed simultaneously, as the RLF report/SHR and SCGFailureInformation/SPR have been enhanced to support the CHO and legacy PSCell addition/change or CPAC for MRO separately, the key point of this two scenarios is how to support the association of MCG failure/near failure information and SCG failure/near failure information, e.g., logging the MCG failure/near failure information and SCG failure/near failure information in the same report or the MCG failure/near failure information and SCG failure/near failure information are logged in different reports with the association information in the two reports.

Proposal 6: RAN2 to consider how to associate the MCG failure/near failure and SCG failure/near failure information with the following two options:

Option 1: UE records the MCG failure/near failure and SCG failure/near failure information in the same report;

Option 2: UE records the MCG failure/near failure and SCG failure/near failure information in different reports, and the association information is needed in the two reports.

In case of CHO with candidate SCGs, both PCell and PSCell are configured with execution conditions, and only the PCell and PSCell execution conditions are fulfilled at the same time, the UE triggers the CHO with candidate SCGs execution. If one of the execution condition (e.g., the execution condition of PSCell) is not fulfilled in a long time but another execution condition (e.g., the execution condition of PCell) is fulfilled, the UE will also not execute the CHO with candidate SCGs, which will cause the unexpected situation, i.e., the CPAC hinders the CHO execution, and vice versa. The ideal result of execution condition evaluation for CHO with candidate SCGs is the execution condition of PCell and PSCell should be fulfilled as closely as possible in terms of time. A trigger condition can be considered for successful CHO with candidate SCGs execution.

Proposal 7: The timing different threshold between PCell execution condition being fulfilled and PSCell execution condition being fulfilled is considered as the trigger condition for successful CHO with candidate SCGs execution.

2.3 Subsequent CPAC

For subsequent CPAC, frequent configuration of candidate PSCell add/update/release may occur which will introduce much XN interface signaling overhead, e.g., for MN-initiated subsequent CPAC, the UE initial is configured with 5 candidate cells (i.e., candidate cell 1~candidate cell 5), if the MN expects to add a candidate cell 6, the MN needs to request all candidate cell 1~5 to configure the execution condition for candidate cell 6, and request the candidate cell 6 to configure the execution condition for candidate cell 1~5. In order to reduce such overhead, it is better to configure suitable initial subsequent CPAC configuration without follow-up configuration update. The subsequent CPAC configuration optimization can be performed based on SON/MDT enhancement.

Proposal 8: RAN2 to consider initial subsequent CPAC configuration optimization for reducing the follow-up configuration update.

3. Conclusion

According to the discussion in section 2, we propose:

MRO enhancements for LTM

Proposal 1: Enhance RLF report for LTM MRO to include the LTM candidate cell(s).

Proposal 2: The following legacy SHR trigger conditions can also be applied to LTM. RAN2 further discusses whether new trigger condition is needed for LTM.

- T310 threshold;

- T312 threshold;

- T304 threshold.

Proposal 3: Enhance SHR for LTM MRO to include at least the following information:

- L1 measurement of serving cell, target cell and other LTM candidate cell(s);

- LTM candidate cell(s).

Proposal 4: RAN2 to discuss how to address the issue of frequent override report for subsequent LTM, recording multiple reports can be considered as a candidate solution.

CHO with candidate SCGs

Proposal 5: For bullet “CHO with candidate SCG” in MRO enhancement for mobility, also consider the R18 leftover scenario of “CHO with target SCG”.

Proposal 6: RAN2 to consider how to associate the MCG failure/near failure and SCG failure/near failure information with the following two options:

Option 1: UE records the MCG failure/near failure and SCG failure/near failure information in the same report;

Option 2: UE records the MCG failure/near failure and SCG failure/near failure information in different reports, and the association information is needed in the two reports.

Proposal 7: The timing different threshold between PCell execution condition being fulfilled and PSCell execution condition being fulfilled is considered as the trigger condition for successful CHO with candidate SCGs execution.

Subsequent CPAC

Proposal 8: RAN2 to consider initial subsequent CPAC configuration optimization for reducing the follow-up configuration update.

4. Reference

- Draft_R2_126_Meeting_Report_v2