3GPP TSG RAN WG2 #127                                                                                   R2-2407006

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Source:	CATT

Title:	Consideration on leftovers from Rel-18 SONMDT

Agenda Item:	8.10.5

Document for:	Discussion and Decision

Introduction

In last RAN2#126 meeting [1], for R18 leftover issues, RAN2 has achieved the agreements as below:

For SDT: For the purpose of SON enhancements for SDT, include RSRP/data volume related information Downlink RSRP value and buffered uplink data volume at the time when the UE evaluates if it should perform SDT. When SDT failure happens, the UE can indicate the failure cause of SDT to the network, e.g. T319a expiration. Details are TBD, e.g. if RSRP and data volume can also be included in such report. For SCG failure: Reply to RAN3 that we will only do EN-DC. RAN2 understands that whether also supporting (NG)EN-DC has no additional RAN2 impact hence RAN3 can decide. If later we get time we can consider other options.

In this contribution, we will continue to discuss the R18 leftover issues and provide our views.

Discussion

RACH optimization for SDT

Background

SDT is consisted of R17 MO-SDT and R18 MT-SDT. In SDT initiation procedure, the CG-SDT has a high priority than RA based SDT. For MO-SDT, when MO-SDT data is arriving at UE side, UE will initiate the MO-SDT procedure with RA partitioning resource if CG-SDT initiation is not selected or failed. For MT-SDT, if the paging with MT-SDT indication is received by UE, UE will initiate the MT-SDT procedure with the normal RA resource if CG-SDT initiation is not selected or failed. Besides, MT-SDT based RA procedure could have MT-SDT specific RSRP threshold, MO-SDT specific RSRP threshold will be applied if MT-SDT specific RSRP threshold is not configured for MT-SDT based RA procedure.

RA report for MO-SDT

According the above agreement for SDT, RSRP/data volume related information at the time when the UE evaluates if it should perform SDT will be logged in RA report. However, it is not clear in which case to log RSRP value and buffered uplink data volume. Obviously, if the SDT based RA procedure is successful, RSRP value and buffered uplink data volume could be included in RA report. If SDT based RA procedure is failed, such as T319a, the RSRP and data volume is better to be reported to network to see if the parameters associated to these two values are configured reasonable.

Proposal 1: The RSRP and data volume at the time when the UE evaluates if it should perform SDT should be included in RA report in both successful and failed SDT based RA procedure.

According to legacy procedure, the parameters including DL RSRP and pending UL data volume size are also used by CG-SDT procedure, in which the UE specific UL grant is configured for initial CG-SDT data transmission. So, it is better to make network realize the thresholds for RA based SDT is common to CG-SDT or not(i.e, CG-SDT is not configured) for a history RA procedure, then the network will consider the impact to CG-SDT or not when adjusting the thresholds. For example, if the indication of CG-SDT not configured is included in RA report for a history RA procedure, but now the CG-SDT is configure by network, then the network will adjust the parameters more carefully to consider the impact to CG resource.

Proposal 2: Whether to perform the CG-SDT selection prior to RA-SDT needs to be considered in RA report.

RA report for MT-SDT

In R18, RA report has been enhanced for RA partitioning which includes the RA procedure triggered by R17 MO-SDT or its combination with other features, some remaining issues, such as whether to optimize the threshold for channel quality or data volume size, are left in the scope of R19 SON/MDT. However, the MT-SDT was introduced from R18, its initiation to RA-SDT is different from the MO-SDT and will apply the RA configuration for common RA procedure rather than the RA resource of MT-SDT, hence, the RA enhancement for MO-SDT in R18 could not be reused for MT-SDT, the network could also not identify the RA report is for MT-SDT or legacy RA procedure. Thus, it is suggested to consider MT-SDT in RACH optimization for SDT. For MT-SDT, the legacy RA report for normal RA procedure (rather than RA partitioning) could be reused, but an additional RA purpose of MT-SDT should be included in the RA report, then the network could identify the RA report is for MT-SDT and adjust the RA resource for MT-SDT.

Based on above analysis, the basic enhancement of RA report for MT-SDT is just to introduce a new RA purpose for MT-SDT. So, it is suggested to consider the RACH optimization for MT-SDT.

Proposal 3: It is suggested to consider the RACH optimization for MT-SDT, and introduce a new RA purpose of MT-SDT in RA report.

MHI for SCG Deactivation/Activation

The Mobility History Information (MHI) for PCell has been supported in Rel-16. In Rel-17, the enhancement for Dual Connectivity is introduced, where the mobility history information of most recently visited PSCell and time spent in the PSCell across all the primary cells is included in the VisitedCellInfoList, the list includes PSCells visited by UE in RRC connect states for NR or E-UTRA.

MHI for PSCell was introduced in R17 SON/MDT. Meanwhile, the SCG activation/deactivation was introduced in R17 MR DC/CA WI [2], in which the SCG could be configured with activation/deactivation state to enable reasonable UE battery consumption while having fast usage of SCG. According to below MHI procedure to log PSCell information in TS 38.331, it can be seen that MHI information will be logged for PSCell upon addition, change, or release of a PSCell etc., more details could be seen in the spec.

TS 38.331 5.7.9.2 Initiation If the UE supports storage of mobility history information, the UE shall: 1> If the UE supports PSCell mobility history information and upon addition of a PSCell: <skip> 2> for the included entry: 3> set the field timeSpent of the entry according to following: 4> if this is the first PSCell entry for the current PCell since entering the current PCell in RRC_CONNECTED: 5> include the entry as the time spent with no PSCell since entering the current PCell in RRC_CONNECTED; 4> else: 5> include the time spent with no PSCell since last PSCell release since entering the current PCell in RRC_CONNECTED; 1> If the UE supports PSCell mobility history information and upon change, or release of a PSCell while being connected to the current PCell: <skip> 2> for the included entry: 3> if the global cell identity of the previous PSCell is available: 4> include the global cell identity of that cell in the field visitedCellId of the entry; 3> else: 4> include the physical cell identity and carrier frequency of that cell in the field visitedCellId of the entry; 3> set the field timeSpent of the entry as the time spent in the previous PSCell while being connected to the current PCell;

For above legacy procedure, RAN2 needs to clarify SCG could be in activated or deactivated status when MHI is performed upon addition, change of PSCell. In our view, it is natural to think that UE will log MHI for PSCell upon addition, change of a PSCell no matter SCG is configured as deactivated or not.

Observation 1: In current spec, UE will log MHI for PSCell upon addition, or change of a PSCell no matter SCG is configured as activated or deactivated.

Based on the current spec, when SCG is activated, UE needs to resume performing radio link monitoring on the SCG and indicate the lower layers to resume beam failure detection on the PSCell. When SCG is activated, UE will stay under the coverage of SCG and it is correct that UE logs this PSCell in MHI. However, when SCG is deactivated, UE may stop radio link monitoring on the SCG and stop the beam failure detection on the PSCell. Then it is possible that UE may move to out of the coverage of current PSCell during SCG deactivation. But UE will still log this PSCell and the spent time into the MHI.

For example, Network configures UE to activate PSCell1 for data transmission at T1 and deactivate the SCG1 at T2 because of traffic is finished. If the data is arriving again at T3, the network could activate the SCG. However, since UE is moving out of SgNB1 and in the coverage of SgNB2, the network will configure the change of PSCell and the SCG2 of SgNB2 with activation.

Figure 1 UE moving and SCG configuration in DC scenario

In above procedure, the time duration between T2 and T3 will be associated to PSCell1 by legacy MHI procedure. If the network receives this MHI information, network may believe that UE is still under the coverage of PSCell1 and available for data transmission, but the UE is out of the PSCell1 coverage and into PSCell2 coverage in fact.

The MHI is used for network to estimate the UE mobility state and then optimum parameters for handover. For example, for fast UEs, the network can choose a macro cell as a target cell which can reduce the signalling overhead for mobility and reduce the mobility failure. Besides, the network can set optimum parameters (e.g. timeToTrigger) for handover measurements. The misunderstanding of the PSCell UE staying to network will cause the incorrect handover optimization.

Observation 2: Based on the UE behavior in current spec, SCG deactivation may result in biased MHI recording and incorrect handover optimization.

Proposal 4: When UE records the MHI of PSCell, UE should record the time duration of SCG deactivation as least to avoid the network estimating the UE’s mobility state incorrectly.

Furthermore, if UE move out of the coverage of PSCell1 during SCG deactivation, the TA can be expired or RLF/beam failure is detected during SCG deactivation, which means UL/DL is un-sync between UE and PSCell1. In this scenario, some extra assistant information, such as whether TAT is expired or whether RLF/beam failure is detected during SCG deactivation, even the RRM measurement results, may be needed to help network realizes the real situation when PSCell1 is in deactivated status and avoid the incorrect handover optimization.

Proposal 5: When UE records the MHI of PSCell, if SCG is deactivated, UE can record some extra information to help network understand the real situation, such as whether TAT is expired or whether RLF/beam failure is detected or not.

MRO for MR-DC SCG failure

In RAN3#123bis meeting, RAN3 had some discussion and intended to keep the R18 agreement in R19, i.e. to support MRO for SCG failure in EN-DC, NGEN-DC and NE-DC scenarios. But in the LS sent to RAN2 [3], RAN3 asked whether RAN2 will support MRO for SCG failure in these scenarios. And then in RAN2#126 meeting, RAN2 decided only to support EN-DC scenario first and replied the LS to RAN3.

In R17, based on the request by RAN3 LS [4], RAN2 only agree to support the MRO for MR-DC SCG failure in NR-DC scenario, and the enhanced parameters are all added in the SCGFailureInformation message in TS38.331.

Table 1 NR-DC SCG failure report content

| Parameter | IE/value & condition |
| --- | --- |
| failedPSCellId and previousPSCellId | Frequency and PCI |
| timeSCGFailure | 0-1023 |
| perRAInfoList | Only can be included if failureType is set to synchReconfigFailureSCG, or is set to randomAccessProblem while T304 is running |
| failureType | Reuse current failure types |

We think the parameters reported in NR-DC scenario can be the baseline for EN-DC scenario.

Proposal 6: The parameters reported in NR-DC scenario can be the baseline for SCG failure report in EN-DC scenario.

Since the parameters used for SCG failure optimization should be reported to the MN, the SCG failure enhancement for EN-DC scenario (or NG-EN-DC scenario) may have spec impact on TS36.331.

Observation 3: Since the parameters used for SCG failure optimization should be reported to the MN, the EN-DC scenario (also include NG-EN-DC scenario) may have spec impact on TS36.331.

During the early discussion in R17, it is concerned that whether the SCGFailureInformation message is appropriate to transmit the parameters for SCG MRO purpose. The usage of SCG failure report related messages is to request the network to resume the data transmission by a second link as soon as possible. Some companies thought that SON related UE stored measurements are not needed immediately after a failure and thus the message size should not be increased for the SON purposes [5]. At the end of R17 discussion, it is compromise to use the SCGFailureInformation message for NR-DC scenario.

Therefore, it is necessary to consider whether the SCG failure report related message (e.g. SCGFailureInformationNR in TS36.331) can still be used for transmitting the parameters for MRO purpose, or other messages should be used.

Proposal 7: Identify whether the SCGFailureInformationNR message in TS36.331 can be used for transmitting the parameters for MRO purpose in EN-DC scenario.

If the SCGFailureInformationNR message can be used for transmitting the parameters for MRO purpose in EN-DC scenario, to analyze the enhanced parameters in the message:

Table 2 Possible EN-DC SCG failure report content

| Parameter | IE/value & condition |
| --- | --- |
| failedPSCellId and previousPSCellId | Frequency and PCI |
| timeSCGFailure | 0-1023 |
| perRAInfoList | Only can be included if failureType is set to synchReconfigFailureSCG, or is set to randomAccessProblem while T304 is running |
| failureType | Reuse current failure types |

For failedPSCellId and previousPSCellId

The frequency and the PCI of the PSCell could be introduced for these two fields similar as NR-DC scenario. No CGI is needed since the SCGFailureInformationNR message will be sent to the network immediately after the SCG failure occurred and there will be no addressing error.

For timeSCGFailure

The value range 0-1023 defined in NR-DC scenario can be introduced.

For perRAInfoList

The RA information for the SgNB should be recorded with NR format, so the NR PerRAInfoList related IEs which are specified in TS 38.331 may need to be introduced in i.e. OCTET STRING:

- PerRAInfoList-r16: Need to be introduced similar as NR-DC scenario;

- PerRAInfoList-v1660: No need to be introduced since the value range in this IE will only be used for beam failure recovery;

- PerRAInfoList-v1800: No need to be introduced since this IE only include LBT related information.

Since the failureType in SCGFailureInformationNR message also includes the same values of synchReconfigFailureSCG and randomAccessProblem as in NR-DC scenario, these two legacy values and corresponding descriptions can be directly reused.

For failureType

The legacy failure types can be reused.

Proposal 8: The parameters reported in EN-DC scenario for SCG failure report may include:

- failedPSCellId and previousPSCellId: Introduce frequency and the PCI of the PSCell;

- timeSCGFailure: Introduce with value range 0-1023;

- perRAInfoList: Introduce to indicate the performed random access procedure related information:

  - Set NR IE PerRAInfoList-r16 in OCTET STRING;

  - Only be set if the failureType is set to synchReconfigFailureSCG or the failureType is set to randomAccessProblem and the SCG failure was declared while T304 was running;

- failureType: Reuse the legacy field.

For SCG Failure Report for MRO in NR-DC scenario, an optional feature without UE radio access capability parameter is introduced in TS38.306, and no explicit UE capability bit is needed.

SCG Failure Report for MRO It is optional for UE to support the delivery of the SCG failure related parameters for MRO in SCGFailureInformation message to the network.

For the SCG Failure Report for MRO in EN-DC scenario, a similar optional feature without UE radio access capability parameter should be introduced in TS36.306.

Proposal 9: Introduce an optional feature without UE radio access capability parameter for the SCG Failure Report for MRO in EN-DC scenario.

Conclusion

According to the analysis in section 2, the observations and proposals are summarized as follows:

RACH optimization for SDT

Proposal 1: The RSRP and data volume at the time when the UE evaluates if it should perform SDT should be included in RA report in both successful and failed SDT based RA procedure.

Proposal 2: Whether to perform the CG-SDT selection prior to RA-SDT needs to be considered in RA report.

Proposal 3: It is suggested to consider the RACH optimization for MT-SDT, and introduce a new RA purpose of MT-SDT in RA report.

MHI for SCG Deactivation/Activation

Observation 1: In current spec, UE will log MHI for PSCell upon addition, or change of a PSCell no matter SCG is configured as activated or deactivated.

Observation 2: Based on the UE behavior in current spec, SCG deactivation may result in biased MHI recording and incorrect handover optimization.

Proposal 4: When UE records the MHI of PSCell, UE should record the time duration of SCG deactivation as least to avoid the network estimating the UE’s mobility state incorrectly.

Proposal 5: When UE records the MHI of PSCell, if SCG is deactivated, UE can record some extra information to help network understand the real situation, such as whether TAT is expired or whether RLF/beam failure is detected or not.

MRO for MR-DC SCG failure

Proposal 6: The parameters reported in NR-DC scenario can be the baseline for SCG failure report in EN-DC scenario.

Observation 3: Since the parameters used for SCG failure optimization should be reported to the MN, the EN-DC scenario (also include NG-EN-DC scenario) may have spec impact on TS36.331.

Proposal 7: Identify whether the SCGFailureInformationNR message in TS36.331 can be used for transmitting the parameters for MRO purpose in EN-DC scenario.

Proposal 8: The parameters reported in EN-DC scenario for SCG failure report may include:

- failedPSCellId and previousPSCellId: Introduce frequency and the PCI of the PSCell;

- timeSCGFailure: Introduce with value range 0-1023;

- perRAInfoList: Introduce to indicate the performed random access procedure related information:

  - Set NR IE PerRAInfoList-r16 in OCTET STRING;

  - Only be set if the failureType is set to synchReconfigFailureSCG or the failureType is set to randomAccessProblem and the SCG failure was declared while T304 was running;

- failureType: Reuse the legacy field.

Proposal 9: Introduce an optional feature without UE radio access capability parameter for the SCG Failure Report for MRO in EN-DC scenario.

Reference

- Chairman’s Notes RAN2#126, Fukuoka, Japan, May 20th – 24th, 2024

- RP-201040 Revised WID on Further Multi-RAT Dual-Connectivity enhancements

- R3-242195 LS on support of MRO for MR-DC SCG failure, RAN3

- R3-211332 LS on information needed for MRO in SCG Failure Report, RAN3

- R2-2108964 Report of [AT115e][871][SON/MDT] Modeling aspects related to information required by SN/SCG (CATT)