3GPP TSG-RAN WG2 Meeting #127		R2-2407007

Maastricht, Netherlands, 19th Aug. – 23rd Aug. 2024

Source:	vivo

Title:		Discussion on multi-hop U2N Relay discovery and (re)selection

Agenda Item:		8.13.2

Document for:	Discussion and Decision

- Introduction

In the WI NR sidelink multi-hop relay[1], the objectives are shown as follows:

- Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays. - Relay discovery and (re)selection [RAN2] - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3] - Impact on SRAP and QoS handling for multi-hop [RAN2] - Control plane procedures [RAN2, RAN3]

In this contribution, we mainly focus on discussing the relay discovery and (re)selection for multi-hop case, including:

- Discovery conditions for U2N remote UE, U2N relay UE and Intermediate Relay UE;

- Relay reselection triggering/criterion;

- Uu threshold condition for U2N remote UE, U2N relay UE and Intermediate Relay UE.

- Discussion

  - Background

Similarly, as in Rel-17 U2N Relay and Rel-18 U2U Relay, the relay discovery and (re)selection procedures for multi-hop U2N relay are highly dependent on the progress in SA2. Their conclusions so far[2] are as follows (highlighted text related to discovery and (re)selection):

## 8.1 Conclusions for KI#1 The following principles are used for normative work: - 5G ProSe Multi-hop Layer-3 UE-to-Network Relay Communication with N3IWF support: - The 5G ProSe Layer Remote UE connects to the N3IWF over 5G ProSe Layer-3 Intermediate Relay and Layer-3 UE-to-Network Relay. - The 5G ProSe Layer-3 Intermediate Relay neither selects N3IWF nor connects to N3IWF. - Support of multi-hop UE-to-Network Relays: - Layer-3 multi-hop UE-to-Network Relay can proceed into normative work. - The service authorization and policy/parameter provisioning procedures will be described in the normative phase. - Standalone discovery & link setup/management is considered and both Model A and Model B are supported. - When Model A discovery is performed, the Remote UE may choose the Intermediate Relay based on the announcement message sent by the Intermediate Relay. - When Model B discovery is performed, the Remote UE selects both the UE-to-Network Relay and the path to reach the UE-to-Network Relay. To perform link management, the DCR message is unicasted between Relays according to the path information included in the message. The path information is an (ordered) list of User Info ID of Relays in the selected path. The Remote UE sends the selected path information to the Intermediate UE-to-Network Relay for communication setup. - The 5G ProSe Direct Discovery message will be extended with an indication that multi-hop relay is supported, along with the hop count and the maximum number of hops. - The maximum number of hops is provisioned by the network to the 5G ProSe Remote UE, Intermediate Relay and UE-to-Network Relays per RSC. - The 5G ProSe Direct Discovery message may be dropped if the maximum number of hops is reached. - The relay path is selected based on e.g. the PC5 signal strength, number of hops between the Remote UE and the UE-to-Network Relay UE, per-hop or cumulative QoS information. The path selection criteria and the support of these metrics can be determined during the normative phase. - The 5G ProSe Direct Communication procedure defined in TS 23.304 [4] is reused for the Layer-2 link establishment between each pair of UEs in the path. - End-to-end QoS management for multi-hop U2N Relays is done similarly to the end-to-end QoS management for single hop L3 U2N Relay as defined in TS 23.304 [4], with enhancement to handle QoS split over multiple legs of the PC5 interface. - The QoS flows setup can be initiated by the network or the 5G ProSe Layer-3 Remote UE during Layer-2 link establishment or link modification procedure. - The UE-to-Network Relay allocates the IP address/prefix, the Intermediate Relay may act as DHCP proxy or may relay the IP allocation message (e.g. Router Solicitation, Advertisement). - Details about remote UE report and additional discovery message handling can be determined in the normative phase. NOTE: Security aspects will be addressed by SA WG3.

  - Discovery

According to SA2 conclusion [2], both model A and model B are supported for multi-hop U2N relay operation. The following two figures are examples in solution #2 in SA2 TR [2] for model-A and model-B:

Figure 6.2.2.1.1-1: 5G ProSe multi-hop UE-to-Network Relay Discovery with Model A

Figure 6.2.2.1.2-1: 5G ProSe multi-hop UE-to-Network Relay Discovery with Model B

Observation 1: According to SA2 conclusion, both Model A and Model B discovery are supported in multi-hop U2N relay operation.

From RAN2’s point of view, we should focus on the AS conditions to transmit discovery messages. This including three aspects:

      - Discovery threshold conditions for U2N Remote UE to transmit mode-B solicitation message;

      - Discovery threshold conditions for U2N relay UE to transmit discovery mode-A announcement message and discovery mode-B response message;

      - Discovery threshold conditions for Intermediate Relay UE to forward multi-hop discovery model A/model B messages.

Observation 2: AS conditions to transmit discovery messages consists of the following three aspects:

- Discovery threshold conditions for U2N Remote UE to transmit mode-B solicitation message;

- Discovery threshold conditions for U2N relay UE to transmit discovery mode-A announcement message and discovery mode-B response message;

- Discovery threshold conditions for Intermediate Relay UE to forward multi-hop discovery model A/model B messages.

### Discovery threshold conditions for U2N Remote UE

In Rel-18 U2U relay operation, for U2U remote UE, it should satisfy the discovery threshold condition that:

- the RSRP of peer U2U remote UE < RSRP-ThreshU2U – HystMinU2U

Similar in multi-hop U2N relay operation, we need to consider the following question:

Question: Whether the multi-hop U2N remote UE should satisfy the following condition to transmit mode-B solicitation message:

- PC5 RSRP between U2N remote UE and U2N relay UE < threshold

we think the answer should be ‘yes’ because it follows the same logic in U2U relay that two UEs should be communicated directly on PC5 unless the PC5 RSRP between them is really low. Therefore, we propose:

Proposal 1: Similar as in Rel-18 U2U relay operation, for multi-hop U2N relay operation the U2N remote UE can transmit Discovery Solicitation message with Model B only when PC5 RSRP between U2N remote UE and U2N relay UE is below a threshold.

### Discovery threshold conditions for U2N Relay UE

In Rel-18 U2U relay operation, for target U2U remote UE, it should satisfy the discovery threshold condition that:

- the RSRP of U2U relay UE > RSRP-ThreshU2U

Similar in multi-hop U2N relay operation, we need to consider the following question:

Question: Whether the U2N relay UE should satisfy the condition to transmit discovery mode-B response message:

- PC5 RSRP between multi-hop U2N relay UE and Intermediate Relay UE > threshold

We think the answer should also be ‘yes’ because otherwise the link quality between U2N relay UE and Intermediate Relay UE is not considered. If we make the U2N remote UE to consider this link, it would be much more complicated, and also it may lead to much more discovery mode-B response message in the system. We would like to follow the same principle in U2U relay. Therefore,

Proposal 2: Similar as in Rel-18 U2U relay operation, for multi-hop U2N relay operation the U2N relay UE can transmit Discovery response message with Model B only when PC5 RSRP between U2N relay UE and Intermediate Relay UE is above a threshold.

For mode-A announcement message at U2N Relay UE, in U2U relay it follows the ‘Neighbor UE(s) in proximity conditions’ and we think this may not be applicable in multi-hop U2N relay operation. No proposal is given now.

### Discovery threshold conditions for Intermediate Relay UE

If we look at the ‘Remote UE <-> Intermediate Relay UE <-> U2N relay UE’ path, the Intermediate Relay UE serves like a U2U relay UE. We understand in multi-hop U2N relay operation, the situation does not change as the Intermediate Relay UE should also have good link quality between remote UE and U2N relay UE to make the whole road passable. Therefore, we would like to simply reuse the conditions of Rel-18 U2U discovery message forwarding.

Proposal 3: Similar as in Rel-18 U2U relay operation, the Intermediate Relay UE can forward the U2N discovery model-A announcement message received from U2N relay UE only when the PC5 RSRP of the message is above a threshold.

Proposal 4: Similar as in Rel-18 U2U relay operation, the Intermediate Relay UE can forward the U2N discovery model-B solicitation message received from U2N remote UE only when the PC5 RSRP of the message is above a threshold.

For the forwarding of U2N discovery model-B response message at Intermediate Relay UE, we can also follow the U2U relay operation, i.e., no RSRP conditions. In U2U relay discussion, this is because that when the target remote UE responses to the discovery model-B solicitation message, it already considers the PC5 RSRP between it and the U2U relay UE. we understand the same assumption can be made in multi-hop U2N relay UE as well as we proposed in Proposal 2.

Observation 3: In U2U relay, the Discovery Response message with Model B can be forward by U2U relay UE without RSRP conditions, because the target remote UE already considers the PC5 RSRP between it and the U2U relay UE before replying to solicitation message.

Proposal 5: Similar as in Rel-18 U2U relay operation, the Intermediate Relay UE can forward the U2N discovery model-B response message received from U2N relay UE without considering the PC5 RSRP between it and the U2N relay UE.

  - Relay (re)selection

### A. Relay (re)selection criterion

According to SA2 TR[2], the most relevant conclusions for relay selection is as follows:

- The Remote UE selects both the UE-to-Network Relay and the path to reach the UE-to-Network Relay;

- The relay path is selected based on e.g. the PC5 signal strength, number of hops between the Remote UE and the UE-to-Network Relay UE, per-hop or cumulative QoS information. The path selection criteria and the support of these metrics can be determined during the normative phase.

Observation 4: SA2 concludes that the PC5 signal strength, number of hops between the Remote UE and the UE-to-Network Relay UE, per-hop or cumulative QoS information may be considered in Relay selection.

In legacy Rel-17 U2N relay and Rel-18 U2N relay, RAN2 mainly discussed the AS criterion for relay (re)selection, which is basically the RSRP. We understand the same principle applies here. According to SA2 conclusion, the number of hops and per-hop or cumulative QoS information should be discussed and considered in SA2, e.g., how to convey that information in discovery messages.

In RAN2, it is simple that the U2N remote UE just consider the RSRP between it and the Intermediate Relay to be above a threshold. For the link of Intermediate Relay UE <-> U2N relay UE, as proposed in Proposal 5, the quality has already been considered when U2N relay UE replies with U2N discovery model-B response message. Therefore, we propose:

Proposal 6: Similar as in Rel-17 U2N relay and Rel-18 U2U relay operation, for multi-hop U2N relay operation the U2N remote UE selects an Intermediate Relay UE whose PC5 RSRP is above a threshold.

Proposal 7: The U2N remote UE selects an U2N relay UE based on upper layer criterion e.g. number of hops and per-hop or cumulative QoS information.

### B. Relay reselection triggering

For relay reselection triggering, we would like to discuss who, either the U2N remote UE or Intermediate Relay UE, should be allowed to perform the relay reselection. It may include two aspects:

- Intermediate Relay UE reselection by U2N remote UE;

- U2N relay UE reselection by Intermediate Relay UE;

On the one hand, in legacy Rel-17 U2N relay and Rel-18 U2U relay, it is the remote UE to trigger relay reselection based on RSRP of current hop. And as we discussed previously, in Rel-19 multi-hop U2N relay operation, the remote UE selects both the UE-to-Network Relay and the path to reach the UE-to-Network Relay in relay selection. Therefore, one simple option is that it is also left to U2N remote UE for relay reselection, and it should reselect both Intermediate Relay UE and U2N relay UE when the PC5 RSRP between Intermediate Relay UE and U2N remote UE is below a threshold.

On the other hand, it is mentioned in WID that:

- A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays.

Consider two additional hop case, it may be easier to allow U2N relay UE reselection by Intermediate Relay UE especially when only the link between them deteriorates. It can be further discussed whether/how to inform U2N remote UE after this kind of reselection. Therefore, we propose:

Proposal 8: U2N remote UE performs relay reselection for both Intermediate Relay UE and U2N relay UE when the PC5 RSRP between Intermediate Relay UE and U2N remote UE is below a threshold.

Proposal 9: Intermediate Relay UE performs relay reselection for U2N relay UE when the PC5 RSRP between Intermediate Relay UE and U2N relay UE is below a threshold. FFS whether/how to inform U2N remote UE.

  - Uu Threshold condition

In Rel-17 U2N relay operation, one of the most important topics is to define the ‘threshold condition’ for both Relay and Remote UE. The threshold conditions for Rel-17 U2N relay UE and U2N remote UE are summarized as follows (simplified):

- For relay UE:

  - threshLowRelay + hystMinRelay < the RSRP of PCell < threshHighRelay - hystMaxRelay

- For remote UE:

  - the RSRP of PCell < threshHighRemote – hystMaxRemote

In Rel-19 multi-hop U2N relay operation, we understand for U2N relay UE and U2N remote UE, the conditions should remain the same, because for U2N relay UE, it serves more or less the same role which is to provide the gNB backhaul. For U2N remote UE, it is also the same that a UE should be performing U2N relay operation only when its Uu link deteriorates.

The only question is that whether we need separate threshold for U2N remote UE in multi-hop case as the U2N remote UE now needs to firstly select an Intermediate U2N Relay UE. In our opinion, there seems no clear motivation to have separate threshold and whether to select U2N relay UE or Intermediate U2N Relay UE most relates to e.g. the service requirement.

Therefore, we propose:

Proposal 10: For U2N relay UE, the Uu threshold condition of Rel-17 U2N relay UE is reused.

Proposal 11: For U2N remote UE, the Uu threshold condition of Rel-17 U2N remote UE is reused.

For Intermediate U2N relay UE, in our understanding, it can be both in-coverage or out-of-coverage, and the multi-hop U2N relay is mainly for coverage enhancement. This understanding is shared by SA2 if one looks at their e.g. solution #1 in [2].

Observation 5: Intermediate U2N relay UE can be both in-coverage or out-of-coverage.

Therefore, we understand for Intermedia U2N relay UE, there is no need of any lower limit for Uu threshold condition, which means no matter how low is the Uu RSRP. For upper limit, i.e., whether the RSRP of PCell of Intermedia U2N relay UE should be restricted to be below a threshold, should be discussed. This can be understood to answer the below question:

- Whether the Intermedia U2N relay UE should not be too close to gNB?

On one hand, if we only look at the ‘Intermediate Relay UE <-> U2N relay UE <-> gNB’ path, the Intermediate Relay UE is like the Rel-17 U2N remote UE, which has the upper limit for Uu threshold. Meanwhile, if the Intermediate Relay UE is too close to gNB (i.e. has a very high Uu RSRP), it is questionable why it is not serving as the U2N relay directly.

On the other hand, if we look at the ‘Remote UE <-> Intermediate Relay UE <-> U2N relay UE’ path, the Intermediate Relay UE serves like a U2U relay UE, and it also seems that the Uu condition for Intermediate Relay UE does not matter at all because anyway the Intermediate Relay UE is just connecting two UEs on PC5.

Therefore, we think the Uu threshold condition of Intermedia U2N relay UE should be discussed in RAN2.

Proposal 12: For Intermediate U2N relay UE, lower limit condition of Uu RSRP is not needed (i.e. the Uu RSRP needn’t be above a threshold).

Proposal 13: For Intermediate U2N relay UE, RAN2 to further discuss whether upper limit condition of Uu RSRP (i.e. the Uu RSRP must be below a threshold) is needed or not.

- Conclusion

In this contribution, we discussed the issues related to discovery, relay (re)selection and Uu threshold conditions. We have the following observations and proposals:

Discovery

Observation 1: According to SA2 conclusion, both Model A and Model B discovery are supported in multi-hop U2N relay operation.

Observation 2: AS conditions to transmit discovery messages consists of the following three aspects:

- Discovery threshold conditions for U2N Remote UE to transmit mode-B solicitation message;

- Discovery threshold conditions for U2N relay UE to transmit discovery mode-A announcement message and discovery mode-B response message;

- Discovery threshold conditions for Intermediate Relay UE to forward multi-hop discovery model A/model B messages.

Observation 3: In U2U relay, the Discovery Response message with Model B can be forward by U2U relay UE without RSRP conditions, because the target remote UE already considers the PC5 RSRP between it and the U2U relay UE before replying to solicitation message.

Proposal 1: Similar as in Rel-18 U2U relay operation, for multi-hop U2N relay operation the U2N remote UE can transmit Discovery Solicitation message with Model B only when PC5 RSRP between U2N remote UE and U2N relay UE is below a threshold.

Proposal 2: Similar as in Rel-18 U2U relay operation, for multi-hop U2N relay operation the U2N relay UE can transmit Discovery response message with Model B only when PC5 RSRP between U2N relay UE and Intermediate Relay UE is above a threshold.

Proposal 3: Similar as in Rel-18 U2U relay operation, the Intermediate Relay UE can forward the U2N discovery model-A announcement message received from U2N relay UE only when the PC5 RSRP of the message is above a threshold.

Proposal 4: Similar as in Rel-18 U2U relay operation, the Intermediate Relay UE can forward the U2N discovery model-B solicitation message received from U2N remote UE only when the PC5 RSRP of the message is above a threshold.

Proposal 5: Similar as in Rel-18 U2U relay operation, the Intermediate Relay UE can forward the U2N discovery model-B response message received from U2N relay UE without considering the PC5 RSRP between it and the U2N relay UE.

Relay (re)selection

Observation 4: SA2 concludes that the PC5 signal strength, number of hops between the Remote UE and the UE-to-Network Relay UE, per-hop or cumulative QoS information may be considered in Relay selection.

Proposal 6: Similar as in Rel-17 U2N relay and Rel-18 U2U relay operation, for multi-hop U2N relay operation the U2N remote UE selects an Intermediate Relay UE whose PC5 RSRP is above a threshold.

Proposal 7: The U2N remote UE selects an U2N relay UE based on upper layer criterion e.g. number of hops and per-hop or cumulative QoS information.

Proposal 8: U2N remote UE performs relay reselection for both Intermediate Relay UE and U2N relay UE when the PC5 RSRP between Intermediate Relay UE and U2N remote UE is below a threshold.

Proposal 9: Intermediate Relay UE performs relay reselection for U2N relay UE when the PC5 RSRP between Intermediate Relay UE and U2N relay UE is below a threshold. FFS whether/how to inform U2N remote UE.

Uu threshold condition

Observation 5: Intermediate U2N relay UE can be both in-coverage or out-of-coverage.

Proposal 10: For U2N relay UE, the Uu threshold condition of Rel-17 U2N relay UE is reused.

Proposal 11: For U2N remote UE, the Uu threshold condition of Rel-17 U2N remote UE is reused.

Proposal 12: For Intermediate U2N relay UE, lower limit condition of Uu RSRP is not needed (i.e. the Uu RSRP needn’t be above a threshold).

Proposal 13: For Intermediate U2N relay UE, RAN2 to further discuss whether upper limit condition of Uu RSRP (i.e. the Uu RSRP must be below a threshold) is needed or not.

- References

- RP-241609, New WID on NR sidelink multi-hop relay;

- TR 23.700-03, Study on system enhancement for Proximity based Services (ProSe) in the 5G System (5GS); Phase 3 (Release 19);