3GPP TSG-RAN WG2 Meeting #127		R2-2407008

Maastricht, Netherlands, 19th Aug. – 23rd Aug. 2024

Source:	vivo

Title:		Discussion on multi-hop U2N Relay Control Plane Procedures

Agenda Item:		8.13.3

Document for:	Discussion and Decision

# Introduction

In the WI of NR sidelink multi-hop relay[1], only Layer 2 multi-hop U2N relay will be defined. The objectives are shown as following:

- Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays. - Relay discovery and (re)selection [RAN2] - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3] - Impact on SRAP and QoS handling for multi-hop [RAN2] - Control plane procedures [RAN2, RAN3]

In this contribution, we mainly focus on discussing the potential CP impacts to support up to two additional hops relays on top of Rel-17 Layer 2 U2N relay.

# Discussion

## Terms alignment for multi-hop relay

In Rel-17 single-hop U2N relay, according to TS 38.331, the following terms are defined to describe a single-hop U2N relay path. See highlighted as below:

U2N Relay UE: A UE that provides functionality to support connectivity to the network for U2N Remote UE(s).

U2N Remote UE: A UE that communicates with the network via a U2N Relay UE.

Observation 1	In Rel-17 single-hop U2N relay, the terms of “U2N Remote UE” and “U2N Relay UE” are defined in TS 38.331.

Meanwhile, in the Rel-19 WID, “remote UE”, “first relay UE” and “last relay UE” are used to describe a multi-hop U2N relay path. See highlighted as below:

Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB).

Observation 2	In Rel-19 multi-hop U2N relay WID, the terms of “remote UE”, “first relay UE” and “last relay UE” are used.

Moreover, according to the latest SA2 TR [2], “5G ProSe Intermediate Relay” is introduced to describe a multi-hop U2N relay path. See highlighted as below:

5G ProSe Intermediate Relay: A 5G ProSe-enabled UE that provides functionality to support connectivity to the network for 5G ProSe Remote UE(s) by using the PC5 reference point with other 5G ProSe-enabled UEs, The 5G ProSe Intermediate Relay is located on the path between 5G ProSe Remote UE and 5G ProSe UE-to-Network Relay.

Observation 3	According to SA2 TR 23.700-03, the terms of “Remote UE”, “Intermediate Relay UE” and “U2N Relay UE” are defined for describing a multi-hop relay path.

Observation 4	Compared with terms in WID, the terms defined in SA2 are more backward compatible with legacy Rel-17 single-hop U2N relay and more extensible to multi-hop scenario.

Therefore,

Proposal 1 	RAN2 to consider Rel-17 single-hop U2N relay definitions as baseline and extend them for multi-hop U2N relay by referring to SA2 TR 23.700-03 terms, as following:

- U2N Remote UE: A UE that communicates with the network via one or more Intermediate U2N Relay UEs and a U2N Relay UE.

- Intermediate U2N Relay UE: A UE that provides functionality to support connectivity to the network for U2N Remote UE(s). The Intermediate U2N Relay UE is located on the path between U2N Remote UE and U2N Relay UE.

- U2N Relay UE: A UE that provides functionality to support connectivity to the network for U2N Remote UE(s) and Intermediate U2N Relay UE(s).

## Basic assumption for multi-hop relay

According to SA2 TR [2], the Intermediate U2N Relay UE is located on the path between U2N Remote UE and U2N Relay UE. Based on that observation, a multi-hop relay path should be set up based on only hop-by-hop PC5 connection as baseline, i.e. across hop PC5 connection is not needed for multi-hop U2N relay. Moreover, in order to perform subsequent relay service for U2N Remote UE over the relay path, the end-to-end Uu connection should also be setup on top of the hop-by-hop PC5 connections. Given that both of the U2N Remote UE and Intermediate U2N Relay UE can be out-of-coverage especially for public safety scenario, the end-to-end Uu connection setup via relay path should be considered as a basic assumption.

Observation 5	In case of a multi-hop U2N relay path, only hop-by-hop PC5 connection is setup and across hop PC5 connection is not needed.

Observation 6	In case of a multi-hop U2N relay path, end-to-end Uu connection is setup on top of the hop-by-hop PC5 connections.

Figure 1. Basic assumption of Multi-hop U2N Relay

As illustrated in above Figure 1, the basic modelling from U2N Relay UE relay path is proposed from U2N Relay UE perspective, U2N intermediate Relay UE perspective, and U2N Remote UE perspective, as follows:

Proposal 2	RAN2 to agree the basic assumption for Multi-hop U2N relay path setup (see Figure 1):

  - From U2N Relay UE perspective:

  - A per-hop PC5 connection with U2N intermediate Relay UE, and

  - A per-hop Uu connection with serving gNB.

  - From Intermediate U2N Relay UE perspective:

  - A per-hop PC5 connection with U2N Relay UE, and

  - A per-hop PC5 connection with U2N Remote UE, and

  - An end-to-end Uu connection with serving gNB.

  - From U2N Remote UE perspective:

  - A per-hop PC5 RRC connection with U2N Intermediate Relay UE, and

  - An end-to-end Uu RRC connection with serving gNB.

Following the Rel-17 single-hop U2N relay, all the UEs over the multi-hop U2N relay path follows the same serving cell when in RRC_CONNECTED to perform unicast data transfer for the U2N Remote UE.

Proposal 3	U2N Remote UE, Intermediate U2N Relay UE and U2N Relay UE follow the same serving cell when in RRC_CONNECTED to perform relaying of unicast data.

## Protocol stack for multi-hop relay

In line with our assumption for multi-hop U2N relay (see Proposal 2 in Section 2.2), the CP protocol stack and UP Protocol stack for Multi-hop U2N relay are illustrated and proposed in the following Figure 2 and Figure 3.

Figure 2. Control plane protocol stack for Multi-hop L2 UE-to-Network Relay

Figure 3. User plane protocol stack for Multi-hop L2 UE-to-Network Relay

Proposal 4	RAN2 to agree the Multi-hop CP protocol stack as shown in Figure 2.

Proposal 5	RAN2 to agree the Multi-hop UP protocol stack as shown in Figure 3.

## RRC connection establishment for U2N Remote UE

Figure 4. RRC connection establishment for U2N Remote UE (via two additional hops relays)

The general procedure of RRC connection establishment for U2N Remote UE is illustrated in above Figure 4 and described as following steps:

1.	The U2N Remote UE, Intermediate U2N Relay UE and U2N Relay UE perform discovery procedure, and establish a PC5-RRC connection between every adjacent UEs using the NR sidelink PC5 unicast link establishment procedure.

2.	The U2N Remote UE sends the first RRC message (i.e., RRCSetupRequest) for its connection establishment with gNB via the Intermediate U2N Relay UE, using a specified PC5 Relay RLC channel configuration. The Intermediate U2N Relay UE sends the SidelinkUEInformationNR message via U2N relay UE to request for the dedicated configurations required to support the multi-hop relay operation for the L2 U2N Remote UE. If the Intermediate U2N Relay UE is not in RRC_CONNECTED, it needs to do its own Uu RRC connection establishment upon reception of a message on the specified PC5 Relay RLC channel via the U2N Relay UE (parent node), e.g. following similar actions as R17 U2N Remote UE. Furthermore, if the U2N Relay UE is not in RRC_CONNECTED, it needs to do its own Uu RRC connection establishment upon reception of a message from the Intermediate U2N Relay UE (child node) on the specified PC5 Relay RLC channel. The U2N Relay UE firstly completes its RRC connection establishment procedure, and receives Uu Relay RLC channel configuration for the Intermediate U2N Relay UE from gNB. Secondly, after the Intermediated U2N Relay UE's RRC connection establishment procedure and sending the SidelinkUEInformationNR message, gNB configures SRB0 (for U2N Remote UE) relaying Uu Relay RLC channel to the U2N Relay UE. The gNB responds with an RRCSetup message to U2N Remote UE. The RRCSetup message is sent to the U2N Remote UE using SRB0 relaying Uu Relay RLC channel over Uu and a specified PC5 Relay RLC channel over two PC5.

3.	The gNB and U2N Relay UE perform relaying channel setup procedure over Uu. According to the configuration from gNB, the Intermediate U2N Relay/U2N Remote UE establishes a PC5 Relay RLC channel for relaying of SRB1 towards the U2N Remote UE/Intermediate U2N Relay UE over PC5, and the Intermediate U2N Relay UE/ U2N Relay UE establishes a PC5 Relay RLC channel for relaying of SRB1 towards the U2N Relay UE/ Intermediate U2N Relay UE over PC5.

4.	The RRCSetupComplete message is sent by the U2N Remote UE to the gNB via the  Intermediate U2N Relay UE and the U2N Relay UE using SRB1 relaying channel over PC5 and SRB1 relaying channel configured to the U2N Relay UE over Uu. Then the U2N Remote UE is RRC_CONNECTED with the gNB.

5.	The U2N Remote UE and gNB establishes security following the Uu security mode procedure and the security messages are forwarded through the Intermediate U2N Relay UE and the U2N Relay UE.

6.	The gNB sends an RRCReconfiguration message to the U2N Remote UE via the Intermediate U2N Relay UE and the U2N Relay UE, to setup the end-to-end SRB2/DRBs of the U2N Remote UE. The U2N Remote UE sends an RRCReconfigurationComplete message to the gNB via the Intermediate U2N Relay UE and the U2N Relay UE as a response. In addition, the gNB may configure additional Uu Relay RLC channels between the gNB and U2N Relay UE, PC5 Relay RLC channels between Intermediate U2N Relay UE and U2N Remote UE, and PC5 Relay RLC channels between Intermediate U2N Relay UE and U2N Relay UE for the relaying traffic.

Proposal 6	RAN2 to agree the general procedure of RRC connection establishment for U2N Remote UE via one additional hops relay (illustrated in Figure 4) as the discussion start point. The procedure includes:

  - U2N Remote UE, Intermediate U2N Relay UE and U2N Relay UE performing discovery and PC5 Connection Establishment;

  - U2N Remote UE sends the first RRCSetupRequest to gNB, which triggers Intermediate U2N Relay UE and/or U2N Relay UE to enter RRC_CONNECTED if they are in RRC_IDLE/INACTIVE;

  - U2N Remote UE and gNB complete connection establishment, security establishment, RRC reconfiguration and so on.

## System information delivery for U2N Remote UE

In Rel-17, a L2 U2N Relay UE can perform unsolicited SIB1 forwarding to a U2N Remote UE connected with this L2 U2N Relay UE via PC5 link, when the SIB1 has not been sent to this U2N Remote UE before or an updated SIB1 has been received by this L2 U2N Relay UE. The similar function should be certainly supported for multiple-hop L2 U2N relay network as the U2N Remote UE served by L2 multiple-hop U2N Relay network also needs SIB1 for RRC connection setup with the gNB. In multiple-hop L2 U2N Relay network, unicast SIB1 propagation from the U2N Relay UE to all its downstream UEs (Intermediate U2N Relay UE to U2N Remote UE) respectively should be supported, which comprises:

- The U2N Relay UE performs unsolicited SIB1 forwarding to the U2N Remote UEs/ Intermediate U2N Relay UE connected to this U2N Relay UE via PC5 link;

- Any Intermediate U2N Relay UE performs unsolicited SIB1 forwarding to the U2N Remote UEs connected with this Intermediate U2N Relay UE via PC5 link.

Proposal 7	U2N Relay UE should support unsolicited forwarding of the acquired SIB1 to Intermediate U2N Relay UEs/U2N Remote UEs connected to this U2N relay UE via PC5 link.

Proposal 8	Intermediate U2N Relay UE should support unsolicited forwarding of the acquired SIB1 to U2N Remote UEs connected to this Intermediate U2N relay UE via PC5 link.

Further, in Rel-17, a L2 U2N Remote UE in RRC_IDLE or RRC_INACTIVE, can request the desired SIBs from its serving U2N Relay UE. Naturally, this should also be supported for U2N Remote UE in RRC_IDLE or RRC_INACTIVE in Layer 2 multiple-hop U2N Relay network. When a U2N Remote UE is connected with an Intermediate U2N Relay UE via PC5 link, it can send SI request to its serving Intermediate U2N Relay UE, the serving Intermediate U2N Relay UE can send the requested SIBs to U2N Remote UE, and the serving Intermediate U2N Relay UE forward the requested SIBs to the U2N remote UE if the requested SIBs are available. Otherwise, the serving Intermediate Relay UE should acquire the request SIBs first either in the similar way as a legacy U2N Remote UE or acquire the requested SIBs in its Uu interface, and forward the acquired SIBs to the U2N Remote UE afterwards.

Proposal 9	Upon reception of SI request from U2N Remote UE, the Intermediate U2N Relay UE can perform one of the following actions:

  - send the requested SIBs, if available, to U2N Remote UE by itself.

  - acquire the requested SIBs, if unavailable, by further forwarding the SI request to U2N Relay UE, and forward the requested SIBs to U2N Remote UE accordingly afterwards.

## Paging delivery for U2N Remote UE

For Rel-17 L2 U2N relay, a U2N Remote UE can send its paging information to its serving U2N relay UE to request its serving U2N relay to monitor the paging message for this U2N Remote UE on behalf, wherein the paging information comprises the UE ID information (ng-5G-S-TMSI, or, ng-5G-S-TMSI & fullI-RNTI) and the paging cycle configuration. The same function should also be supported for a U2N Remote UE served by Rel-19 multiple-hop U2N network. In Rel-19 L2 multiple-hop U2N relay network, a U2N Relay UE should be able to monitor the paging message for all downstream UEs, including downstream Intermediate U2N Relay UEs and downstream U2N Remote UEs, based on the paging information from these Intermediate U2N Relay UE and U2N Remote UEs. If a U2N Remote UE connected with an Intermediate U2N Relay UE, it should transmit the paging information to the Intermediate U2N Relay UE and the Intermediate U2N Relay UE can either monitor paging by itself e.g., if it is in-coverage and support basic paging function), or further forward the paging information of this U2N Remote UE to its serving U2N Relay UE to request its serving U2N Relay UE to monitor the paging message for this U2N Remote UE on-behalf. In R19 multiple-hop U2N network, in case of an Intermediate U2N Relay UE sending its paging information to its serving U2N Relay UE to request its serving U2N Relay UE to monitor the paging message on-behalf, it should forward the received paging message from the U2N Relay UE to U2N Remote UE accordingly afterwards.

Proposal 10	U2N Relay UE should support monitoring the paging message for a downstream UE (Intermediate U2N Relay UE or U2N Remote UE) based on the paging information (ng-5G-S-TMSI, fullI-RNTI, paging cycle info) received from the downstream UE.

Proposal 11	Upon reception of the paging information from U2N Remote UE, U2N intermediate Relay UE can perform one of the following actions:

  - Monitor paging for U2N Remote UE, and forward the paging message to U2N Remote UE by itself, e.g., if it is capable of paging and in-coverage.

  - Further forward paging information to U2N Relay UE, and forward the received paging message from the U2N Relay UE to U2N Remote UE accordingly afterwards.

A U2N Remote UE in RRC_IDLE/RRC_INACTIVE connected with an Intermediate Relay UE via PC5 link may leave the L2 multiple-hop Relay network due to mobility. In this case, the Intermediate U2N Relay UE should notify the corresponding U2N Relay UE which monitors the paging information for this U2N Remote UE on-behalf to release the paging information of this U2N Remote UE. This can help the U2N Relay UE to stop paging message monitoring for this U2N Remote UE, which is beneficial for power saving of the U2N Relay UE.

Proposal 12	The U2N Relay UE should be notified when U2N Remote UE left so that the U2N Relay UE can stop paging message monitoring on behalf of U2N Remote UE.

## TAU/RNAU for U2N Remote UE

In Rel-17, the L2 U2N Remote UE performs TAU/RNAU based on the L2 U2N Relay UE's serving cell when it is connected with the L2 U2N Relay UE. A L2 U2N Remote UE in RRC_IDLE or RRC_INACTIVE initiates TAU/RNAU procedure if the serving cell changes (due to cell change by the L2 U2N Relay UE) and the new serving cell is outside of the L2 U2N Remote UE's configured TA/RNA. Basically, we assume that the U2N Remote UE can be aware of the U2N Relay UE's serving cell via multi-hop relay discovery procedure, and thus a similar principle can be applied to multi-hop Layer-2 UE-to-Network Relay operation for minimum the specification efforts. Therefore,

Proposal 13	The U2N Remote UE can be aware of the U2N Relay UE's serving cell change via multi-hop relay discovery procedure and performs legacy Rel-17 TAU/RNAU procedure as baseline.

## E2E QoS handling

According to SA2 TR, below conclusion is made in case of multi-hop Layer-3 UE-to-Network Relay operation, which is done similarly to legacy single-hop case.

End-to-end QoS management for multi-hop U2N Relays is done similarly to the end-to-end QoS management for single hop L3 U2N Relay as defined in TS 23.304 [4], with enhancement to handle QoS split over multiple legs of the PC5 interface.

And in solution #1 it is illustrated how it may work:

Figure 6.1.2.6.2.2-1: End-to-End QoS translation for 5G ProSe multi-hop Layer-3 UE-to-Network Relay operation

When it comes to multi-hop Layer-2 UE-to-Network Relay operation, the situation is also similar. In other words, the serving gNB performs E2E QoS split to derive a SL-PDB value for each PC5 hop and will further send the spilt value to UE associated to that PC5 hop.

Proposal 14	A SL-PDB value is configured by serving gNB to U2N Remote UE, U2N Intermediate Relay UE, U2N Relay UE. How to perform E2E QoS split is up to gNB implementation.

## Backward compatibility issue

L2 multiple-hop U2N relay can extend the network coverage even more than the R17/18 L2 U2N Relay network. By the time of L2 multiple-hop U2N relay deployment, legacy U2N Remote UE may have already considerable penetration in the market. Similar to the case that an advance NR gNB is usually required to serve legacy NR UEs in the market by the operators, an L2 multiple-hop U2N relay network may also be probably required to serve legacy U2N Remote UEs so that the legacy U2N Remote UEs can also benefit from the coverage extension by the L2 multiple-hop U2N relay network.

In the following, the network structure and the related protocol architecture are discussed on how to enable serving legacy U2N Remote UE via L2 multiple-hop U2N relay network.

In Figure 5 below, an example discovery procedure is illustrated. To serve the legacy U2N Remote UE, the Intermediate U2N Relay UE can also serve as a legacy U2N Relay UE to provide legacy discovery service to the legacy U2N Remote UEs. For instance, for Mode A based discovery procedure, the Intermediate U2N Relay UE should adapt the received discovery message from U2N Relay UE from R19 discovery message format to the legacy one and transmit the legacy discovery message. For model B, the solicitation message received from the legacy U2N Remote UE should also be adapted by the Intermediate U2N Relay UE from format to the R19 format, and afterwards the Intermediate U2N Relay UE send the solicitation message of R19 format to the U2N Relay UE. In such way, only intermediate U2N relay UE is impacted for serving legacy U2N Remote UE. In the aforementioned discovery/solicitation message format adaptation procedure, some issues need to be considered: there can be hop information in the R19 discovery/solicitation message but no this information in the legacy discovery/solicitation message, and the RSC in legacy discovery/solicitation message can be different from the RSC in the legacy discovery/solicitation message and a mapping between legacy RSC and R19 RSC may be needed.

Figure 5. Discovery procedure to serve legacy L2 U2N remote UE in L2 U2N multi-hop relay network

The U2N Relay UE can also provide legacy discovery service for serving legacy U2N Remote UE, according to the received configuration by upper layers.

Observation 7	For serving legacy U2N Remote UE, two issues regarding discovery should be considered, for which the feasibility may be studied in SA2:

  - the Intermediate U2N Relay UE/U2N relay UE should provide legacy discovery service to legacy U2N Remote UE in addition to R19 discovery service to R19 U2N Remote UE.

  - the Intermediate U2N Relay UE should support the format adaptation between legacy discovery/solicitation message formats and the R19 ones.

The second issue is the impact of user plane protocol for supporting legacy U2N Remote UE in L2 multiple-hop U2N Relay network. If the R17 SRAP is reused for multiple-hop U2N Relay, no special enhancement is needed for the Intermediate U2N Relay UE to hand the data/signaling forwarding for legacy U2N Remote UE.

If new SRAP header format is used for multiple-hop U2N Relay, there can be two aspects:

- If the legacy remote UE is served by U2N Relay UE: In this case, the U2N Relay UE can serve as legacy U2N Relay UE to serve the legacy U2N Remote UE and in the meanwhile it should also serve as a R19 U2N Relay UE to serve the R19 U2N Remote UEs and Intermediate U2N Relay UEs. For all these UEs of different types served by the same U2N Relay UE, the best option is to use the same SRAP PDU format for data/signaling forwarding in the Uu interface. As illustrated in Figure 6, for the legacy U2N Remote UE, the U2N Relay UE should adapt the SRAP PDU format accordingly for serving legacy U2N Remote UE: in downstream direction, the U2N Relay UE should adapt the SRAP PDU received from the U2N Relay UE from new format to legacy format; while in the upstream direction, the reverse operation is performed by the U2N Relay UE.

Figure 6. User plane protocol stack to serve legacy remote UE via U2N Relay UE in Multi-hop L2 UE-to-Network Relay network

- If the legacy remote UE is served by an Intermediate U2N Relay UE, the example user plane architecture is exemplified in Figure 7, the similar SRAP format adaptation should be performed by the intermediate U2N Relay UE.

Figure 7. User plane protocol stack to serve legacy remote UE via Intermediate U2N Relay UE in Multi-hop L2 UE-to-Network Relay network

Observation 8	If SRAP PDU format is enhanced for multiple-hop U2N Relay, the U2N Relay UE and the Intermediate U2N Relay UE should adapt the SRAP PDU format between legacy SRAP PDU format and the R19 one for serving legacy U2N Remote UE.

Proposal 15	RAN2 to discuss whether backward-compatibility of multi-hop L2 U2N relay network is required, i.e. whether the multi-hop L2 U2N relay network should also be able to serve R17/R18 L2 U2N Remote UEs.

# Conclusion

This contribution mainly discussed the potential CP impacts. The contribution concludes with:

Observation 1	In Rel-17 single-hop U2N relay, the terms of “U2N Remote UE” and “U2N Relay UE” are defined in TS 38.331.

Observation 2	In Rel-19 multi-hop U2N relay WID, the terms of “remote UE”, “first relay UE” and “last relay UE” are used.

Observation 3	According to SA2 TR 23.700-03, the terms of “Remote UE”, “Intermediate Relay UE” and “U2N Relay UE” are defined for describing a multi-hop relay path.

Observation 4	Compared with terms in WID, the terms defined in SA2 are more backward compatible with legacy Rel-17 single-hop U2N relay and more extensible to multi-hop scenario.

Observation 5	In case of a multi-hop U2N relay path, only hop-by-hop PC5 connection is setup and across hop PC5 connection is not needed.

Observation 6	In case of a multi-hop U2N relay path, end-to-end Uu connection is setup on top of the hop-by-hop PC5 connections.

Observation 7	For serving legacy U2N Remote UE, two issues regarding discovery should be considered, for which the feasibility may be studied in SA2:

  - the Intermediate U2N Relay UE/U2N relay UE should provide legacy discovery service to legacy U2N Remote UE in addition to R19 discovery service to R19 U2N Remote UE.

  - the Intermediate U2N Relay UE should support the format adaptation between legacy discovery/solicitation message formats and the R19 ones.

Observation 8	If SRAP PDU format is enhanced for multiple-hop U2N Relay, the U2N Relay UE and the Intermediate U2N Relay UE should adapt the SRAP PDU format between legacy SRAP PDU format and the R19 one for serving legacy U2N Remote UE.

Terms alignment

Proposal 1 	RAN2 to consider Rel-17 single-hop U2N relay definitions as baseline and extend them for multi-hop U2N relay by referring to SA2 TR 23.700-03 terms, as following:

  - U2N Remote UE: A UE that communicates with the network via one or more Intermediate U2N Relay UEs and a U2N Relay UE.

  - Intermediate U2N Relay UE: A UE that provides functionality to support connectivity to the network for U2N Remote UE(s). The U2N Intermediate Relay UE is located on the path between U2N Remote UE and U2N Relay UE.

  - U2N Relay UE: A UE that provides functionality to support connectivity to the network for U2N Remote UE(s) and Intermediate U2N Relay UE(s).

Basic assumption

Proposal 2	RAN2 to agree the basic assumption for Multi-hop U2N relay path setup (see Figure 1):

  - From U2N Relay UE perspective:

  - A per-hop PC5 connection with U2N intermediate Relay UE, and

  - A per-hop Uu connection with serving gNB.

  - From Intermediate U2N Relay UE perspective:

  - A per-hop PC5 connection with U2N Relay UE, and

  - A per-hop PC5 connection with U2N Remote UE, and

  - An end-to-end Uu connection with serving gNB.

  - From U2N Remote UE perspective:

  - A per-hop PC5 RRC connection with U2N Intermediate Relay UE, and

  - An end-to-end Uu RRC connection with serving gNB.

Proposal 3	U2N Remote UE, Intermediate U2N Relay UE and U2N Relay UE follow the same serving cell when in RRC_CONNECTED to perform relaying of unicast data.

Protocol stack

Proposal 4	RAN2 to agree the Multi-hop CP protocol stack as shown in Figure 2.

Proposal 5	RAN2 to agree the Multi-hop UP protocol stack as shown in Figure 3.

RRC connection establishment

Proposal 6	RAN2 to agree the general procedure of RRC connection establishment for U2N Remote UE via one additional hops relay (illustrated in Figure 4) as the discussion start point. The procedure includes:

  - U2N Remote UE, Intermediate U2N Relay UE and U2N Relay UE performing discovery and PC5 Connection Establishment;

  - U2N Remote UE sends the first RRCSetupRequest to gNB, which triggers Intermediate U2N Relay UE and/or U2N Relay UE to enter RRC_CONNECTED if they are in RRC_IDLE/INACTIVE;

  - U2N Remote UE and gNB complete connection establishment, security establishment, RRC reconfiguration and so on.

System information delivery

Proposal 7	U2N Relay UE should support unsolicited forwarding of the acquired SIB1 to Intermediate U2N Relay UEs/U2N Remote UEs connected to this U2N relay UE via PC5 link.

Proposal 8	Intermediate U2N Relay UE should support unsolicited forwarding of the acquired SIB1 to U2N Remote UEs connected to this Intermediate U2N relay UE via PC5 link.

Proposal 9	Upon reception of SI request from U2N Remote UE, the Intermediate U2N Relay UE can perform one of the following actions:

  - send the requested SIBs, if available, to U2N Remote UE by itself.

  - acquire the requested SIBs, if unavailable, by further forwarding the SI request to U2N Relay UE, and forward the requested SIBs to U2N Remote UE accordingly afterwards.

Paging delivery

Proposal 10	U2N Relay UE should support monitoring the paging message for a downstream UE (Intermediate U2N Relay UE or U2N Remote UE) based on the paging information (ng-5G-S-TMSI, fullI-RNTI, paging cycle info) received from the downstream UE.

Proposal 11	Upon reception of the paging information from U2N Remote UE, U2N intermediate Relay UE can perform one of the following actions:

  - Monitor paging for U2N Remote UE, and forward the paging message to U2N Remote UE by itself, e.g., if it is capable of paging and in-coverage.

  - Further forward paging information to U2N Relay UE, and forward the received paging message from the U2N Relay UE to U2N Remote UE accordingly afterwards.

Proposal 12	The U2N Relay UE should be notified when U2N Remote UE left so that the U2N Relay UE can stop paging message monitoring on behalf of U2N Remote UE.

TAU/RNAU

Proposal 13	The U2N Remote UE can be aware of the U2N Relay UE's serving cell change via multi-hop relay discovery procedure and performs legacy Rel-17 TAU/RNAU procedure as baseline.

E2E QoS handling

Proposal 14	A SL-PDB value is configured by serving gNB to U2N Remote UE, U2N Intermediate Relay UE, U2N Relay UE. How to perform E2E QoS split is up to gNB implementation.

Backward compatibility issue

Proposal 15	RAN2 to discuss whether backward-compatibility of multi-hop L2 U2N relay network is required, i.e. whether the multi-hop L2 U2N relay network should also be able to serve R17/R18 L2 U2N Remote UEs.

# References

- RP-241609, New WID on NR sidelink multi-hop relay;

- TR 23.700-03, Study on system enhancement for Proximity based Services (ProSe) in the 5G System (5GS); Phase 3 (Release 19).