3GPP TSG-RAN WG2 #127 	R2-2407015

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:	8.7.5  RLC AM Enhancements

Source:	NEC

Title:	RLC AM enhancements

Document for:	Discussion, Decision

# 1.Introduction

One objective of Rel-19 XR is to enhance RLC AM for XR service with small packet delay budget, which further consist of two aspects, and we made good progress in last RAN2 meeting respectively:

- Avoid unnecessary retransmissions (when it is too late)

For avoiding unnecessary RLC AM retransmissions, RAN2 to enhance the RLC AM by adopting enhancements from one of the following perspectives: - Rx initiated approach • For proper advancing of the transmitting window, RLC AM is enhanced with a way for the receiver to indicate abandoned SDUs to the transmitter. • Tx side just processes the status report as in legacy • FFS how Rx side determines that an SDU should be abandoned - Tx initiated approach • The transmitting side of AM RLC entity notifies the receiving RLC side about the obsolete SDUs • Tx side stops retransmit obsolete SDUs • Rx side updates state variables according to the information from Tx side RAN2 will discuss details of both approaches, compare them and choose one once the details are clearer.

- Achieve timely retransmissions (before it is too late)

To achieve timely retransmissions on RLC layer for XR traffic, RAN2 will consider the following options: • Autonomous retransmission (i.e. without status report) of PDUs based on some triggers (existing or new triggers can be considered) • Retransmission based on enhanced status report • Retransmission based on enhanced polling FFS whether any enhancements are needed or this can be solved with proper configuration and current mechanism Impact on capacity should be considered RAN2 focuses on the enhancements for UL traffic

In this document, we will discuss further details of each solutions, comparing them and share our preference.

# 2 Discussion

## 2.1 Avoid unnecessary retransmissions

To avoid unnecessary retransmission, either Tx or Rx side shall initiate the procedure to give up a AMD PDU transmission or reception, and more importantly, both Tx and Rx side window should be advanced in synchronized way. Last meeting, we have identified two possible solutions one is RX initiated and another one is TX initiated:

- Rx initiated approach

•	For proper advancing of the transmitting window, RLC AM is enhanced with a way for the receiver to indicate abandoned SDUs to the transmitter.

•	Tx side just processes the status report as in legacy

•	FFS how Rx side determines that an SDU should be abandoned

With Rx initiated approach, the main question is how RX side determines that an SDU should be abandoned. In RLC UM, Rx side determines that an SDU should be abandoned based on timer t-Reassembly and variable RX_Timer_Trigger, as shown in below

Figure 1 RLC UM Rx side window operation

the timer t-Reassembly is initiated when missing RLC PDU(s) are detected (i.e., gap appears between received RLC PDUs), RX_Timer_Trigger variable holds the value of the SN following the SN which triggered t-Reassembly. when the timer t-Reassembly expires, receiving side no longer waits for the missed RLC PDU(s) and move the reassembly window forward.

A similar approach can be adopted for RLC AM if Rx determines to give up a RLC PDU reception. Because the timer t-Reassembly has been used in RLC AM currently for STATUS report purpose,  we can give another name to the timer and variable which are used for giving up reception of a SDU.

In details, as shown below we name the new timer as t-Giveup, and new variable named Rx-Giveup-Trigger holds the SN following the SN which triggered t-Giveup. Same as t-Reassembly in RLC AM, the timer t-Giveup is initiated when missing RLC PDU(s) are detected (i.e., gap appears between received RLC PDUs). when the timer t-Giveup expires, receiving side no long waits for the missed RLC PDU(s) and move the lower edge of Rx window (i.e., RX-Next) forward to the first missed RLC PDU after Rx-Giveup-Trigger.

Figure 2: Rx initiated approach

According to the agreement made in last meeting, RX side will give a fake ACK for the abandoned RLC PDUs, this will trigger the Tx side window to move forward.

With this option, there is no impact to Tx side. At the Rx side, we need to introduce a new timer and new variable and specify the Rx window operation based on the timer and variable, same as RLC UM. Moreover when Rx sends a status report, it has to treat the abandoned RLC PDUs as same as successfully received ones .

Proposal 1: with Rx initiated approach,

- 1a: Rx will determines that an SDU should be abandoned based on a new timer and a new variable, operated same as in the RLC UM.

- 1b: Rx will send fake ACK in STATUS report for the abandoned SDUs

- Tx initiated approach

•	The transmitting side of AM RLC entity notifies the receiving RLC side about the obsolete SDUs

•	Tx side stops retransmit obsolete SDUs

•	Rx side updates state variables according to the information from Tx side

The first question of Tx initiated approach is how Tx side stop retransmit obsolete SDUs.  Discardtimer at PDCP layer is configured based on PDB. When discardTimer expires, PDCP layer will indicate this to RLC, so PDCP discard indication could be reused as a trigger to terminate the RLC ARQ retransmission. A separated timer could be considered too. The Tx side window operation is shown as below, the Tx window will move forward to the first not ACKed and not discarded SDU, i.e., the variable Tx_Next_Ack will be updated either when a STATUS report is received from peer RLC entity, or a PDCP discard indication is received from upper layer.

Figure 3: Tx initiated approach

The next question is how the transmitting side will inform the receiving side the RLC PDUs which it does not need to wait for anymore, this could be done e.g., via a RLC SN gap report, similar to PDCP SN gap report. This further triggers that the receiving side window moves forward.

With this option, we need to introduce RLC SN gap report, specify Tx window operation based on discard indication , and specify Rx window operation based on RLC SN gap report.

Proposal 2: with Tx initiated approach:

- 2a: Tx determines an RLC PDU is obsolete upon receiving the discard indication from PDCP layer

- 2b: introduce RLC SN gap report taking PDCP SN gap report as baseline

- Independent Rx and Tx approach

•	Tx side stops retransmit an obsolete SDUs based on the discard indication as for Tx initiated approach

•	Rx side stops to receive an obsolete SDU based on local timer and variable as for Rx initiated approach

Above we propose a hybrid option for RAN2 discussion: The Tx side is same as Tx initiated approach, using the discard indication to terminate unnecessary retransmission. At the same time, the Tx side is same as Rx initiated approach, using a new timer for giving up a missing RLC PDUs. This would avoid to introduce RLC SN gap report in Tx initiated approach, and avoid to fake ACK for the abandoned RLC PDUs in Rx initiated approach and the operation will not depend on signalling delay between Rx and Tx. The synchronization between Tx and Rx side windows is guaranteed by proper NW configuration.

Proposal 3 RAN2 consider independent Rx and Tx approach, where

•	Tx side stops to retransmit an obsolete SDUs based on the discard indication as for Tx initiated approach

•	Rx side stops to receive an obsolete SDU based on local timer and variable as for Rx initiated approach

For each approach in table below, we summarize the impacts to Tx and Rx side operation of RLC entity respectively, to have an idea on the difference and the complexity of each approach:

Table 1 Specification impact of different approaches

|  | Impact to Tx side | Impact to Rx side |
| --- | --- | --- |
| Rx initiated approach | No impact | - introduce a new timer and new variable for triggering to abandon SDU(s), this includes to specify the initiation/update condition of the new timer and variable (same/similar as RLC UM) - update existing variables and timers to advance the Rx window considering the abandoned SDUs. - In Status Report, send ACK for abandoned SDUs |
| Tx initiated approach | - specify the trigger to abandon a SDU e.g., discard indication from PDCP - update existing variables and timers to advance the Tx window considering abandoned SDUs - introduce RLC SN gap report including triggers, format and behaviour | - update existing variables and timers to advance the Rx window upon receiving the RLC SN gap report. |
| Independent Rx and Tx approach NOTE: synchronization between Rx and Tx window is guaranteed by proper configuration | - specify the trigger to abandon a SDU e.g., discard indication from PDCP - update existing variables and timers to advance the Tx window considering abandoned SDUs | - introduce a new timer and new variable for triggering to abandon SDU(s), this includes to specify the initiation/update condition of the new timer and variable (same/similar as RLC UM) - update existing variables and timers to advance the Rx window considering the abandoned SDUs. |

Proposal 4 RAN2 choose one approach considering the specification impact summarized in the table

## 2.2 Timely retransmission

To achieve timely RLC retransmission for XR, following possible solutions have been identified in last meeting:

•	Autonomous retransmission (i.e. without status report) of PDUs based on some triggers (existing or new triggers can be considered)

•	Retransmission based on enhanced status report

•	Retransmission based on enhanced polling

As shown in below the copied RLC specification, STATUS reporting will be triggered either by polling from its peer AM RLC entity or detection of reception failure of an AMD PDU, and STATUS report transmission is further subject to t-StatusProhibit timer:

Triggers to initiate STATUS reporting include: - Polling from its peer AM RLC entity: - When an AMD PDU with SN = x and the P field set to "1" is received from lower layer, the receiving side of an AM RLC entity shall: - if the AMD PDU is to be discarded as specified in clause 5.2.3.2.2; or - if x < RX_Highest_Status or x >= RX_Next + AM_Window_Size: - trigger a STATUS report. - else: - delay triggering the STATUS report until x < RX_Highest_Status or x >= RX_Next + AM_Window_Size. NOTE 1: This ensures that the RLC Status report is transmitted after HARQ reordering. - Detection of reception failure of an AMD PDU - The receiving side of an AM RLC entity shall trigger a STATUS report when t-Reassembly expires. NOTE 2: The expiry of t-Reassembly triggers both RX_Highest_Status to be updated and a STATUS report to be triggered, but the STATUS report shall be triggered after RX_Highest_Status is updated. When STATUS reporting has been triggered, the receiving side of an AM RLC entity shall: - if t-StatusProhibit is not running: - at the first transmission opportunity indicated by lower layer, construct a STATUS PDU and submit it to lower layer. - else: - at the first transmission opportunity indicated by lower layer after t-StatusProhibit expires, construct a single STATUS PDU even if status reporting was triggered several times while t-StatusProhibit was running and submit it to lower layer. When a STATUS PDU has been submitted to lower layer, the receiving side of an AM RLC entity shall: - start t-StatusProhibit.

- For polling, transmitting side can send polling bit after number of PDUs or number of bytes transmitted out , this is fully under NW configuration via parameters pollPDU and pollByte, they can be as small as 4PDUs and 1000bytes.

- For detection of reception failure of an AMD PDU, this is controlled by parameter t-Reassembly. t-Reassembly starts as long as a receiving SN gap appears, the value of t-Reassemby is configured by network, it can be as small as 0, in which case, STATUS report will be triggered right after detection a reception gap

- In addition, a new STATUS report transmission is not allowed during the time window following the previous STATUS report transmission. This is controlled by parameter t-StatusProhibit, to avoid too frequent signalling. but this parameter can be configured as small as 0.

And the possible configurable value of these parameters

PollPDU ::= ENUMERATED { p4, p8, p16, p32, p64, p128, p256, p512, p1024, p2048, p4096, p6144, p8192, p12288, p16384,p20480, p24576, p28672, p32768, p40960, p49152, p57344, p65536, infinity, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1} PollByte ::= ENUMERATED { kB1, kB2, kB5, kB8, kB10, kB15, kB25, kB50, kB75, kB100, kB125, kB250, kB375, kB500, kB750, kB1000, kB1250, kB1500, kB2000, kB3000, kB4000, kB4500, kB5000, kB5500, kB6000, kB6500, kB7000, kB7500, mB8, mB9, mB10, mB11, mB12, mB13, mB14, mB15, mB16, mB17, mB18, mB20, mB25, mB30, mB40, infinity, spare20, spare19, spare18, spare17, spare16, spare15, spare14, spare13, spare12, spare11, spare10, spare9, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1} T-Reassembly ::= ENUMERATED { ms0, ms5, ms10, ms15, ms20, ms25, ms30, ms35, ms40, ms45, ms50, ms55, ms60, ms65, ms70, ms75, ms80, ms85, ms90, ms95, ms100, ms110, ms120, ms130, ms140, ms150, ms160, ms170, ms180, ms190, ms200, spare1} T-StatusProhibit ::= ENUMERATED { ms0, ms5, ms10, ms15, ms20, ms25, ms30, ms35, ms40, ms45, ms50, ms55, ms60, ms65, ms70, ms75, ms80, ms85, ms90, ms95, ms100, ms105, ms110, ms115, ms120, ms125, ms130, ms135, ms140, ms145, ms150, ms155, ms160, ms165, ms170, ms175, ms180, ms185, ms190, ms195, ms200, ms205, ms210, ms215, ms220, ms225, ms230, ms235, ms240, ms245, ms250, ms300, ms350, ms400, ms450, ms500, ms800, ms1000, ms1200, ms1600, ms2000, ms2400, spare2, spare1}

NW could make the feedback procedure as fast as possible by configuring small value for these parameters, with the cost of very frequent STATUS report signalling from Rx to Tx. For example, with T-StatusProhibit=0, T-Reassembly=0 and PollPDU=4, Status Report will be sent in following conditions:

- Upon receive every 4th PDU, due to Rx side sets the polling bit at every 4th PDU transmission. In this case  Status report may or may not be necessarily needed.

- Upon detect a receiving SN gap, this is necessary and timely

One possible enhancement is to introduce remaining time based polling. This means Rx will poll only if the remaining time is short

Proposal 5: RAN2 consider remaining time based polling.

For blind retransmission, we can see following slightly different options:

- Timer based: If the remaining time of a RLC PDU has been less than a configured threshold, retransmission can be triggered

- Counter-based: If a RLC PDU has been retransmitted for certain number of times, a retransmission can be triggered

Counter-based solution seems not solve the issue fully. A given number of retransmission could last longer or shorter , which depends on the variable round trip delay, consequently counter-based solution cannot guarantee timely retransmission. Remaining time based solution sounds more reasonable to us.

Proposal 6: RAN2 consider to introduce autonomous retransmission based on remaining time

## 2.3 Disable RLF trigger from RLC AM

There is also proposal to not trigger RLF when maximum ARQ retransmission reaches.

Maximum times of ARQ retransmissions reaches, it means a RLC PDU cannot be successfully received after (x times of ARQ retransmissions * y time of HARQ retransmissions) . This was considered as a rare case if we assume that x and y would not be small numbers. With this assumption, RLF can be triggered.

However, if the event of reaching the maximum ARQ retransmissions become very common e.g., configured maximum retransmission times are very small due to the small PDB. we believe there is no point to trigger RLF.

At the same time, If we terminate ARQ retransmission based on PDB, it may stop to reach the maximum retransmissions. If we speed up retransmission or introduce other trigger for retransmission, it may impact how fast the maximum retransmissions will reaches. so we propose:

Proposal 7: Delay discussion on disabling RLF declaration at RLC layer, till other RLC AM enhancements become clear.

# 3 Summary

In this contribution, we shared our view on RLC AM enhancement.

Avoid unnecessary retransmissions:

Proposal 1: with Rx initiated approach,

- 1a: Rx determines that an SDU should be abandoned based on a new timer and a new variable, operated same/similar as in the RLC UM.

- 1b: Rx sends fake ACK in STATUS report for the abandoned SDUs

Proposal 2: with Tx initiated approach:

- 2a: Tx will determines an RLC PDU is obsolete upon receiving the discard indication from PDCP layer

- 2b: introduce RLC SN gap report taking PDCP SN gap report as baseline

Proposal 3 RAN2 consider independent Rx and Tx approach, where

•	Tx side stops to retransmit an obsolete SDUs based on the discard indication as for Tx initiated approach

•	Rx side stops to receive an obsolete SDU based on local timer and variable as for Rx initiated approach

Proposal 4 RAN2 choose one approach considering the specification impact summarized in the table

Table 1 Specification impact of different approaches

|  | Impact to Tx side | Impact to Rx side |
| --- | --- | --- |
| Rx initiated approach | No impact | - introduce a new timer and new variable for triggering to abandon SDU(s), this includes to specify the initiation/update condition of the new timer and variable (same/similar as RLC UM) - update existing variables and timers to advance the Rx window considering the abandoned SDUs. - In Status Report, send ACK for abandoned SDUs |
| Tx initiated approach | - specify the trigger to abandon a SDU e.g., discard indication from PDCP - update existing variables and timers to advance the Tx window considering abandoned SDUs - introduce RLC SN gap report including triggers, format and behaviour | - update existing variables and timers to advance the Rx window upon receiving the RLC SN gap report. |
| Independent Rx and Tx approach NOTE: synchronization between Rx and Tx window is guaranteed by proper configuration | - specify the trigger to abandon a SDU e.g., discard indication from PDCP - update existing variables and timers to advance the Tx window considering abandoned SDUs | - introduce a new timer and new variable for triggering to abandon SDU(s), this includes to specify the initiation/update condition of the new timer and variable (same/similar as RLC UM) - update existing variables and timers to advance the Rx window considering the abandoned SDUs. |

Timely retransmission:

Proposal 5: RAN2 consider remaining time based polling.

Proposal 6: RAN2 consider to introduce autonomous retransmission based on remaining time

Disable RLF trigger from RLC AM:

Proposal 7: Delay discussion on disabling RLF declaration at RLC layer, till other RLC AM enhancements become clear.