3GPP TSG-RAN WG2 #127	R2-2407016

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:	8.8.5 Support of regenerative payload

Source:	NEC

Title:	Mobility with regenerative payload

Document for:	Discussion, Decision

# 1.Introduction

In this document, we discuss mobility aspects with regenerative mode NTN.

# 2 Discussion

With transparent mode NTN, we have introduced different enhancements on mobility for different scenarios, in order to cope with the more frequent handover experimented by UE and to reduce the signaling overhead, shorten the handover interruption and increase the reliability:

- Satellite switch scenario: satellite switch with re-sync

- Quasi earth fixed cell: Time-based CHO, which can be performed via RACH-based or RACH-less

- Earth moving cell: Location-based CHO, can be performed via RACH-based

## 2.1 Satellite switch with re-sync

There was discussion for regenerative mode NTN, whether we can reuse satellite switch with re-sync agreed for transparent mode.

For satellite switch in transparent mode, because UE is connected to the same GW/gNB on ground via different satellites before and after satellite switch, satellite switch with re-sync(it was called unchanged PCI) was introduced in Rel18 with no involvement of any RRC signalling, no user plane stack reset, no security key change and no PCI change. Note that Rel17 UE or UE does not support this feature, they would rely on time-based CHO most likely and or RLF/RRC-re-establishement.

With regenerative mode, because UE will be connected to different gNB located on different satellite, satellite switch means gNB switch. Firstly this will involve UE context relocation, which should be discussed in RAN3; secondly, whether it is possible somehow to keep the security key during satellite/gNB switch, which should be discussed in SA3:

Proposal 1: satellite switch with regenerative mode involves UE context relocation, security key continuity, we could ask or wait for RAN3/SA3 discussion

from RAN2 perspective, assuming it is not an issue with the UE context relocation and the security key continuity, we could discuss 1) reset or not reset user plane stack, 2)the possibility to reduce or skip RRC signalling involvement. Unlike the intra-gNB satellite switch with transparent mode, with inter-gNB switch, It is different to maintain and synchronize target gNB-side with the UE-side protocol variables and status e.g., DRX timers in MAC, transmission/reception window in RLC, PDB relevant timer in PDCP:

Proposal 2: satellite switch with regenerative mode involves at least UP stack re-establishment from RAN2 perspective

On the other hand, relying purely on connected mode mobility would mean high signalling overhead. It would be beneficial to enhance/simplify the mobility procedures, with it, all connected mode UEs under the coverage will switch the RRC connection to the new satellite with less L3 signalling evolvement and or shorter interruption in regenerative mode scenario.

Proposal3:  RAN2 could discuss to reduce L3 mobility signalling upon satellite switch with regenerative mode

## 2.2 Quasi earth fixed cell

For Quasi earth fixed cell, in rel-17, we identified that all UEs under a Quasi earth fixed cell will experience handover at the time upon cell replacement. The solution is to introduce timer-based CHO. The same solution without change can be applicable to regenerative mode, assuming RAN3 will work on the inter-satellites coordination for CHO preparation.

RACH-less can be configured too, thanks to the predictable CHO execution time.

Proposal 4: Timer-based CHO, including in combine with RACH-less, could be reused for Quasi earth fixed cell with regenerative mode

On the other hand, considering the frequent satellite/gNB change. We propose to re-consider RRC re-establishment procedure in case time-based CHO is not configured

Proposal 5: upon satellite switch with regenerative mode, RAN2 discuss to support enhanced RRC re-establishment procedure if CHO is not configured

## 2.3 Earth moving cell

For Earth moving cell, location-based and or RRM-based CHO could be configured, however it cannot be configured with RACH-less, because NW cannot know when the CHO will be performed hence when to send dynamic grants from target cell to UE, or in case per-allocated grant is configured, it is not sure when CG will be used. In worst case, the pre-allocated grant might be not used and wasted.

To overcome this timing issue, one option is to make target gNB aware of the CHO execution timing. When CHO execution condition is fulfilled or is going to fulfilled, UE can report to the source cell ( current serving cell) that UE will execute CHO to the selected target cell. Source gNB then inform target gNB/cell the upcoming CHO of the UE (if it is inter-gNB CHO) to the target cell. this would be helpful for target cell to start dynamic scheduling for the UE at right time, or know when the pre-configured CG will be used in real.

Figure 3: CHO notification

Alternatively, the handover timing would be largely predictable even for location-based CHO in earth moving cell by network, if we assume that UE is stationary, and the UE will move out of the cell due the movement of the cell coverage only. So a simple enhancement is to introduce a valid time window for RACH-skip config, if the execution condition is fulfilled within the time window, RACH-skip config is valid, UE transmit on pre-allocated CG or UE will expect DG from target gNB during this time window, i.e., RACH-less CHO is performed. Otherwise if the execution condition is fulfilled out of the configured time window. RACH-skip should be invalid, and RACH-based CHO can be performed.

Proposal 6: Enable RACH-less CHO with location/RRM-based condition, with following enhancements

- Option 1 : Introduce CHO notification from UE to source gNB to support RACH-less CHO

- Option 2: Introduce a valid time window for RACH-skip configuration. UE executes RACH-less CHO if the time is in the configured time window, otherwise executes RACH-based CHO

# 3 Summary

In this document, we have following proposal regarding support mobility with regenerative mode:

Proposal 1: Satellite switch with regenerative mode involves UE context relocation, security key continuity, we could ask or wait for RAN3/SA3 discussion

Proposal 2: Satellite switch with regenerative mode involves at least UP stack re-establishment from RAN2 perspective

Proposal3: RAN2 could discuss to reduce L3 mobility signalling upon satellite switch with regenerative mode

Proposal 4: Timer-based CHO, including in combine with RACH-less, could be reused for Quasi earth fixed cell with regenerative mode

Proposal 5: Upon satellite switch with regenerative mode, RAN2 discuss to support enhanced RRC re-establishment procedure if CHO is not configured

Proposal 6: Enable RACH-less CHO with location/RRM-based condition, with following enhancements

- Option 1 : Introduce CHO notification from UE to source gNB to support RACH-less CHO

- Option 2: Introduce a valid time window for RACH-skip configuration. UE executes RACH-less CHO if the time is in the configured time window, otherwise executes RACH-based CHO