3GPP TSG-RAN WG2 #127 	R2-2407017

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:	8.8.6 LTE to NR NTN mobility

Source:	NEC

Title:	LTE TN to NR NTN Idle Mode Mobility

Document for:	Discussion, Decision

# 1.Introduction

We made good progress regarding support LTE TN to NR NTN idle mode mobility:

Agreements: 1. For idle mode mobility from EUTRA TN to NR NTN, NB-IoT UEs are considered not in the scope. 2. For idle mode mobility from EUTRA TN to NR NTN, we don’t consider specific optimizations for BL UEs and UEs in CE. 3. SIB24 is reused to provide the NR NTN cell reselection related information (e.g. frequency information, SMTC config, etc.), introducing a satellite ID list in per frequency. The EUTRA cell provides the satellite assistance information for NR neighbor cell per satellite, as identified by the satellite ID. 4. To support the idle mode mobility from EUTRA TN to NR NTN, the satellite assistance information for NR NTN neighbor cells is needed and should include the following parameters: - Satellite ephemeris information/TA common information/k-Mac/epoch time/validity duration/ntn-PolarizationDL (FFS if mandatory or optional) 5. The Ephemeris information/epoch time/k-mac/validity duration IEs defined in SIB33 specified in TS36.331 should be reused for NR satellite assistance information. 6. The signalling format for ntn-PolarizationDL and TA common related configurations within NTN-Config specified in TS38.331 should be introduced in TS36.331 for NR satellite assistance information. 7. RAN2 will decide in the next meeting which of the following options to adopt for the provision of the NR satellite assistance information (based on TPs provided by the WI RRC Rapporteur): Option 1: Introduce a new SIB to include the NR satellite assistance information. Option 2: Define new IE for NR satellite assistance information and define separate neighbour satellite information list to provide the NR satellite information in SIB33. Option 3: Extend the NeighSatelliteInfo defined for IoT NTN to include the parameters needed for NR satellite, and reuse the neighSatelliteInfoList defined in SIB33 to provide either NR or IoT NTN information. 8. Introduce the clarification in the field description of measTimingConfig (configured via SIB24 in TS 36.331) that it is configured based on the assumption that the gNB-UE propagation delay equals to 0 ms, and UE can adjust the offset based on the actual propagation delay, when the corresponding frequency is associated with a satellite ID. Working Assumption: 1. NR NTN cell reselection evaluation is based on RRM measurements as legacy; no spec impact foreseen for EUTRA TN to NR NTN cell (can come back in the next meeting to see if the WA can be confirmed)

In this contribution, we discuss a few remaining issues.

# 2 Discussion

## 2.1 Provision the NR satellite assistance information

One open issue captured in last meeting is how to provide the NR satellite assistance information:

RAN2 will decide in the next meeting which of the following options to adopt for the provision of the NR satellite assistance information (based on TPs provided by the WI RRC Rapporteur): - Option 1: Introduce a new SIB to include the NR satellite assistance information. - Option 2: Define new IE for NR satellite assistance information and define separate neighbour satellite information list to provide the NR satellite information in SIB33. - Option 3: Extend the NeighSatelliteInfo defined for IoT NTN to include the parameters needed for NR satellite, and reuse the neighSatelliteInfoList defined in SIB33 to provide either NR or IoT NTN information.

Technical speaking, SIB33 provide a satellite information, identified by satellite ID. When the satellite ID is linked with a LTE frequency, we know this is a IOT NTN satellites. Hence it is straightforward SIB33 provide a satellite information regardless it will be linked to a LTE frequency/cell or a NR frequency/cell.

One concern of reusing SIB33 is the maximum 4 satellite assistance information capacity due to TB size limit. Considering the small coverage of a LTE TN cell, The total number of its neighbouring satellites including both IOT NTN and NR NTN likely will be more than 4. In fact, it is more likely that network will configure either IOT NTN or NR NTN neighbouring satellites for a given LTE TN cell.

Another question is behaviour of a UE not supporting NR NTN, simply if the UE support one of NR NTN and IOT NTN, it will read SIB33, and only use the satellites information that the linkage of the satellites and frequency is supported; if the UE support neither of NR NTN and IOT NTN, it will not read SIB33.

Introducing a new SIB would have most specification impact, but it seems unnecessary.

Proposal 1 Provide NR satellite assistance information in SIB33 instead of a new SIB

Among option 2 and 3, our preference is option3. With option3, all the satellites of IOT and NR will have unique satellite ID and no need to repeat the same IEs for NR and IOT NTN.

Proposal 2 adopt option3 for NR satellites provision, i.e., extend the NeighSatelliteinfor to include the extra parameters needed for NR satellites

## 2.2 Measurement Initiation

Last meeting, we discussed if any enhancement on measurement initiation is needed:

Proposal 8: NR NTN neighbor cell measurement initiation is performed as in legacy, i.e. based on RRM measurement as in cell reselection from EUTRA TN to NR TN. No spec impact is needed on cell reselection procedure in TS 36.304. Come back in the next meeting

All enhancements introduced for measurement initiation (i.e., time-based, location-based measurement initiation, and TN coverage assistance info) are only for the case that serving cell is a NTN cell rather than a TN cell.

When serving cell is a TN cell, measurement initiation condition is always based on RRM measurement of SERVING CELL The principle is to not start measurement on intra-frequency, lower or equal priority frequency if the radio condition of serving cell is good enough. measurement initiations condition could be distinguished for following cases:

- Intra frequency: measure if Srxlev <= SIntraSearchP and Squal <= SIntraSearchQ

- Inter-RAT or inter-frequencies with higher priority than serving frequency: already measure

- Inter-RAT or inter-frequencies with equal or lower priority than serving frequency: measure if Srxlev < SnonIntraSearchP and Squal < SnonIntraSearchQ

But it does not distinguish between TN/NTN or intra/inter RAT neighbouring cells as of now.

We think as long as the serving cell is EUTRAN cell, the measurement initiation for cell reselection to NR NTN cell should be same as the measurement initiation for cell reselection to IOT NTN cell. No specific enhancement is needed for NR NTN neighbour cell measurement. i.e., it will reply on frequency priority only. If any enhancement is necessary, it should be equally applicable to all TN to NTN scenarios.

Proposal 3: NR NTN neighbour cell measurement initiation is performed as legacy, no enhancement is needed

## 2.3 Cell Reselection Evaluation

Last meeting we also discussed if any enhancement is needed for cell reselection evaluation considering NR NTN neighbouring cell, and made following working assumption:

Working Assumption: NR NTN cell reselection evaluation is based on RRM measurements as legacy; no spec impact foreseen for EUTRA TN to NR NTN cell (can come back in the next meeting to see if the WA can be confirmed

This has been discussed in past in the context of NR TN to NR NTN cell reselection, there was interest to distinguish TN and NTN cell type for cell reselection evaluation, e.g., TN cell have absolute higher priority over NTN cell or prioritize TN or NTN cell with different offset. But we did not agree any enhancement in the end

The situation of LTE TN to NR NTN would not be different from case of NR TN to NR NTN

Proposal 4: confirm the working assumption that no enhancement on LTE TN to NR NTN cell reselection evaluation

# 3 Summary

This contribution presented our view on remaining issues of support LTE TN to NR NTN idle mode mobility:

Proposal 1 Provide NR satellite assistance information in SIB33 instead of a new SIB

Proposal 2 adopt option3 for NR satellites information provision, i.e., extend the NeighSatelliteinfor to include the extra parameters needed for NR satellites

Proposal 3: NR NTN neighbour cell measurement initiation is performed as legacy, no enhancement is needed

Proposal 4: confirm the working assumption that no enhancement on LTE TN to NR NTN cell reselection evaluation