3GPP TSG-RAN WG2 Meeting #127                        R2-2407024

Maastricht, Netherlands, Aug.19th – Aug. 23rd, 2024

Agenda Item:	8.6.3

Source: 	Transsion Holdings

Title:  	Discussion on event triggered L1 measurement configuration for LTM

Document for:	Discussion and Decision

- Introduction

In the last RAN2#126 meeting, the measurement enhancements for LTM was discussed and the following agreements were made[1]:

Agreements on measurement enhancements for LTM:

- Event triggered L1 measurement should be designed for the following LTM purposes:

- Select the candidate beam/cell to trigger early synchronization.

- Select the target beam/cell and trigger LTM cell switch procedure.

2. 	For event triggered L1 measurement, use of beam level measurement result for event evaluation is baseline. FFS for the cell level measurement.

3.	Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events.

-	Event LTM2: Beam of serving cell becomes worse than absolute threshold;

-	Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

-	Event LTM4: Beam of candidate cell becomes better than absolute threshold;

-	Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

FFS on what beam(s) of the serving cell and neighboring cell is used for event evaluation. FFS on the need of Event LTM1.

4.	Support the beam config of both SSB and CSI-RS in L1 measurement resource configuration in LTM config. Working assumption: Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5.

5.	RAN2 assumes filtering of the L1 measure results is needed. It’s up to RAN1 whether the specified L1 filtering is needed or ok to leave it to UE implementation.

6.	For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

In this contribution, we continue to discuss the measurement enhancement for LTM.

- Discussion

2.1 Measurement event configuration for LTM

In last meeting it has been agreed that the L1 filtering、TTT and hysteresis can be applied in event triggerd L1 measurement report to avoid piong-pong cell switch in LTM. However the event triggered L1 measurement report with filter、Hysteresis、TimeToTrigger to avoid ping-pong may bring addition latency. So it needs to make a trade-off between ping-pong effect and latency. The ping-pong HO maybe has different effects in different scenarios:

- For intra-DU case, as the UE doesn’t need to perform RLC reestablishment and PDCP data recovery when performing handover, the ping-pong effect is smaller, the parameters(filter, Hysteresis, TimeToTrigger, etc) for avoid ping-pong may be not needed or can be set smaller to make a balance between ping-pong rates and latency.

- For inter-DU/inter-CU case, as the UE shall perform RLC reestablishment and PDCP data recovery procedure, the ping-pong effect is bigger, the parameters(filter, Hysteresis, TimeToTrigger, etc) for avoid ping-pong is needed or set bigger then intra-DU case.

Proposal 1:  L1 Measurement filtering, Hysteresis and TTT may need to be configured differently for Inter-DU and Intra-DU/inter-CU scenarios to make a trade-off between ping-pong rates and latency.

2.2 Event triggered L1 measurement purpose for LTM

As agreed in last meeting, the event triggered L1 measurement shoud be designed for the following LTM purposes:

- Select the candidate beam/cell to trigger early synchronization.

- Select the target beam/cell and trigger LTM cell switch procedure.

The R18 LTM supports UE-based TA measurement for early synchronization and the exact time the UE performs TA measurement is up to UE implementation, thus the network doesn’t  know whether the UE has performs TA measurement or not. If the event triggered L1 MR is used for trigger early synchronization, it may cause repeated triggering early synchronization if the UE has already get the TA of candidates cell by UE-based TA measurement. To avoid this the purpose of the event triggered L1 measurement can be included in the measurement configuration. If the purpose is to select candidate beam/cell to trigger early synchronization and the UE has already get the TA of candidate cells, the UE can consider these candidate cells unavailable or doesn’t perform L1 measurement on these candidate cells or doesn’t trigger L1 measurement report if event condition is met to avoid repeated triggering early synchronization.

Proposal 2: The purpose of event triggered L1 measurement can be included in the measurement configuration to avoid repeated triggering early synchronization.

- 3 Conclusions

In this contribution we discussed issues related to measurement enhancement for LTM and made the following proposals:

Proposal 1: L1 Measurement filtering, Hysteresis and TTT may need to be configured differently for Inter-DU and Intra-DU/inter-CU scenarios to make a trade-off between ping-pong rates and latency.

Proposal 2: The purpose of event triggered L1 measurement can be included in the measurement configuration to avoid repeated triggering early synchronization.

- 4 References

- “Draft_R2_126_Meeting_Report_v1”