3GPP TSG-RAN WG2 Meeting #127                        R2-2407025

Maastricht, Netherlands, Aug.19th – Aug. 23rd, 2024

Agenda Item:	8.6.4

Source: 	Transsion Holdings

Title:  	Discussion on L1 measurement reporting for LTM

Document for:	Discussion and Decision

- Introduction

In the last RAN2#126 meeting, the measurement enhancements for LTM was discussed and the following agreements were made[1]:

Agreements on measurement enhancements for LTM:

- Event triggered L1 measurement should be designed for the following LTM purposes:

- Select the candidate beam/cell to trigger early synchronization.

- Select the target beam/cell and trigger LTM cell switch procedure.

2. 	For event triggered L1 measurement, use of beam level measurement result for event evaluation is baseline. FFS for the cell level measurement.

3.	Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events.

-	Event LTM2: Beam of serving cell becomes worse than absolute threshold;

-	Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

-	Event LTM4: Beam of candidate cell becomes better than absolute threshold;

-	Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

FFS on what beam(s) of the serving cell and neighboring cell is used for event evaluation. FFS on the need of Event LTM1.

4.	Support the beam config of both SSB and CSI-RS in L1 measurement resource configuration in LTM config. Working assumption: Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5.

5.	RAN2 assumes filtering of the L1 measure results is needed. It’s up to RAN1 whether the specified L1 filtering is needed or ok to leave it to UE implementation.

6.	For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

In this contribution, we continue to discuss the measurement enhancement for LTM.

- Discussion

2.1 L1 measurement reporting for LTM

As agreed in last meeting, the event triggered L1 measurement shoud be designed for the following LTM purposes:

- Select the candidate beam/cell to trigger early synchronization.

- Select the target beam/cell and trigger LTM cell switch procedure.

The R18 LTM supports UE-based TA measurement and PDCCH order for early synchronization and the exact time the UE performs TA measurement is up to UE implementation, thus the network doesn’t know whether the UE has performs TA measurement or not. If the event triggered L1 MR is used for trigger early synchronization, it may cause repeated triggering early synchronization if the UE has already get the TA of candidates cell by UE-based TA measurement. To avoid this the TA available information can be included in the L1 measurement report. If the network receives L1 measurement report with TA available information for a candidate cell, then the network will not send PDCCH order for this candidate cell to trigger early synchronization as the UE has already get the TA. The TA available information can be just one bit to indiate the network that the UE has already get the TA of the candidate cell.

Proposal 1: The TA available information can be included in the measurement report to avoid repeated triggering early synchronization.

2.2 How to report event triggered L1 measurements

There are two options to carry the L1 measurement report: MAC CE and UCI. The R18 LTM L1 measurement report is carried by UCI and the the L1 measurement report can be periodic、semi-persistent and aperiodic. For periodic report the UL resource has to be reserved for the UE for a long time. The R19 event triggered L1 measurement report is used to save the UL transmission resource. So the MAC CE is more suitable to convey the event triggered L1 measurement report, moreover the MAC CE is more flexibility and can convey more information.

Proposal 2: Event-triggered L1-measurements are reported by the UE to the network via a MAC CE.

- 3 Conclusions

In this contribution we discussed issues related to measurement enhancement for LTM and made the following proposals:

Proposal 1: The TA available information can be included in the measurement report to avoid repeated triggering early synchronization.

Proposal 2: Event-triggered L1-measurements are reported by the UE to the network via a MAC CE.

- 4 References

- “Draft_R2_126_Meeting_Report_v1”