3GPP TSG-RAN WG2 Meeting #R2-127		R2-2407033 Maastricht, Netherlands, Aug 19-23, 2024

Agenda Item:	8.6.2

Source: 	Rakuten

Title:  	Security impacts of Inter-CU LTM

Document for:	Discussion, Decision

# Introduction

In the WID [1], the following objective is defined for inter-CU LTM:

- Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - As secondary priority, support the case when NR-DC is configured and CU is acting as SN and MCG is unchanged - As secondary priority, support the case when NR-DC is configured, CU is acting as MN and SCG is unchanged or SCG is released - Note: The case that LTM is configured in both MCG and SCG is excluded - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support

The WID objective mentions that for inter-CU LTM Rel-18 intra-CU LTM is the baseline. This implies that any enhancements for the inter-CU LTM use cases are going to be on top of the Rel 18 framework.

In this contribution, we describe and discuss security aspects involved in extending LTM to support inter-CU use-cases. We also propose a way forward for some of these.

# Discussion

## Security handling in inter-CU LTM scenario

It is obvious that security key refresh is required at inter gNB mobility as the PDCP anchor point i.e gNB-CU-UP is bound to change. There is no difference to this even when LTM solutions are applied for UE mobility.

The below agreement was made in RAN2 #125-bis.

Agreements on LTM cell switch execution phase:

- Upon inter-CU LTM execution, UE performs

- MAC reset

- RLC re-establishment

- PDCP re-establishment

- Security key update

FFS if there is an inter-CU LTM w/o security key change.

Here in this paper, we discuss the FFS: if there is an inter-CU LTM w/o security key change.

A UE configured with LTM may have both intra and inter-frequency candidate cells. In scenarios where two different gNBs are on two different frequency layers (eg: one on FR1 and another on FR2), the inter-CU LTM is not limited just to the gNB edges (as in case of intra-frequency candidate cells), it may have to be configured at all locations inside a gNB.

Hence, the logic that inter-CU LTM is just limited to gNB edges and when the UE moves inside a gNB, we may not need inter-CU LTM candidate cells is not true. This also entails that inter-CU LTM cannot be sustained without security key change by anchoring the PDCP entity from the old serving gNB, as it may cause significant signaling overhead and compromise on data transmission latency between CU-UP and the gNB-DU.

Figure 1: Exemplary deployment scenario for inter-CU LTM

In the above figure, the Red cells belong to a FR1 macro gNB and the Green cells are small cells of FR2 frequency range and belong to a different gNB. A UE is shown in one of the cells where it can be easily visualized to have inter-CU LTM candidate cells. In this case, the concept of PDCP anchoring from a previous serving gNB becomes unnecessarily complex.

Figure 2: Exemplary deployment scenario for inter-CU LTM without security key change

Another issue that we foresee is that the candidate/target gNB may need a direct Xn interface with the gNB hosting the anchor CU-UP to establish the UL/DL datapath. In such a case inter-CU LTM without security key change will not work and hence CU-UP has to be relocated.

For example, in the above figure 2, let’s consider the case where the UE moves from gNB1 (DU1) to gNB2 (DU2) to gNB3(DU3). In the scenario where the CU-UP is anchored from the first serving gNB, it may work when the UE moves under the coverage of gNB2, albeit with some unnecessary and complex signaling to establish data path between CU-UP1 and DU2. But when the UE moves to gNB3, this doesn’t work as there is no Xn interface between gNB3 and gNB1. In such cases, the CU-UP has to be relocated.

In summary

- Inter-CU LTM without security key change consumes additional signaling and latency between a serving gNB-CU-CP and anchor gNB-CU-CP (to setup and manage anchor gNB-CU-UP).

- Inter-CU LTM doesn’t provide any benefit wrt RLC handling, as the gNB-DU is going to be different in anycase.

- Inter-CU LTM without security key change doesn’t require data forwarding as the PDCP is not changed. Since this is over the backhaul, it cannot be treated as a gamechanger.

- Even if Inter-CU LTM without security key change is agreed, gNB-CU-UP relocation (PDCP + security key change) becomes mandatory when there is no direct Xn interface between the target gNB and the gNB hosting the anchor PDCP. This leads to dual implementation causing additional effort.

Therefore, we don’t think it is worth the trade-off having an inter-CU LTM solution without security key change, especially when Inter-CU LTM with security key change is already in place.

We also think that inter-CU LTM with security key change is inevitable, and hence we see no benefit in inter-CU LTM without security key change.

Inter-CU LTM scenarios are not limited to gNB edges, they could occur at any location in a gNB.

gNB-CU-UP relocation (PDCP + security key change) becomes mandatory when there is no direct Xn interface between the target gNB and the gNB hosting the anchor PDCP.

This handover without security key change is not specific to LTM. Any other handover, for eg L3 handover could also be impacted due to this.

There is no visible benefit of inter-CU LTM without security key change compared to inter-CU LTM with security key change.

Inter-CU LTM with security key change solution is sufficient to address all use-cases of inter-CU LTM.

Security Key refresh is always executed for inter-CU LTM.

## Intra-gNB inter gNB-CU-UP relocation

In Rel 18, it was agreed that security key update is not supported for intra-gNB LTM scenarios.

The below agreement was made in RAN2 #125-bis which indicates that security key refresh is supported for inter-CU LTM.

Agreements on LTM cell switch execution phase:

- Upon inter-CU LTM execution, UE performs

- MAC reset

- RLC re-establishment

- PDCP re-establishment

- Security key update

FFS if there is an inter-CU LTM w/o security key change.

However, there is an intra gNB use case that requires security key refresh. In this section, we describe the inter gNB-DU LTM scenario not considered in R18 and discuss the security aspects associated with it.

Figure 2: Inter gNB-DU LTM HO with intra gNB gNB-CU-UP relocation

Let us consider the above figure which depicts a gNB with the following elements,

  - A gNB having a gNB-CU-CP and two gNB-CU-UPs serving a subset of gNB-DUs.

  - gNB-CU-UP1 serves gNB-DUs, DU1..DU5.

  - gNB-CU-UP2 serves gNB-DUs, DU6..DU10.

While the gNB internal architecture is not relevant to RAN2, it is interesting to notice here that when a UE undergoes LTM cell switch from a cell belonging to DU5 to a cell belonging to DU6, it also undergoes intra gNB inter-gNB-CU-UP relocation. When a UE undergoes intra gNB gNB-CU-UP relocation, it changes the PDCP anchor point and hence there is a need to refresh the security keys. This implies that new security keys generated have to be delivered to the UE. This also implies that the gNB-CU-UP has to be included while preparing such an LTM candidate cell.

Security key refresh is required when a UE undergoes inter gNB-DU mobility that results in an intra gNB gNB-CU-UP relocation.

Security Key refresh is supported for inter gNB-DU LTM, when gNB-CU-UP relocation occurs inside the same gNB.

RAN2 discusses and agrees a way forward for the security impacts due to intra gNB gNB-CU-UP relocation scenario and sends an LS to RAN3 to discuss and address the same.

# Conclusion

We have the following observations:

Observation 1	Inter-CU LTM scenarios are not limited to gNB edges, they could occur at any location in a gNB.

Observation 2 	gNB-CU-UP relocation (PDCP + security key change) becomes mandatory when there is no direct Xn interface between the target gNB and the gNB hosting the anchor PDCP.

Observation 3	This handover without security key change is not specific to LTM. Any other handover, for eg L3 handover could also be impacted due to this.

Observation 4	There is no visible benefit of inter-CU LTM without security key change compared to inter-CU LTM with security key change.

Observation 5	Inter-CU LTM with security key change solution is sufficient to address all use-cases of inter-CU LTM.

Observation 6	Security key refresh is required when a UE undergoes inter gNB-DU mobility that results in an intra gNB gNB-CU-UP relocation.

We have the following proposals:

Proposal 1	Security Key refresh is always executed for inter-CU LTM.

Proposal 2	Security Key refresh is supported for inter gNB-DU LTM, when gNB-CU-UP relocation occurs inside the same gNB.

Proposal 3	RAN2 discusses and agrees a way forward for the security impacts due to intra gNB gNB-CU-UP relocation scenario and sends an LS to RAN3 to discuss and address the same.

# References

RP-240299, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom, 3GPP TSG RAN #103, Maastricht, Netherlands, March 18-21, 2024

RAN2 Chairman Notes R2_125bis_ChairNotes

RAN2 Chairman Notes R2_126_ChairNotes