3GPP TSG-RAN WG2 #127	R2-2407034

Maastricht, Netherlands, 19th – 23th August 2024

Agenda Item:	8.13.3

Source:	Ericsson, FirstNet, AT&T

Title:	Discussion on control plane procedures

Document for:	Discussion, Decision

# 1	Introduction

For the WI on NR sidelink multiple-hop relay, the below objectives on control plane procedures have been defined in the WID [1].

The objective of this work item is to specify solutions that are needed to support multi-hop Layer-2 UE-to-Network relay for a single indirect path via SL relay UEs based on Rel-17/18 SL relay functionalities [RAN2, RAN3]

- Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays.

  - Relay discovery and (re)selection [RAN2]

  - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3]

  - Impact on SRAP and QoS handling for multi-hop [RAN2]

  - Control plane procedures [RAN2, RAN3]

We discuss aspects on control plane procedures in this paper. All discussions refer to L2 U2N Remote UE or L2 U2N Relay UE unless otherwise stated.

# 2	Discussion

Figure 1: an example of a multi-hop L2 U2N relay.

According to the above study objectives, RAN2 needs to specify mechanisms to support up to two additional hops relays. RAN2 can focus on one additional hop relay scenario prior to RAN#107. It will be further checked in RAN#107 on whether the specified mechanism can be easily extended to two additional hop relays. Given this in mind, we believe that RAN2 should study and specify mechanisms to minimize the impact of hop count on the mechanism as much as possible.

RAN2 to focus on developing mechanisms aiming to minimize the impact of hop count on the developed multi-hop relay mechanisms.

## 2.1 RRC connection management

After a Remote UE has completed the discovery procedure to reach the target U2N Relay UE, the Remote UE needs to establish its own PDU sessions/DRBs with the network before user plane data transmission. The NR sidelink PC5 unicast link establishment procedures can be used to setup a secure unicast link between Remote UE and Intermediate Relay UE, and between Intermediate Relay UE and U2N Relay UE, before Remote UE establishes a Uu RRC connection with the network via U2N Relay UE. Same approach can be used for NR sidelink PC5 unicast link establishment procedure to setup a secure unicast link between Intermediate Relay UE and another Intermediate Relay UE, before Remote UE establishes a Uu RRC connection with the network via U2N Relay UE when we consider 2 additional Relays scenario in later meetings. In this case, the NR sidelink PC5 unicast link establishment procedures would need to be performed independently for each hop. However, the NR sidelink PC5 unicast link establishment procedures will propagate back towards the network, until it reaches the L2 U2N Relay.

The Remote UE selects all Relay UEs after Relay (re)selection, i.e., selects the path to reach U2N Relay UE. After that, the Remote UE can initiate the PC5 unicast link establishment towards the selected Intermediate Relay UE. However, in order to trigger the Intermediate Relay UE to initiate the PC5 unicast link establishment towards its next Relay UE (e.g., U2N Relay UE), the Intermediate UE needs to know which UE is selected as the U2N Relay UE).

To establish PC5 unicast link between a selected Intermediate Relay UE and a selected U2N Relay UE, the Intermediate Relay UE needs to be aware of which UE has been selected as the U2N Relay UE.

It seems that this issue has been also somewhat discussed in SA2. As shown in the below, how path information is applied in link management, has been captured in SA2 conclusions for multi-hop U2N relay (see clause 8.1 Conclusion for K1#1 in TR 23.700-03).

-	When Model B discovery is performed, the Remote UE selects both the UE-to-Network Relay and the path to reach the UE-to-Network Relay. To perform link management, the DCR message is unicasted between Relays according to the path information included in the message. The path information is an (ordered) list of User Info ID of Relays in the selected path. The Remote UE sends the selected path information to the Intermediate UE-to-Network Relay for communication setup.

It is sufficient for each Relay UE to apply the path information in the unicast link establishment towards the U2N Relay UE. The path information is built by Remote UE. therefore, Remote UE would need to signal each Relay UE of the path information. In addition, the link establishment procedure involving the path information for L3 multi-hop relay, being discussed in SA2 can be reused for L2 multi-hop U2N relay. Since the link establishment procedure is common for L2 multi-hop U2N relay and L3 multi-hop U2N relay.

As captured in TR 23.700-03, for multi-hop U2N relay, Remote UE performs link management procedure to reach U2N Relay UE according to the path information which is an ordered list of User Info ID of Relays in the selected path.

As captured in TR 23.700-03, the L2 link establishment procedure involving the path information for multi-hop U2N relay, being discussed in SA2 is also applicable for L2 multi-hop relay. Since the link establishment procedure is common for L2 multi-hop relay and L3 multi-hop relay.

After relay (re)selection, Remote UE can build the path information for the selected path based on the selected Relays towards the selected U2N Relay UE.

After relay (re)selection, remote UE builds the path information based on selected Relays for the selected path towards U2N Relay UE.

Upon initiating the link establishment procedure for the first PC5 hop, Remote UE sends the DCR including the path information to Intermediate Relay UE. After reception of the DCR message from Remote UE, the Intermediate Relay UE stores the path information. When Intermediate Relay UE determines to initiate the link establishment procedure towards the selected U2N Relay UE according to the path information, Intermediate Relay UE includes the path information (without any update) in the DCR and sends the DCR to the selected U2N Relay UE. Upon reception of the DCR, the selected U2N Relay UE may also store the path information, based on which the U2N Relay UE knows that the Remote UE has selected it to be the U2N Relay UE. After that, Remote UE and the U2N Relay UE can follow the legacy procedure (i.e., as captured in clause 16.12.5.1 of TS 38.300) can be reused to establish RRC connection for Remote UE towards the gNB serving the U2N Relay UE.

During the L2 link establishment procedure, Remote UE sends the DCR including the path information to the selected Intermediate Relay UE for the first hop link establishment. The selected Intermediate Relay UE sends the DCR including the path information to the selected U2N Relay UE for the second hop link establishment.

RAN2 can assume the L2 link establishment is the same for L2 multi-hop U2N relay and L3 multi-hop U2N relay. Based on this, it is recommended that RAN2 sends LS to SA2 indicating this assumption to trigger SA2 further work on L2 multi-hop U2N relay.

RAN2 assumes the L2 link establishment is the same for L2 multi-hop U2N relay and L3 multi-hop U2N relay. RAN2 sends LS to SA2 indicating this.

It may be beneficial for the gNB to be aware of the path information for the selected path. In Rel-17 L2 U2N relay, it was the relay UE to report to the gNB of the remote UE ID via SidelinkUEInformationNR. Similarly, in case of multi-hop U2N relay, the L2 U2N Relay UE can report the path information (provided by upper layer) to the gNB via SidelinkUEInformationNR. It is expected that the gNB (as described in our accompany paper [2], it is assumed all Relay UEs are served by the same gNB) can use the path information at least for the below purposes

- Allocate local ID for Remote UE and each Relay UE on the selected path

- configure (PC5 or Uu) RLC channels for relaying SRB1/2 purpose

- configure (PC5 or Uu) RLC channels for relaying DRBs purpose

- perform QoS split between hops for the selected path

It is beneficial for L2 U2N Relay UE to inform the gNB of the path information for the selected path.

L2 U2N Relay UE signals the gNB of the path information (provided by upper layer) for the selected path via SidelinkUEInformationNR.

The legacy procedure for L2 U2N Remote UE RRC connection establishment (i.e., as captured in clause 16.12.5.1 of TS 38.300) can be reused for Remote UE in multi-hop relay with update due to support of additional Relay UE(s) on the path. An example of the procedure is illustrated in the below Figure 2.

Figure 2: Procedure for L2 U2N Remote UE connection establishment in multi-hop relay.

## 2.2 Uu RLF

In legacy, after Uu RLF is declared, the L2 U2N Relay UE takes the following action on top of the actions described in clause 9.2.7:

-	a PC5-RRC message can be used for sending an indication to its connected L2 U2N Remote UE(s), which may trigger RRC connection re-establishment for L2 U2N Remote UE; or

-	indicating to upper layer to trigger PC5 unicast link release.

Upon detecting PC5 RLF, the L2 U2N Remote UE may trigger RRC connection re-establishment.

The above legacy procedure can be reused for L2 U2N Relay UE and L2 U2N Remote UE with the below enhancements

- An indication on detection of Uu RLF at L2 U2N Relay UE needs to be forwarded to connected L2 U2N Remote UE(s) and L2 Intermediate Relay UEs; how to signal the indication to all connected L2 U2N Remote UE(s) are FFS, since the per hop legacy PC5 RRC cannot be used.

- L2 U2N Relay UE need to signal U2N Intermediate Relay UE(s) to trigger PC5 unicast link release for all PC5 hops.

L2 U2N Relay UE cannot use per hop legacy PC5 RRC to inform L2 U2N Remote UE of detected Uu RLF.

Upon detection of Uu RLF at L2 U2N Relay UE, L2 U2N Relay UE signals an indication of the link failure to all connected L2 U2N Remote UEs. FFS on the signaling details.

Upon detection of Uu RLF at L2 U2N Relay UE, L2 U2N Relay UE needs to signal U2N Intermediate Relay UE(s) to trigger PC5 unicast link release for all PC5 hops. FFS on the signaling details.

## 2.3 RRC re-establishment

As described in other clauses, new trigger conditions may need to be defined for RRC connection re-establishment.

Study and discuss additional trigger conditions for RRC connection re-establishment.

In Rel-17, the L2 U2N Remote UE may perform the following actions during the RRC connection re-establishment procedure:

-	If only suitable cell(s) are available, the L2 U2N Remote UE initiates RRC re-establishment procedure towards a suitable cell;

-	If only suitable L2 U2N Relay UE(s) are available, the L2 U2N Remote UE initiates RRC re-establishment procedure towards a suitable relay UE's serving cell via selected suitable L2 U2N Relay;

-	If both a suitable cell and a suitable relay are available, the L2 U2N Remote UE can select either one to initiate RRC re-establishment procedure based on implementation.

For multi-hop relay, upon triggering of RRC connection re-establishment, Remote UE performs similar actions. If Remote UE finds only available Relay UEs, Remote UE initiates RRC re-establishment procedure towards the selected suitable U2N Relay UE.

The legacy L2 U2N Remote UE actions during the RRC connection re-establishment procedure can be reused for L2 multi-hop U2N relay.

## 2.4 System Information and Paging

In Rel-17, the L2 U2N Remote UE in RRC_CONNECTED can use the on-demand SIB framework to request the SIB(s) via L2 U2N Relay UE. The L2 U2N Remote UE in RRC_IDLE or RRC_INACTIVE can inform L2 U2N Relay UE of its requested SIB type(s) via PC5-RRC message. Then, L2 U2N Relay UE triggers on-demand SI/SIB acquisition procedure according to its own RRC state (if needed) and sends the acquired SI(s)/SIB(s) to L2 U2N Remote UE via PC5-RRC message.

In Rel-17, Remote UE needs to provide paging assistance information (e.g., its ID and paging DRX cycle) to a Relay UE so that the Relay UE can monitor Remote UE’s POs. The Relay UE sends the paging messages to Remote UE via PC5-RRC.

For multi-hop relay, Remote UE connects to the network via one or multiple Intermediate Relay UEs and one U2N Relay UE. In such case, it is reasonable for L2 U2N Remote UE to use the legacy mechanisms only towards the L2 U2N Relay UE, rather than any L2 Intermediate Relay UE, since L2 Intermediate Relay UE may be served in a different cell of the same gNB than L2 U2N Relay UE.

It is reasonable for L2 U2N Remote UE to use the legacy mechanisms only towards the L2 U2N Relay UE, rather than any L2 Intermediate Relay UE, since L2 Intermediate Relay UE may be served in a different cell than L2 U2N Relay UE.

Similar as Uu RLF, the legacy per hop PC5-RRC signalling cannot be directly applied by L2 U2N Remote UE and L2 U2N Relay UE to signal information on system information or paging between each other because Remote UE and they are separated by more than one hop.

RAN2 to discuss how to signal system information and/or paging information between L2 U2N Relay UE and L2 U2N Remote UE.

## 2.6 QoS split

In legacy releases, RAN2 had agreed the below regarding QoS split for U2N relay

- It is up to gNB implementation to perform PDB split between Uu and PC5 (non-standardized PDB values are not precluded). No specification impact is foreseen in RAN2.

We think the same agreement/conclusion applies to multi-hop L2 U2N relay. It will be up to gNB implementation to perform PDB split among all hops including PC5 hops and Uu hop.

It is up to gNB implementation to perform PDB split among all hops including PC5 hops and Uu hop.

## 2.7 RAN sharing

In Rel-17, RAN sharing is supported for L2 U2N Relay UE. In particular, the L2 U2N Relay UE may forward, via discovery message, cell access related information before the establishment of a PC5-RRC connection.

In our view, RAN sharing can be directly supported for multi-hop relay without additional spec changes foreseen. It is sufficient that cell access related information of U2N Reay UE is forwarded to Remote UE via discovery message before the establishment of a PC5-RRC connection.

RAN sharing can be supported for multi-hop L2 U2N relay without additional spec changes foreseen.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	To establish PC5 unicast link between a selected Intermediate Relay UE and a selected U2N Relay UE, the Intermediate Relay UE needs to be aware of which UE has been selected as the U2N Relay UE.

Observation 2	As captured in TR 23.700-03, for multi-hop U2N relay, Remote UE performs link management procedure to reach U2N Relay UE according to the path information which is an ordered list of User Info ID of Relays in the selected path.

Observation 3	As captured in TR 23.700-03, the L2 link establishment procedure involving the path information for multi-hop U2N relay, being discussed in SA2 is also applicable for L2 multi-hop relay. Since the link establishment procedure is common for L2 multi-hop relay and L3 multi-hop relay.

Observation 4	After relay (re)selection, remote UE builds the path information based on selected Relays for the selected path towards U2N Relay UE.

Observation 5	During the L2 link establishment procedure, Remote UE sends the DCR including the path information to the selected Intermediate Relay UE for the first hop link establishment. The selected Intermediate Relay UE sends the DCR including the path information to the selected U2N Relay UE for the second hop link establishment.

Observation 6	It is beneficial for L2 U2N Relay UE to inform the gNB of the path information for the selected path.

Observation 7	L2 U2N Relay UE cannot use per hop legacy PC5 RRC to inform L2 U2N Remote UE of detected Uu RLF.

Observation 8	It is reasonable for L2 U2N Remote UE to use the legacy mechanisms only towards the L2 U2N Relay UE, rather than any L2 Intermediate Relay UE, since L2 Intermediate Relay UE may be served in a different cell than L2 U2N Relay UE.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to focus on developing mechanisms aiming to minimize the impact of hop count on the developed multi-hop relay mechanisms.

Proposal 2	RAN2 assumes the L2 link establishment is the same for L2 multi-hop U2N relay and L3 multi-hop U2N relay. RAN2 sends LS to SA2 indicating this.

Proposal 3	L2 U2N Relay UE signals the gNB of the path information (provided by upper layer) for the selected path via SidelinkUEInformationNR.

Proposal 4	Upon detection of Uu RLF at L2 U2N Relay UE, L2 U2N Relay UE signals an indication of the link failure to all connected L2 U2N Remote UEs. FFS on the signaling details.

Proposal 5	Upon detection of Uu RLF at L2 U2N Relay UE, L2 U2N Relay UE needs to signal U2N Intermediate Relay UE(s) to trigger PC5 unicast link release for all PC5 hops. FFS on the signaling details.

Proposal 6	Study and discuss additional trigger conditions for RRC connection re-establishment.

Proposal 7	The legacy L2 U2N Remote UE actions during the RRC connection re-establishment procedure can be reused for L2 multi-hop U2N relay.

Proposal 8	RAN2 to discuss how to signal system information and/or paging information between L2 U2N Relay UE and L2 U2N Remote UE.

Proposal 9	It is up to gNB implementation to perform PDB split among all hops including PC5 hops and Uu hop.

Proposal 10	RAN sharing can be supported for multi-hop L2 U2N relay without additional spec changes foreseen.

# 4	References

[1] RP-241609, “New WID on NR sidelink multi-hop relay”, 3GPP TSG RAN Meeting #104, Shanghai, China, 17 – 20 June 2024.

[2] R2-2407035, “discussion on discovery and relay (re)selection”, 3GPP TSG-RAN WG2 #127, Maastricht, Netherlands, 19th – 23th August 2024.