3GPP TSG-RAN WG2 #127	R2-2407035

Maastricht, Netherlands, 19th – 23th August 2024

Agenda Item:	8.13.2

Source:	Ericsson, FirstNet, AT&T

Title:	Discussion on relay discovery and relay (re)selection

Document for:	Discussion, Decision

# 1	Introduction

For the WI on NR sidelink multiple-hop relay, the below objectives on relay discovery and relay (re)selection have been defined in the WID [1].

The objective of this work item is to specify solutions that are needed to support multi-hop Layer-2 UE-to-Network relay for a single indirect path via SL relay UEs based on Rel-17/18 SL relay functionalities [RAN2, RAN3]

- Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays.

  - Relay discovery and (re)selection [RAN2]

  - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3]

  - Impact on SRAP and QoS handling for multi-hop [RAN2]

  - Control plane procedures [RAN2, RAN3]

.

We discuss aspects on relay discovery and relay (re)selection in this paper. All discussions refer to L2 U2N Remote UE or L2 U2N Relay UE unless otherwise stated.

# 2	Discussion

Figure 1: an example of a multi-hop L2 U2N relay.

According to the above study objectives, RAN2 needs to specify mechanisms to support up to two additional hops relays. RAN2 can focus on one additional hop relay scenario prior to RAN#107. It will be further checked in RAN#107 on whether the specified mechanism can be easily extended to two additional hop relays. Given this in mind, we believe that RAN2 should study and specify mechanisms to minimize the impact of hop count on the mechanism as much as possible.

RAN2 to focus on developing mechanisms aiming to minimize the impact of hop count on the developed multi-hop relay mechanisms.

An example of a multi-hop relay is shown in Figure 1, where the L2 U2N remote UE may connect to the gNB via an L2 Intermediate Relay UE and a L2 U2N Relay UE. We discuss different aspects on discovery and relay (re)selection in the below based on this example. To simplify the discussion, we refer to Relay UE 1 as an Intermediate Relay UE (i.e., which is an Intermediate Relay UE between Remote UE and the last Relay UE). Depending on hop count, the number of Intermediate Relay UE may be one or larger than one. In case there are two additional hops, Relay UE 1 and Relay UE 2 can be referred to as Intermediate Relay UE 1 and Intermediate Relay UE 2. The last Relay UE can be referred to as U2N Relay UE. These terminologies are also aligned with SA2 (see TR 23.700-03). They can be used during Stage 2 phase. Whether other terminologies are needed can be checked during Stage 3 phase.

Aligned with SA2, RAN2 to use terminologies including L2 U2N Remote UE, L2 Intermediate Relay UE(s) and L2 U2N Relay UE at least in Stage 2 phase.

According to the WID [1], RAN2 will specify service continuity for intra-gNB scenarios. It means that Remote UE, and all Relay UEs for the same path are served by the same gNB. The first question that RAN2 needs to answer is whether Intermediate Relay UE(s) can be served in a different cell than U2N Relay UE. There are two scenarios expected.

Scenario 1: Intermediate Relay UE(s) and U2N Relay UE are in the same serving cell.

Scenario 2: Intermediate Relay UE(s) and U2N Relay UE are in different serving cells of the same gNB. With more additional Intermediate Relay UEs on the same path, Intermediate Relay UEs may be in different cells.

Scenario 2 may require additional efforts for RAN2 than Scenario 1. However, we think the additional efforts are manageable.

Therefore, we would like to RAN2 to confirm this before executing any detailed study.

RAN2 to work on two scenarios including:

  - Scenario 1: Intermediate Relay UE(s) and U2N Relay UE are in the same serving cell.

  - Scenario 2: Relay UEs can be in different serving cells of the same gNB.

## 2.1 Relay discovery

In this section, we discuss aspects regarding Relay discovery including discovery model, RRC state, discovery configuration, Uu RSRP threshold, resource pool, resource allocation mode, power control and security.

In the legacy, standalone discovery model (i.e., Model A and Model B discovery model) are supported for single hop U2N relay. It is natural to support both of these models same as in the legacy for multi-hop relay.

Same as in legacy, support standalone discovery model (i.e., Model A and Model B discovery model) for multi-hop U2N relay.

In the legacy, both U2N/U2U remote UE and U2N/U2U relay UE can be in any RRC state (i.e., RRC_IDLE, RRC_INACTIVE or RRC_CONNECTED. The RRC states combination of Remote UE in RRC_CONNECTED and relay UE in RRC_IDLE is not supported. The RRC states combination of remote UE in RRC_CONNECTED and relay UE in RRC_INACTIVE is not supported. The same rules would be applicable for Remote UE, U2N Relay. Since L2 Intermediate Relay UE may be out of coverage. It seems unnecessary for Intermediate Relay UE to be mandated in RRC_CONNECTED.

Same as in legacy, when L2 U2N Remote UE is in RRC_CONNECTED, L2 U2N Relay UE also needs to be RRC_CONNECTED.

L2 Intermediate Relay UE doesn’t need to be in RRC_CONNECTED when L2 Remote UE is in RRC_CONNECTED.

In the legacy, a U2N remote UE can be configured with a Uu RSRP threshold, based on which the U2N remote UE determines if it can transmit discovery messages. Similarly, a U2N relay UE can be configured with a maximum Uu RSRP threshold, a minimum Uu RSRP threshold, or both, based on which the U2N relay UE determines if it can transmit discovery messages. It is reasonable to reuse these thresholds for Remote UE and U2N Relay UE in multi-hop relay scenario. If the gNB supports both single hop U2N relay and multi-hop U2N relay, it is reasonable for the gNB to configure the same Uu RSRP thresholds. In other words, the gNB doesn’t need to distinguish between single hop Remote UEs and multi-hop Remote UEs.

Reuse the same Uu RSRP thresholds as in the legacy (i.e., single hop L2 U2N relay) for L2 Remote UE and L2 U2N Relay UE in multi-hop U2N relay.

Regarding resource pools, same as in the legacy, a dedicated resource pool and shared resource pools can be (pre)configured for discovery in multi-hop relay. Same as Uu RSRP threshold, the gNB can configure the same discovery resource pool(s) for both single hop U2N relay and multi-hop U2N relay.

Reuse the same discovery resource pool(s) and configurations as in the legacy (i.e., single hop L2 U2N relay) in multi-hop U2N relay.

In the legacy, for a U2N Remote UE connecting to the gNB via a U2N Relay UE, only resource allocation Mode 2 is supported. Different from U2N relay, U2U Remote UE supports both resource allocation Mode 1 and Mode 2. For multi-hop relay, it may be necessary to further discuss if resource allocation Mode 1 can be supported for Remote UE. In our view, it is feasible to introduce the support of resource allocation Mode 1 for Remote UE. For instance, at least, configured grant Type 1 can be supported. The configuration of configured grant Type 1 is based on RRC signaling, which doesn’t add much design complexity for gNB. Meanwhile, it is beneficial for gNB to directly allocate sidelink resources for Remote UE(s) by supporting configured grant Type 1.

In legacy, for a U2N Remote UE connecting to the gNB via a U2N Relay UE, only resource allocation Mode 2 is supported.

To discuss if resource allocation Mode 1 (e.g., configured grant Type 1) can be supported for L2 U2N Remote UE connecting to the gNB via multi-hop U2N relays.

## 2.2 Relay (re)selection

In legacy, the U2N Remote UE triggers U2N Relay selection in following cases:

-	Direct Uu signal strength of current serving cell of the U2N Remote UE is below a configured signal strength threshold;

-	Indicated by upper layer of the U2N Remote UE.

The U2N Remote UE may trigger U2N Relay reselection in following cases:

1)	PC5 signal strength of current U2N Relay UE is below a (pre)configured signal strength threshold;

2)	Cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure has been indicated by U2N Relay UE via PC5-RRC signalling;

3)	When U2N Remote UE receives a PC5-S link release message from U2N Relay UE;

4)	When U2N Remote UE detects PC5 RLF;

5)	Indicated by upper layer.

For Relay selection, the existing conditions can be directly reused.

Therefore, we would like to make the below proposal.

Supports the same trigger conditions of relay selection for L2 U2N Remote UE as in legacy.

For Relay reselection,

The trigger conditions 1) is applicable to both L2 Intermediate Relay UE and L2 U2N Relay UE. For the latter, FFS on how the latter is informed to L2 U2N Remote UE.

The condition 3) is only applicable to L2 Intermediate Relay UE.

the legacy trigger conditions including 4) and 5) can be directly reused for L2 U2N Remote UE.

We think the condition 2) is only applicable for L2 U2N Relay UE. In addition, upon detection of any such event, the Relay UE would need to inform Remote UE of the event. RAN2 can further discuss the signaling details. In legacy, PC5-RRC signalling is exchanged between UEs in a per hop manner, which would not be applicable for the Relay UE which is not directly connected to Remote UE.

In addition, an L2 Intermediate Relay UE may detect PC5 RLF towards L2 U2N Relay UE, in this case, the Intermediate Relay UE can use PC5-RRC message to inform L2 U2N Remote UE of the detected PC5 RLF. Based on which, L2 U2N Remote UE may trigger relay reselection.

Therefore, we would like to make the below proposal.

Similar as in legacy, L2 U2N Remote UE may trigger relay reselection in following cases:

  - PC5 signal strength of current L2 Intermediate Relay UE or current L2 U2N Relay UE is below a (pre)configured signal strength threshold. FFS on how the latter is informed to L2 U2N Remote UE.

  - Cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure has been indicated by L2 U2N Relay UE. FFS on signaling details.

  - When L2 U2N Remote UE receives a PC5-S link release message from L2 Intermediate Relay UE

  - When L2 U2N Remote UE detects PC5 RLF

  - PC5 RLF towards L2 U2N Relay UE has been indicated by L2 Intermediate Relay UE

  - Indicated by upper layer.

In Rel-17 single hop U2N relay, the PLMN ID and cell ID can be used as additional AS criteria. Similarly, Remote UE may need to consider both PLMN ID and cell ID in the relay (re)selection procedure. In the procedure, Remote UE may first select a U2N Relay UE considering its PLMN ID and cell ID. After that, Remote UE selects one or multiple intermediate Relay UE by checking if they are authorized to access the cell of U2N Relay UE. Therefore, each Relay UE candidate can include the PLMN info and cell ID in discovery messages as a container (which is transmitted or forwarded). In legacy, the discovery message can already contain PLMN ID and cell ID for single Relay UE. For multiple U2N relay, the discovery message can be extended to include PLMN ID and cell ID for each Relay UE in the path. Upon reception of discovery messages/responses, Remote UE considers received PLMN info and cell ID info in relay (re)selection procedure.

Same as in legacy, PLMN info and cell ID info of each Relay UE may be considered as additional AS criteria in relay (re)selection.

Each L2 U2N Relay UE candidate can include PLMN info and cell ID in discovery messages (which is transmitted or forwarded).

In addition, in Solution #1(TR 23.700-03), SA2 has discussed how to use Hop-Count value in relay (re)selection procedure. In an example, Remote UE selects the Relay UE announcing the least Hop-Count value to build the path with shortest hop numbers. We think it is not necessary to disclose Hop-Count value into AS, due to below reasons

1) Hop-count is included in Discovery or DCR/DCA messages, which are not seen by AS by nature.

2) It is sufficient to categorize such criteria as Upper layer criteria, and leave them to be handled by Upper layers. Disclose them to AS may introduce double control/usage of these criteria at both Upper layer and AS layer, which increases additional complexity.

Therefore, we propose that

Hop-Count can be categorized as an upper layer criterion and left to be handled by upper layer, and therefore, no need to be disclosed to AS layer.

In Rel-18, U2U Relay UE may consider PC5 RSRP threshold when transmitting or forwarding discovery messages. We think such mechanisms can be reused by an Intermediate Relay UE. A UE can determine whether itself is capable to operate as an Intermediate Relay UE or a U2N Relay UE depending on e.g., whether the UE has direct connection to the network.

A UE can determine whether itself is capable to operate as an Intermediate Relay UE or a U2N Relay UE depending on e.g., whether the UE has direct connection to the network.

An Intermediate Relay UE can consider PC5 RSRP threshold when transmitting or forwarding discovery messages.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	In legacy, for a U2N Remote UE connecting to the gNB via a U2N Relay UE, only resource allocation Mode 2 is supported.

Observation 2	A UE can determine whether itself is capable to operate as an Intermediate Relay UE or a U2N Relay UE depending on e.g., whether the UE has direct connection to the network.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to focus on developing mechanisms aiming to minimize the impact of hop count on the developed multi-hop relay mechanisms.

Proposal 2	Aligned with SA2, RAN2 to use terminologies including L2 U2N Remote UE, L2 Intermediate Relay UE(s) and L2 U2N Relay UE at least in Stage 2 phase.

Proposal 3	RAN2 to work on two scenarios including:

a.	Scenario 1: Intermediate Relay UE(s) and U2N Relay UE are in the same serving cell.

b.	Scenario 2: Relay UEs can be in different serving cells of the same gNB.

Proposal 4	Same as in legacy, support standalone discovery model (i.e., Model A and Model B discovery model) for multi-hop U2N relay.

Proposal 5	Same as in legacy, when L2 U2N Remote UE is in RRC_CONNECTED, L2 U2N Relay UE also needs to be RRC_CONNECTED.

Proposal 6	L2 Intermediate Relay UE doesn’t need to be in RRC_CONNECTED when L2 Remote UE is in RRC_CONNECTED.

Proposal 7	Reuse the same Uu RSRP thresholds as in the legacy (i.e., single hop L2 U2N relay) for L2 Remote UE and L2 U2N Relay UE in multi-hop U2N relay.

Proposal 8	Reuse the same discovery resource pool(s) and configurations as in the legacy (i.e., single hop L2 U2N relay) in multi-hop U2N relay.

Proposal 9	To discuss if resource allocation Mode 1 (e.g., configured grant Type 1) can be supported for L2 U2N Remote UE connecting to the gNB via multi-hop U2N relays.

Proposal 10	Supports the same trigger conditions of relay selection for L2 U2N Remote UE as in legacy.

Proposal 11	Similar as in legacy, L2 U2N Remote UE may trigger relay reselection in following cases:

1)	PC5 signal strength of current L2 Intermediate Relay UE or current L2 U2N Relay UE is below a (pre)configured signal strength threshold. FFS on how the latter is informed to L2 U2N Remote UE.

2)	Cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure has been indicated by L2 U2N Relay UE. FFS on signaling details.

3)	When L2 U2N Remote UE receives a PC5-S link release message from L2 Intermediate Relay UE

4)	When L2 U2N Remote UE detects PC5 RLF

5)	PC5 RLF towards L2 U2N Relay UE has been indicated by L2 Intermediate Relay UE

6)	Indicated by upper layer.

Proposal 12	Same as in legacy, PLMN info and cell ID info of each Relay UE may be considered as additional AS criteria in relay (re)selection.

Proposal 13	Each L2 U2N Relay UE candidate can include PLMN info and cell ID in discovery messages (which is transmitted or forwarded).

Proposal 14	Hop-Count can be categorized as an upper layer criterion and left to be handled by upper layer, and therefore, no need to be disclosed to AS layer.

Proposal 15	An Intermediate Relay UE can consider PC5 RSRP threshold when transmitting or forwarding discovery messages.

# 4	References

[1] RP-241609, “New WID on NR sidelink multi-hop relay”, 3GPP TSG RAN Meeting #104, Shanghai, China, 17 – 20 June 2024.