3GPP TSG-RAN WG2 Meeting #127															          R2-2407036

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda item:	8.8.6

Source:	MediaTek Inc.

Title:	Discussion on cell reselection from E-UTRA TN to NR NTN

Document for:	Discussion and Decision

# Introduction

In the last meeting, a basic framework to support mobility from E-UTRA TN to NR NTN is agreed, i.e., the content of satellite assistance information, NR NTN cell reselection related information in SIB24, reusing the existing IEs in 36.331/38.331, etc. The related agreements are listed below [1]:

Agreements:

- For idle mode mobility from EUTRA TN to NR NTN, NB-IoT UEs are considered not in the scope.

- For idle mode mobility from EUTRA TN to NR NTN, we don’t consider specific optimizations for BL UEs and UEs in CE.

- SIB24 is reused to provide the NR NTN cell reselection related information (e.g. frequency information, SMTC config, etc.), introducing a satellite ID list in per frequency. The EUTRA cell provides the satellite assistance information for NR neighbor cell per satellite, as identified by the satellite ID.

4.	To support the idle mode mobility from EUTRA TN to NR NTN, the satellite assistance information for NR NTN neighbor cells is needed and should include the following parameters:

-	Satellite ephemeris information

-	TA common information

-	k-Mac

-	epoch time

-	validity duration

-	ntn-PolarizationDL (FFS if mandatory or optional)

5.	The Ephemeris information/epoch time/k-mac/validity duration IEs defined in SIB33 specified in TS36.331 should be reused for NR satellite assistance information.

6.	The signalling format for ntn-PolarizationDL and TA common related configurations within NTN-Config specified in TS38.331 should be introduced in TS36.331 for NR satellite assistance information.

7.	RAN2 will decide in the next meeting which of the following options to adopt for the provision of the NR satellite assistance information (based on TPs provided by the WI RRC Rapporteur):

Option 1: Introduce a new SIB to include the NR satellite assistance information.

Option 2: Define new IE for NR satellite assistance information and define separate neighbour satellite information list to provide the NR satellite information in SIB33.

Option 3: Extend the NeighSatelliteInfo defined for IoT NTN to include the parameters needed for NR satellite, and reuse the neighSatelliteInfoList defined in SIB33 to provide either NR or IoT NTN information.

8.	Introduce the clarification in the field description of measTimingConfig (configured via SIB24 in TS 36.331) that it is configured based on the assumption that the gNB-UE propagation delay equals to 0 ms, and UE can adjust the offset based on the actual propagation delay, when the corresponding frequency is associated with a satellite ID.

Working Assumption:

- NR NTN cell reselection evaluation is based on RRM measurements as legacy; no spec impact foreseen for EUTRA TN to NR NTN cell (can come back in the next meeting to see if the WA can be confirmed)

In this paper, we discuss some remaining FFS issues.

# Discussion

How to provide the NR NTN satellite assistance information?

During the last meeting, there are three options considered:

- Option 1: Introduce a new SIB to include the NR satellite assistance information.

- Option 2: Define new IE for NR satellite assistance information and define separate neighbour satellite information list to provide the NR satellite information in SIB33.

- Option 3: Extend the NeighSatelliteInfo defined for IoT NTN to include the parameters needed for NR satellite, and reuse the neighSatelliteInfoList defined in SIB33 to provide either NR or IoT NTN information.

Option 1 can benefit UE which only supports either IoT-NTN or NR-NTN by reading the necessary SIB only. But it may cause extra SIB signalling overhead and scheduling overhead.  For a UE capable of supporting NR-NTN and IoT-NTN, it may cause UE overhead to read both SIBs.

Option 2 can reduce the SIB scheduling overhead and UE decoding overhead (i.e., only decoding the necessary IEs).  In addition, UE can easily differentiate IEs for IoT-NTN or NR-NTN (like Option1). However, it may cause some redundant IEs (or repeated IEs) in SPEC.  Furthermore, UE would receive both IoT-NTN and NR-NTN information even if UE only supports one of them.

Option 3 can slightly minimize the SIB message size by reusing the current configurations/IEs, but it will cause the extra decoding overhead for UE which only supports one of IoT-NTN and NR-NTN. In addition, the number of maximum satellites (4 currently) shall be re-considered when the deployment includes IoT-NTN and NR-NTN simultaneously.

Based on the above analysis, we slightly prefer Option2 or Option 3.

In addition, based on the agreements, the TA-common related information and ntn-PolarizationDL shall reuse the format in 38.331, which means that new IEs for TA-common and ntn-PolarizationDL are needed. The next issue is whether to add NR-NTN common-TA and ntn-PolarizationDL into the current NeighSatelliteInfo or to define a new IE for NR-NTN. From ASN.1 coding overhead point of view, a new IE is more suitable and easy. Therefore, we tend to think option 2 is best approach.

Proposal 1: Define a new IE for NR satellite assistance information and define a separate neighbour satellite information list to provide the NR satellite information in SIB33. [Option 2]

NR NTN cell reselection evaluation

We have below working assumption from last meeting:

Working Assumption:

- NR NTN cell reselection evaluation is based on RRM measurements as legacy; no spec impact foreseen for EUTRA TN to NR NTN cell (can come back in the next meeting to see if the WA can be confirmed)

In the current NR TN to NR NTN and EUTRA TN to EUTRA NTN, the legacy cell reselection evaluation is reused.  Therefore, the RRM measurements for EUTRA TN to NR NTN shall be the baseline.  As a result, the WA could be confirmed.

Proposal 2:  Confirm the work assumption below:

- NR NTN cell reselection evaluation is based on RRM measurements as legacy; no spec impact foreseen for EUTRA TN to NR NTN cell (can come back in the next meeting to see if the WA can be confirmed)

# Conclusion

Based on the above discussion, we have the following observations and proposals.

How to provide the NR NTN satellite assistance information?

Proposal 1: Define a new IE for NR satellite assistance information and define a separate neighbour satellite information list to provide the NR satellite information in SIB33. [Option 2]

NR NTN cell reselection evaluation

Proposal 2:  Confirm the work assumption below:

- NR NTN cell reselection evaluation is based on RRM measurements as legacy; no spec impact foreseen for EUTRA TN to NR NTN cell (can come back in the next meeting to see if the WA can be confirmed)

# References

- R2-2405703    Report from Break-Out Session on NR NTN and IoT NTN

# Appendix

#### –	SystemInformationBlockType33

The IE SystemInformationBlockType33 contains satellite assistance information for neighbour cells.

SystemInformationBlockType33 information element

-- ASN1START

SystemInformationBlockType33-r18 ::= SEQUENCE {

neighSatelliteInfoList-r18		NeighSatelliteInfoList-r18			OPTIONAL,	-- Need OR

neighValidityDuration-r18		ENUMERATED {s5, s10, s15, s20, s25, s30, s35, s40,

s45, s50, s55, s60, s120, s180, s240, s900}

OPTIONAL,	-- Need OP

lateNonCriticalExtension		OCTET STRING							OPTIONAL,

...

}

NeighSatelliteInfoList-r18 ::=	SEQUENCE (SIZE(1..maxSat-r17)) OF NeighSatelliteInfo-r18

NeighSatelliteInfo-r18 ::=	SEQUENCE {

satelliteId-r18				SatelliteId-r18,

ephemerisInfo-r18				CHOICE {

stateVectors-r18				EphemerisStateVectors-r17,

orbitalParameters-r18			EphemerisOrbitalParameters-r17

},

nta-CommonParameters-r18		SEQUENCE {

nta-Common-r18					INTEGER (0..8316827)		OPTIONAL,	-- Need OP

nta-CommonDrift-r18			INTEGER (-261935..261935)	OPTIONAL,	-- Need OP

nta-CommonDriftVariation-r18	INTEGER (0..29479)			OPTIONAL	-- Need OP

},

epochTime-r18					SEQUENCE {

startSFN-r18					INTEGER (0..1023),

startSubFrame-r18				INTEGER (0..9)

}																OPTIONAL,	-- Need OP

k-Mac-r18						INTEGER (1..512)				OPTIONAL,	-- Need OP

t-ServiceStartNeigh-r18		TimeOffsetUTC-r17				OPTIONAL	-- Need OR

}

-- ASN1STOP