3GPP TSG-RAN WG2 Meeting #127	R2-2407037

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda item:	8.10.5

Source:	Samsung

Title:	SON/MDT enhancements for leftover topics from R18

Document for:	Discussion and Decision

# Introduction

As per [1], one of the objectives of R19 SON WI is to support the leftover issues from R18.

- Support of the leftovers in Rel-18 SON/MDT [RAN3, RAN2]: - RACH optimization for SDT - MHI Enhancement for SCG Deactivation/Activation - MRO for MR-DC SCG failure

In this contribution, we discuss the UE reporting for these leftover topics from R18.

# Discussion

MRO for MR-DC SCG failure

In RAN2#127, it is agreed to support MRO for EN-DC SCG failure. In EN-DC, the SCGFailureInformationNR is sent to the MN (eNB or ng-eNB) in case of SCG failure. To support MRO, the following information should be added to the SCGFailureInformationNR similar like NR-NR DC:

•	the CGI of the Source PSCell

•	CGI of the Failed PSCell

•	timeSCGFailure

•	connectionFailureType

•	random-access related information set by the PSCell

Proposal 1: Include the following information in the SCGFailureInformationNR.

•	the CGI of the Source PSCell

•	CGI of the Failed PSCell

•	timeSCGFailure

•	connectionFailureType

•	random-access related information set by the PSCell

MHI Enhancement for SCG Deactivation/Activation

UE may report whether SCG deactivation was there for a PSCell in MHI. It could be interesting to know whether the SCG deactivation had any performance impacts. For e.g. the UE may log the duration when there was UL data in SCG Bearer, but the SCG was deactivated.

Proposal 2: RAN2 to discuss reporting of the duration when there was UL data in SCG Bearer, but the SCG was deactivated in MHI.

RACH optimization for SDT

In RAN2#126, RAN2 made the following agreements in last meeting:

For the purpose of SON enhancements for SDT, include RSRP/data volume related information

Downlink RSRP value and buffered uplink data volume at the time when the UE evaluates if it should perform SDT.

When SDT failure happens, the UE can indicate the failure cause of SDT to the network, e.g. T319a expiration. Details are TBD, e.g. if RSRP and data volume can also be included in such report.

RAN2 agreed to log downlink RSRP value and it is important for the network to perform SDT related optimization. On the other hand, UE can initiate SDT when the DL RSRP is not higher than sdt-RSRP-Threshold. Due to this reason, it would be beneficial for the UE to log the SDT RSRP threshold (sdt-RSRP-Threshold-r17).

Proposal 3: If the UE evaluates if it should perform SDT, UE logs sdt-RSRP-Threshold.

RAN2 agreed to indicate the failure cause of SDT to the network and the details are TBD. UE may log the following causes for SDT failure.

  - if indication from the MCG RLC that the maximum number of retransmissions has been reached is received while SDT procedure is ongoing.

  - if random access problem indication is received from MCG MAC while SDT procedure is ongoing.

  - if the lower layers indicate that cg-SDT-TimeAlignmentTimer or the configuredGrantTimer expired before receiving network response for the UL CG-SDT transmission with CCCH message while SDT procedure is ongoing.

  - if integrity check failure indication is received from lower layers while SDT procedure is ongoing.

  - if T319a expires.

  - if network send UE to RRC_IDLE in response to the resume procedure initiated for SDT.

  - if network send UE to RRC_INACTIVE in response to the resume procedure initiated for SDT.

Proposal 4: UE logs the following failure causes of SDT to the network:

  - if indication from the MCG RLC that the maximum number of retransmissions has been reached is received while SDT procedure is ongoing.

  - if random access problem indication is received from MCG MAC while SDT procedure is ongoing.

  - if the lower layers indicate that cg-SDT-TimeAlignmentTimer or the configuredGrantTimer expired before receiving network response for the UL CG-SDT transmission with CCCH message while SDT procedure is ongoing.

  - if integrity check failure indication is received from lower layers while SDT procedure is ongoing.

  - if T319a expires.

# Conclusion

Based on the discussion, we have the following proposals.

Proposal 1: Include the following information in the SCGFailureInformationNR.

•	the CGI of the Source PSCell

•	CGI of the Failed PSCell

•	timeSCGFailure

•	connectionFailureType

•	random-access related information set by the PSCell

Proposal 2: RAN2 to discuss reporting of the duration when there was UL data in SCG Bearer, but the SCG was deactivated.

Proposal 3: If the UE evaluates if it should perform SDT, UE logs sdt-RSRP-Threshold.

Proposal 4: UE logs the following failure causes of SDT to the network:

  - if indication from the MCG RLC that the maximum number of retransmissions has been reached is received while SDT procedure is ongoing.

  - if random access problem indication is received from MCG MAC while SDT procedure is ongoing.

  - if the lower layers indicate that cg-SDT-TimeAlignmentTimer or the configuredGrantTimer expired before receiving network response for the UL CG-SDT transmission with CCCH message while SDT procedure is ongoing.

  - if integrity check failure indication is received from lower layers while SDT procedure is ongoing.

  - if T319a expires.

# References

[1] RP-234038: Data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR standalone and MR-DC (Multi-Radio Dual Connectivity) Phase 4

[2] RAN2#126 meeting report.