3GPP TSG-RAN WG2 Meeting #127	R2-2407038

Maastricht, Netherlands,  Aug 19th – 23rd, 2024

Agenda item:	7.13.2

Source:	Samsung, Ericsson

Title:	RRC Corrections for SON/MDT

Document for:	Discussion and Decision

# Introduction

In this contribution, we discuss RRC corrections for SON/MDT including

-Handling SPR configuration during CHO/LTM based recovery.

-Handling SHR/SPR configuration during full configuration.

# Discussion

## SPR configuration during CHO/LTM based recovery

In the current specification, UE releases SPR configuration configured by both PCell and PSCell upon initiating RRC Reestablishment procedure, as shown below.

#### 5.3.7.2 Initiation Upon initiation of the procedure, the UE shall: 1> stop timer T310, if running; 1> stop timer T312, if running; 1> stop timer T304, if running; 1> start timer T311; 1> stop timer T316, if running; 1> if UE is not configured with attemptCondReconfig; and 1> if UE is not configured with attemptLTM-Switch: 2> reset MAC; 2> release spCellConfig, if configured; 2> suspend all RBs, and BH RLC channels for IAB-MT, and Uu Relay RLC channels for L2 U2N Relay UE, except SRB0 and broadcast MRBs; 2> release the MCG SCell(s), if configured; 2> if MR-DC is configured: 3> perform MR-DC release, as specified in clause 5.3.5.10; 2> perform the LTM configuration release procedure for the MCG and the SCG as specified in clause 5.3.5.18.7; 2> release delayBudgetReportingConfig, if configured and stop timer T342, if running; 2> release overheatingAssistanceConfig, if configured and stop timer T345, if running; 2> release idc-AssistanceConfig, if configured; 2> release btNameList, if configured; 2> release wlanNameList, if configured; 2> release sensorNameList, if configured; 2> release drx-PreferenceConfig for the MCG, if configured and stop timer T346a associated with the MCG, if running; 2> release maxBW-PreferenceConfig for the MCG, if configured and stop timer T346b associated with the MCG, if running; 2> release maxCC-PreferenceConfig for the MCG, if configured and stop timer T346c associated with the MCG, if running; 2> release maxMIMO-LayerPreferenceConfig for the MCG, if configured and stop timer T346d associated with the MCG, if running; 2> release minSchedulingOffsetPreferenceConfig for the MCG, if configured stop timer T346e associated with the MCG, if running; 2> release rlm-RelaxationReportingConfig for the MCG, if configured and stop timer T346j associated with the MCG, if running; 2> release bfd-RelaxationReportingConfig for the MCG, if configured and stop timer T346k associated with the MCG, if running; 2> release releasePreferenceConfig, if configured stop timer T346f, if running; 2> release onDemandSIB-Request if configured, and stop timer T350, if running; 2> release referenceTimePreferenceReporting, if configured; 2> release sl-AssistanceConfigNR, if configured; 2> release obtainCommonLocation, if configured; 2> release musim-GapAssistanceConfig, if configured and stop timer T346h, if running; 2> release musim-GapPriorityAssistanceConfig, if configured; 2> release musim-LeaveAssistanceConfig, if configured; 2> release musim-CapabilityRestrictionConfig, if configured and stop timer T346n, if running; 2> release ul-GapFR2-PreferenceConfig, if configured; 2> release scg-DeactivationPreferenceConfig, if configured, and stop timer T346i, if running; 2> release propDelayDiffReportConfig, if configured; 2> release rrm-MeasRelaxationReportingConfig, if configured; 2> release maxBW-PreferenceConfigFR2-2, if configured; 2> release maxMIMO-LayerPreferenceConfigFR2-2, if configured; 2> release minSchedulingOffsetPreferenceConfigExt, if configured; 2> release multiRx-PreferenceReportingConfigFR2, if configured, and stop timer T346m, if running; 2> release aerial-FlightPathAvailabilityConfig, if configured; 2> release ul-TrafficInfoReportingConfig, if configured, and stop all instances of timer T346l, if running; 1> release successHO-Config, if configured; 1> release successPSCell-Config configured by the PCell, if configured; 1> release successPSCell-Config configured by the PSCell, if configured;

RIL issue S254 discussed in last meeting noted that in the current specification, even if the UE performs CHO based recovery, the SPR configuration from the PSCell will be released. As the network doesn’t know whether UE has executed CHO through CHO based recovery or a CHO based on the satisfaction of conditions, it has no means to know if SPR config from the PSCell is released. The same issue is present for LTM based recovery also. During RAN2#126, companies in general agreed that the issue exists, but wanted more time to check the solution.

As it is agreed that only the SPR configuration from PCell needs to be released after a PCell change, UE needs to keep the SPR configuration configured by PSCell if either attemptCondReconfig or attemptLTM-Switch is configured while initiating RRCReestablishment procedure. A TP showing the relevant spec changes is provided in section 5.

Proposal 1: If attemptCondReconfig or attemptLTM-Switch are configured while initiating RRC Reestablishment procedure, UE keeps the successPSCell-Config configured by the PSCell. RIL S254 is closed.

## SHR/SPR configuration during full configuration

In the current specification, UE releases dedicated configurations in the current configurations during full configuration. This means SHR/SPR configurations from the source cell also will be released at this point. However, the SHR/SPR configurations from the source cell is used for determining the SHR/SPR at a later point of time. Therefore, the UE should keep them during full configuration. The configurations will be released after the SHR/SPR is determined, as in current spec. A TP showing the relevant spec changes is provided in section 6.

Proposal 2: UE keeps the successHO-Config and successPSCell-Config during full configuration.

# Conclusion

Based on the discussion, we have the following proposals.

Proposal 1: If attemptCondReconfig or attemptLTM-Switch are configured while initiating RRC Reestablishment procedure, UE keeps the successPSCell-Config configured by the PSCell. RIL S254 is closed.

Proposal 2: UE keeps the successHO-Config and successPSCell-Config during full configuration.

# References

[1] TS 38.331 V18.2.0

# TP1: RRC Reestablishment

#### 5.3.7.2	Initiation

<some not relevant text>

Upon initiation of the procedure, the UE shall:

1>	stop timer T310, if running;

1>	stop timer T312, if running;

1>	stop timer T304, if running;

1>	start timer T311;

1>	stop timer T316, if running;

1>	if UE is not configured with attemptCondReconfig; and

1>	if UE is not configured with attemptLTM-Switch:

2>	reset MAC;

2>	release spCellConfig, if configured;

2>	suspend all RBs, and BH RLC channels for IAB-MT, and Uu Relay RLC channels for L2 U2N Relay UE, except SRB0 and broadcast MRBs;

2>	release the MCG SCell(s), if configured;

2>	if MR-DC is configured:

3>	perform MR-DC release, as specified in clause 5.3.5.10;

2>	perform the LTM configuration release procedure for the MCG and the SCG as specified in clause 5.3.5.18.7;

2>	release delayBudgetReportingConfig, if configured and stop timer T342, if running;

2>	release overheatingAssistanceConfig, if configured and stop timer T345, if running;

2>	release idc-AssistanceConfig, if configured;

2>	release btNameList, if configured;

2>	release wlanNameList, if configured;

2>	release sensorNameList, if configured;

2>	release drx-PreferenceConfig for the MCG, if configured and stop timer T346a associated with the MCG, if running;

2>	release maxBW-PreferenceConfig for the MCG, if configured and stop timer T346b associated with the MCG, if running;

2>	release maxCC-PreferenceConfig for the MCG, if configured and stop timer T346c associated with the MCG, if running;

2>	release maxMIMO-LayerPreferenceConfig for the MCG, if configured and stop timer T346d associated with the MCG, if running;

2>	release minSchedulingOffsetPreferenceConfig for the MCG, if configured stop timer T346e associated with the MCG, if running;

2>	release rlm-RelaxationReportingConfig for the MCG, if configured and stop timer T346j associated with the MCG, if running;

2>	release bfd-RelaxationReportingConfig for the MCG, if configured and stop timer T346k associated with the MCG, if running;

2>	release releasePreferenceConfig, if configured stop timer T346f, if running;

2>	release onDemandSIB-Request if configured, and stop timer T350, if running;

2>	release referenceTimePreferenceReporting, if configured;

2>	release sl-AssistanceConfigNR, if configured;

2>	release obtainCommonLocation, if configured;

2>	release musim-GapAssistanceConfig, if configured and stop timer T346h, if running;

2>	release musim-GapPriorityAssistanceConfig, if configured;

2>	release musim-LeaveAssistanceConfig, if configured;

2>	release musim-CapabilityRestrictionConfig, if configured and stop timer T346n, if running;

2>	release ul-GapFR2-PreferenceConfig, if configured;

2>	release scg-DeactivationPreferenceConfig, if configured, and stop timer T346i, if running;

2>	release propDelayDiffReportConfig, if configured;

2>	release rrm-MeasRelaxationReportingConfig, if configured;

2>	release maxBW-PreferenceConfigFR2-2, if configured;

2>	release maxMIMO-LayerPreferenceConfigFR2-2, if configured;

2>	release minSchedulingOffsetPreferenceConfigExt, if configured;

2>	release multiRx-PreferenceReportingConfigFR2, if configured, and stop timer T346m, if running;

2>	release aerial-FlightPathAvailabilityConfig, if configured;

2> release ul-TrafficInfoReportingConfig, if configured, and stop all instances of timer T346l, if running;1>	release successHO-Config, if configured;

1>	release successPSCell-Config configured by the PCell, if configured;

1>	if any DAPS bearer is configured:

2>	reset the source MAC and release the source MAC configuration;

2>	for each DAPS bearer:

3>	release the RLC entity or entities as specified in TS 38.322 [4], clause 5.1.3, and the associated logical channel for the source SpCell;

3>	reconfigure the PDCP entity to release DAPS as specified in TS 38.323 [5];

2>	for each SRB:

3>	release the PDCP entity for the source SpCell;

3>	release the RLC entity as specified in TS 38.322 [4], clause 5.1.3, and the associated logical channel for the source SpCell;

2>	release the physical channel configuration for the source SpCell;

2>	discard the keys used in the source SpCell (the KgNB key, the KRRCenc key, the KRRCint key, the KUPint key and the KUPenc key), if any;

1>	release sl-L2RelayUE-Config, if configured;

1>	release sl-L2RemoteUE-Config, if configured;

1>	release the SRAP entity, if configured;

1>	release ncr-FwdConfig, if configured;

# TP2: Full Configuration

#### 5.3.5.11	Full configuration

The UE shall:

1>	release/ clear all current dedicated radio configurations except for the following:

-	the MCG C-RNTI;

-	the AS security configurations associated with the master key;

-	the SRB1/SRB2 configurations and DRB/multicast MRB configurations as configured by radioBearerConfig or radioBearerConfig2.

NOTE 1:	Radio configuration is not just the resource configuration but includes other configurations like MeasConfig. Radio configuration also includes the RLC bearer configurations as configured by RLC-BearerConfig, PC5 Relay RLC channel as configured by SL-RLC-ChannelConfig, and Uu Relay RLC channel as configured by Uu-RelayRLC-ChannelConfig. In case NR-DC or NE-DC is configured, this also includes the entire NR or E-UTRA SCG configuration which are released according to the MR-DC release procedure as specified in 5.3.5.10.

NOTE 1a:	For NR sidelink communication/discovery, the radio configuration includes the sidelink RRC configuration received from the network, but does not include the sidelink RRC reconfiguration and sidelink UE capability received from other UEs via PC5-RRC. In addition, the UE considers the new NR sidelink configurations as full configuration, in case of state transition and change of system information used for NR sidelink communication/discovery.

NOTE 1b:	To establish the RLC bearer of SRB(s) after release due to fullConfig, the network can include the srb-Identity within srb-ToAddModList (i.e. the UE applies RLC default configuration) and/or provide rlc-BearerToAddModList of concerned SRB(s) explicitly.

-	the logged measurement configuration;

-  the successHO-Config;

-  the successPSCell-Config.