3GPP TSG-RAN WG2 #127	Tdoc R2-2407039

Maastricht, Netherlands, August 19th – 23rd, 2024

Agenda Item:	8.5.2

Source:	Ericsson

Title:	Discussion on on-demand SSB for NES

Document for:	Discussion, Decision

# 1	Introduction

In RAN#103, the WID for network energy savings in Rel-19 was revised [1]. The first objective concerns the on-demand SSB SCell operation, as highlighted below.

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum. - Study procedures and signaling method(s) to support on-demand SIB1 for UEs in idle/inactive mode, including: [RAN1/2/3] - Triggering method by uplink wake-up-signal using an existing signal/channel. - Wake-up-signal configuration provisioning to UE - Note: No modification of SSB will be discussed under this objective - Information exchange between gNBs at least for the configuration of wake-up signal, if necessary. - Checkpoint for normative work in RAN#105 - Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Study adaptation of PRACH in spatial domain, e.g. non-uniform PRACH resources per SSB, and specify if found beneficial - This study is to be done in 2Q’2024 only - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown - Specify the corresponding core requirements, for the above features [RAN4].

The list of RAN1 agreements is available in the Appendix.

# 2	Discussion

## This contribution focuses on the first objective highlighted in the above WID. In particular, the contribution addresses the aspects concerning the configuration, signaling, and activation/deactivation of on-demand SSB transmissions.

According to the WID, the UE is in connected mode and configured with CA and, hence, it is configured with a PCell and one or more SCell(s). Note that the PCell is always activated and not dormant. The SCell(s) can be activated and dormant/not dormant, or deactivated.

## 2.1	On-demand SSB configuration and signalling aspects

RAN1, who is the leading WG for this work, has started the discussions in RAN1#116 meeting and continued the discussions in RAN1#116bis and RAN1#117 meetings. The agreements reached by RAN1 can serve as starting point for RAN2. One of the agreements from RAN1#116 meeting concerns a common design for the on-demand SSB operation for all applicable CA scenarios:

Agreement

RAN1 to strive for a common design for on-demand SSB operation considering all applicable CA configurations.

We propose to adopt the same conclusion in RAN2, i.e., to agree to have a common design for on-demand SSB operation for all scenarios.

Have a common design for on-demand SSB operation for all applicable CA configurations from RAN2 standpoint.

Concerning the configuration aspects, the discussion in RAN1#117 meeting concluded with the following agreements:

Agreement

- For a cell supporting on-demand SSB SCell operation, at least the following for on-demand SSB via higher layer RRC signaling is supported.

  - Frequency of the on-demand SSB

  - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst

  - Periodicity of the on-demand SSB

  - FFS: Whether more than one on-demand SSB configurations can be configured for the cell to UE

  - FFS: Whether the RRC is newly introduced or existing RRC is reused

As highlighted in the agreement above, one of the possible solutions is that the network provides the UE with different options for the SSB transmission patterns by an RRC configuration. These options for the SSB transmission patterns could be given outside of the SCell configuration since we think that the SSB patterns can be commonly configured for various SCells. Furthermore, to support additional flexibility, the configurations could be different for FR1 and FR2. In this way, configurations for FR1 and FR2 would be shared among serving cells operating on FR1 and FR2, respectively. It is important to note that while the options for the SSB transmission patterns would be per FR, the activation of a certain SSB transmission pattern could be different for different SCells. This way of configuring on-demand SSB per FR requires fewer bits compared to, for example, configuring on-demand SSB per SCell. Additionally, we think that FR1 and FR2 have different characteristics (e.g., number of potential beams employed in each FR) and as such this scheme allows for more flexibility than, for example, configuring the same on-demand SSB for both FR1 and FR2. For example, as the maximum number of beams is different in FR1 and FR2, the potential number of deployed OnDemand SSB beams would vary. This leads to potential different periodicity and therefore different starting point/offsets for the OnDemand SSBs. Therefore, configuring on-demand SSB per FR can be seen as a solution that requires fewer bits and provides a reasonable level of flexibility.

Define one on-demand SSB configuration per FR which is applicable for all SCells in that FR.

Concerning the signalling aspects, the discussion in RAN1#117 meeting concluded with the following agreements:

Agreement

- For a cell supporting on-demand SSB SCell operation,

  - Support RRC based signaling to indicate on-demand SSB transmission on the cell.

  - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell.

  - FFS: Whether to support DCI based signaling to indicate on-demand SSB transmission on the cell.

    - This DCI signaling does not provide SCell activation/deactivation.

    - If supported, details on DCI including UE-specific or group-common DCI, DCI contents, etc.

  - FFS: Scenarios where the above signalings are applicable

As highlighted in the agreements above, RAN1 agreed to support RRC and MAC CE based signalling, while whether to support DCI based signalling was left as an FFS. The details concerning the signalling aspects are, however, an open question. Furthermore, as captured in the above agreement, when addressing the signalling aspect, it is important to take into consideration the scenarios in which on-demand SSB will be used. In this regard, we recall the agreements concerning the scenarios and cases from RAN1#116 and RAN1#116-bis meetings, respectively.

During RAN1#116 meeting, RAN1 reached the following agreements on scenarios and cases:

Agreement

Regarding the UE assumption on SSB transmission on a cell supporting on-demand SSB SCell operation, the following cases are identified for further study:

- Case #1: No always-on SSB on the cell

- Case #2: Always-on SSB is periodically transmitted on the cell

- FFS: Whether always-on SSB and on-demand SSB are not cell-defining SSB if transmitted.

FFS: Which scenario the above applies for

Agreement

For the following identified scenarios for on-demand SSB SCell operation, focus future RAN1 discussion to down-select (both may be selected) between the two scenarios.

- Scenario #2: SCell is configured to a UE but before the UE receives SCell activation command (e.g., as defined in TS 38.321)

- Scenario #3: After UE receives SCell activation command (e.g., as defined in TS 38.321)

  - This does not preclude SCell for which activation is completed

  - FFS: The case where SCell activation is completed

FFS: Application timing between NW triggering message and on demand SSB transmission

The discussion on the same topic was continued during RAN1#116bis meeting and resulted in the following agreements:

Agreement

For the identified scenarios and cases (as per RAN1#116 agreement), on-demand SSB can be triggered by gNB at least for the following scenarios/cases:

- Scenario #2 and Case #1

- Scenario #2 and Case #2

- Scenario #2A and Case #1

- Scenario #2A and Case #2

- FFS: Scenario #3A and Case #1

- FFS: Scenario #3A and Case #2

- FFS: Scenario #3B and Case #1

- FFS: Scenario #3B and Case #2

- For Case #1, once on-demand SSB is triggered, its transmission is in a periodic manner.

  - Note: This does not imply periodic on-demand SSB is transmitted indefinitely after triggered.

- Notes:

  - Scenario #2A refers to

    - “When UE receives SCell activation command (e.g., as defined in TS 38.321)”

  - Scenario #3A refers to

    - “After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed”

  - Scenario #3B refers to

    - “When SCell activation is completed and SCell is activated” or

    - “After SCell activation is completed and SCell is activated”

  - For discussion purpose under AI 9.5.1, always-on SSB is SSB supported in Rel-18 specifications.

  - Timing for on-demand SSB transmission (e.g. when the triggered SSB starts and ends) will be separately discussed.

Given that RAN1 down-selected the Scenario #2 (i.e., on-demand SSB operation is supported upon the configuration of SCell and before the UE receives SCell activation command) and Scenario #2A (i.e., on-demand SSB operation is supported when the UE receives SCell activation command) and given the agreement that “RAN1 to strive for a common design for on-demand SSB operation considering all applicable CA configurations.”, our understanding is that the common solution should be used for the on-demand SSB activation/deactivation in all scenarios.

In the context of MAC CE, a simple common solution that can support both Scenario#2 and Scenario#2A is to define separate signalling for the activation/deactivation of on-demand SSB, e.g., a new MAC CE. RAN2 can design this new MAC CE such that it is solely used for the activation/deactivation of on-demand SSB or such that it can be used for both SCell and SSB activation/deactivation. Our preference is the joint design according to which a new MAC CE can also be used for both SCell and SSB activation/deactivation, respectively. Since the SSB activation MAC CE anyway needs to include the SCell information, sending two MAC CEs where the other repeats the SCell info and only adds activation flag seems like overhead.  Compared to the design according to which a new MAC CE is solely used for the activation/deactivation of on-demand SSB along with the existing MAC CE for SCell activation/deactivation, the benefit of the joint design is that when NW activates both SCell and the corresponding on-demand SSBs (i.e., Scenario #2A), these are in the same MAC CE and there is no risk that one of the MAC CE(s) is lost, even if this would be a corner case.

A simple solution for MAC CE based signaling that fulfils the objective of having a common design for all cases and scenarios is to define a new MAC CE for on-demand SSB operation.

UEs are informed of on-demand SSB transmissions, via new MAC CE signalling that supports both SCell and on-demand SSB activation/deactivation, respectively.

Since the legacy MAC CE can still be used for SCell activation/deactivation, the new MAC CE needs to support the functionality of activating/deactivating SSB only and joint activation of SCell and respective SSB. This can be achieved by defining the fields for each functionality (e.g., SCell activation/deactivation and SSB activation/deactivation), which can be optionally present.

The new MAC CE supports at least functionality of activating/deactivating SSB and joint activation of SCell and respective SSB.

# 3	Conclusion

In this contribution we discuss the second objective highlighted in the WID. Based on the discussion in the previous sections we made the following observations:

Observation 1	A simple solution for MAC CE based signaling that fulfils the objective of having a common design for all cases and scenarios is to define a new MAC CE for on-demand SSB operation.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Have a common design for on-demand SSB operation for all applicable CA configurations from RAN2 standpoint.

Proposal 2	Define one on-demand SSB configuration per FR which is applicable for all SCells in that FR.

Proposal 3	UEs are informed of on-demand SSB transmissions, via new MAC CE signalling that supports both SCell and on-demand SSB activation/deactivation, respectively.

Proposal 4	The new MAC CE supports at least functionality of activating/deactivating SSB and joint activation of SCell and respective SSB.

# References

RP-240170, “Revised WID: Enhancements of network energy savings for NR”, RAN103, Maastricht, Netherlands, March 2024.

# Appendix

### RAN1#116 Agreements

Agreement

Regarding the UE assumption on SSB transmission on a cell supporting on-demand SSB SCell operation, the following cases are identified for further study:

- Case #1: No always-on SSB on the cell

- Case #2: Always-on SSB is periodically transmitted on the cell

- FFS: Whether always-on SSB and on-demand SSB are not cell-defining SSB if transmitted.

FFS: Which scenario the above applies for

Agreement

RAN1 to strive for a common design for on-demand SSB operation considering all applicable CA configurations.

Agreement

For the following identified scenarios for on-demand SSB SCell operation, focus future RAN1 discussion to down-select (both may be selected) between the two scenarios.

- Scenario #2: SCell is configured to a UE but before the UE receives SCell activation command (e.g., as defined in TS 38.321)

- Scenario #3: After UE receives SCell activation command (e.g., as defined in TS 38.321)

  - This does not preclude SCell for which activation is completed

  - FFS: The case where SCell activation is completed

FFS: Application timing between NW triggering message and on demand SSB transmission

Agreement

Support on-demand SSB SCell operation triggered by gNB.

- FFS Details of associated signaling/indication/configuration provided to UE

Agreement

- For SSB burst(s) triggered by on-demand SSB SCell operation, study at least the following options.

  - Option 1: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A.

  - Option 1A: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A until gNB turns OFF the on demand SSB

  - Option 2: UE expects that on-demand SSB burst(s) is transmitted from time instance A to time instance B and not transmitted after time instance B.

  - Option 3: UE expects that on-demand SSB burst(s) is transmitted N times after time instance A and not transmitted after N on-demand SSB bursts are transmitted.

  - Option 4: UE expects that on-demand SSB burst(s) is transmitted with a periodicity from time instance A to time instance B and with the other periodicity after time instance B.

  - FFS: The combination of above options

  - FFS: How to define time instance A/B and the value of N per option

  - FFS: Each option is applicable to which Cases or Scenarios (as per the previous agreement)

### RAN1#116bis Agreements

Agreement

For the identified scenarios and cases (as per RAN1#116 agreement), on-demand SSB can be triggered by gNB at least for the following scenarios/cases:

- Scenario #2 and Case #1

- Scenario #2 and Case #2

- Scenario #2A and Case #1

- Scenario #2A and Case #2

- FFS: Scenario #3A and Case #1

- FFS: Scenario #3A and Case #2

- FFS: Scenario #3B and Case #1

- FFS: Scenario #3B and Case #2

- For Case #1, once on-demand SSB is triggered, its transmission is in a periodic manner.

  - Note: This does not imply periodic on-demand SSB is transmitted indefinitely after triggered.

- Notes:

  - Scenario #2A refers to

    - “When UE receives SCell activation command (e.g., as defined in TS 38.321)”

  - Scenario #3A refers to

    - “After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed”

  - Scenario #3B refers to

    - “When SCell activation is completed and SCell is activated” or

    - “After SCell activation is completed and SCell is activated”

  - For discussion purpose under AI 9.5.1, always-on SSB is SSB supported in Rel-18 specifications.

  - Timing for on-demand SSB transmission (e.g. when the triggered SSB starts and ends) will be separately discussed.

Agreement

- For a cell supporting on-demand SSB SCell operation,

  - Note: It is up to gNB implementation whether always-on SSB (if transmitted) on the cell is cell-defining SSB or not.

  - For on-demand SSB on the cell, downselect between the following alternatives

    - Alt-1: It is up to gNB implementation whether on-demand SSB is cell-defining SSB or not.

    - Alt-2: On-demand SSB is limited to non-cell-defining SSB.

      - FFS: Further limitations to on-demand SSB

Agreement

- For a cell supporting on-demand SSB SCell operation,

  - L1 and/or L3 measurement based on on-demand SSB is supported for the cell.

    - FFS further details on L1 and/or L3 measurement

Agreement

The following agreement from RAN1#116 is modified (in red)

- For SSB burst(s) triggeredindicated by on-demand SSB SCell operation, study at least the following options.

  - Option 1: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A.

  - Option 1A: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A until gNB turns OFF the on demand SSB

  - Option 2: UE expects that on-demand SSB burst(s) is transmitted from time instance A to time instance B and not transmitted after time instance B.

  - Option 3: UE expects that on-demand SSB burst(s) is transmitted N times after time instance A and not transmitted after N on-demand SSB bursts are transmitted.

  - Option 4: UE expects that on-demand SSB burst(s) is transmitted with a periodicity from time instance A to time instance B and with the other periodicity after time instance B.

  - FFS: The combination of above options

  - FFS: How to define time instance A/B and the value of N per option

  - FFS: Each option is applicable to which Cases or Scenarios (as per the previous agreement)

Agreement

For a cell supporting on-demand SSB SCell operation, further study the following options.

- Option 1: Separate signaling between legacy/existing signaling (e.g., RRC, MAC CE) providing SCell activation/deactivation and signaling providing On-demand SSB transmission indication.

- Option 2: A single signaling in which both SCell activation/deactivation and On-demand SSB transmission indication are provided.

  - FFS: Details of the signaling

- Other options are not precluded.

- FFS: Details on On-demand SSB transmission indication

### RAN1#117 Agreements

Agreement

- For a cell supporting on-demand SSB SCell operation,

  - Support RRC based signaling to indicate on-demand SSB transmission on the cell.

  - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell.

  - FFS: Whether to support DCI based signaling to indicate on-demand SSB transmission on the cell.

    - This DCI signaling does not provide SCell activation/deactivation.

    - If supported, details on DCI including UE-specific or group-common DCI, DCI contents, etc.

  - FFS: Scenarios where the above signalings are applicable

Agreement

- For a cell supporting on-demand SSB SCell operation, at least the following for on-demand SSB via higher layer RRC signaling is supported.

  - Frequency of the on-demand SSB

  - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst

  - Periodicity of the on-demand SSB

  - FFS: Whether more than one on-demand SSB configurations can be configured for the cell to UE

  - FFS: Whether the RRC is newly introduced or existing RRC is reused

Agreement

- At least support L1 measurement based on on-demand SSB

  - For L1 measurement based on on-demand SSB, periodic, semi-persistent, [and aperiodic] L1 measurement reports based on existing CSI framework are supported.

    - FFS on potential enhancements of CSI report configuration and/or triggering/activation mechanisms for L1 measurement based on on-demand SSB

Agreement

For SSB burst(s) indicated by on-demand SSB SCell operation via MAC CE, UE expects that on-demand SSB burst(s) is transmitted from time instance A which is determined as follows.

- Alt 3-1: Time instance A is [the slot boundary of] the first SSB time domain position [of actually transmitted on-demand SSB burst] which is T [slots or symbols] after the [slot or symbol] where UE receives a signalling from gNB to indicate on-demand SSB transmission

  - The SSB time domain positions of on-demand SSB burst are configured by gNB.

- FFS: Details of the value of T (≥ 0) including possibility of T comprising of multiple components

- Note: The value of T is not less than existing timeline required for UE’s MAC CE processing for SCell activation

- FFS: Whether the value of T is predefined or indicated/configured by gNB

- FFS: Details of “the [slot or symbol] where UE receives a signalling from gNB” or “the [slot or symbol] where UE transmits HARQ-ACK corresponding to a signalling from gNB to trigger on-demand SSB”

Above applies at least for the case where SCell with on demand SSB transmission and cell with signalling transmission have the same numerology.

Agreement

- For a cell supporting on-demand SSB SCell operation, at least the followings for on-demand SSB are known to UE.

  - Sub-carrier spacing of the on-demand SSB

  - Physical Cell ID of the on-demand SSB

  - Location of on-demand SSB burst

  - Downlink transmit power of on-demand SSB

  - FFS: Other parameters

  - FFS: Whether each of above parameters is configured/indicated explicitly or not