3GPP TSG-RAN WG2 Meeting #127                                       	R2-2407040

Maastricht, Netherlands, 19 – 23 August 2024

Agenda item:	8.1.3

Source:	Samsung

Title:	Discussion on NW-side data collection

Document for:	Discussion and Decision

# 1. Introduction

In the last RAN2 meeting, some further agreements were made for NW-side data collection. In this contribution, we continue to discuss each aspect of NW-side data collection for NW-side model training.

# 2. Discussion

## 2.1 Discussion on beam management use case

In RAN2#125bis meeting, RAN2 discussed NW-side data collection and agreed the following [1]:

Agreements

1	For the NW-side data collection related to beam management use cases, RAN2 to consider gNB-centric and OAM-centric approaches

2	We aim that the same measurement framework is applied to both gNB-centric data collection and OAM-centric data collection for NW-side data collection.

3	RAN2 supports enhancements to MDT for data collection framework for training.  FSS Whether to enhance logged or immediate MDT

In RAN2#126, the following was additionally agreed:

Agreements for beam management

1. 	For gNB centric and OAM centric (for RRC signaling between UE and gNB), reporting multiple instances of logged L1 measurement result from UE to gNB via a RRC message as configured by gNB is an optional feature.  FFS how to handle case when single RRC message is not sufficient.   FFS if there will be any further enhancement needed pending RAN1 agreement.

2.	Immediate MDT is the baseline framework for OAM-centric data collection for the training of a network-sided model

3.	Enhance the immediate MDT framework to support periodical reporting.   FFS whether and what event-based reporting is supported and FFS on network request reporting

The following issues related to NW-side data collection and related reporting by the UE require further discussion in our view:

- Whether to enhance the immediate MDT further.

- Whether and how to consider enhancements to data collection in different RRC states (including consistent collection across states), in order to support data collection for NW-side model training,.

- Whether and how to support UE logging and reporting of logged data.

In the following sections of this contribution we discuss aspects related to:

- Limitations of existing RRM/MDT mechanisms.

- RAN2 discussion on data collection in RRC states.

- Support of data logging at the UE.

- Need of input from RAN1.

### 2.1.1 Limitations of existing RRM/MDT mechanisms

In RAN2#122 meeting, RAN2 made the assumption that the study on data collection frameworks should focus on data generation and data reporting in RRC_CONNECTED state [9]:

- P6a: RAN2 assumes that the analysis/selection of the data collection frameworks should focus on the RRC_CONNECTED state (for both data generation and reporting). Analysis and potential enhancement on the non-connected state can be revisited when needed. - P6b: LS to RAN1 to confirm the WA (in P6a) on RRC state of data collection.

However, RAN2 did agree (as per above) that IDLE state can be revisited when needed (e.g. when discussing MDT). Related to this, in RAN3#123bis meeting, RAN3 had the same discussion on the Rel-18 leftovers [7] and agreed to support continuous data collection across RRC states [8]:

Continuous MDT collection targeting the same UE across RRC states

Observation 1: RAN3 agreed to support continuous MDT collection across RRC states in Rel-19.

In our understanding of RAN3’s work, the continuous MDT collection of AIML data is only beneficial for training at OAM. For signalling-based MDT, the core network is responsible for setting the configuration and selecting the UEs and can therefore configure the signalling-based MDT for the same UE. Thus, there is no issue for signalling-based MDT when it comes to ensuring continuous MDT – the focus is on management-based MDT, where the configuration comes from the OAM but the UE selection is done at RAN side.

Observation 2: The focus of discussion on continuous MDT collection across RRC states is for the specific case of management-based MDT.

If a UE in RRC connected state is selected by gNB1 to do logged MDT, UE goes to RRC INACTIVE/IDLE state and does the logged MDT based on configuration from gNB1. If the same UE comes back to RRC CONNECTED state with the connection to gNB2, the issue arises of how to enable the gNB2 to select this UE to continue the MDT.

Observation 3: For the scenario of UE switching from RRC CONNECTED state to RRC INACTIVE/IDLE state, and then switching back to RRC CONNECTED state while connecting to a different gNB, it is unclear how this new gNB would enable continuous MDT.

Based on above observations, we propose the following:

Proposal 1: RAN2 to discuss whether any RAN2 work is needed to enable continuous MDT collection (management-based), in line with the related work in RAN3.

### 2.1.2 RAN2 discussion on data collection in RRC states

RAN2 communicated the agreements quoted in the previous section, in addition to other data collection requirements and assumptions, in the LS to RAN1 [10]. For example, Assumption 3 in relation to data collection in RRC_CONNECTED:

Assumption 3: RAN2 assumes that the analysis/selection of the data collection frameworks should focus on the RRC_CONNECTED state (for both data generation and reporting). Analysis and potential enhancement of the non-connected state can be revisited when needed.

In response, in RAN1#114, RAN1 discussed the assumptions in RAN2 LS (i.e. Part A: Assumption 1, 2, 3, 4), and provided updates/confirmation on PART A in the reply LS [11]. For example, on Assumption 3:

RAN1 confirms RAN2’s Assumption 3 for CSI compression, CSI prediction, beam prediction and Positioning use cases. For positioning, it is noted that existing specification supports DL PRS measurement and UE positioning in both RRC_CONNECTED and RRC_INACTIVE state.

Observation 4: RAN1 and RAN2 confirmed during the Study Item phase that the analysis/selection of the data collection frameworks, for different use cases, should focus on data generation and reporting in the RRC_CONNECTED state.

In RAN2#123 meeting, RAN2 re-discussed the aspect of data collection in RRC_CONNECTED state, based on [4], and captured the following in RAN2#123 Chair notes [12], however not yet agreed:

Chair: The proposals below are almost agreeable. It is a narrowing proposal (more specific than the physical entity mapping agreed) and is a reasonable baseline for further work: Proposal 5 If feasibility of OAM-centric data collection for NW-side models is assessed by RAN1, RAN2 considers enhancements to logged MDT, such as logging measurements in RRC Connected mode.

Based on above, we propose the following:

Proposal 2. RAN2 to discuss whether consistent RRM measurements across RRC states is needed to improve model-training quality for the general use-case of data collection related to AIML-model training.

Regarding the outstanding FFSs from RAN2#126:

Agreements for beam management

1. 	For gNB centric and OAM centric (for RRC signaling between UE and gNB), reporting multiple instances of logged L1 measurement result from UE to gNB via a RRC message as configured by gNB is an optional feature.  FFS how to handle case when single RRC message is not sufficient.   FFS if there will be any further enhancement needed pending RAN1 agreement.

2.	Immediate MDT is the baseline framework for OAM-centric data collection for the training of a network-sided model

3.	Enhance the immediate MDT framework to support periodical reporting.   FFS whether and what event-based reporting is supported and FFS on network request reporting

In our view, whether reporting multiple instances of logged L1 measurement would require the use of multiple RRC messages or segmentation of a single RRC message, this could be up to the network in the CONNECTED as it can control the volume of the reported measurements (e.g. measurement period, number of reported values). Nonetheless, RAN2 would need to further discuss the handling in the case RRM measurement enhancements in IDLE/INACTIVE are pursued:

Proposal 3a. RAN2 will not pursue RRC segmentation for logged L1 measurement in RRC CONNECTED state.

Proposal 3b. RAN2 to discuss RRC segmentation for logged L1 measurement if measurement for training data collection is introduced for RRC IDLE/INACTIVE state.

Moreover, regarding the FFS related to “event-based reporting”, in our understanding, given that NW-sided data collection is for model training, we do not see any scenario nor motivation for event-based reporting, as to train the model would require the use of non-biased data. That is, periodically reported data.

Proposal 3c. RAN2 will not pursue event-based reporting.

### 2.1.4 Issues related to supporting data logging at the UE

According to TR 38.843, the following aspects on data logging were captured [14]:

- UE to support data logging.

However, the logging and reporting of UE measurements of AI/ML data will significantly increase the usage of UE memory, processing power and energy consumption. Especially so, if the UE needs to continuously store collected data across RRC states.

Consequently, the UE may fail to complete the data collection procedure (e.g. including logging and reporting of data), if the UE runs out of memory, processing power and/or energy. RAN2 has not yet discuss this problem.

Proposal 4a: RAN2 to discuss the UE behaviour in the case of UE failure to log collected data due to the UE running out of memory.

In our view, the gNB should configure the UE behaviour for data collection (e.g. including logging and reporting) taking into consideration the UE memory status, which the UE reports, e.g. periodically, on-demand or as event-triggered (memory shortage). Otherwise, the gNB may configure the UE data collection behaviour, e.g. data collection size, latency, and/or other data collection requirements according to the use case (and/or LCM purpose), then it is up to the UE how to handle the data collection process according to it is available memory. That is, the UE may assess its available memory to determine for whether it is able to perform data collection (i.e. logging and reporting) according to the configured data collection requirements. For example, the UE may decide that it is not able to complete the data collection process (according to the data collection requirements) due to lack of memory, so the UE may stop data logging and/or delete any logged data and indicate this failure to the gNB. Moreover, the gNB may configure the reporting periodicity of the collected (or stored) data, i.e. whether the UE report this data periodically or on-demand.

Observation 5: Network need to receive UE assistance information in order to configure data collection procedure properly.

Proposal 4b: RAN2 to discuss how the UE reports its data logging capability/status to proceed with data collection procedure.

### 2.1.4 Need of input from RAN1

For BM Case1 and Case2, due to the usage of different AI models, the contents of training data (for labelling purposes) are very different. We believe it would help us move the discussion along (and help RAN2 to design related signalling) in both RAN1 and RAN2, if RAN1 could provide some specific information to RAN2 on content of collected training data.

In line with above, we propose the following:

Proposal 5. RAN2 to send an LS to RAN1 to ask for input on the contents for data collection for training, before designing the related signalling.

## 2.2 Discussion on positioning use case

According to WID [17], there are five positioning sub use cases, which can be summarized as below.

| Case | Positioning type | Model entity | AI/ML for Positioning type |
| --- | --- | --- | --- |
| 1 (1st priority) | UE-based positioning | UE-side model | Direct |
| 2a (2nd priority) | UE-assisted/LMF-based positioning | UE-side model | assisted |
| 2b (2nd priority) | UE-assisted/LMF-based positioning | LMF-side model | Direct |
| 3a (1st priority) | NG-RAN node assisted positioning | gNB-side model | assisted |
| 3b (1st priority) | NG-RAN node assisted positioning | LMF-side model | Direct |

Among the five cases, the case 2b/3a/3b are relevant to NW-side model. Thus, in the following, we discuss the data collection for NW-side model training for the case 2b, 3a, 3b one by one.

### 2.2.1 Case 2b (Direct AI/ML positioning with LMF-side model)

The case 2b is for direct AI/ML positioning with LMF-side model. For model training, the LMF is the termination point for training data as captured in the TR [14] as below.

For LMF-side model, the LMF is the termination point for training data.

Proposal 6: RAN2 confirm that the termination entity of model training data is the LMF for the case 2b.

For the case 2b, the positioning type is “Direct AI/ML positioning”. The label data for model training can consist of multiple pairs of input and output data. According to the TR, the following information can be assumed for the input/output data.

- Input data: CIR, PDP, DP measurement results

- Output data:  Known UE location

Since the output data for the label is known UE location, the source of training data can be the UE with known location like PRU.

Proposal 7: RAN2 confirm that the generation entity of model training data is the UE with known location for the case 2b.

For the case 2b, the generated label data needs to be delivered from the UE to the LMF. In our view, the legacy LPP signalling (i.e., LPP ProvideLocationInformation message) can be used for this. Note that, with the existing LPP, the LMF can configure PRU to report both measurement results and location estimate (i.e., known location) already. Thus, we can reuse the existing LPP message with minor enhancement (e.g., adding AI/ML specific measurement results like CIR, PDP, DP, etc.).

Proposal 8: The existing LPP ProvideLocationInformation message can be reused to deliver the training data from the UE with known location to the LMF for the case 2b.

Proposal 9: RAN2 can discuss potential enhancement on LPP (e.g., adding AI/ML specific measurement results to LPP ProvideLocationInformation message) based on the input from RAN1 later.

### 2.2.2 Case 3a (AI/ML assisted positioning with gNB-side model)

The case 3a is for AI/ML assisted positioning with gNB-side model. For model training, gNB or LMF or OAM can be the termination point for training data as captured in the TR [14] as below.

For gNB-side model, training data can be generated by the gNB, while the termination point for training data may include the gNB, or OAM.  Note: RAN2 identified the case in which LMF may be used for gNB-side model training. However, no conclusion was reached, as this depends on the RAN1 progress.

For the case 3a, the positioning type is “AI/ML assisted positioning”. The label data for model training can consist of multiple pairs of input and output data. According to the TR, the following information can be assumed for the input/output data.

- Input data: CIR, PDP, DP measurement results

- Output data:  Intermediate results (e.g., ToA, AoA, LOS/NLOS indicator, etc.)

Since the positioning mode is NG-RAN node assisted for the case 3a, the input/output data can come from the gNB.

Proposal 10: RAN2 confirm that the generation entity of model training data is the gNB for the case 3a.

The generated training data needs to be delivered from the gNB to the gNB/LMF/OAM. Before discussing the detailed signalling method to support the data collection, RAN2 need to discuss whether to support the training data delivery to each entity (i.e., gNB, LMF, OAM). For the gNB case, we can support it without any spec. impact and thus don’t need to exclude this option. However, considering the computation capability of gNB, the model training at gNB may be impractical. In that sense, we think at least one of LMF-side or OAM-side training can be supported for the case 3a in addition to gNB-side training.

Proposal 11: RAN2 confirm that the gNB can be the termination point of training data for case 3a without spec impact.

Proposal 12: RAN2 discuss whether OAM and/or LMF can be the termination point of training data for case 3a.

### 2.2.3 Case 3b (Direct AI/ML positioning with LMF-side model)

The case 3b is for direct AI/ML positioning with LMF-side model. For model training, the LMF is the termination point for training data as captured in the TR [14] as below.

For LMF-side model, the LMF is the termination point for training data.

Proposal 13: RAN2 confirm that the termination entity of model training data is the LMF for the case 3b.

For the case 3b, the positioning type is “Direct AI/ML positioning”. The label data for model training can consist of multiple pairs of input and output data. According to the TR, the following information can be assumed for the input/output data.

- Input data: CIR, PDP, DP measurement results

- Output data:  Known UE location

For the input data, the measurement results can be delivered from the gNB to the LMF since the positioning mode is NG-RAN node assisted for the case 3b. However, the gNB cannot provide the output data (i.e., known UE location) to the LMF. Instead, the LMF can associate the measurement results with known UE location to generate the label data. More specifically, the LMF can configure the UE with known location (e.g., PRU) to transmit positioning SRS and request the gNB/TRP to measure the SRS and report the measurement results. Then, the LMF can generate the label data by associating the measurement results from the gNB with the known location of the UE. The LMF with known UE location needs only the measurement results from the gNB for training data collection in the case 3b.

Proposal 14: RAN2 confirm that the generation entity of model training data is the gNB for the case 3b.

For the case 3b, it is obvious that NRPPa can be used for the training data collection from the gNB to the LMF. However, the detailed signalling design should be up to RAN3.

Proposal 15: RAN2 assume that NRPPa is used for training data collection from the gNB to the LMF for the case 3b and the detailed signalling is up to RAN3.

# 4. Conclusion

In this contribution, we discussed existing MDT/RRM limitations and potential enhancements for MDT/RRM mechanisms to support data collection for AI/ML. Additionally, the potential problem of increased usage of UE memory, processing power and energy consumption due to logging of collected data at the UE across different RRC states was discussed. We additionally covered the positioning use case. The following are the proposals we would like RAN2 to discuss and agree:

Proposal 1: RAN2 to discuss whether any RAN2 work is needed to enable continuous MDT collection (management-based), in line with the related work in RAN3.

Proposal 2. RAN2 to discuss whether consistent RRM measurements across RRC states is needed to improve model-training quality for the general use-case of data collection related to AIML-model training.

Proposal 3a. RAN2 will not pursue RRC segmentation for logged L1 measurement in RRC CONNECTED state.

Proposal 3b. RAN2 to discuss RRC segmentation for logged L1 measurement if measurement for training data collection is introduced for RRC IDLE/INACTIVE state.

Proposal 3c. RAN2 will not pursue event-based reporting.

Proposal 4a: RAN2 to discuss the UE behaviour in the case of UE failure to log collected data due to the UE running out of memory.

Proposal 4b: RAN2 to discuss how the UE reports its data logging capability/status to proceed with data collection procedure.

Proposal 5. RAN2 to send an LS to RAN1 to ask for input on the contents for data collection for training, before designing the related signalling.

Proposal 6: RAN2 confirm that the termination entity of model training data is the LMF for the case 2b.

Proposal 7: RAN2 confirm that the generation entity of model training data is the UE with known location for the case 2b.

Proposal 8: The existing LPP ProvideLocationInformation message can be reused to deliver the training data from the UE with known location to the LMF for the case 2b.

Proposal 9: RAN2 can discuss potential enhancement on LPP (e.g., adding AI/ML specific measurement results to LPP ProvideLocationInformation message) based on the input from RAN1 later.

Proposal 10: RAN2 confirm that the generation entity of model training data is the gNB for the case 3a.

Proposal 11: RAN2 confirm that the gNB can be the termination point of training data for case 3a without spec impact.

Proposal 12: RAN2 discuss whether OAM and/or LMF can be the termination point of training data for case 3a.

Proposal 13: RAN2 confirm that the termination entity of model training data is the LMF for the case 3b.

Proposal 14: RAN2 confirm that the generation entity of model training data is the gNB for the case 3b.

Proposal 15: RAN2 assume that NRPPa is used for training data collection from the gNB to the LMF for the case 3b and the detailed signalling is up to RAN3.

# 5. Reference

- RAN2#125 Chair Notes

- 3GPP TR 38.913 Study on scenarios and requirements for next generation access technologies

- RAN3#119bis Chair Notes

- RAN3#120 Chair Notes

- RAN2#121bis-e Chair Notes

- RAN2#123 Chair Notes

- RP-240323 Revised SID on Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN (Rel-19 WI)

- RAN3#123bis Chair Notes

- RAN2#122 Chair Notes

- R2-2306906, LS out on Data Collection Requirements and Assumptions (LS to RAN1)

- R2-2309435, Reply LS on Data Collection Requirements and Assumptions (LS to RAN2)

- RAN2#123 Chair Notes

- TS 38.304 NR; User Equipment (UE) procedures in Idle mode and RRC Inactive state

- TR 38.843 Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Rel-18)

- 3GPP TS 38.331 NR; Radio Resource Control (RRC) protocol specification

- TS 38.133 NR; Requirements for support of radio resource management

- RP-240774, ‘Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface’, Qualcomm