3GPP TSG-RAN WG2 #127	Tdoc R2-2407041

Maastricht, Netherlands, August 19th – 23rd, 2024

Agenda Item:	8.5.3

Source:	Ericsson

Title:	Discussion on on-demand SIB1 for NES

Document for:	Discussion, Decision

# 1	Introduction

In RAN#103, the WID for network energy savings in Rel-19 was revised [1], with the objective to study on-demand SIB1 for idle/inactive UEs as highlighted below:

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum. - Study procedures and signaling method(s) to support on-demand SIB1 for UEs in idle/inactive mode, including: [RAN1/2/3] - Triggering method by uplink wake-up-signal using an existing signal/channel. - Wake-up-signal configuration provisioning to UE - Note: No modification of SSB will be discussed under this objective - Information exchange between gNBs at least for the configuration of wake-up signal, if necessary. - Checkpoint for normative work in RAN#105 - Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Study adaptation of PRACH in spatial domain, e.g. non-uniform PRACH resources per SSB, and specify if found beneficial - This study is to be done in 2Q’2024 only - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown Specify the corresponding core requirements, for the above features [RAN4].

In this paper, we discuss the remaining issues raised in RAN WG2 Meeting #126 [2]. In the Appendix we also provide the list of RAN1 agreements.

# 2	UL WUS configuration and UE behaviour

During RAN WG2 Meeting #125bis [3], RAN2 agreed to focus on “Scenario 1a: Cell A SIB assisted intra-cell WUS” according to which on-demand SIB1 for NES cell is supported through following steps:

- The UL WUS configuration for NES Cell is provided to the UE in a SIB of Cell A,

- UE transmits UL WUS to the NES Cell to request on-demand SIB1, and

- NES Cell transmits on-demand SIB1 in response to the UL WUS from the UE.

Furthermore, the following agreements concerning the scenarios for on-demand SIB1 were made during RAN WG2 Meeting #126 [2]:

Further clarification

- RRC release message assisted intra-cell WUS can be discussed as option of signaling details in stage 3.

Scenario 3 (case 1 in RAN1)

- RAN2 to wait for RAN1’s progress whether to support scenario 3.

The above agreements imply that RAN2 can focus on finalizing the work for “Scenario 1a: Cell A SIB assisted intra-cell WUS” and pause the discussions concerning other scenarios. Taking this into consideration, in this section we address the following aspects:

- Provisioning of the UL WUS configuration and associated updates,

- UL WUS configuration in the case of multiple NES Cells, and

- RA procedure for on-demand SIB1 acquisition.

## 2.1		Provisioning of the UL WUS configuration and its updates

One of the aspects that was addressed during RAN WG2 Meeting #126 [2] is how to convey the information concerning the UL WUS configuration in Scenario 1a. The discussion on this topic concluded with the following relevant agreements:

Information of WUS configuration

- For Message 1 based on-demand SIB1 request, the on-demand SI request configuration that currently included in SIB1 may be used as the design baseline.

Transmission of WUS configuration

- Cell A’s SIB can be used to configure on-demand SIB1 related configuration for neighbour NES cells, e.g., via new SIB or the existing SIB.

As highlighted in the agreement on “transmission of WUS configuration” a pending question that needs to be addressed is whether Cell A provides the UL WUS configuration for NES Cell via an existing SIBx or a new SIB. One of the considered options is to include the UL WUS configuration for NES Cell into SIB1 or some other existing SIBx of Cell A. In our opinion, using SIB1 of Cell A to provide the information concerning the UL WUS configuration of NES Cell is not the best choice for the following reasons. SIB1 carries the essential information concerning the cell in which it is transmitted, and the UL WUS configuration of NES Cell cannot be considered essential information of Cell A. There are also practical concerns with extending SIB1 with non-essential contents as SIB1 already has a considerable size, especially in multi-PLMN deployments where many information elements are provided per PLMN. It would therefore be unwise to further extend SIB1 message and compromise SIB1 coverage unless it is absolutely necessary. Finally, reusing SIB1 or an existing SIBx of Cell A may impose unnecessary constrains on the size of the UL WUS configuration, which may be especially problematic in cases where the same Cell A assists multiple NES Cells in OD-SIB1 provisioning, i.e., when the same Cell A needs to provide UL WUS configurations for multiple NES Cells. Therefore, for the above-mentioned reasons, we prefer to define a new SIB for Cell A for the purpose of providing the UL WUS configuration(s) of the NES Cell(s).

SIB1 carries the essential information (of Cell A), and the UL WUS configuration of NES Cell cannot be considered essential information of Cell A.

SIB1 has already a considerable size, especially in multi-PLMN deployments. Extending SIB1 message has impact on coverage.

One Cell A may assist multiple NES cells, including multiple WUS configurations. Extending an existing SIBx may be unnecessarily limiting on number of UL WUS configuration that can be broadcast.

UL WUS configuration for the NES Cell is provided in a new SIB of Cell A.

Another aspect that was discussed during WG2 Meeting #126 [2] concerns the updates of the UL WUS configuration and SIB1, respectively. While there was no agreement concerning the UL WUS configuration updates, the following agreements were made concerning the on-demand SIB1 updates:

SIB1 acquisition upon SIB change notification in NES cell

- Once the NES UE camps on the NES cell, if the UE receives SIB change notification, the UE is expected to receive SIB1 from NES cell.

Paging and SIB1 update in NES cell

- After Rel-19 NES UEs camp in NES cell, the UE behaviour is same as the one defined as legacy normal camped state, e.g. paging reception, SIB1 update, etc.

The pending question is from which cell a Rel-19 NES UE should expect to receive the updates of the UL WUS configuration and on-demand SIB1 while the UE is in the NES Cell (i.e., after the UE camps in NES Cell). In what follows we discuss the pros and cons of the solutions according to which Cell A and NES Cell provide the UL WUS configuration updates, respectively. On one hand, if Cell A provides the UL WUS configuration update, then the UE would need to reselect Cell A to obtain the updates. Given that Cell A and NES Cell will likely be on different carriers, the UE would have to switch frequencies to receive the UL WUS configuration updates, which is not favourable from the perspective of the UE. On the other hand, providing the UL WUS configuration updates in the NES cell (e.g., via legacy SI modification procedures) would not have a significant impact on the energy consumption in the NES Cell since the UL WUS configuration is expected to be rather static without the need for frequent updates. Therefore, in our opinion, it is reasonable to follow the same logic as in the case of SIB1 updates, and thus to support the solution according to which the NES Cell provides UL WUS configuration updates once the NES UE camps on the NES Cell.

If the UE were to receive the updates for UL WUS configuration in Cell A it would have to switch frequencies assuming that Cell A and NES Cell are on different carriers.

Once Rel-19 NES UE camps on the NES cell, the UE expects to receive UL WUS configuration updates from the NES Cell, e.g., via legacy SI modification procedures.

## 2.2		UL WUS configuration in case of multiple NES Cells

The versions of Scenario 1a with multiple Cells A and NES Cells were discussed during the WG2 Meeting #126 [2], and the following agreements were made:

Further clarification

- Study on-demand SIB1 provisioning for NES Cell(s) in versions of Scenario 1a with multiple Cells A and/or NES Cells:

- More than one Cell A may provide configuration for the same NES cell.

- The same Cell A may assist more than one NES Cells.

Information of WUS configuration

- Can use the PCI and frequency of a NES Cell to associate the UL WUS configuration with NES Cell.

As highlighted in the agreement on “further clarification”, the same Cell A may assist multiple NES Cells. One of the remaining questions that needs to be addressed is whether the same UL WUS configuration can be used for multiple NES Cells. The benefit of using the same UL WUS configuration for multiple NES Cells is that it allows for minimizing the size of the message that carries the UL WUS configuration(s). However, using different UL WUS configurations for different NES Cells may be needed if Cell A assists multiple NES Cells that are in different bands (e.g., NES Cells in FR1 and FR2) or have different deployment characteristics such as different cell sizes. The benefit of using different UL WUS configurations is that it allows for the flexibility to provide different UL WUS configuration parameters for different NES Cells. Furthermore, in order to avoid waking up multiple NES cells by the same WUS configuration/transmission, distinguished WUS configurations for various NES cell are desired. Therefore, in order to achieve the balance between the message size, unnecessary invocations in multi-NES-Cell deployments, and configuration flexibility, the NW should be able to decide whether to use the same or different UL WUS configurations for multiple NES Cells.

Using the same UL WUS configuration for multiple NES Cells allows for minimizing the size of the message that carries the UL WUS configuration(s).

Using different UL WUS configurations for different NES Cells avoids unnecessary/false NES Cell wakeups, allows for the flexibility to provide different UL WUS configuration parameters for different NES Cells when needed (e.g., if Cell A assists both NES cells in FR1 and FR2, different cell sizes, etc.).

It should be possible for the network to indicate one UL WUS configuration for multiple NES cells and/or separate UL WUS configurations.

## 2.3		On-demand SIB1 acquisition procedure

During RAN WG2 Meeting #125bis [3], RAN2 agreed that the existing MSG1 procedure is reused for on-demand SIB1 acquisition:

Agreements on on-demand SIB1:

- Existing Msg 1 based on-demand procedure is reused for on-demand SIB1 acquisition procedure. FFS on Msg 3. FFS if / when the UE monitors the OD-SIB1 upon reception of RAR. FFS: whether introduce specified UE behavior if RACH failure of OD-SIB1 request.

In what follows, we discuss the remaining questions related to the on-demand SIB1 acquisition procedure and UE behavior.

One of the remaining questions is whether there is a need for a MSG3-based on-demand SIB1 acquisition procedure on top of the already agreed MSG1-based on-demand SIB1 acquisition procedure. To answer this question, it is beneficial to recall the motivation behind the four-step-based RA and preamble reservation in the case of legacy on-demand SI and to discuss differences between the legacy on-demand SI and on-demand SIB1.

In the case of legacy on-demand SI, the four-step-based RA procedure can be useful when it is desirable to avoid preamble reservation. Furthermore, in the case of legacy on-demand SI, the four-step-based RA procedure is reasonable for the following two reasons. First, because there may be multiple SIs that can be requested on-demand, and hence more information may need to be conveyed to the NW (e.g., in MSG3) about which of the SIs the UE is interested in. Second, because the UE already knows how to contact the NW as the RACH configuration is included in the always-present SIB1. The reasoning is quite different in the case of on-demand SIB1 as we will discuss in the following. Since SIB1 is part of the minimum system information, it is reasonable to reserve a single specific preamble and PRACH resources for its request, rather than rely on a random preamble and contention resolution at a later stage of the RA process (e.g., after receiving MSG3). Observe that even if several UEs request on-demand SIB1 simultaneously, the solution does not suffer from the preamble collision problems since all the requests are intended for the same functionality, and the same response message would be delivered to all the requesting UEs. Furthermore, in the case of on-demand SIB1, the RA procedure would be used for a single SIB, and hence differently from the case of legacy on-demand SI, there would be no need for using MSG3 to convey additional information to the NW. Finally, regardless of the two-/four-step procedure, the UE would need to be configured for how to initiate the first message (RACH configuration). Introducing a four-step-based RA procedure for on-demand SIB1 would partly require the configuration of a preamble and RACH configuration for sending the first message, and hence further communication for asking for the SIB1 in successive messages. Consequently, compared to the two-step-based RA procedure, the four-step-based RA procedure where MSG3 is used for the on-demand SIB1 acquisition leads to increased latency. For the above reasons, we see no reason to use the four-step-based RA procedure and MSG3 for on-demand SIB1.

The four-step RA procedure is beneficial for the purpose of legacy on-demand SI because there may be multiple SIs that can be requested on-demand. Information on which SIs the UE is interested in can be conveyed via MSG3 instead of reserving multiple preambles.

In case of on-demand SIB1, a single preamble is enough for requesting SIB1, i.e., there is no need to use MSG3 to convey additional information to the NW.

In case of legacy on-demand SI four-step RA procedure the UE already knows from RACH configuration in SIB1 how to initiate access to the NW. In case of on-demand SIB1, the UE needs a preamble and how to transmit that preamble, i.e., using which time/frequency resources, irrespective of two-/four-step RA procedure.

Compared to the two-step-based RA procedure, the four-step-based RA procedure where MSG3 is used for the on-demand SIB1 acquisition procedure leads to increased latency, and there is no obvious reason to introduce the four-step-based RA procedure for on-demand SIB1.

MSG3 is not used for the on-demand SIB1 request procedure.

The discussion on the MSG1- based on-demand SIB1 acquisition procedure and UE behaviour continued in RAN WG2 Meeting #126 [2] and it was concluded with the following agreement:

RAR monitoring for MSG1 based OD-SIB1 REQ

- RAN2 assumes the UE is expected to receive the RAR responding to the preamble transmission for Msg1-based on-demand SIB1 procedure, as the baseline.

UE behaviour on RACH failure of OD-SIB1 REQ

- As baseline, upon random access procedure failure of OD-SIB1 request, UE regards OD-SIB1 can’t be acquired in the NES cell and considers it as barred. It doesn’t exclude the option to leave the determination to the UE implementation.

According to the current TS 38.331 [4], the following is specified:

“SIB1 includes information regarding the availability and scheduling (e.g. mapping of SIBs to SI message, periodicity, SI-window size) of other SIBs with an indication whether one or more SIBs are only provided on-demand and, in that case, the configuration needed by the UE to perform the SI request…. Each SI message is associated with an SI-window and the SI-windows of different SI messages do not overlap. That is, within one SI-window only the corresponding SI message is transmitted.”

The above specification text states that SIB1 carries the information regarding the availability and scheduling of the other SIBs. However, similar information is not available for SIB1, and given that SIB1 carries the essential information, there is a need for mechanisms that would allow the NW to convey the information to the UE regarding the requested on-demand SIB1 transmissions.

The existing mechanisms cannot be directly used for defining the UE behaviour upon transmitting MSG1 that carries the request for on-demand SIB1.

According to the above-highlighted agreement on “RAR monitoring for MSG1 based OD-SIB1 REQ”, the assumption is that as the baseline the UE is expected to receive a RAR message as a response to the UE’s on-demand SIB1 request contained in MSG1. It is important to note that a NES Cell can send the same RAR message to all UEs that requested on-demand SIB1 since all requests are intended for the same functionality. Consequently, sending a RAR message as a response to on-demand SIB1 request(s) does not cause excessive signalling nor significant energy consumption in the NES Cell.

NES Cell can send the same RAR message to all UEs that requested on-demand SIB1 since all requests are intended for the same functionality, and hence sending a RAR message as a response to on-demand SIB1 request(s) does not cause excessive signalling nor significant energy consumption in the NES Cell.

Furthermore, similarly as the UE can use MSG1 to request on-demand SIB1, the NES Cell can use RAR as a feedback message to instruct the UE how to behave upon its reception. For example, the RAR can indicate the information when the UE can expect on-demand SIB1 transmissions relative to the RAR transmission (e.g., an offset value to on-demand SIB1 transmission and/or a time window during which the UE can expect on-demand SIB1 transmission can be introduced for this purpose), whether on-demand SIB1 is transmitted in all directions or only in some SSBs, and the information concerning the UE behaviour if the UE does not receive on-demand SIB1 according to the timeline specified in RAR message (e.g., whether the UE should attempt to repeat the request for on-demand SIB1 and if yes for how long the UE should wait before repeating the request). The RAR can contain the explicit parameter values or a pointer to the values preconfigured in the UL WUS configuration.

RAR message is used for indicating the UE behaviour concerning when to expect on-demand SIB1 transmission, in which directions the transmission of on-demand SIB1 can be expected, and how to proceed if on-demand SIB1 is not received when expected.

# 4	Cell barring, reselection, and camping aspects

In this section we address the aspects related to:

- Barring of legacy UEs,

- Barring and camping of Rel-19 NES UEs, and

- Cell reselection criteria for Cell A and NES Cell.

## 4.1		Barring aspects for legacy and Rel-19 NES UEs

We start with discussing the remaining questions concerning the barring of legacy UEs. During RAN WG2 Meeting #126 [2], RAN2 reached the following agreement on legacy UE impact:

Legacy UE impact

- NW need to bar the legacy UE from accessing the on-demand SIB1 cell (e.g. based on the existing barring mechanism)

The above agreement on “Legacy UE impact” states that there is a need to bar legacy UEs from accessing a NES Cell with on-demand SIB1. The exact solution for barring is, however, still an open question. In our understanding, two candidate solutions for barring are being considered for a NES Cell that transmits on-demand SIB1. The first solution is baring with an explicit flag (e.g., similar to the existing barring mechanism used for the Rel-18 Cell DTX/DRX feature). The second solution, which is being discussed in RAN1, is an implicit barring of legacy UEs based on the lack of SIB1 configuration in PBCH. While waiting for input from RAN1 regarding the implicit barring, RAN2 can also discuss the pros and cons of the two barring options. Note that regardless of the discussion above, the cell is implicitly barred for a Rel-19 UE supporting NES Cell if no WUS configuration can be found for the NES Cell. In the end, question is when does UE decide a cell is barred, directly from explicit indication, or after checking for WUS configuration.

Discuss explicit and implicit barring mechanisms for a NES Cell that transmits on-demand SIB1.

Regardless of legacy UE barring options, a Rel-19 UE supporting OD-SIB1 is implicitly barred from a NES cell for which no WUS configuration is found.

We continue with discussing the question related to barring relaxation for Rel-19 NES UEs in case the UE is unable to acquire SIB1. During RAN WG2 Meeting #126 [2], RAN2 reached the following agreements concerning the barring of Rel-19 NES UEs:

Barring for R19 NES UE

- If the NES UE is unable to acquire the SIB1 of NES Cell before UE initiates OD-SIB1 procedure, it will not consider it as barred at that moment. R19 NES UE bars the cell if the UE fails to acquire SIB1 via on-demand SIB1 for NES cell.

The above agreement on “barring for R19 NES UE” implies that the legacy behavior according to which UE considers the cell as barred if it is unable to acquire the SIB1 should be relaxed for Rel-19 NES UEs supportive of the on-demand SIB1 feature. However, an open question related to the above agreement is how long a UE should wait before initiating on-demand SIB1 procedure, i.e., once it identifies a NES Cell and obtains the corresponding UL WUS configuration. It is important to observe that even in a NES Cell with on-demand SIB1, a UE may still be able to acquire SIB1 without requesting it, i.e., if the NES cell is transmitting on-demand SIB1 as a response to another UE’s request, or if the gNB decides to revert the NES cell to its legacy mode of transmitting SIB1 periodically. Acquiring SIB1 without initiating on-demand SIB1 procedure implies fewer UL WUS transmissions, and hence the potential for further minimization of the energy consumption on both, the NW, and the UE side. One way of supporting SIB1 acquisition without an explicit on-demand SIB1 request is to instruct the UE (e.g., in the UL WUS configuration) to wait for a certain time before requesting on-demand SIB1, and/or to cancel the UL WUS process in case it finds SIB1 in the meantime. To avoid prolonged waiting, and hence a negative impact in terms of latency at the UE, the UE could be instructed, for example, to wait with on-demand SIB1 request for the periodicity at which the cell would have been transmitting SIB1 if it was operating in a legacy mode. In this case, the impact on the UE in terms of latency would not be larger than that of legacy periodic SIB1 transmissions. Furthermore, the UE could while waiting for the upcoming WUS transmission occasion, decode potential OD-SIB1 RAR occasions in case another UE has already requested OD-SIB1 in the previous WUS occasion.

Acquiring SIB1 without initiating on-demand SIB1 procedure implies fewer UL WUS transmissions, and hence the potential for further minimization of the energy consumption on both, the NW, and the UE side.

Discuss how to support SIB1 acquisition without transmitting UL WUS for NES Cell with on-demand SIB1, e.g., before WUS transmission the UE attempts to decode SIB1 and/or OD-SIB1 RAR at given occasions.

## 4.2		Cell reselection and camping aspects

Cell reselection and camping aspects were discussed during RAN WG2 Meeting #126 [2], and the following related agreements were reached:

Triggering condition of WUS transmission

- If the UE chooses the NES cell using legacy intra-F/inter-F cell re-selection procedure (as baseline), the UE triggers WUS transmission.

- After UE successfully receives OD-SIB1 for that NES Cell and if it is a suitable cell, UE camps in the NES Cell “similar” to a legacy Cell.

UE request SIB1 to perform initial access in NES cell

- RAN2 not to support on-demand SIB1 request that is combined with an initial access to perform RRC connection establishment/resume on the NES cell.

Legacy UE impact

- How to avoid/deprioritize the legacy UE camping at the Cell A attempting to switch to the NES Cell (but allowing the R19 NES UE to do that).

The above agreements on “triggering condition of WUS transmission” imply that the UE can trigger the request for on-demand SIB1 upon intending to camp on NES cell, i.e., before cell (re-)selection to NES cell. However, as highlighted in the agreement on the “legacy UE impact”, one of the pending questions from the previous meeting concerns cell reselection priorities for the legacy and Rel-19 UEs capable of on-demand SIB1 feature. In what follows we discuss the question of NES Cell prioritization/deprioritization.

In certain scenarios it could be beneficial to prioritize or deprioritize the reselection of NES Cell and Cell A. From the perspective of the UEs, the benefit is in preventing unnecessary energy consumption and delays, which occur while the UE is applying barring procedures for the NES Cell and searching for another suitable cell. Similarly, at the NW side, it could be beneficial in terms of energy consumption to deprioritize the reselection of the NES Cell if a NES Cell experiences an excessive number of wakeups without or with very few UEs accessing the NES cell. On the contrary, if for example, the NW have had intended the use of NES Cell, but configuration is done such a way that only a few UEs are connected or paged for transmission in the DL, it could be beneficial to incentivize the reselection of the NES Cell.

Reusing the existing reselection parameters for incentivizing/disincentivizing the reselection of a NES Cell is not suitable for the following reason. The existing reselection parameters are broadcasted for the cell reselection, and hence the same for all UEs camping on the cell. This implies that the existing cell reselection mechanisms do not support partitioning of the UEs in groups that have different reselection criteria (e.g., one for legacy and one for Rel-19 NES UEs). Therefore, in order to provide the NW with the desired flexibility in terms of cell reselection, one of the possible approaches is to define new NES-specific cell reselection priority parameters. These new NES-specific cell reselection priority parameters can be provided, for example, by Cell A.

The existing reselection parameters for cell reselection are broadcasted for, and hence do not support partitioning of the UEs in groups that have different reselection criteria (e.g., one for legacy and one for Rel-19 NES UEs).

New NES-specific reselection priority parameters are defined for the purpose of prioritizing/deprioritizing a NES Cell.

# 5	On-demand SIB1 and Automatic Neighbour Relation

One of the important features that may be affected by on-demand SIB1 is Automatic Neighbour Relation (ANR). Introduction of on-demand SIB1 affects the ANR in a way that NES Cell with on-demand SIB1 cannot be discovered by the ANR. According to the current specification (c.f., Section 15.3.3 of 38.330 [5]), if a cell is not broadcasting SIB1, the UE aborts the Cell Global Identity (CGI) reporting procedure and reports "noSIB1":

“In case the detected NR cell does not broadcast SIB1, the UE may report noSIB1 indication as specified in TS 38.331 [12].”

Therefore, if the coexistence of on-demand SIB1 and ANR is not addressed, operators may need to manually provision and manage NES cells with on-demand SIB1 in the neighbour relation tables. Since this is not a practical and desirable solution, there is a need for new mechanisms that would allow for a functional coexistence of on-demand SIB1 and ANR. For example, one solution could be that the NW configures the UE whether CGI-Info is to be captured for NES cells with OD-SIB1, another solution could be that CGI-Info (or parts of it) of the NES cell is included in the WUS configuration such that the UE already knows or only needs to capture the WUS config and hence avoids asking for SIB1 for the sake of ANR.

Discuss how to support a functional coexistence of on-demand SIB1 and ANR. Example solutions could be that NW configured the UE with whether SIB1 is to be requested for the sake of ANR, another is that CGI-Info is included as part of WUS config so that UE avoids asking for SIB1.

# 6	Conclusion

In this contribution we discuss the second objective highlighted in the WID. Based on the discussion in the previous sections we made the following observations:

Observation 1	SIB1 carries the essential information (of Cell A), and the UL WUS configuration of NES Cell cannot be considered essential information of Cell A.

Observation 2	SIB1 has already a considerable size, especially in multi-PLMN deployments. Extending SIB1 message has impact on coverage.

Observation 3	One Cell A may assist multiple NES cells, including multiple WUS configurations. Extending an existing SIBx may be unnecessarily limiting on number of UL WUS configuration that can be broadcast.

Observation 4	If the UE were to receive the updates for UL WUS configuration in Cell A it would have to switch frequencies assuming that Cell A and NES Cell are on different carriers.

Observation 5	Using the same UL WUS configuration for multiple NES Cells allows for minimizing the size of the message that carries the UL WUS configuration(s).

Observation 6	Using different UL WUS configurations for different NES Cells avoids unnecessary/false NES Cell wakeups, allows for the flexibility to provide different UL WUS configuration parameters for different NES Cells when needed (e.g., if Cell A assists both NES cells in FR1 and FR2, different cell sizes, etc.).

Observation 7	The four-step RA procedure is beneficial for the purpose of legacy on-demand SI because there may be multiple SIs that can be requested on-demand. Information on which SIs the UE is interested in can be conveyed via MSG3 instead of reserving multiple preambles.

Observation 8	In case of on-demand SIB1, a single preamble is enough for requesting SIB1, i.e., there is no need to use MSG3 to convey additional information to the NW.

Observation 9	In case of legacy on-demand SI four-step RA procedure the UE already knows from RACH configuration in SIB1 how to initiate access to the NW. In case of on-demand SIB1, the UE needs a preamble and how to transmit that preamble, i.e., using which time/frequency resources, irrespective of two-/four-step RA procedure.

Observation 10	Compared to the two-step-based RA procedure, the four-step-based RA procedure where MSG3 is used for the on-demand SIB1 acquisition procedure leads to increased latency, and there is no obvious reason to introduce the four-step-based RA procedure for on-demand SIB1.

Observation 11	The existing mechanisms cannot be directly used for defining the UE behaviour upon transmitting MSG1 that carries the request for on-demand SIB1.

Observation 12	NES Cell can send the same RAR message to all UEs that requested on-demand SIB1 since all requests are intended for the same functionality, and hence sending a RAR message as a response to on-demand SIB1 request(s) does not cause excessive signalling nor significant energy consumption in the NES Cell.

Observation 13	Acquiring SIB1 without initiating on-demand SIB1 procedure implies fewer UL WUS transmissions, and hence the potential for further minimization of the energy consumption on both, the NW, and the UE side.

Observation 14	The existing reselection parameters for cell reselection are broadcasted for, and hence do not support partitioning of the UEs in groups that have different reselection criteria (e.g., one for legacy and one for Rel-19 NES UEs).

Based on the discussion in the previous section, we propose the following:

Proposal 1	UL WUS configuration for the NES Cell is provided in a new SIB of Cell A.

Proposal 2	Once Rel-19 NES UE camps on the NES cell, the UE expects to receive UL WUS configuration updates from the NES Cell, e.g., via legacy SI modification procedures.

Proposal 3	It should be possible for the network to indicate one UL WUS configuration for multiple NES cells and/or separate UL WUS configurations.

Proposal 4	MSG3 is not used for the on-demand SIB1 request procedure.

Proposal 5	RAR message is used for indicating the UE behaviour concerning when to expect on-demand SIB1 transmission, in which directions the transmission of on-demand SIB1 can be expected, and how to proceed if on-demand SIB1 is not received when expected.

Proposal 6	Discuss explicit and implicit barring mechanisms for a NES Cell that transmits on-demand SIB1.

Proposal 7	Regardless of legacy UE barring options, a Rel-19 UE supporting OD-SIB1 is implicitly barred from a NES cell for which no WUS configuration is found.

Proposal 8	Discuss how to support SIB1 acquisition without transmitting UL WUS for NES Cell with on-demand SIB1, e.g., before WUS transmission the UE attempts to decode SIB1 and/or OD-SIB1 RAR at given occasions.

Proposal 9	New NES-specific reselection priority parameters are defined for the purpose of prioritizing/deprioritizing a NES Cell.

Proposal 10	Discuss how to support a functional coexistence of on-demand SIB1 and ANR. Example solutions could be that NW configured the UE with whether SIB1 is to be requested for the sake of ANR, another is that CGI-Info is included as part of WUS config so that UE avoids asking for SIB1.

# References

RP-240170, “Revised WID: Enhancements of network energy savings for NR”, RAN103, Maastricht, Netherlands, March 2024.

R2-2405701, “Report from session on V2X/SL, R19 NES and MOB”, 3GPP TSG-RAN WG2 Meeting #126, Fukuoka, Japan, May 20 – 24, 2024.

R2-2403731, “Report from session on V2X/SL, R19 NES and MOB”, 3GPP TSG-RAN WG2 Meeting #125bis, Changsha, China, April 15 – 19, 2024.

3GPP TS 38.331 V18.1.0, Technical Specification Group Radio Access Network; NR; Radio Resource Control (RRC) protocol specification, Release 18.

3GPP TS 38.300 V18.1.0, Technical Specification Group Radio Access Network; NR; NR and NG-RAN Overall Description, Release 18.

# Appendix A

### RAN1#116 Agreements

Agreement

For discussion purpose, the following assumption will be used in RAN1

- Cell A: A cell that is periodically transmitting at least its own SIB1

- NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE

For the further study of on-demand SIB1 for idle/inactive mode UE, RAN1 studies the following options.

On target cell of UL WUS transmission:

- Option 1: UE transmits UL WUS to NES Cell

- Option 2: UE transmits UL WUS to Cell A

On configuration provision for UL WUS transmission

- Option A: UE obtains the UL WUS configuration from NES Cell

- Option B: UE obtains the UL WUS configuration from Cell A

Other options are not precluded

Agreement

For further study of achievable NES gain with on-demand SIB1 for idle/inactive mode UE,

- Assume the following for network energy evaluation of non-NES cell in FR1:

  - Empty/low/medium cell load as defined in 38.864

  - Cat 1/Cat 2 BS as defined in 38.864

  - 30kHz SCS, DDDSU TDD pattern

    - Case A: 20ms SSB period with 20ms SIB1 period;

    - Case C: 20ms SSB period with 160ms SIB1 period;

    - Case D: 20ms SSB period with 40ms SIB1 period;

Note: Other SSB/SIB1 periodicity assumptions are not precluded (up to companies to report)

  - 4 or 8 SSBs in a SSB burst with SSB pattern case C

  - 20ms or 160ms PRACH monitoring period

- Assume the following for network energy evaluation of NES cell in FR1:

  - Empty/low/medium cell load as defined in 38.864

  - Cat 1/Cat 2 BS as defined in 38.864

  - 30kHz SCS, DDDSU TDD pattern

    - Case 1: 20ms SSB period with no SIB1 transmitted;

Note: Other SSB/SIB1 assumptions are not precluded (up to companies to report)

  - 4 or 8 SSBs in a SSB burst with SSB pattern case C

  - 20ms/160ms UL WUS monitoring period

- Note: SSB/CORESET0 multiplexing pattern 1 is used

Agreement

- For study of UL WUS design, consider at least PRACH as a starting point

  - FFS: Whether there is dedicated PRACH resource for SIB1 request

- Other option(s) not precluded

Agreement

For the further study of on-demand SIB1 for idle/inactive mode UE, RAN1 to discuss triggering conditions for sending UL-WUS.

Agreement

For the study of on-demand SIB1 for idle/inactive mode UE, RAN1 to further study whether feedback from gNB in response to the SIB1 request is supported including associated details.

Agreement

For the further study on UL WUS configuration among the following options:

- Option 1: Pre-defined UL WUS configuration

- Option 2: UL WUS configuration that applies to multiple NES cell

- Option 3: UL WUS configuration that applies to a single NES cell

### RAN1#116bis Agreements

Agreement

For the further study of on-demand SIB1 for idle/inactive mode UE, RAN1 focuses its studies on the following cases:

- Case 1: Option 1+A+X

- Case 2: Option 1+B+X

- Case 3: Option 2+B+Y

Where the options 1/2/A/B/X/Y are defined below:

- On target cell of UL WUS transmission:

  - Option 1: UE transmits UL WUS to NES Cell

  - Option 2: UE transmits UL WUS to Cell A

- On configuration provision for UL WUS transmission

  - Option A: UE obtains the UL WUS configuration from NES Cell

  - Option B: UE obtains the UL WUS configuration from Cell A

- On receiving of SIB1

  - Option X: UE receives on-demand SIB1 from NES Cell

  - Option Y: UE receives on-demand SIB1 from Cell A

Agreement

RAN1 to further study the following UE operation scenarios in the UL WUS design:

- Scenario 1: UE requests SIB1 to camp on NES cell

- Scenario 2: UE request SIB1 to perform random access procedure to make RRC connection to NES cell

Agreement

RAN1 to further study UE identification of NES cell with on-demand SIB1 based on one, both, or combination of the following options:

- Option 1: By WUS configuration

- Option 2: By PBCH payload of NES cell

Agreement

Companies to report at least the following key settings used in the evaluation/simulation of achievable NES gain with on-demand SIB1 in idle/inactive mode

- Setting A: SIB1 period (20ms/40ms/160ms)

- Setting B1: Cell load (Empty/low/medium)

- Setting B2: Traffic model

- Setting C: SIB1 PDSCH time domain resource index in 38.214 Table 5.1.2.1.1-2

- Setting D: CORESET0/SSB multiplexing pattern including controlResourceSetZero (index) in 38.213 Table 13-6, and searchSpaceZero (index) in 38.213 Table 13-11

- Setting E: PRACH configurations (including PRACH configuration index in 38.211 Table 6.3.3.2-3) for WUS and initial/random access

- Setting F: Cat1/Cat2 BS

- Setting G: Number of SSB beams

- Setting H: NES gain/loss on Cell A

- Setting I: On-demand SIB1 transmission rate (how often UE requests on-demand SIB1)

Agreement

For further study of the NES gain/loss evaluation assumption on Cell A with on-demand SIB1 on NES cell for idle/inactive mode UE,

- Assume the following for network energy evaluation of Cell A in FR1:

  - Company to report among empty/low/medium cell load as defined in 38.864

  - Same Cat BS as the Non-NES cell

  - 30kHz SCS, DDDSU TDD pattern

    - Same SSB period as the Non-NES cell and company to report SIB1 period

  - Same number of SSBs in a SSB burst as the Non-NES cell with SSB pattern case C

  - 20ms PRACH configuration periodicity for WUS and/or initial access RACH and company to report RACH configuration index in 38.211 Table 6.3.3.2-3

  - Same SSB/CORESET0 multiplexing pattern and same SIB1 PDSCH time domain resource allocation as the Non-NES cell

  - Same traffic model as the Non-NES cell

  - Companies to report the assumption of WUS configuration provision or UL WUS monitoring or on-demand SIB1 transmission on Cell A if Case 2 (Option 1+B+X) or Case 3 (Option 2+B+Y) is considered

Agreement

For UL WUS design for SIB1 request, at least dedicated PRACH resource is the assumption for further study in RAN1

- FFS: Details on time, frequency, and/or PRACH preamble resources for UL WUS

- FFS: whether RACH resource for SIB1 request could be used for an initial access procedure and/or an on-demand SI procedure

Agreement

Conditions for triggering UL WUS transmission is up to RAN2. Any related work in RAN1 to be triggered by RAN2 LS. Send an LS to RAN2. Final LS in R1-2403779.

### RAN1#117 Agreements

Agreement

For SIB1 in idle/inactive mode, prioritize RAN1 discussions on Case 2 and Case 3

- Case 2 (Option 1+B+X) is feasible from RAN1 perspective.

- Further study Case 3, focusing on the additional NES benefits over Case 2, feasibility, complexity, and spec impact.

Agreement

For further study of on-demand SIB1 in idle/inactive mode, it is assumed that always-on SSB is transmitted on the NES cell with on-demand SIB1.

Agreement

At least for Case-2: For further study of type 0 PDCCH monitoring occasions for on demand SIB1, after UE transmits the UL WUS in idle/inactive mode, RAN1 assumes following as a starting point:

- Option 1: One or more type 0 PDCCH monitoring occasions for on demand SIB1 within a time window

  - FFS: How the search space zero configuration is provided (e.g. from  searchSpaceZero  in  MIB  or  from a new search space that is indicated by UL-WUS configuration)

  - FFS: Details of the time window, including at least the starting time and duration

  - FFS: Whether/how to support transmission of on-demand SIB1 with the association with SSB(s) based on a received UL-WUS

Agreement

From RAN1 point of view, the following is feasible. It is up to RAN2 to decide whether/how to support it.

At least for Case 2 (Option 1+B+X) design, a unified configuration format that can support both Option 2 (i.e. a UL-WUS configuration applies to multiple NES cells) and Option 3 (i.e. a UL-WUS configuration applies to a single NES cell).

Agreement

For further study of on-demand SIB1 in idle/inactive mode, on the spatial relationships among PDCCH/PDSCH of on-demand SIB1, SSB, and UL WUS, as UL WUS is using dedicated PRACH resource, it is assumed that spatial relationships among PDCCH/PDSCH of on-demand SIB1, SSB and UL WUS can follow legacy mechanism.

Agreement

For further study of on-demand SIB1 in idle/inactive mode, use the following Table I (from R1-2405106, Ericsson) as a starting point to discuss the required parameters/contents inside the UL WUS configuration.

Table I.

| Purpose | Parameters |  |  |
| --- | --- | --- | --- |
| To which cell does the config applies | NES-CellId | PhysCellId |  |
|  |  | ARFCN-ValueNR |  |
| WUS transmission | SIB1-RequestConfig | ss-PBCH-BlockPower |  |
|  |  | rach-OccasionsSIB1 | Prach-ConfigurationIndex |
|  |  |  | msg1-FDM |
|  |  |  | msg1-FrequencyStart |
|  |  |  | zeroCorrelationZoneConfig |
|  |  |  | preambleReceivedTargetPower |
|  |  |  | preambleTransMax |
|  |  |  | powerRampingStep |
|  |  |  | ra-ResponseWindow |
|  |  |  | ssb-perRACH-Occasion |
|  |  | sib1-RequestPeriod |  |
|  |  | sib1-RequestResource | ra-PreambleStartIndex |
|  |  |  | ra-AssociationPeriodIndex |
|  |  |  | ra-ssb-OccasionMaskIndex |
|  |  | rsrp-ThresholdSSB |  |
|  |  | prach-RootSequenceIndex |  |
|  |  | msg1-SubcarrierSpacing |  |
|  |  | restrictedSetConfig |  |
|  |  | tdd-UL-DL-ConfigurationCommon |  |
|  | frequencyInfoUL | frequencyBandList |  |
|  |  | absoluteFrequencyPointA |  |
|  |  | offsetToCarrier |  |
|  |  | p-Max |  |
|  |  | frequencyShift7p5khz |  |
| SIB1 reception | pdcch-ConfigSIB1 | ssb-SubcarrierOffset |  |
|  |  | controlResourceSetZero |  |
|  |  | searchSpaceZero |  |
| RAR Reception | pdcch-ConfigOD-SIB1-RAR | controlResourceSet |  |
|  |  | monitoringSlotPeriodicityAndOffset |  |
|  |  | Duration |  |
|  |  | monitoringSymbolsWithinSlot |  |
|  |  | aggregationLevels |  |

Agreement

For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 20ms SIB1 period (Case A), FR1, empty load

- For Cat 1 BS, 8 beams, 0% on-demand SIB1 transmission rate,

  - the NES gain is 39.37% - 47.69% from the following 7 sources

    - R1-2404463, R1-2405595, R1-2405341, R1-2404859, R1-2404507, R1-2405363, R1-2404625

  - One source (R1-2405162) reports 25% NES gain with PRACH configuration index = 152

  - One source (R1-2403979) reports the following NES gain with PRACH configuration index = 98

    - 24.6% with one beam of SIB1 PDSCH transmitted in each slot

    - 17.4% with two beams of SIB1 PDSCH transmitted in each slot

  - One source (R1-2404122) reports the following with FDMed SSB and PDSCH of SIB1

    - 9.27%~18.94% NES gain with UL WUS configuration transmitted in legacy SIB on Cell A

    - 0%~9.66% NES gain with UL WUS configuration transmitted in separated SIB on Cell A. The separated SIB is TDMed with legacy SIB and transmitted in a 20ms period using dedicated resource.

    - 0% NES gain with Case 1 (Options 1+A+X) and WUS configuration transmitted in separate SIB on the NES cell

    - 13.25% NES gain with Case 1 (Options 1+A+X) and pre-configured for fixed WUS configuration

      - NES cell does not transmit the WUS configuration

- For Cat 1 BS, 4 beams, 0% on-demand SIB1 transmission rate,

  - the NES gain is 22.1% - 42.6% from the following 5 sources

    - R1-2404408, R1-2405341, R1-2404758, R1-2405363, R1-2404625

  - One source (R1-2403979) reports the following NES gain with PRACH configuration index = 98

    - 16.8% with one beam of SIB1 PDSCH transmitted in each slot

    - 10.9% with two beams of SIB1 PDSCH transmitted in each slot

- For Cat 1 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%,

  - the NES gain is 30.43% - 45.79% from the following 4 sources

    - R1-2404463, R1-2404859, R1-2405363, R1-2404625

  - One source (R1-2405162) reports the following NES gain with PRACH configuration index = 152

    - 25% NES gain with Case 3 (Options 2+B+Y) assuming the UE can camp on the NES cell and the SIB1 is only transmitted in a single beam corresponding to the UL WUS beam

    - 23% with Case 2 (Options 1+B+X)

- For Cat 1 BS, 4 beams, 0% < on-demand SIB1 transmission rate <= 30%,

  - the NES gain is 21.79% - 33.11% from the following 2 sources

    - R1-2405363, R1-2404625

- For Cat 2 BS, 8 beams, 0% on-demand SIB1 transmission rate,

  - the NES gain is 20.5% - 39.34% from the following 8 sources

    - R1-2404463, R1-2403942, R1-2405341, R1-2404224, R1-2404859, R1-2404507, R1-2404561, R1-2405595

  - One source (R1-2404224) reports 11.9% NES gain with medium cell load on Cell A

- For Cat 2 BS, 4 beams, 0% on-demand SIB1 transmission rate, the NES gain is 19.31 – 19.39% from one source (R1-2405341)

- For Cat 2 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%,

  - the NES gain is 29.19% - 36.87% from the following 3 sources

    - R1-2404463, R1-2403942, R1-2404859

  - One source (R1-2404561) reports 15.87%~20.18% NES gain with PRACH configuration index = 148

- For Cat 1 BS, 4/8 beams, 4% < on-demand SIB1 transmission rate < 24%, the NES gain is 17.98%~24.48% from one source (R1-2404408).

Agreement

For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 160ms SIB1 period (Case C), FR1, empty load

- For Cat 1 BS, 8 beams, 0% on-demand SIB1 transmission rate, the NES gain is 2.21 – 10.6% from the following 10 sources

  - R1-2405595, R1-2403979, R1-2405341, R1-2404859, R1-2405162, R1-2404122, R1-2404507, R1-2405363, R1-2404625, R1-2404561

- For Cat 1 BS, 4 beams, 0% on-demand SIB1 transmission rate,

  - the NES gain is 1.7% - 6.23% from the following 5 sources

    - R1-2403979, R1-2405341, R1-2405363, R1-2404625, R1-2404758

  - One source (R1-2404758) reports 28.9% NES gain with Case 3 where no PRACH and paging is activated for the NES cell

- For Cat 1 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 1.75% - 8.48% from the following 6 sources

  - R1-2405106, R1-2405162, R1-2404859, R1-2405363, R1-2404625, R1-2404561

- For Cat 1 BS, 4 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 1.01% - 5.8% from the following 3 sources

  - R1-2405106, R1-2405363, R1-2404625

- For Cat 2 BS, 8 beams, 0% on-demand SIB1 transmission rate, the NES gain is 3.24% - 7.76% from the following 5 sources

  - R1-2403942, R1-2405341, R1-2404859, R1-2404507, R1-2404561

- For Cat 2 BS, 4 beams, 0% on-demand SIB1 transmission rate, the NES gain is 2.96% from one source (R1-2405341)

- For Cat 2 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 2.5% - 7.44% from the following 3 sources

  - R1-2404561, R1-2403942, R1-2404859

Agreement

For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 20ms SIB1 period (Case A), FR1, low load

- For Cat 1 BS, 8 beams, 0% on-demand SIB1 transmission rate,

  - the NES gain is 30.56% - 41.8% from the following 5 sources

    - R1-2405595, R1-2405341, R1-2404859, R1-2405363, R1-2404625

  - One source (R1-2403979) reports the following NES gain with PRACH configuration index = 98

    - 13.9% with one beam of SIB1 PDSCH transmitted in each slot

    - 7.9% with two beams of SIB1 PDSCH transmitted in each slot

- For Cat 1 BS, 4 beams, 0% on-demand SIB1 transmission rate,

  - the NES gain is 16% - 36.2% from the following 5 sources

    - R1-2405247, R1-2405341, R1-2404033, R1-2405363, R1-2404625

  - One source (R1-2404033) reports 10% NES gain with FDMed SSB and PDSCH of SIB1

  - One source (R1-2403979) reports the following NES gain with PRACH configuration index = 98

    - 5.2% with one beam of SIB1 PDSCH transmitted in each slot

    - 1.9% with two beams of SIB1 PDSCH transmitted in each slot

- For Cat 1 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%,

  - the NES gain is 22.44% - 37.64% from the following 3 sources

    - R1-2404859, R1-2405363, R1-2404625

  - One source (R1-2404122) reports the following with FDMed SSB and PDSCH of SIB1

    - 4.73%~9.64% NES gain with UL WUS configuration transmitted in legacy SIB on Cell A

    - 0%~4.8% NES gain with UL WUS configuration transmitted in separated SIB on Cell A. The separated SIB is TDMed with legacy SIB and transmitted in a 20ms period using dedicated resource.

    - 0% NES gain with Case 1 (Options 1+A+X) and WUS configuration transmitted in separate SIB on the NES cell

    - 7% NES gain with Case 1 (Options 1+A+X) and pre-configured for fixed WUS configuration

      - NES cell does not transmit the WUS configuration

- For Cat 1 BS, 4 beams, 0% < on-demand SIB1 transmission rate <= 30%,

  - the NES gain is 13.17% - 36.2% from the following 3 sources

    - R1-2404625, R1-2405247, R1-2405363

- For Cat 2 BS, 8 beams, 0% on-demand SIB1 transmission rate,

  - the NES gain is 16.8% - 28.77% from the following 6 sources

    - R1-2403942, R1-2405341, R1-2404224, R1-2404859, R1-2404561, R1-2405595

  - One source (R1-2404224) reports 10.5% NES gain with medium cell load on Cell A

- For Cat 2 BS, 4 beams, 0% on-demand SIB1 transmission rate, the NES gain is 12.71% – 12.88% from one source (R1-2405341)

- For Cat 2 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 14.43% - 27.47% from the following 3 sources

  - R1-2404561, R1-2403942, R1-2404859

Agreement

For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 160ms SIB1 period (Case C), FR1, low load

- For Cat 1 BS, 8 beams, 0% on-demand SIB1 transmission rate,

  - the NES gain is 1.2 – 8.3% from the following 7 sources

    - R1-2405595, R1-2403979, R1-2405341, R1-2404859, R1-2405363, R1-2404625, R1-2404561

- For Cat 1 BS, 4 beams, 0% on-demand SIB1 transmission rate, the NES gain is 0.2% - 5.46% from the following 4 sources

  - R1-2403979, R1-2405341, R1-2405363, R1-2404625

- For Cat 1 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 2.87% - 7.38% from the following 4 sources

  - R1-2404859, R1-2405363, R1-2404625, R1-2404561

- For Cat 1 BS, 4 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 1.97% - 4.48% from the following 2 sources

  - R1-2405363, R1-2404625

- For Cat 2 BS, 8 beams, 0% on-demand SIB1 transmission rate, the NES gain is 3.04% - 5.57% from the following 4 sources

  - R1-2403942, R1-2405341, R1-2404859, R1-2404561

- For Cat 2 BS, 4 beams, 0% on-demand SIB1 transmission rate, the NES gain is 1.79% from one source (R1-2405341)

- For Cat 2 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 2.12% - 5.26% from the following 3 sources

  - R1-2404561, R1-2403942, R1-2404859

Agreement

For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 20ms SIB1 period (Case A), FR1, medium load

- For Cat 1 BS, 8 beams, 0% on-demand SIB1 transmission rate,

  - the NES gain is 9.88% - 14.4% from the following 2 sources

    - R1-2405341, R1-2405363

  - One source (R1-2403979) reports the following NES gain with PRACH configuration index = 98

    - 4.5% with one beam of SIB1 PDSCH transmitted in each slot

    - 2.3% with two beams of SIB1 PDSCH transmitted in each slot

- For Cat 1 BS, 4 beams, 0% on-demand SIB1 transmission rate,

  - the NES gain is 4.92% - 8.4% from the following 2 sources

    - R1-2405341, R1-2405363

  - One source (R1-2403979) reports the following NES gain with PRACH configuration index = 98

    - 1.5% with one beam of SIB1 PDSCH transmitted in each slot

    - 0.5% with two beams of SIB1 PDSCH transmitted in each slot

- For Cat 1 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%,

  - the NES gain is 11.5% - 12.88% from one source (R1-2405363)

  - One source (R1-2404122) reports the following with FDMed SSB and PDSCH of SIB1

    - 1.94%~4% NES gain with UL WUS configuration transmitted in legacy SIB on Cell A

    - 0%~1.94% NES gain with UL WUS configuration transmitted in separated SIB on Cell A. The separated SIB is TDMed with legacy SIB and transmitted in a 20ms period using dedicated resource.

    - 0% NES gain with Case 1 (Options 1+A+X) and WUS configuration transmitted in separate SIB on the NES cell

    - 2.64% NES gain with Case 1 (Options 1+A+X) and pre-configured for fixed WUS configuration

      - NES cell does not transmit the WUS configuration

- For Cat 1 BS, 4 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 6%~7.53% from one source (R1-2405363)

- For Cat 2 BS, 8 beams, 0% on-demand SIB1 transmission rate, the NES gain is 8.08% - 15.47% from the following 4 sources

  - R1-2405341, R1-2404224, R1-2404561, R1-2403942

- For Cat 2 BS, 4 beams, 0% on-demand SIB1 transmission rate, the NES gain is 4.09% – 4.19% from one source (R1-2405341)

- For Cat 2 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 10.78% - 15.3% from the following 2 sources

  - R1-2404561, R1-2403942

Agreement

For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 160ms SIB1 period (Case C), FR1, medium load

- For Cat 1 BS, 8 beams, 0% on-demand SIB1 transmission rate, the NES gain is 0.3 – 2.44% from the following 4 sources

  - R1-2403979, R1-2405341, R1-2405363, R1-2404561

- For Cat 1 BS, 4 beams, 0% on-demand SIB1 transmission rate, the NES gain is 0.1% - 1.3% from the following 3 sources

  - R1-2403979, R1-2405341, R1-2405363

- For Cat 1 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 1.24% - 2.26% from the following 2 sources

  - R1-2405363, R1-2404561

- For Cat 1 BS, 4 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 0.67% - 1.6% from one source (R1-2405363)

- For Cat 2 BS, 8 beams, 0% on-demand SIB1 transmission rate, the NES gain is 0.95% - 4.13% from the following 3 sources

  - R1-2403942, R1-2405341, R1-2404561

- For Cat 2 BS, 4 beams, 0% on-demand SIB1 transmission rate, the NES gain is 0.48% from one source (R1-2405341)

- For Cat 2 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 1.44% - 4.02% from the following 2 sources

  - R1-2404561, R1-2403942

Agreement

For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 40ms SIB1 period (Case D), FR1, empty load

- For Cat 1 BS, 8 beams, 0% on-demand SIB1 transmission rate,

  - the NES gain is 24.51% - 31.3% from the following 6 sources

    - R1-2405595, R1-2405341, R1-2404859, R1-2404507, R1-2405363, R1-2404625

  - One source (R1-2403979) reports the following NES gain with PRACH configuration index = 98

    - 16.3% with one beam of SIB1 PDSCH transmitted in each slot

    - 10.6% with two beams of SIB1 PDSCH transmitted in each slot

- For Cat 1 BS, 4 beams, 0% on-demand SIB1 transmission rate,

  - the NES gain is 12.4% - 35.5% from the following 5 sources

    - R1-2404408, R1-2405341, R1-2404758, R1-2405363, R1-2404625

  - One source (R1-2403979) reports the following NES gain with PRACH configuration index = 98

    - 10.1% with one beam of SIB1 PDSCH transmitted in each slot

    - 6.1% with two beams of SIB1 PDSCH transmitted in each slot

- For Cat 1 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 19.44% - 28.81% from the following 3 sources

  - R1-2404859, R1-2405363, R1-2404625

- For Cat 1 BS, 4 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 12.87% - 20.09% from the following 2 sources

  - R1-2405363, R1-2404625

- For Cat 2 BS, 8 beams, 0% on-demand SIB1 transmission rate, the NES gain is 11.77% - 25.46% from the following 5 sources

  - R1-2403942, R1-2405341, R1-2404859, R1-2404507, R1-2404561

- For Cat 2 BS, 4 beams, 0% on-demand SIB1 transmission rate, the NES gain is 10.69 – 10.74% from one source (R1-2405341)

- For Cat 2 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%,

  - the NES gain is 16.46% - 24.24% from the following 2 sources

    - R1-2403942, R1-2404859

  - One source (R1-2404561) reports 8.92%~10.37% NES gain with PRACH configuration index = 148

Agreement

For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 40ms SIB1 period (Case D), FR1, low load

- For Cat 1 BS, 8 beams, 0% on-demand SIB1 transmission rate,

  - the NES gain is 18.38% - 26.4% from the following 5 sources

    - R1-2405595, R1-2405341, R1-2404859, R1-2405363, R1-2404625

  - One source (R1-2403979) reports the following NES gain with PRACH configuration index = 98

    - 8.1% with one beam of SIB1 PDSCH transmitted in each slot

    - 4.3% with two beams of SIB1 PDSCH transmitted in each slot

- For Cat 1 BS, 4 beams, 0% on-demand SIB1 transmission rate,

  - the NES gain is 9.84% - 17.41% from the following 3 sources

    - R1-2405341, R1-2405363, R1-2404625

  - One source (R1-2403979) reports the following NES gain with PRACH configuration index = 98

    - 2.8% with one beam of SIB1 PDSCH transmitted in each slot

    - 1% with two beams of SIB1 PDSCH transmitted in each slot

- For Cat 1 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 13.36% - 23.68% from the following 3 sources

  - R1-2404859, R1-2405363, R1-2404625

- For Cat 1 BS, 4 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 7.27% - 15.59% from the following 2 sources

  - R1-2405363, R1-2404625

- For Cat 2 BS, 8 beams, 0% on-demand SIB1 transmission rate, the NES gain is 10.48% - 17.72% from the following 4 sources

  - R1-2403942, R1-2405341, R1-2404859, R1-2404561

- For Cat 2 BS, 4 beams, 0% on-demand SIB1 transmission rate, the NES gain is 7.01% from one source (R1-2405341)

- For Cat 2 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 7.7% - 17.05% from the following 3 sources

  - R1-2403942, R1-2404859, R1-2404561

Agreement

For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 40ms SIB1 period (Case D), FR1, medium load

- For Cat 1 BS, 8 beams, 0% on-demand SIB1 transmission rate,

  - the NES gain is 5.07% - 7.8% from the following 2 sources

    - R1-2405341, R1-2405363

  - One source (R1-2403979) reports the following NES gain with PRACH configuration index = 98

    - 2.4% with one beam of SIB1 PDSCH transmitted in each slot

    - 1.2% with two beams of SIB1 PDSCH transmitted in each slot

- For Cat 1 BS, 4 beams, 0% on-demand SIB1 transmission rate,

  - the NES gain is 2.46% - 4.4% from the following 2 sources

    - R1-2405341, R1-2405363

  - One source (R1-2403979) reports the following NES gain with PRACH configuration index = 98

    - 0.7% with one beam of SIB1 PDSCH transmitted in each slot

    - 0.3% with two beams of SIB1 PDSCH transmitted in each slot

- For Cat 1 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 5.62~7.01% from one source (R1-2405363)

- For Cat 1 BS, 4 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 2.77% - 3.85% from one source (R1-2405363)

- For Cat 2 BS, 8 beams, 0% on-demand SIB1 transmission rate, the NES gain is 4.08%~10.1% from the following 3 sources

  - R1-2403942, R1-2405341, R1-2404561

- For Cat 2 BS, 4 beams, 0% on-demand SIB1 transmission rate, the NES gain is 2.02%~2.09% from one source (R1-2405341)

- For Cat 2 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 5.61% - 9.94% from the following 2 sources

  - R1-2404561, R1-2403942

Agreement

For the evaluation of NES loss on cell A, the following is observed

  - NES loss of 0%~0.98% from the following 15 sources with UL WUS configuration transmitted in legacy SIB on Cell A (with different assumptions on SIB periodicity and cell loading):

    - R1-2404408, R1-2404463, R1-2403942, R1-2403979, R1-2405341, R1-2404224, R1-2404859, R1-2404758, R1-2405162, R1-2404122, R1-2404507, R1-2404033, R1-2405363, R1-2404625, R1-2404561 (for Case 2)

  - One source (R1-2405106) reports for the case with UL WUS configuration transmitted in separated SIB on Cell A where the separated SIB is TDMed with legacy SIB and transmitted in a 320ms period using dedicated resource.

    - 0.62%~2.65% NES loss for Case 2 (Options 1+B+X) with empty load

    - 0.73%~3.20% NES loss for Case 3 (Options 2+B+Y) with empty load

  - One source (R1-2404122) reports

    - 2.95%(medium load)~15.49%(empty load) NES loss with UL WUS configuration transmitted in separated SIB on Cell A

      - The separated SIB is TDMed with legacy SIB and transmitted in a 20ms period using dedicated resource.

    - 1.28%(medium load)~3.62%(empty load) NES loss with UL WUS configuration transmitted in legacy SIB on Cell A

Conclusion

For further study of on-demand SIB1 in idle/inactive mode, enabling or disabling specific operations (e.g. paging, RACH receiving, OSI request …) of the NES cell with on-demand SIB1 is up to RAN2.

Agreement

For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 20ms SIB1 period (Case A), FR1, 8 beams, on-demand SIB1 transmission rate > 30%

- For Cat 1 BS, empty load, the NES gain is 37.77% from one source (R1-2404463)

- For Cat 2 BS, empty load,

  - the NES gain is 8.9% - 28.71% from the following 3 sources

    - R1-2404224, R1-2404561, R1-2404463

  - The NES gain is 1.47% from one source (R1-2404561) with 75% on-demand SIB1 transmission rate

- For Cat 2 BS, low load,

  - the NES gain is 7.9% - 14.65% from the following 2 sources

    - R1-2404224, R1-2404561

  - The NES gain is 0.42% from one source (R1-2404561) with 75% on-demand SIB1 transmission rate

- For Cat 2 BS, medium load,

  - the NES gain is 6.2% - 10.4% from the following 2 sources

    - R1-2404224, R1-2404561

  - The NES gain is 0.12% from one source (R1-2404561) with 75% on-demand SIB1 transmission rate

Agreement

For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 40ms SIB1 period (Case D), FR1, 8 beams, on-demand SIB1 transmission rate > 30%

- For Cat 2 BS, empty/low/medium load, the NES gain is 3.77% - 5.97% from one source (R1-2404561)

Agreement

For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 160ms SIB1 period (Case C), FR1, 8 beams, on-demand SIB1 transmission rate > 30%

- For Cat 1/Cat 2 BS, empty/low/medium load, the NES gain is 0.31%~3.33% from the following 2 sources

  - R1-2404561, R1-2404122

Agreement

For the evaluation of the energy consumption to transmit WUS configuration on NES Cell:

- One source (R1-2405595) reports that for broadcasting WUS configuration using DCI occupying 2 symbols per SSB beam) on NES cell every 80 ms on CORESET#0 (48 RBs):

  - For BS category 1, empty/low load, the energy consumption is increased by 6.26%~8.51% over SSB transmission only. The energy saving gain with Case 1 is 40.52% / 32.41% for empty/low load over the baseline with 8 beams, SIB-1 period 20ms (baseline), 0% SIB-1 transmission rate (for on demand SIB1).

  - For BS category 2, empty/low load, the energy consumption is increased by 1.55%~2.11% over SSB transmission only. The energy saving gain with Case 1 is 28.25% / 23.03% for empty/low load over the baseline with 8 beams, SIB-1 period 20ms (baseline), 0% SIB-1 transmission rate (for on demand SIB1).