3GPP TSG-RAN WG2 Meeting #R2-127		R2-2407042 Maastricht, Netherlands, Aug 19-23, 2024

Agenda Item:	8.5.3

Source: 	Rakuten

Title:  	Discussion on on-demand SIB1 for NES

Document for:	Discussion and decision

# Introduction

The Rel-19 WI on enhancements of network energy savings for NR (RP-234065) includes the objective to study on-demand SIB1 for idle/inactive UEs as mentioned below:

Study procedures and signaling method(s) to support on-demand SIB1 for UEs in idle/inactive mode, including: [RAN1/2/3] - Triggering method by uplink wake-up-signal using an existing signal/channel. - Wake-up-signal configuration provisioning to UE - Note: No modification of SSB will be discussed under this objective - Information exchange between gNBs at least for the configuration of wake-up signal, if necessary. - Checkpoint for normative work in RAN#105

In this contribution we discuss some important aspects associated with OD-SIB1, focussed on the highlighted objective.

# Discussion

In this document, we refer to the following nomenclature.

- The gNB associated with the NES cell as NES cell gNB

- The gNB-CU associated with the NES cell as NES cell gNB-CU

- The gNB-DU associated with the NES cell as NES cell gNB-DU

- The gNB associated with the Cell A as Cell A gNB

- The gNB-CU associated with the Cell A as Cell A gNB-CU

- The gNB-DU associated with the Cell A as Cell A gNB-DU

## Introduction

RAN1#116 (February 2024) made the following assumptions for the purpose of discussion:

For discussion purpose, the following assumption will be used in RAN1

- Cell A: A cell that is periodically transmitting at least its own SIB1.

- NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE

The cell A in the below example is a macro cell and the NES cell is a small cell which has at least partial overlapping coverage with the macro cell.

Figure 1: NES Cell and Anchor cell topography for OD-SIB1

## IDLE/INACTIVE mode overload in Cell A

When an overload condition is detected in Cell A with respect to IDLE/INACTIVE mode UEs, a percentage of IDLE/INACTIVE UEs have to be offloaded to the NES cells. Further, all new UEs coming to Cell A have to be re-directed to the NES cells until the overload condition eases.

When the IDLE/INACTIVE UEs overload condition is detected, the anchor cell gNB shall offload UEs to one or more NES cells having overlapping coverage with it, an exemplary topography of anchor cell with overlapping NES cell(s) is shown in Figure 1.

When an overload condition of IDLE/INACTIVE UEs is detected in Cell A, the Cell A gNB shall initiate offloading of IDLE/INACTIVE UEs to NES cells.

To execute this offloading, the Cell A modifies its SIB1 to include the list of NES cell IDs to which offloading is intended. In addition to this, the cause “Overload_Based_Offloading” may also be indicated, so that the UEs can transmit a UL WUS to the NES cell and perform cell-reselection to NES cell after acquiring UL WUS configuration. Cell A gNB sends a Paging with System Info Modification to all UEs camped on it to read SIB1 modification. This enables the UEs to read the modified Cell A SIB1 and evaluate the radio condition of NES cell and perform cell re-selection to NES cell, if feasible.

Alternatively, if the Cell A gNB wants to take proactive action to just prevent new incoming UEs, it could send a backhaul signaling to the NES cell gNB to initiate OD-SIB1 transmission, so that all the incoming UEs could directly camp on the NES cell without coming to Cell A.

During overload, Cell A gNB initiates a Paging message with System Info Modification to trigger camped UEs to read modified SIB1 and transmit UL WUS and perform cell re-selection to NES cell.

The Paging message with modified SIB1 may include the list of NES Cell IDs selected for offloading UEs and the cause.

Cell A gNB may request NES cell gNB to trigger OD-SIB1 transmission without a UL WUS. RAN2 asks RAN3 to discuss and agree a solution to handle this issue.

## SIB1 reacquisition in an NES cell

Once a UE in IDLE/INACTIVE mode camps on NES cell, it may have to re-acquire SIB1 for reasons mentioned below.

RRC TS 38.331:

#### 5.2.2.2	SIB validity and need to (re)-acquire SIB

##### 5.2.2.2.1	SIB validity

The UE shall apply the SI acquisition procedure as defined in clause 5.2.2.3 upon cell selection (e.g. upon power on), cell-reselection, return from out of coverage, after reconfiguration with sync completion, after entering the network from another RAT, upon receiving an indication that the system information has changed, upon receiving a PWS notification, upon receiving request (e.g., a positioning request) from upper layers; and whenever the UE does not have a valid version of a stored SIB or posSIB or a valid version of a requested SIB.

We observe that UEs might need to reacquire SIB1 under certain circumstances as mentioned above.

UEs might need to reacquire SIB1 of an NES cell after having camped there.

If the NES cell is transmitting OD-SIB1, then it can be assumed that the UE can perform a SIB1 re-acquisition. In case the NES cell is in Energy saving mode i.e there is no periodic SIB transmission or OD-SIB1 transmission is stopped, then the most straightforward solution for UEs to reacquire SIB1 is to perform cell reselection to cell A where it can reacquire NES Cell UL WUS configuration. Subsequently it sends UL WUS to NES cell i.e performs RACH procedure and requests OD-SIB1 transmission from the NES cell. Thus it can re-acquire the latest SIB1. This is sub-optimal as the UE has to perform cell reselection to cell A all over again. It incurs additional UE power as well as signaling towards the network

For a UE camped on NES cell, it is suboptimal to perform cell re-selection to Cell A to acquire UL WUS configuration of NES cell, request NES cell to transmit OD-SIB1 again and re-acquire SIB1 of NES cell.

## Instead, the UL WUS configuration of an NES cell could be considered to be valid until explicitly indicated. This implies that whenever the UL WUS configuration of an NES cell is modified when the NES cell is in ES mode, the NES cell initiates OD-SIB1 broadcast proactively (i.e deactivates ES mode) and sends a Paging message with System Info Broadcast to IDLE/INACTIVE UEs camped on it. This triggers the camped UEs to acquire the UL WUS configuration from the NES cell itself. Further, the UEs may use the UL WUS to re-acquire SIB1 whenever required.

UL WUS configuration of an NES cell acquired by a UE is considered valid unless explicitly indicated to the UE.

Whenever an NES cell is in non-ES mode, the NES cell broadcasts its own UL WUS configuration in its SIB1.

Whenever there is a modification in the UL WUS configuration of the NES cell, the NES cell proactively initiates OD-SIB1 transmission and sends a Paging message with System Info Broadcast to all IDLE/INACTIVE UEs.

## Synergizing OD-SIB1 transmission in an NES cell

OD-SIB1 transmission is based on UL wake-up signal directly sent to NES cell (or via Cell A) from individual UEs and there could be multiple UEs in the coverage area of an NES cell who are eligible to camp on the NES cell, waiting for OD-SIB1 transmission.

NES cell triggering OD-SIB1 transmission per UE is sub-optimal from energy savings perspective. Moreover, the other eligible UEs in IDLE/INACTIVE mode are not aware of OD-SIB1 transmission, as they may not try to perform cell re-selection at the same time.

NES cell triggering OD-SIB1 transmission per UE is sub-optimal from energy savings perspective.

Eligible UEs in IDLE/INACTIVE mode which are camped on Cell A are not aware of NES cell’s OD-SIB1 transmission.

To mitigate this, the NES cell may decide when to start the OD-SIB1 transmission. For example, the gNB could configure a threshold number of “n” UEs requesting OD-SIB1, before initiating OD-SIB1 transmission. i.e the OD-SIB1 transmission could be initiated only after ‘n’ UEs have sent UL wake-up signal to the NES cell.

In such a case where the NES is not going to start the OD-SIB1 transmission immediately upon receiving the UL WUS, the gNB may indicate to the UE, for example in the Random Access Response(RAR), that the UE need not monitor PDCCH in the RAR-window. The UE may also implicitly understand that the network will communicate whenever OD-SIB1 transmission is initiated.

gNB shall be able to be configured to wait until a threshold number of “n” UEs request OD-SIB1 before initiating OD-SIB1 transmission in an NES cell.

Post receiving a UL WUS request, the gNB shall be able to indicate to the UE to not monitor PDCCH in the RAR-window, when the OD-SIB1 transmission is not initiated immediately.

Subsequently, when the ‘n’ number of UL WUS requests are received, the gNB may iniitate the OD-SIB1 transmission at the NES cell. Since the intention is to inform all the eligible UEs (including those that have already sent a UL WUS requesting OD-SIB1 earlier), the NES cell gNB has to notify the cell A gNB to notify the IDLE/INACTIVE UEs camped on cell A, so that they can access the OD-SIB1 transmission.

The NES cell gNB shall notify the Cell A gNB about the NES Cell ID and its OD-SIB1 transmission. RAN2 asks RAN3 to discuss and agree a way forward for this issue.

The Cell A gNB shall send a Paging message to all the IDLE/INACTIVE UEs to inform them about the NES Cell ID and its OD-SIB1 transmission.

The IDLE/INACTIVE UEs on Cell A, who already have the WUS configuration of the given NES cell, may access OD-SIB1 of NES cell and perform cell re-selection to NES cell.

# Conclusion

We have the following observations:

Observation 1	UEs might need to reacquire SIB1 of an NES cell after having camped there.

Observation 2	For a UE camped on NES cell, it is suboptimal to perform cell re-selection to Cell A to acquire UL WUS configuration of NES cell, request NES cell to transmit OD-SIB1 again and re-acquire SIB1 of NES cell.

Observation 3	NES cell triggering OD-SIB1 transmission per UE is sub-optimal from energy savings perspective.

Observation 4	Eligible UEs in IDLE/INACTIVE mode which are camped on Cell A are not aware of NES cell’s OD-SIB1 transmission.

We have the following proposals:

Proposal 1	When an overload condition of IDLE/INACTIVE UEs is detected in Cell A, the Cell A gNB shall initiate offloading of IDLE/INACTIVE UEs to NES cells.

Proposal 2	During overload, Cell A gNB initiates a Paging message with System Info Modification to trigger camped UEs to read modified SIB1 and transmit UL WUS and perform cell re-selection to NES cell.

Proposal 3	The Paging message with modified SIB1 may include the list of NES Cell IDs selected for offloading UEs and the cause.

Proposal 4	Cell A gNB may request NES cell gNB to trigger OD-SIB1 transmission without a UL WUS. RAN2 asks RAN3 to discuss and agree a solution to handle this issue.

Proposal 5	UL WUS configuration of an NES cell acquired by a UE is considered valid unless explicitly indicated to the UE.

Proposal 6	Whenever an NES cell is in non-ES mode, the NES cell broadcasts its own UL WUS configuration in its SIB1.

Proposal 7	Whenever there is a modification in the UL WUS configuration of the NES cell, the NES cell proactively initiates OD-SIB1 transmission and sends a Paging message with System Info Broadcast to all IDLE/INACTIVE UEs.

Proposal 8	gNB shall be able to be configured to wait until a threshold number of “n” UEs request OD-SIB1 before initiating OD-SIB1 transmission in an NES cell.

Proposal 9	Post receiving a UL WUS request, the gNB shall be able to indicate to the UE to not monitor PDCCH in the RAR-window, when the OD-SIB1 transmission is not initiated immediately.

Proposal 10	The NES cell gNB shall notify the Cell A gNB about the NES Cell ID and its OD-SIB1 transmission. RAN2 asks RAN3 to discuss and agree a way forward for this issue.

Proposal 11	The Cell A gNB shall send a Paging message to all the IDLE/INACTIVE UEs to inform them about the NES Cell ID and its OD-SIB1 transmission.

Proposal 12	The IDLE/INACTIVE UEs on Cell A, who already have the WUS configuration of the given NES cell, may access OD-SIB1 of NES cell and perform cell re-selection to NES cell.

# Reference

RP-240170, “Revised WID: Enhancements of network energy savings for NR”, RAN103, Maastricht, Netherlands, March 2024.

3GPP TR 38.864 V18.1.0, Study on network energy savings for NR.

RAN1 Chairman Notes : RAN1 #116bis