3GPP TSG-RAN WG2 #127	R2-2407043

Maastricht, Netherlands, 19 – 23 Aug 2024

Agenda item:		8.5.3	On-demand SIB1

Title:	Discussion on On-demand SIB1 procedure and UL WUS configuration

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

During RAN2 #126, the following agreements were made:

Further clarification Study on-demand SIB1 provisioning for NES Cell(s) in versions of Scenario 1a with multiple Cells A and/or NES Cells: - More than one Cell A may provide configuration for the same NES cell. - The same Cell A may assist more than one NES Cells. Information of WUS configuration Can use the PCI and frequency of a NES Cell to associate the UL WUS configuration with a NES Cell. For Message 1 based on-demand SIB1 request, the on-demand SI request configuration that currently included in SIB1 may be used as the design baseline. Triggering condition of WUS transmission If the UE chooses the NES cell using legacy intra-F/inter-F cell re-selection procedure (as baseline), the UE triggers WUS transmission. After UE successfully receives OD-SIB1 for that NES Cell and if it is a suitable cell, UE camps in the NES Cell “similar” to a legacy Cell. UE request SIB1 to perform initial access in NES cell RAN2 not to support on-demand SIB1 request that is combined with an initial access to perform RRC connection establishment/resume on the NES cell. Barring for R19 NES UE If the NES UE is unable to acquire the SIB1 of NES Cell before UE initiates OD-SIB1 procedure, it will not consider it as barred at that moment. R19 NES UE bars the cell if the UE fails to acquire SIB1 via on-demand SIB1 for NES cell. Paging and SIB1 update in NES cell After Rel-19 NES UEs camp in NES cell, the UE behaviour is same as the one defined as legacy normal camped state, e.g. paging reception, SIB1 update, etc. RAR monitoring for MSG1 based OD-SIB1 REQ RAN2 assumes the UE is expected to receive the RAR responding to the preamble transmission for Msg1-based on-demand SIB1 procedure, as the baseline. UE behaviour on RACH failure of OD-SIB1 REQ As baseline, upon random access procedure failure of OD-SIB1 request, UE regards OD-SIB1 can’t be acquired in the NES cell and considers it as barred. It doesn’t exclude the option to leave the determination to the UE implementation. SIB1 acquisition upon SIB change notification in NES cell Once the NES UE camps on the NES cell, if the UE receives SIB change notification, the UE is expected to receive SIB1 from NES cell.

In this contribution, we discuss further considerations on Cell selection, OD-SIB1 request procedure, and remaining issues related to WUS configuration and support of different cases.

# 2. Discussion

RAN1 and RAN2 have agreed to discuss some cases/scenarios in this SI.

Case 1 (in RAN1, Scenario 3 in RAN2): Standalone NES Cell, UL WUS configuration and OD-SIB1 is achieved at NES Cell B.

Case 2 (in RAN1, Scenario 1a in RAN2): Cell A provides UL WUS configuration and UE performs UL WUS and acquires OD-SIB1 at NES Cell B.

Case 3 (in RAN1, Scenario 1b in RAN2): Full Cell A, UE performs UL WUS and acquires OD-SIB1 from NES cell B at Cell A.

Note: Cell A, a.k.a. anchor cell, is a cell that can provide UL WUS configuration for an OD-SIB1 NES cell.

2.1	Cell (re)selection for NES cell

In the past RAN2 meetings, there were some agreements related to cell reselection to a NES cell, where OD-SIB1 is supported:

In RAN2#125bis,

- The UE first should acquire valid SIB1 (e.g. via SIB1 request) for camping to NES cell (if the UE knows the cell doesn’t broadcast SIB1 and supports on-demand SIB1).

In RAN2#126,

If the UE chooses the NES cell using legacy intra-F/inter-F cell re-selection procedure (as baseline), the UE triggers WUS transmission.

After UE successfully receives OD-SIB1 for that NES Cell and if it is a suitable cell, UE camps in the NES Cell “similar” to a legacy Cell.

The agreements in RAN2#126 implies that the UE transmits the OD-SIB1 request after cell reselection procedure and before determining to camp on that NES cell. Based on this understanding, we discuss some open issues for cell reselection to a NES cell of OD-SIB1 with assuming the scenario 1a (Case 2 in RAN1). When the NES cell is the best cell, the UE initiates the OD-SIB1 acquisition on the NES cell. After the WUS for OD-SIB request is transmitted, the UE waits for SIB1 transmission on the NES cell. One is issue on the paging reception. While the UE is waiting for SIB1 transmission, the UE may not receive a paging on the current camping cell. To avoid missing paging reception, the UE should ensure that the UE does not miss the paging occasion in the current camping cell. It can be left to UE implementation about exactly how this is ensured.

Proposal 1: The UE should not miss paging occasion(s) on the current camping cell, while the UE is wating for SIB1 transmission on NES cell. It is up to UE implementation how to ensure this.

Another issue is the UE behaviour when the NES cell is not suitable for the UE due to some reasons. If this happens, the UE can simply select the 2nd best cell. Similar issue is the UE behaviour when the UE cannot receive the SIB1 on the NES cell after a while (FFS on how long it is). The situation may be a corner case, as the UE selects the NES cell based on best cell selection policy basically. So, the simplest way is to do the same way as the previous case where the NES cell is not suitable, i.e. selects the 2nd best cell. This will work well, especially if the time to wait for SIB1 transmission is relatively short. Another way is to perform cell reselection procedure again by removing the NES cell from the candidate at this time. However, unless the UE is moving so fast or the UE was waiting for very long time for SIB1 transmission on the NES cell, this could be too much. We assume that the UE should select the 2nd best cell if the UE cannot receive the SIB1 on the NES cell as baseline. Once RAN2 has more progress, this may be reconsidered later.

Proposal 2: The UE selects the 2nd best cell, if the UE considers the NES cell is not suitable for camping or the UE cannot receive the SIB1 on the NES cell, as baseline.

2.2	Msg3-based OD-SIB1 request

In the last meeting, some more details were agreed for the Msg1-based OD-SIB1 request. Remaining issue is whether to also support the Msg3-based OD-SIB1 request or not. Unlike the legacy OD-SI request, the target is only the SIB1 and thus no big resource fragmentation is expected due to introducing OD-SIB1 request. Unless the RACH preamble transmission is successful, the Msg3 transmissions does not happen anyway. In other words, as long as the Msg1 preamble transmission is successful, the Msg1-based transmission works well apart from the concern on resource fragmentation which could be considered minor issue. We assume the Msg3-based OD-SIB1 request will not be needed.

Proposal 3: RAN2 to assume the Msg3-based OD-SIB1 request is not considered in Rel-19.

2.3	WUS update

In the draft RAN2 #127 agenda, the following potential topic is listed:

Leftover issues from RAN2#126 and remaining essential issues for study (e.g. need of WUS configuration acquisition from NES cell (if needed, what’s use cases and how it works)?,

Following the RAN2 #126 agreement: “Once the NES UE camps on the NES cell, if the UE receives SIB change notification, the UE is expected to receive SIB1 from NES cell”, this can be interpreted as the fact that UL WUS configuration for the targeted NES cell is not needed for UEs camping on NES cell. Indeed, SIB1 will remain valid until is updated, and legacy procedure can handle SIB1 update, as per RAN2 #126 agreement.

Observation: Once a UE has successfully camped on an OD-SIB1 cell, it will not need its UL WUS configuration to acquire up-to-date SIB1 for this cell.

Then, it is unclear what the UE should do with the UL WUS configuration. If the UE stays on the NES cell, then it has no need for it and could discard it, and potentially reacquire it in the future if needed.

However, in a frequent mobility scenario between nearby small NES cells, the UE could keep the UL WUS configuration and even keep it up to date if it were to leave the NES cell and then switch back to it again. Similarly, it can be discussed whether keeping other cells’ SIB1 stored could be beneficial.

Proposal 4: RAN2 to discuss how UE handles the serving cell’s UL WUS configuration (store it, keep it up to date, discard it) in case of future mobility once an OD-SIB1 procedure has been successfully completed and a UE is camping on the OD-SIB1 NES Cell.

2.4	SIB1 validity

In previous releases, unlike other SIBs, SIB1 is neither on-demand broadcasting nor subject to valueTag/validity feature. SIB1 carries all the basic information for UE to perform the initial access procedure at least up to RRCSetup. More importantly, SIB1 carries scheduling information for other SIBs in the IE SI-SchedulingInfo. This IE contains information needed for acquisition of other SIBs, also contains the information for UE to decide whether a stored version of other SIBs can be used.

However along with introduction of on-demand SIB1 feature

- SIB1 validity is feasible at least for case 2 . A Cellx’s SIB1valuetag can be provided together with its WUS configuration over neighboring cell(s) as shown in following figure. A UE who has up-to-date WUS config will has up-to-date SIB1valuetag of a on-demand SIB1 cell. After a UE reselects to the on-demand SIB1 cell, it will validify the stored SIB1 of this cell based on the received SIB1Valuetag from neighboring cell and decide to use the stored SIB1 version or to send WUS and request SIB1.

- SIB1 validity apparently will bring in a lot of more power and signalling overhead gain. By using an stored version of SIB1, UE will save to send WUS, and NW will save to send SIB1 in response.  If on-demand SIB1 is introduced, but a stored version of SIB1 cannot be valid and used after UE moving out to other cell and back to the same cell in RRC idle/inactive mode, we will not achieve optimal  signalling/power saving. For example, when one or multiple UEs are moving at the boarder of an on-demand SIB1 cell, NW will end up constantly receiving WUS/SIB1 request signalling and constantly broadcasting SIB1 in response to the requests

Proposal 5: Support SIB1 validity feature along with on-demand SIB1 feature.

2.5	Support of Case 1and Case 3

During last RAN2 meeting, it was agreed that:

Scenario 3 (case 1 in RAN1)

RAN2 to wait for RAN1’s progress whether to support scenario 3.

Some operators have shown interest in this topic; however, little progress has been made and RAN1 has not come to a conclusion either.

Proposal 6: Wait for RAN1 for support of Case 1.

Provision of UL WUS configuration may be heavy in Case 2 (“at least PRACH configuration” was agreed). Likewise, provision of full SIB1 would be even heavier in Case 3.

Since there may be many OD-SIB1 NES cells in a Cell A’s area, constantly broadcasting all UL WUS configurations or SIB1s may be cumbersome. Relying on OD-SI to convey UL WUS configuration or SIB1 can significantly reduce the signalling load at Cell A’s side.

The Cell A still needs to indicate to the UE that information regarding neighbouring NES cells is available. However, it only needs to indicate minimal information in SI, at least Frequency and PCI (as per RAN2 agreement), and the UE can then request required UL WUS configuration /SIB1 for targeted NES cells via OD-SI. This information would typically be located in a new SIBx.

Proposal 7: If Case 3 is supported, RAN2 to consider that SIB1 of a neighbouring NES Cells are encapsulated in a new SIx/SIBx of Cell A which can be requested on-demand.

# 3. Conclusion

In this contribution we discussed support of on-demand SIB1 for UEs in Idle/Inactive and made the following proposals.

Proposal 1: The UE should not miss paging occasion(s) on the current camping cell, while the UE is wating for SIB1 transmission on NES cell. It is up to UE implementation how to ensure this.

Proposal 2: The UE selects the 2nd best cell, if the UE considers the NES cell is not suitable for camping or the UE cannot receive the SIB1 on the NES cell, as baseline.

Proposal 3: RAN2 to assume the Msg3-based OD-SIB1 request is not considered in Rel-19.

Proposal 4: RAN2 to discuss how UE handles the serving cell’s UL WUS configuration (store it, keep it up to date, discard it) in case of future mobility once an OD-SIB1 procedure has been successfully completed and a UE is camping on the OD-SIB1 NES Cell.

Proposal 5: Support SIB1 validity feature along with on-demand SIB1 feature.

Proposal 6: Wait for RAN1 for support of Case 1.

Proposal 7: If Case 3 is supported, RAN2 to consider that SIB1 of a neighbouring NES Cells are encapsulated in a new SIx/SIBx of Cell A which can be requested on-demand.

# References

[1] R2-2403731, Report from session on V2X/SL, R19 NES and MOB

[2] RAN1 Chair’s Notes (RAN1#116bis)