3GPP TSG-RAN WG2 Meeting #127	R2-2407044

# Maastricht, Netherlands, 19th Aug – 23rd Aug 2024

Agenda Item:	8.7.1.1

Source:	Ericsson

Title:	Discussion on LSs from SA2 and RAN3

Document for:	Discussion, Decision

# 1	Introduction

In S2-2407351 SA2 pose two questions to RAN2 asking for feedback on the CN providing dynamic in-band signaling of periodicity information over the GTP-U tunnel and the usefulness of “time to next burst” information provided on the first packet in the burst. The questions are repeated below highlighting the RAN2 queries.

-          Question1 [for SA4, RAN2 and RAN3]:

  - SA2 discusses indicating periodicity via in-band signaling (i.e. in GTP-U) for dynamic changes of the periodicity and kindly asks RAN2 and RAN3 to feedback on that approach.

  - To SA4: is it possible for application server to provide the periodicity to the PSA UPF in RTP header extension?

- Question2 [for SA4 and RAN2]: There is some discussion about time to next burst.

  - To SA4: is it possible that the application server provides the time to next burst (i.e. the time interval between the current burst and the next burst) in the 1st packet of the burst via N6?

  - Does RAN2 think the time to next burst is useful for RAN resource scheduling, especially with regard to the possible jitter between the NG-RAN and the Application Server?

In R3-243957 RAN3 provide an LS to RAN2 asking for feedback on UL PSI discard activation for split bearer mode of operation. The LS read the following:

RAN3 is currently discussing XR DC signaling enhancements for various bearer types.

In case of NR-DC split bearer, MN DU and SN DU congestion is local to each node. Based on congestion situation in MN DU and/or SN DU, each of the nodes independently send PSI-Based SDU Discard Activation/Deactivation MAC CE to UE for activation/deactivation of UL PSI based PDU set discarding of the split bearer. when UE receives MAC CE from both nodes or from a single node, it is not clear how UE will activate/deactivate UL PSI based PDU set discarding when split bearer is configured.

Thus, RAN3 kindly asks RAN2 to clarify the UE behavior for handling uplink PSI based PDU discarding when MN and/or SN node sends the PSI-Based SDU Discard Activation/Deactivation MAC CE.

In this contribution we discuss the RAN2 response to these questions.

# 2	Discussion on questions from SA2

## 2.1 Discussion on Question 1

Dynamic updates of periodicity information in the case of the traffic pattern change due to e.g. (frame)rate-adaptation may be useful to optimize resource scheduling in the RAN e.g. aligning grants with traffic arrival. It may also be useful to configure specific features such as optimal DRX pattern, alignment of SPS/CG allocations and PDCCH skipping. Therefore, we propose that RAN2 replies that such dynamic information may be useful.

RAN2 replies that dynamic in-band signaling indicating periodicity is useful for optimized resource scheduling and the configuration of features, e.g. DRX, SPS/CG and PDCCH skipping.

## 2.2 Discussion on Question 2

On this question it should be noted that this topic has already been discussed in Rel-18 handling the following LS S4-230739 from SA4 in RAN2#122. In this LS SA4 queried the following:

SA4 thanks SA2 for confirming the progress of the normative work timeline. As indicated in S4-230419, the new RTP header extension under SA4 5G_RTP will signal the PDU set information, including PDU set sequence number, PDU set boundary indication, PDU sequence number within a PDU set, PDU set size, and PDU set importance.

During SA4#123-e, it was agreed to add a 3-bit End of Data Burst indication in the new header extension. SA4 has committed to progressing the semantics of the fields and developing normative guidelines for the Application Server on how to populate the fields of the RTP header extension for the supported media codecs. Upon completing such an effort, SA4 will continue to provide guidelines on how the UPF may extract some of the supported PDU set information from existing RTP/SRTP headers, header extensions, and payloads in case the newly defined RTP header extension is absent.

In addition to marking the last PDU of the data burst, SA4 sees the benefit of using additional bits to indicate inter-burst time, which may change dynamically due to various reasons, including application-layer rate control. SA4 believes that this can enable the RAN to switch to the most appropriate power state. SA4 kindly requests feedback from SA2 and RAN2 on the value and feasibility of such solution and if that can be supported within Rel. 18 timeframe.

And RAN2 provided the following reply in [R2-2306572]:

RAN2 would like to thank SA4 for their LS on N6 PDU Set Identification and inform that RAN can benefit from getting XR related information about End of Burst (1 bit). RAN2 have not identified any other dynamic information that is useful.

Considering that the usefulness of ‘inter-burst time’ have already been discussed during RAN2#122 (Rel-18) and concluded that it was not useful, we do not see the need to repeat the same discussion again.

RAN2 has already discussed ‘inter burst time’ information in RAN2#122 (Rel-18) and concluded that it is not useful.

Additionally, as SA2 suggest in the question, making good use of inter-burst time information is likely difficult considering the jitter between NG-RAN and application server, making the information unreliable to be used for dynamic scheduling purposes. We think RAN2 should reply to SA2 that inter-burst time is not needed.

Dynamic ‘Inter-burst time’ information is likely unreliable considering the jitter between NG-RAN and application server.

RAN2 replies to SA2 that inter-burst time is not useful.

# 3	Discussion on questions from RAN3

The behavior for PSI activation is described in the MAC specification and the PSI feature behavior is described in the PDPC specification.

[MAC specification 38.321]

### 5.18.34	Activation/deactivation of PSI-based SDU discard

The network activates and deactivates PSI-based SDU discard by sending the PSI-Based SDU Discard Activation/Deactivation MAC CE described in clause 6.1.3.73. The PSI-based SDU discard is initially deactivated upon (re-)configuration by upper layers and after reconfiguration with sync.

The MAC entity shall for each DRB configured with discardTimerForLowImportance:

1>	if a PSI-Based SDU Discard Activation/Deactivation MAC CE is received activating the PSI-based SDU discard for the DRB:

2>	indicate the activation of the PSI-based SDU discard for the DRB to upper-layers.

1>	if a PSI-Based SDU Discard Activation/Deactivation MAC CE is received deactivating the PSI-based SDU discard for the DRB:

2>	indicate the deactivation of the PSI-based SDU discard for the DRB to upper-layers.

[PDCP specification 38.323]

### 5.2.1	Transmit operation

At reception of a PDCP SDU from upper layers, the transmitting PDCP entity shall:

-	if discardTimerForLowImportance is configured and PSI based SDU discard is activated, and the PDCP SDU belongs to a low importance PDU Set:

-	start the discardTimerForLowImportance associated with this PDCP SDU;

-	else:

-	start the discardTimer associated with this PDCP SDU (if configured).

NOTE 0:	Identification of PSI of a PDU Set and determination of low importance PDU Set are left up to UE implementation.

The PSI activation/deactivation MAC CE can be sent by the network for each MAC entity and in the MAC CE indicate for each associated DRB if activating or deactivating. When such indication is received, the MAC entity will propagate the indication for each DRB to higher layers.

The PDCP entity corresponding to the DRB will simply act on any PSI activation/deactivation indication received and apply the appropriate behavior, i.e. the correct discard timer. Thus, the PDCP entity will act on the last (de-)activation order provided from lower layers and no special handling is introduced for split bearer.

It should also be noted that RAN2 discussed the initiation of PSI based PDU discarding in multiple meetings, i.e. #123, #124, #125 and #125bis. In these discussion proposals for handling of split bearers were treated, e.g. to add limitations such that activation indication need to be provided for all legs to activate the PSI based discarding. However, because of arguments such as one leg affecting the load on another leg in split bearer and increased complexity by adding dependencies between legs it was agreed in #125bis that no optimization should be introduced for UL split bearers.

[RAN#125bis agreement]

### => No further optimizations will be supported in Rel-18 for UL split-bearer

We propose RAN2 should clarify to RAN3 that the PDCP entity will act on the last (de)activation order received from either of the MN or SN.

RAN2 clarify to RAN3 that the PDCP entity will act on the last (de-)activation order received from either of the MN or SN.

# Conclusion

In the previous sections we made the following observations:

Observation 1	RAN2 has already discussed ‘inter burst time’ information in RAN2#122 (Rel-18) and concluded that it is not useful.

Observation 2	Dynamic ‘Inter-burst time’ information is likely unreliable considering the jitter between NG-RAN and application server.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 replies that dynamic in-band signaling indicating periodicity is useful for optimized resource scheduling and the configuration of features, e.g. DRX, SPS/CG and PDCCH skipping.

Proposal 2	RAN2 replies to SA2 that inter-burst time is not useful.

Proposal 3	RAN2 clarify to RAN3 that the PDCP entity will act on the last (de-)activation order received from either of the MN or SN.

# References

- SA4 LS S4-230739

- RAN2 reply LS R2-2306572