3GPP TSG-RAN WG2 Meeting #127	R2-2407045

# Maastricht, Netherlands, 19th Aug – 23rd Aug 2024

Agenda Item:	8.7.2

Source:	Ericsson

Title:	Discussion on Multi-Modality

Document for:	Discussion, Decision

# 1	Introduction

The XR work item has a study-objective as follows:

- Study and if justified, specify aspects related to multi-modality (intra-UE) (with coordination with SA2/SA4 as needed by LS request). Aim to facilitate efficient and effective support for XR application with Multiple QoS flows with multi-modal inter-dependencies, meeting multi-modal QoS requirements, e.g. synchronization and/or coordination. Efficiency enhancements are expected to be visible in terms of capacity or power consumption. [RAN2]. NOTE: Check in RAN#105 (check also other WG involvement if needed).

This study objective is planned to conclude by the end of this meeting and here we give our view on this objective, including a discussion of DRX enhancements belonging to the multi-modality objective [1], for improving UE energy saving.

# 2	Discussion on Multi-Modality awareness

In the last meeting, RAN2 sent an LS to SA2 to ask what information is available and could be provided to RAN regarding multi-modality. RAN2 has so far not received any answer from SA2 and it is thus unclear what information could be provided, but it is our understanding that among the proposals in RAN2, the MMSID is the information which most companies considered as a candidate. An alternative solution proposed is that, instead of the CN providing the MMSID to RAN, the UE could provide some similar information to RAN in the UAI. In the following sections we summarize the current understanding around these two solutions.

## 2.1         MMSID approach

The MMSID is defined as:

[From: 23.501 (stage 2), V18.5.0]

-	The Multi-modal Service ID is an explicit indication that data flows are related to a multi-modal service. The PCF may use this information to derive the correct PCC rules and to apply appropriate QoS policies for the data flows that are part of a specific multi-modal application.

[From TS 23.503 (stage 2), V18.5.0]

-	Multi-modal Service ID: an identifier that refers to the multi-modal communication service. Data flows belonging to the same multi-modal service share the same Multi-modal Service ID.

According to TS 29.514 the MMSID is a string that is set by the Application Function interfacing the CN (i.e. on the network-side of an application). Currently the MMSID is propagated to the Policy Control Function (PCF) which is a core network node in the 3GPP network, but it is not sent to any other core network node or to RAN. While the actual PCF-behaviour associated with the MMSID is not defined, the assumption is that the MMSID could have impact on the PCF´s policies.

The MMSID is defined and associated with a service (AF session). If one service generates multiple flows, then these flows should be given the same MMSID, as defined in the stage-2 specification and stage 3:

[From: 23.501 (stage 2), V18.5.0]

### 5.37.2	Policy control enhancements to support multi-modal services

A multi-modal service is a communication service that consists of several data flows that relate to each other and that are subject to application coordination. The data flows can transfer different types of data (for example audio, video, positioning, haptic data) and may come from different sources(e.g. a single UE, a single device or multiple devices connected to the single UE, or multiple UEs).

For the single UE case, it is expected that those data flows are closely related and require strong application coordination for the proper execution of the multi-modal application and therefore, all those data flows are transmitted in a single PDU session.

The Nnef_AFsessionWithQoS service allows the AF to provide, at the same time, for each data flow that belongs to the multi-modal service, a Multi-modal Service ID, the service requirements and the QoS monitoring requirements:

-	The Multi-modal Service ID is an explicit indication that data flows are related to a multi-modal service. The PCF may use this information to derive the correct PCC rules and to apply appropriate QoS policies for the data flows that are part of a specific multi-modal application.

-	The AF may provide QoS monitoring requirements for data flows associated to a multi-modal service to the  PCF . The PCF generates the authorized QoS Monitoring policy for each data flow.

[From: 29.514 Policy Authorization Service; stage 3 release 18]

#### 4.2.2.37	Provisioning of multi-modality services

This procedure is used by a NF service consumer to:

-	provide service requirements for each media flow that comprise the multi-modal service; and/or

-	provide QoS monitoring requirements for each media flow that comprise the multi-modal service;

when "MultiMedia" feature is supported.

The NF service consumer may include the multi-modal Service Identifier within the "multiModalId" attribute to indicate that the new AF session relates to a multi-modal service.

NOTE:	For the delivery of multi-modal services, the NF service consumer may request to the PCF multiple media flows for a single UE or for multiple UEs via multiple PDU Sessions and separate request(s) per PDU Session with the same "multiModalId" attribute.

To provide service requirements for a multi-modal service, the NF service consumer shall follow the procedures described in clause 4.2.2.2 for initial provisioning of service information, with the following additional considerations:

-	When the multi-modal service combines several media, the NF service consumer shall provide the service information of each media within the "medComponents" attribute. The media subcomponent(s), when provided for a media component, only contain the description of the service data flow(s).

As outlined in the stage-2/-3 specifications, the MMSID is not itself associated with one particular flow rather it is associated with an AF session carrying one or more data flows.

MMSID can indicate if an AF session and associated flows are multi-modal.

However, according to the SA2 specification, the MMSID is a string used to configure local QoS policies in the PCF. As such, it is not assumed to convey any concrete dependency information between the flows nor to convey any specific requirements associated with any particular flow. Since the MMSID does not convey any requirement of common handling of flows, performing common handling based on it (e.g. joint admission control) in RAN may interfere with application level built in mechanisms.

The MMSID is not assumed to convey dependency information between flows.

The MMSID does not convey requirements associated with any particular flow.

Common flow handling in RAN based on MMSID may interfere with application level built in mechanisms.

It was discussed at the last RAN2 meeting, and also indicated to SA2 in the LS, that RAN2 are considering if the MMSID could be sent to RAN so that RAN could consider it in its internal procedures.

Procedures for common handling (e.g. joint admission control), QoS coordination and media synchronization in the core network were discussed in SA2 Rel-18 work item FS_XRM. Proposals can be found in 3GPP TR 23.700-60 (see for example solution 36 in clause 6.36, solution 38 in clause 6.38, solution 65 in clause 6.65, etc). However, no agreement was reached to pursue such solutions, but rely on application layer mechanisms, as proposed in solution 40 and solution 62.

No mechanisms for common handling, QoS coordination and media synchronization of multi-modal services have been specified in SA2.

## 2.2         UE Assistance Information approach

Another proposed approach in the same spirit of the MMSID is that the UE could send information similar to, but independent of the MMSID to the gNB. The UE could send this information in the UE Assistance Information message. In contrast to the MMSID which is generated in the Application Function (i.e. server side), the UE side (i.e. often the client) is not the one who sets or controls the MMSID. Our understanding of the proposal for UE Assistance Information is therefore to leave completely up to UE implementation how this UE assistance information is generated.

We understand though that the wanted use case for this information in the UAI would be the same as the proposals for the MMSID, i.e. some multi-modal information which is indicated from the UE to the gNB, which the gNB can use as input in various procedures.

Multi-modal information in the UAI is a similar but alternative solution to providing MMSID to RAN.

Multi-modal information provided in the UAI will be up to UE implementation how to generate.

RAN2 to take the above observations into account when making decisions on the multi-modality awareness aspects.

# 3	Discussion on DRX Enhancements for XR

## 3.1 Handling multiple flows with multiple active DRX configurations

The interleaved pattern of multiple (quasi-)periodic XR flows that are running in parallel creates irregular and short sleep opportunities. Current C-DRX solutions allow the NW to configure only one DRX configuration per serving cell, which cannot match the traffic characteristics of all XR flows. Thus, it cannot closely follow the short sleep opportunities and results in either low power saving gains, or in a long traffic delay.

Our proposed solution is to support multiple active DRX configurations. With this solution, the NW provides multiple DRX configurations to a UE, all running in parallel. By doing so, the network could use XR traffic information such as traffic periodicity or jitter characteristics, provided for instance by the application, to configure DRX in a way that the DRX parameters in each configuration match each individual (quasi-)periodic traffic flow. The UE monitors the PDCCH while the drx-onDurationTimer (or drx-InactivityTimer, or Active Time) is running in any of the DRX configurations.

An example is shown in Figure 1, for two DRX configurations that are selected to match the parameter values (periodicity and alignment) of two traffic flows. The DL time slots in the TDD pattern in which the UE monitors the PDCCH are shaded in black. Thus, the multiple active DRX solution is able to follow the irregular time gaps due to the interleaved traffic flows and save UE power, while ensuring a low delay.

Figure 1. Illustration of multiple active DRX with two configurations, each matching a DL traffic flow: the cycle and drx-onDurationTimer in green follow the video flow (16.66 ms period) and the DRX configuration in magenta follows the audio flow (10 ms period). The video traffic experiences a random jitter of ±4 ms with a truncated Gaussian distribution around the mean traffic period.

Table 1, Table 2, and Table 3 show example simulation results for two downlink traffic flows (video and audio) and one UL flow (pose). The difference between the tables consists in the video traffic periodicity and audio packet delay budget (PDB) – underlined in the table titles. For all results, the simulation methodology in [2] and the assumptions in Annex A are considered. The UE power saving gain (PSG) is estimated vs. Always On. According to the capacity definition in [2], there need to be at least 90% satisfied UEs, where a UE is satisfied if all its flows meet their own PDB requirements (99% of the application packets transmitted within their PDB).

Table 1 Results for 8 UEs/cell and VR multi-stream traffic: DL video (60 fps, 30 Mbps, ±4 ms jitter, 10 ms PDB), DL audio (10 ms periodicity, 30 ms PDB), UL pose (4 ms periodicity, 10 ms PDB)

| Power Saving Scheme | DRX cycle (ms) | drx-onDurationTimer (ms) | drx-InactivityTimer (ms) | % of satisfied UE | Mean PSG of all UEs | Mean PSG of satisfied UEs |
| --- | --- | --- | --- | --- | --- | --- |
| Always On | - | - | - | 92.1% | - | - |
| R15/16 Long DRX | 10 | 8 | 4 | 85.5% | 5.7% | 5.8% |
| R15/16 Short DRX | 4 | 2 | 4 | 82.9% | 6.7% | 6.8% |
| Rel-18 non-integer DRX | 16.66 | 10 | 4 | 89.6% | 8.1% | 8.2% |
| Multiple active DRX | 16.66 | 10 | 4 | 91.1% | 6.0% | 6.0% |
|  | 10 | 2 | 4 |  |  |  |

Table 2 Results for 8 UEs/cell and VR multi-stream traffic: DL video (60 fps, 30 Mbps, ±4 ms jitter, 10 ms PDB), DL audio (10 ms periodicity, 10 ms PDB), UL pose (4 ms periodicity, 10 ms PDB)

| Power Saving Scheme | DRX cycle (ms) | drx-onDurationTimer (ms) | drx-InactivityTimer (ms) | % of satisfied UE | Mean PSG of all UEs | Mean PSG of satisfied UEs |
| --- | --- | --- | --- | --- | --- | --- |
| Always On | - | - | - | 92.1% | - | - |
| R15/16 Long DRX | 10 | 8 | 4 | 85.5% | 5.7% | 5.8% |
| R15/16 Short DRX | 4 | 2 | 4 | 82.9% | 6.7% | 6.7% |
| Rel-18 non-integer DRX | 16.66 | 10 | 4 | 63.5% | 8.1% | 14.0% |
| Multiple active DRX | 16.66 | 10 | 4 | 91.1% | 6.0% | 6.0% |
|  | 10 | 2 | 4 |  |  |  |

Table 3 Results for 5 UEs/cell and VR multi-stream traffic: DL video (30 fps, 30 Mbps, ±4 ms jitter, 10 ms PDB), DL audio (10 ms periodicity, 10 ms PDB), UL pose (4 ms periodicity, 10 ms PDB)

| Power Saving Scheme | DRX cycle (ms) | drx-onDurationTimer (ms) | drx-InactivityTimer (ms) | % of satisfied UE | Mean PSG of all UEs | Mean PSG of satisfied UEs |
| --- | --- | --- | --- | --- | --- | --- |
| Always On | - | - | - | 94.3% | - | - |
| R15/16 Long DRX | 10 | 8 | 4 | 89.0% | 6.3% | 6.3% |
| R15/16 Short DRX | 4 | 2 | 4 | 87.4% | 8.1% | 8.1% |
| Rel-18 non-integer DRX | 33.33 | 10 | 4 | 0% | 19.6% | - |
| Multiple active DRX | 33.33 | 10 | 4 | 93.4% | 14.1% | 14.3% |
|  | 10 | 2 | 0 |  |  |  |

The results in all tables show that none of the legacy Long DRX, Short DRX, or non-integer DRX can reach 90% satisfied UEs. All these legacy variants thus decrease the XR capacity compared to Always On. The most severe case is in Table 3 (video 30 fps and audio with 10 ms PDB), where non-integer DRX results in 0% satisfied UEs. By contrast, multiple active DRX always exceeds 90% satisfied UEs and can also save UE power up to 14.3%.

A single DRX configuration matched to a traffic flow may not be suitable to fulfil the PDBs of other traffic flows, resulting in down to zero capacity.

Multiple active DRX configurations, each matching a traffic flow, are suitable to achieve both high UE power saving gains and many satisfied UEs, if a single DRX configuration matched to one flow does not satisfy the PDBs of other flows.

Support multiple active DRX configurations to limit the delay and optimize power saving of UEs with multi-flow XR services.

## 3.2 Relaxing the limitation for configuring secondary DRX group

Two DRX groups can be configured per UE in case of carrier aggregation (CA). However, according to the MAC specifications, only the drx-onDurationTimer and drx-InactivityTimer can be configured with different values for the two DRX groups, whereas the other DRX parameter values must be the same [4]:

When two DRX groups are configured, each Serving Cell is uniquely assigned to either of the two groups. The DRX parameters that are separately configured for each DRX group are: drx-onDurationTimer, drx-InactivityTimer. The DRX parameters that are common to the DRX groups are: drx-SlotOffset, drx-RetransmissionTimerDL, drx-RetransmissionTimerUL, drx-LongCycleStartOffset, drx-NonIntegerLongCycleStartOffset, drx-ShortCycle (optional), drx-NonIntegerShortCycle (optional), drx-ShortCycleTimer (optional), drx-HARQ-RTT-TimerDL, and drx-HARQ-RTT-TimerUL.

This is too restrictive and we show that enhancing the DRX groups to allow independent parameter values per DRX group is beneficial for XR traffic.

As a DRX group enhancement example, we consider different drx-StartOffset values for the different DRX groups. This allows to handle video jitter when CA is employed, by better covering the expected arrival of the video packets, as follows. The first DRX group can be configured for e.g. Serving Cell_1 with a long drx-onDurationTimer that covers the entire jitter range, to limit delay. The secondary DRX group can be configured for e.g. Serving Cell_2 to have a shorter drx-onDurationTimer and an enhanced and shifted drx-StartOffset, to cover the second half of the jitter range. This saves UE power by not monitoring Serving Cell_2 during the first half of the jitter range, and does not increase the traffic delay too much, since the highest probability of traffic arrival is in the middle of the jitter range.

Table 4 shows simulation results for two downlink traffic flows (video and audio) and one UL flow (pose). As in Section 3.1, the simulation methodology in [2] and the assumptions in Annex A are considered. The UE power saving gain (PSG) is estimated vs. Always On. According to the capacity definition in [2], there need to be at least 90% satisfied UEs, where a UE is satisfied if all its flows meet their own PDB requirements (99% of the application packets transmitted within their PDB).

Table 4 Results for FR1+FR2 CA, 4 UEs/cell and VR multi-stream traffic: DL video (60 fps, 60 Mbps, ±4 ms jitter, 10 ms PDB), DL audio (10 ms periodicity, 30 ms PDB), UL pose (4 ms periodicity, 10 ms PDB)

| Power Saving Scheme | DRX cycle (ms) | drx-onDurationTimer (ms) | drx-InactivityTimer (ms) | % of satisfied UE | Mean PSG of all UEs | Mean PSG of satisfied UEs |
| --- | --- | --- | --- | --- | --- | --- |
| Always On | - | - | - | 93.6% | - | - |
| Legacy 1 DRX group | 16.66 | 10 | 4 | 93.3% | 16.0% | 16.1% |
| Legacy 2 DRX groups | Both groups: 16.66 | Group 1: 10 Group 2: 5 | Group 1: 4 Group 2: 2 | 83.8% | 26.8% | 27.2% |
| Enhanced 2 DRX groups (independent drx-StartOffset): 3 ms delayed cycle start of group 2 versus group 1 | Both groups: 16.66 | Group 1: 10 Group 2: 5 | Group 1: 4 Group 2: 2 | 90.0% | 27.5% | 27.7% |

The results show that with the enhancement of independent configuration parameters per DRX group, a significant PSG of around 27% can be achieved, while also increasing the percentage of satisfied UEs versus the legacy case of two DRX groups with only drx-onDurationTimer and drx-InactivityTimer being different among the two groups. The high PSG is due to the shorter drx-onDurationTimer for the secondary DRX group and the high percentage of satisfied UEs is due to the different shifted drx-StartOffset for the secondary DRX group, which covers the second half of the video jitter range and thus limits the delay.

Having different drx-StartOffsets for the DRX groups can improve the number of satisfied UEs when the groups have different drx-onDurationTimer and drx-InactivityTimer.

Support independent configuration parameters for the secondary DRX group.

# Conclusion

In the previous sections we made the following observations:

Observation 1	MMSID can indicate if an AF session and associated flows are multi-modal.

Observation 2	The MMSID is not assumed to convey dependency information between flows.

Observation 3	The MMSID does not convey requirements associated with any particular flow.

Observation 4	Common flow handling in RAN based on MMSID may interfere with application level built in mechanisms.

Observation 5	No mechanisms for common handling, QoS coordination and media synchronization of multi-modal services have been specified in SA2.

Observation 6	Multi-modal information in the UAI is a similar but alternative solution to providing MMSID to RAN.

Observation 7	Multi-modal information provided in the UAI will be up to UE implementation how to generate.

Observation 8	A single DRX configuration matched to a traffic flow may not be suitable to fulfil the PDBs of other traffic flows, resulting in down to zero capacity.

Observation 9	Multiple active DRX configurations, each matching a traffic flow, are suitable to achieve both high UE power saving gains and many satisfied UEs, if a single DRX configuration matched to one flow does not satisfy the PDBs of other flows.

Observation 10	Having different drx-StartOffsets for the DRX groups can improve the number of satisfied UEs when the groups have different drx-onDurationTimer and drx-InactivityTimer.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to take the above observations into account when making decisions on the multi-modality awareness aspects.

Proposal 2	Support multiple active DRX configurations to limit the delay and optimize power saving of UEs with multi-flow XR services.

Proposal 3	Support independent configuration parameters for the secondary DRX group.

# References

3GPP RP-240791, “Revised WID on XR (eXtended Reality) for NR Phase 3”, 3GPP TSG RAN Meeting #103, March 2024.

TR 38.838

TR 38.840

TS 38.321, V18.2.0

# Annex A: Simulation assumptions

For all simulation results, dynamic grant (DG) is used for the DL video flow and DL audio flow, while configured grant (CG) is configured for the UL pose flow, so this flow does not affect the DRX configuration.

The UE power consumption model is adopted from [3], with the additional UL power modelling in [2], namely linear interpolation method in linear scale for UE Tx power values other than 0 dBm and 23 dBm. Other parameters are summarized in Tables A.1 and A.2.

Table A.1: System simulation parameters

| Parameter | Deployment scenarios |
| --- | --- |
|  | Dense Urban (38.913 w/ following parameters) |
| Layout | 9cells with wraparound ISD: 200m |
| Channel model | UMa (38.901) |
| UE Distribution | 80% indoor, 20% outdoor single carrier/ 100% Outdoor carrier aggregation FR1+FR2 |
| Carrier frequency | 4 GHz FR1/ 28 GHz FR2 |
| Subcarrier spacing | 30 kHz FR1/ 120 kHz FR2 |
| BS height | 25m |
| UE height | For Dense urban and Urban Macro, the UE height for indoor UEs is updated as following based on Table 6-1 in TR 36.873. UE height (hUT) in meters; general equation for UE height; hUT=3(nfl – 1) + 1.5. nfl for outdoor UEs; 1. nfl for indoor UEs; nfl ~ uniform(1,Nfl) where Nfl ~ uniform(4,8). |
| BS noise figure | 5 dB |
| UE noise figure | 9 dB |
| BS receiver | MMSE-IRC |
| UE receiver | MMSE-IRC |
| Channel estimation | Realistic |
| UE speed | 3 km/h |
| MCS | Up to 256QAM |
| BS Antenna Pattern | 3-sector antenna radiation pattern, 8 dBi |
| BS Antenna Configuration | 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) (dH, dV) = (0.5λ, 0.5λ), (Carrier aggregation FR1/FR2 32/32 TxRU) |
| UE Antenna Pattern | Omni-directional, 0 dBi |
| UE Antenna Configuration | 2T/4R, (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1;1,2), (dH, dV) = (0.5, N/A)λ |
| Down Tilt | 12 degrees |
| BS Transmit Power | 44 dBm per 20 MHz Note: For system BW larger than above, Tx power scales up accordingly. |
| UE max tx power | 23dBm |
| System Bandwidth | 100 MHz |

Table A.2: Other assumptions

| TDD Configuration | DDDSU single carrier/ DDDU carrier aggregation FR1+FR2 |
| --- | --- |
| Scheduler | SU-MIMO PF scheduler |
| PHY processing delay | UE PDSCH processing Capability #1 DL NACK to retransmission delay 1.5ms |
| DMRS overhead | 1 DMRS symbol per PDSCH |
| Power control parameter | alpha: 0.8 |
| Transmission scheme | Reciprocity-based precoding |
| Configured Grant | On |