3GPP TSG-RAN WG2 Meeting #127	R2-2407047

# Maastricht, Netherlands, 19th August – 23rd August 2024

Agenda Item:	8.7.4

Source:	Ericsson

Title:	Discussion on scheduling enhancements

Document for:	Discussion, Decision

# 1	Introduction

The XR WID [1] for Rel-19 contains the following objective on UL scheduling enhancements:

- Specify Enhancements for Scheduling, as follows:

  - For the UL, Study and if justified, Specify enhancements using delay/deadline information, for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs. [RAN2].

    - Note: LCP implementation complexity need to be taken into account when evaluating solutions.

In RAN2#126 the following agreements were made on the scheduling objective with LCP impacts:

- Delay-aware LCP enhancement to resolve the issue of data with low remaining time being delayed due to data from other LCHs with no delay critical data is supported in Rel-19 XR.

- The solution should consider impact on UE complexity (as already indicated in SI objective description)

- For delay-aware LCP enhancement, RAN2 considers the following option to override/adjust the priority of LCH based on delay/deadline information as a baseline:

- - Use additional priority configured to LCHs in case of these LCHs with delay-critical data

- FFS whether the priority only applies to delay-critical data within the LCH or for the whole LCH

- We try to avoid RAN1 impacts.

- RAN2 assumes no dynamic indications are needed for triggering the delay-aware LCP mechanism. RAN2 assumes this mechanism is configured in a semi-static way.

- For LCP restrictions based solutions, RAN2 will not discuss solutions requiring RAN1 work. FFS whether other LCP restrictions based approaches are needed/beneficial

- The solutions should not disallow non-delay critical data from using an UL grant.

- LCP prioritization within a logical channel will not be considered in RAN2 discussions

- FFS whether a separate remaining time threshold can be configured for delay aware LCP (i.e. different from the one used for DSR).

In RAN2#126 the following agreements were made on the scheduling objective with DSR impacts:

- Enhance DSR to report with multiple pairs of remaining time and buffer size for the LCG.

- FFS whether DSR triggering is impacted

- FFS whether PDU set importance needs to be included

This contribution investigates the impacts on XR capacity when enhancing the LCP mechanism with delay aspects and propose ways forward for DSR enhancements connected to existing issues with the DSR feature.

# 2	Discussion on DSR enhancements

In RAN2#126 it was agreed that DSR will be enhanced to report multiple pairs of remaining time and buffer sizes in Rel19. This will solve the issues brought up with the Rel18 DSR solution having low granularity of reporting. Essentially this will allow the DSR to give a detailed report to the network when there is delay critical data that has arrived at different time instances, e.g. multiple PDU Sets in the queue. This will also enable to configure a high triggering threshold, i.e. early reporting of new delay critical data, but still give a detailed report with the more complete picture of the delay critical data in the UE buffer. What remains to be decided is how the multiple pairs of information are selected, the triggering impacts of the new DSR behaviour and if any further information should be included. This paper will go into details of these aspects.

## 2.1 Selection of information pairs in the DSR

The Rel18 DSR solution is based on a reporting threshold where any data with remaining time below the configured threshold value will be reported as delay critical data. The most straight forward solution for multiple reporting pairs is to extend this solution to multiple reporting thresholds, where each value range between two consecutive thresholds indicates an information pair in the DSR delay. Naturally as in the Rel18 solution the lowest remaining time inside the range would be reported together with the summation of all the buffer size of the data that has a remaining time inside the range. A solution based on multiple thresholds would limit the size of the DSR as there will be a maximum number of ranges, i.e. information pairs, to be included in the report. This solution would also support both PDU Sets, with a single discard timer each for all belonging PDUs, and non-PDU Sets with PDUs spread out in time and many discard timers running in parallel with different expiry time.

As an example, data with time left between 0 ms and 10 ms belong to delay critical data in the first information pair while data with time left between 11 ms and 20 ms belong to the delay critical data in the second information pair. This example is visualised in Figure 1.

Network should be able to configure multiple remaining time thresholds for each LCG to report multiple pairs of remaining time and buffer sizes per LCG.

Figure 1: Multiple remaining time thresholds mapping to multiple information pairs in the DSR.

Above is the baseline solution working for most situations but it needs to be acknowledged that there can be scenarios when the UE buffer is not ordered according to the discard timer (as outlined with more detail in section 3.1 below). In those instances, it would be misleading to only report the lowest remaining time to the network without some indication that this data will not be delivered first. We proposed in an earlier meeting (R2-2403225) to also indicate the transmission order in the DSR. However a simpler solution for these situations would be to simply multiplex the data into one buffer value. In this way the network will at least know how much data that need to be scheduled to support the lowest remaining time value reported. An example of such situation is shown in Figure 2 where the queue in the UE is not ordered by remaining time, i.e. the first packets (PDU Set 1) in the queue has a longer remaining time than the second set of packets (PDU Set 2). In this case PDU Set 2 is below the first threshold (10 ms) and report the lowest remaining time while PDU Set 1, which the UE has decided to place first in queue and thus transmit first, should be included in the buffer size for the first threshold and thus not report any further remaining time value. Any other data placed behind in the queue (in the example PDU Set 3) and belonging to other thresholds can be reported as in the baseline case, e.g. by including a new information pair.

Figure 2: UE with a queue not ordered by the remaining time with three remaining time thresholds and three PDU Sets.

Any data in front of the queue with longer remaining time than the data behind in the queue should report the lowest remaining time and total buffer size.

## 2.2 Triggering of the DSR

Multiple reporting thresholds allow for an easy way to extend the triggering mechanism by selecting which reporting thresholds trigger a DSR. In the Rel18 solution when data becomes delay critical, i.e. remaining time falls below the threshold, a DSR is triggered to report the delay critical data. With multiple reporting thresholds a simple solution is to make it configurable which of the reporting thresholds that are triggering a DSR, i.e. as soon as remaining time of any data fall below the threshold a new DSR is triggered. In this way the network can decide when it deems it useful for the DSR to be triggered. An example shown of such configuration is shown in Figure 3.

Make it network configurable which are the remaining time thresholds that will trigger a DSR.

Figure 3: Example of DSR triggering with multiple thresholds. Threshold 1 is configured at 26 ms while Threshold 2 is configured at 6 ms.

## 2.3 Other information in the DSR

### 2.3.1 Issue with importance levels in the DSR

The Rel-18 DSR solution has an identified issue with reporting when having different importance levels in the UE buffer, which is visualized in Figure 4. The scenario shown highlights the case when two PDU Sets with different importance have arrived in the UE buffer in such a way that they have become interleaved. In Rel-18 only the high importance data is reported as delay critical data in the DSR and thus the existence of the low importance PDUs, with remaining time below the Remaining Time Threshold, will be transparent to the network, i.e. in the depicted scenario the network will only get knowledge of the 4 high importance PDUs being delay critical while in reality there are 7 PDUs in total below the threshold. When network provides a grant for 4 PDUs only 2 high importance PDUs will be scheduled since 2 other low importance PDUs will use up the grant resources.

Figure 4: Issue with interleaved PDU Sets and transmission order.

DSR has issues supporting PDU Sets with multiple importance levels in the UE buffer.

### 2.3.2 DSR with importance indication

Enhancing the DSR to report multiple pairs doesn’t solve the issue with low importance levels being transparent to the network. A possible solution is to also report the low importance values in the DSR. However a concern brought up has been that reporting the low importance data with short discard timers will trigger a DSR with very urgent remaining time values without the network knowing if this is really critical data to serve fast. To remedy such a situation the DSR could be enhanced to report the importance information associated to the pair of remaining time and buffer size. By providing the importance information the network can take the appropriate scheduling decision, i.e. even if a low remaining time is indicated, if the data is also of low importance, the scheduling may not need to be performed as urgent and the network may let this data expire/be discarded in case of overload when some other UE has more urgent data which needs to be scheduled.

Since only two levels of importance are used for the PSI discarding solution, only one bit indication is required for each information pair in the DSR. As the DSR format already contains a reserved R-bit associated to the remaining time and buffer size values, it is a simple change to use this bit for reporting the importance level, i.e. if the remaining time is reported for the high importance data (the discardTimer is used) or the low importance data (the discardTimerForLowImportance is used). If multiple importance levels reside inside the threshold, an appropriate rule can be applied, e.g. the highest importance level is indicated.

Two importance levels to be included in the DSR and the importance level is indicated in the DSR format using a new I-bit instead of the previous R-bit.

An example of an enhanced DSR format is shown in Figure 5. In this format multiple information pairs are used for any LCG that has delay critical data with remaining time belonging to different thresholds. Furthermore, instead of the R-bit there is an I-bit indicating the importance of the associated remaining time and buffer size.

Figure 5: New DSR format with multiple rows/pairs for each LCG. The report indicates the importance level of the reported buffer size and remaining time.

RAN2 to agree on the enhanced DSR format provided in Figure 5.

# 3	Discussion on LCP enhancements

## 3.1 LCP enhancement considerations

When considering LCP enhancements to improve the prioritization of data with short remaining time there are some important aspects that should be taken into consideration:

- Certain data will always require highest priority, e.g. pose traffic, and should always be prioritized first, even when there exist other time critical data in the buffers (the negative impact on XR capacity by not doing such prioritization can be shown through system simulations).

- In current LCP mechanism not all time critical data may be given priority and instead other (less time-critical) data in other LCH will be prioritized.

- Packets may arrive at different times and PDU Sets may even become interleaved in the LCH queue, i.e. the remaining time of packets is not always strictly following the place in the queue. It is even possible that multiple different traffic flows are mapped to the same LCH, potentially increasing the issue with interleaved PDU Sets even more.

- It was agreed that prioritization inside a LCH should not be considered in RAN2 discussion but for completeness it should be noted that above makes it also possible to have both delay critical and non-delay critical packets in the same LCH. Even though configuration can avoid such cases by restricting to only map flows with similar characteristics and requirements to the same LCH, that is an implementation aspect. In a worst case scenario a LCH can contain both delay critical and non-delay critical packets. Then it is possible that the non-delay critical packets are ahead in the queue and will get the benefit of increased priority for the LCH, see some examples in Figure 6.

Figure 6: Examples of worst case scenario where delay critical and non-delay critical packets can be in the same LCHs

If we only want delay critical packets to benefit from increased priority for an LCH, then delay critical and non-delay critical packets can be mapped into separate LCHs as shown in Figure 7.

Figure 7: Delay critical and non-delay critical packets in same or separate LCHs

Another alternative is that the UE itself chooses to transmit delay critical data from a LCH before non-delay critical data, even if the non-delay critical data is ahead in the queue, meaning that the UE itself reorders the packets. However, this is something that is left for UE implementation.

The above aspects and their impacts on the performance of the solution should be given thorough consideration when designing the new LCP solution.

A delay-aware LCP solution should allow protecting transmission of selected data, e.g. pose could be configured to always have a higher priority than the time critical data.

A delay-aware LCP solution should enable fast transmission of the time critical data when possible.

A delay-aware LCP solution should take into consideration that different PDU Sets and traffic flows may be interleaved in a LCH.

There can be both delay critical and non-delay critical packets in a LCH. However, how to handle intra-LCH prioritization is left to UE implementation.

## 3.2 Delay-aware LCP enhancement

A simple solution with low UE complexity is to adjust the priority of selected LCH based on the remaining time of the data in the LCH queue, i.e. introduce dynamic priority of selected LCH. This priority adjustment can be done in advance in the UE, i.e. before the actual selection of LCH and resource allocation and thus has low impact on the UE processing during the time critical procedures. The solution can work such that as soon as any data in the LCH pass the configured remaining time threshold, i.e. data becomes time critical, the scheduling priority for the LCH can be adjusted to a configured higher priority value. This priority value is then used in the legacy LCP mechanism when selecting between the LCHs based on the priority values. The priority values and time threshold should be configured for each LCH by the network which allows for the appropriate prioritization. When there is no more time critical data the priority of the LCH is reverted to the original priority value. The time threshold should be different than the threshold used in the DSR, since the threshold to indicate the delay critical data in a DSR is used for different purpose. Reporting of delay critical data through the DSR can be used by the network to get early indication to do appropriate scheduling but not necessarily to schedule the delay critical data in the UE directly, since there may still be time for other data to be served before the delay critical data. Furthermore, DSR may now be enhanced with multiple reporting thresholds in Rel19, which is another indication that the DSR thresholds have a different purpose than the one used in the delay-aware LCP enhancement.

Lastly to make sure that there is timely transmission of the time critical data another stop condition should be introduced in the delay-aware LCP solution. This can be done by applying a new counter value to keep track of the transmission of the time critical data, so that only when all of the time critical data is transmitted from the LCH the resource allocation move on to another lower priority LCH. This procedure assures that the time critical data is being included in the transmission before any other low important data.

The described delay-aware LCP solution make it possible to remedy the aspects brought up in 3.1, as with proper implementation and configuration the time critical data is prioritized and transmitted as quickly as possible but not at the expense of any potentially higher priority data in the buffer. We thus propose following for the delay-aware LCP enhancements.

A delay-aware LCP solution can configure a LCH with a time threshold to determine if there is time critical data in the buffer. This threshold is different than the thresholds used in the DSR reporting.

A LCH can be configured with an alternative priority value to be applied when there is time critical data in the buffer.

A delay-aware LCP solution uses new counter variables to keep track of the scheduling of the time critical data.

# 4	Conclusion

In the previous sections we made the following observations:

Observation 1	DSR has issues supporting PDU Sets with multiple importance levels in the UE buffer.

Observation 2	There can be both delay critical and non-delay critical packets in a LCH. However, how to handle intra-LCH prioritization is left to UE implementation.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Network should be able to configure multiple remaining time thresholds for each LCG to report multiple pairs of remaining time and buffer sizes per LCG.

Proposal 2	Any data in front of the queue with longer remaining time than the data behind in the queue should report the lowest remaining time and total buffer size.

Proposal 3	Make it network configurable which are the remaining time thresholds that will trigger a DSR.

Proposal 4	Two importance levels to be included in the DSR and the importance level is indicated in the DSR format using a new I-bit instead of the previous R-bit.

Proposal 5	RAN2 to agree on the enhanced DSR format provided in Figure 5.

Proposal 6	A delay-aware LCP solution should allow protecting transmission of selected data, e.g. pose could be configured to always have a higher priority than the time critical data.

Proposal 7	A delay-aware LCP solution should enable fast transmission of the time critical data when possible.

Proposal 8	A delay-aware LCP solution should take into consideration that different PDU Sets and traffic flows may be interleaved in a LCH.

Proposal 9	A delay-aware LCP solution can configure a LCH with a time threshold to determine if there is time critical data in the buffer. This threshold is different than the thresholds used in the DSR reporting.

Proposal 10	A LCH can be configured with an alternative priority value to be applied when there is time critical data in the buffer.

Proposal 11	A delay-aware LCP solution uses new counter variables to keep track of the scheduling of the time critical data.

# References

3GPP RP-240791, “Revised WID on XR (eXtended Reality) for NR Phase 3”, 3GPP TSG RAN Meeting #103, March 2024.