3GPP TSG-RAN WG2 Meeting #127	R2-2407052

Maastricht, Netherlands,  Aug 19th – 23rd, 2024

Agenda item:	8.10.2

Source:	Samsung

Title:	MRO enhancements for Rel-18 mobility features

Document for:	Discussion and Decision

# Introduction

As per [1], one of the objectives of R19 SON WI is to support the MRO enhancement for R18 mobility mechanisms, including, Lower layer triggered mobility (LTM), CHO with candidate SCGs and subsequent CPAC. From RAN2, this involves identification and specification of necessary UE reporting to enhance the mobility parameter tuning as given below.

The objective of this work item is to specify data collection enhancement in NR standalone and MR-DC for SON/MDT purpose. The specific objectives of this work item are: - MRO enhancement for R18 mobility mechanisms, including, Lower layer triggered mobility (LTM), CHO with candidate SCGs, subsequent CPAC [RAN3, RAN2]: - Specification of the inter-node information exchange, including possible enhancements to interfaces [RAN3] - Identify and specify necessary UE reporting to enhance the mobility parameter tuning [RAN2]

# Discussion

## MRO for subsequent CPAC

In RAN2#126, the following agreement was made.

RAN2 will look into failure and near failure scenarios for subsequent execution of SCPAC and see if/what enhancements are needed.

Subsequent CPAC allows subsequent cell group change after changing cell group without reconfiguration and re-initiation of CPC/CPA. Like legacy CPAC, robustness of subsequent CPAC also depends on the configuration of thresholds, offsets, timers etc. related to measurement reporting and mobility. Thus it is straightforward that R18 SCGFailureInformation and SPR are the baseline for MRO for subsequent CPAC.

Proposal 1-1: R18 MRO for SCGFailureInformation/SPR is the baseline for MRO for subsequent CPAC.

Now let us look at the differences between CPAC and subsequent CPAC.

Subsequent CPAC introduces condExecutionCondToAddModList and condExecutionCondToReleaseList as shown below.

CondReconfigToAddModList-r16 ::= SEQUENCE (SIZE (1.. maxNrofCondCells-r16)) OF CondReconfigToAddMod-r16

CondReconfigToAddMod-r16 ::=     SEQUENCE {

condReconfigId-r16               CondReconfigId-r16,

condExecutionCond-r16            SEQUENCE (SIZE (1..2)) OF MeasId                      OPTIONAL,    -- Need M

condRRCReconfig-r16              OCTET STRING (CONTAINING RRCReconfiguration)          OPTIONAL,    -- Cond condReconfigAdd

...,

[[

condExecutionCondSCG-r17         OCTET STRING (CONTAINING CondReconfigExecCondSCG-r17) OPTIONAL     -- Need M

]],

[[

condExecutionCondPSCell-r18      SEQUENCE (SIZE (1..2)) OF MeasId                      OPTIONAL,    -- Cond condReconfigCHO-WithSCG

subsequentCondReconfig-r18       SubsequentCondReconfig-r18                            OPTIONAL,    -- Need M

securityCellSetId-r18            SecurityCellSetId-r18                                 OPTIONAL,    -- Need M

scpac-ConfigComplete-r18         ENUMERATED {true}                                     OPTIONAL     -- Cond CPAC

]]

}

CondReconfigExecCondSCG-r17 ::=  SEQUENCE (SIZE (1..2)) OF MeasId

SubsequentCondReconfig-r18 ::=   SEQUENCE {

condExecutionCondToReleaseList-r18   CondExecutionCondToReleaseList-r18                OPTIONAL,   -- Need N

condExecutionCondToAddModList-r18    CondExecutionCondToAddModList-r18                 OPTIONAL,   -- Need N

...

}

CondExecutionCondToAddModList-r18 ::= SEQUENCE (SIZE (1.. maxNrofCondCells-r16)) OF CondExecutionCondToAddMod-r18

CondExecutionCondToAddMod-r18 ::=    SEQUENCE {

subsequentCondReconfigId-r18         CondReconfigId-r16,

subsequentCondExecutionCond-r18      SEQUENCE (SIZE (1..2)) OF MeasId                      OPTIONAL,    -- Need M

subsequentCondExecutionCondSCG-r18   OCTET STRING (CONTAINING CondReconfigExecCondSCG-r17) OPTIONAL,    -- Need M

...

}

CondExecutionCondToReleaseList-r18 ::= SEQUENCE (SIZE (1.. maxNrofCondCells-r16)) OF CondReconfigId-r16

-- TAG-CONDRECONFIGTOADDMODLIST-STOP

A UE may be configured with conditional reconfiguration configuration without execution conditions. In R18, a UE considers whether a neighbour cell is a CPAC candidate cell based on whether condRRCReconfig is configured or not. With subsequent CPAC, it is possible for the network to release the execution conditions and keep condRRCReconfig-r16. Unless the execution conditions are configured, UE does not evaluate those cells for CPAC. Therefore, the UE needs to report to the network a cell as a candidate cell, only when the execution conditions are also available, since CPAC is possible only to those cells

Proposal 1-2: For MRO of subsequent CPAC, UE considers a neighbour cell as a candidate PSCell only if the candidate PSCell has the execution conditions configured.

Another aspect that needs to be discussed is reporting of the timeSinceCPAC-Reconfig, the time elapsed between the initiation of the execution of conditional reconfiguration for the target PSCell and the reception of the last conditionalReconfiguration for the SCG including the condRRCReconfig of the target PSCell. This is included to identify the case where the near failure occurred because conditionalReconfiguration is received late. Obviously, for subsequent CPAC,the scenario is not applicable. So UE can avoid this reporting. The absence of the field can also help the network to identify if the CPAC is subsequent CPAC.

Proposal 1-3: When the PSCell Addition or PSCellChange is due to subsequent CPAC, the UE doesn’t include the field timeSinceCPAC-Reconfig in SPR.

Proposal 1-4: Absence of timeSinceCPAC-Reconfig  helps network to identify CPAC as subsequent CPAC.No additional field is required for this purpose.

## MRO for LTM

For MRO for LTM, RAN2 decided to consider the following in RAN2#125bis [2].

RAN2 considers SHR, RA report and RLF for MCG LTM SON.

RAN2 will start work on MCG LTM.

RAN2 also identified the scenarios and subcases to be considered for LTM.

In RAN2#126, RAN2 discussed further on the contents of RLF report. In this contribution, we discuss on some of the remaining issues.

Random Access Information for LTM

Proposal 2-1: A new RA purpose is used for LTM cell switch and early synchronisation for TA.

We understand that the random access optimisation for the early TA needs to be done by both the source cell and candidate cell. For e.g. the thresholds at which the early TA is triggered is decided by source cell, while the RA parameters are decided by the candidate cell. The configuration of thresholds will make an impact on whether early TA is successful or not. So UE may include both source cell and candidate cell identifier in the RA report.

Proposal 2-2: UE includes the source and candidate cell identifiers in the RA information for early synchronisation for TA.

It is also important to know whether the random access for early TA is successful and how many attempts are made for early TA.

Proposal 2-3: If the UE has received TA value in LTM cell switch command, UE reports the TA is successful in the RA information.

Proposal 2-4: UE reports the number of PDCCH ordered RACH performed for early TA a candidate cell for a cell switch in the RA information.

Finally, it is important to log the RA report in all the cases-RA report, RLF report and SHR.

Proposal 2-5: RA information for early TA is included in RA report, RLF report and SHR.

SHR trigger for LTM

In our understanding the existing SHR triggers based on T310,T312, T304 are enough for the LTM. LTM is also a type of handover and the optimisations are quite similar. We understand that there may be some specific configurations such as UE based TA measurements or Early Sync that could be of interest to the network. But this can be easily handled by a T304 trigger. For e.g. the value of T304 timer expired for a LTM cell switch which doesn’t use RACH based LTM will be substantially lower than the same for the one which uses RACH. We need to note that the network will configure T304 considering both RACH based LTM and RACHless LTM. So a higher percentage for T304 threshold will be required for RACH based LTM and a lower percentage for RACHless LTM. Thus it is clear that no new trigger is needed for SHR for LTM.

Observation 1: SHR for the cases such as Early TA/UE based LTM can be easily logged using different threshold for T304 triggers.

Proposal 2-6: T310, T312 and T304 based triggers are used for SHR for LTM. No new trigger is needed.

## MRO for CHO with Candidate SCGs

In RAN2#125bis [2], RAN2 agreed to study the failure and near failure scenarios for CHO with candidate SCGs. In RAN2#126, RAN2 decided to enhance the RLF report with additional information regarding the state of the two execution conditions.

UE may report the conditional execution identifiers corresponding to fulfilled CHO conditions and CPAC conditions which are part of CHO with SCG configuration in RLF report/SCGFailureInformation and SHR/SPR. Similarly measurements of both MCG and SCG and the time information (time of fulfilment event to the failure or success of HO/PSCellChange) are also useful for MRO of CHO with SCGs.

Proposal 3-1: For MRO for subsequent CPAC, UE includes following information in RLF report and SHR

- Identifier of fulfilled execution conditions for CHO and CPC

- Time between fulfilment of CHO/CPC conditions and the occurrence of failure/near failure.

- Measurement results of PCells and PSCells which are candidates for CHO with SCG.

UE may be configured with multiple conditional reconfigurations, for e.g. one for CHO and one for CHO with CPAC. To identify the fulfilled event, UE may report the conditional reconfiguration identifier for satisfied condition in RLF report and SHR.

Proposal 3-2: UE includes conditional reconfiguration identifier of the fulfilled condition in RLF report/SHR.

# Conclusion

Based on the discussion, we have the following proposals and observations.

MRO for subsequent CPAC

Proposal 1-1: R18 MRO for SCGFailureInformation/SPR is the baseline for MRO for subsequent CPAC.

Proposal 1-2: For MRO of subsequent CPAC, UE reports a neighbour cell as a candidate PSCell only if the candidate PSCell has the execution conditions configured.

Proposal 1-3: When the PSCell Addition or PSCellChange is due to subsequent CPAC, the UE doesn’t include the field timeSinceCPAC-Reconfig in SPR.

Proposal 1-4:Absence of timeSinceCPAC-Reconfig  helps network to identify CPAC as subsequent CPAC.No additional field is required for this purpose.

MRO for LTM

Proposal 2-1: A new RA purpose is used for LTM cell switch and early synchronisation for TA.

Proposal 2-2: UE includes the source and candidate cell identifiers in the RA information for early synchronisation for TA.

Proposal 2-3: If the UE has received TA value in LTM cell switch command, UE reports that TA is successful in the RA information.

Proposal 2-4: UE reports the number of PDCCH ordered RACH performed for early TA in a candidate cell for a cell switch in the RA information.

Proposal 2-5: RA information for early TA is included in RA report, RLF report and SHR.

Observation 1: SHR for the cases such as Early TA/UE based LTM can be easily logged using different threshold for T304 triggers.

Proposal 2-6: T310, T312 and T304 based triggers are used for SHR for LTM. No new trigger is needed.

MRO for CHO with SCGs

Proposal 3-1: UE includes following information in RLF report/SCGFailureInformation and SHR/SPR:

- Identifier of fulfilled execution conditions for CHO and CPC

- Time between fulfilment of CHO/CPC conditions and the occurrence of failure/near failure.

- Measurement results of PCells and PSCells which are candidates for CHO with SCG.

Proposal 3-2: UE includes conditional reconfiguration identifier of the fulfilled condition in RLF report/SHR.

# References

[1] RP-234038: Data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR standalone and MR-DC (Multi-Radio Dual Connectivity) Phase 4

[2] R2-2404102: RAN2#125bis Chair notes

[2] RAN2#126 Chair notes