3GPP TSG-RAN WG2 Meeting #127	R2-2407060

Maastricht, Netherlands, 19 – 23 August 2024

Agenda item:	7.3.2

Source:	Nokia, Nokia Shanghai Bell

Title:	NW identifying RACH for emergency call

WID/SID:	Netw_Energy_NR-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

In this contribution, we would like to get common understanding on NW identifying RACH for emergency call.

# 2	Discussion

To avoid long latency for emergency call, it has been specified that RACH can be triggered by emergency call during Cell DTX/DRX non-active time and it was captured in stage 2 that the once the gNB recognizes there is an emergency call, it should ensure that there is no impact to the service, e.g. by releasing/deactivating cell DTX/DRX configuration.

38.321: For each Serving Cell configured with cell DRX, the MAC entity shall: 1> if cell DRX is activated for this Serving Cell: 2> if [(SFN × 10) + subframe number] modulo (cellDTX-DRX-Cycle) = (cellDTX-DRX-StartOffset): 3> start cellDTX-DRX-onDurationTimer for this serving cell after cellDTX-DRX-SlotOffset from the beginning of the subframe. 1> if cell DRX is activated and the Serving Cell is not in the cell DRX Active Period: 2> not instruct the physical layer to signal a SR on a PUCCH resource for SR; 2> not increment the SR_COUNTER for a SR; 2> not start the sr-ProhibitTimer for a SR; 2> not deliver any configured uplink grant and the associated HARQ information to the HARQ entity; 2> not instruct a HARQ process associated with a configured uplink grant to trigger a new transmission or a retransmission; 2> not report CSI on PUCCH and semi-persistent CSI configured on PUSCH; 2> if an emergency service is initiated by upper layers and this Serving Cell is the SpCell: 3> initiate a Random Access procedure (as specified in clause 5.1.1). NOTE: How the MAC layer in the UE is aware of an ongoing emergency service is up to UE implementation. 38.300: Once the gNB recognizes there is an emergency call or public safety related service (e.g. MPS or MCS), the network should ensure that there is no impact to that service (e.g. it may release or deactivate cell DTX/DRX configuration). The network should also ensure that there is at least partial overlapping between UE's connected mode DRX on-duration and cell DTX/DRX active duration, i.e. the UE's connected mode DRX periodicity is a multiple of cell DTX/DRX periodicity or vice versa.

It was left to NW implementation on how the NW recognizes there is an emergency call. AS explicit indication was brought up during the discussions since there is no cause value for connect mode emergency call, but it was not agreed with the assumption that based on the RACH during non-active time, the NW can basically know it is for emergency call. However, there are also other RACH triggers to be considered as well, e.g. BFR triggered SR, UL data arrival triggered SR there is no dedicated SR configuration (e.g. when TAT has expired) etc.

For RACH triggered by BFR triggered SR, there seems to be no issue since the NW knows it is not for emergency call since there would be no NAS message for BFR.

Observation 1: No issue with RACH triggered by BFR since NW can differentiate that RACH triggered during Cell DTX non-active period is for BFR or service request depending on whether if there is a NAS message from the UE.

Emergency call trigger RACH has been added independently from the SR trigger so that it can always be sent regardless of whether there is dedicated SR configuration or not. It could be easily distinguished RACH is not for other service reques when there is dedicated SR configuration since it triggers SR which cannot be sent during Cell DTX non-active time and it would not trigger RACH.

Observation 2: when there is dedicated SR, the NW can identify RACH triggered during Cell DTX non-active period is for emergency call since for other services it would use SR which will be sent at next active period.

Proposal 1: RAN2 to confirm that at least for the case when there is dedicated SR configuration, the NW can identify RACH triggered during Cell DTX non-active period is for emergency call if there is NAS request.

While for the case when there is no dedicated SR resource, it is unclear how the NW recognize there is an emergency call or other service request since gNB may not be able to decode NAS message. It has been agreed that SR cannot be sent during Cell DRX non-active period. It is unclear though if it also prevents also RA-SR during Cell DTX non-active period. If that is the case, the NW knows RACH during non-active period with NAS message is only for emergency call it is independent from SR. Otherwise, the NW might only be able to recognize there is an emergency call after further interactions with CN which comes much later.

Observation 3: when there is no dedicated SR (e.g. TAT expired), if RA-SR is prevented during Cell DTX non-active period, then the NW can identify RACH triggered during Cell DTX non-active period with NAS service request is for emergency call. Otherwise, it can be for any service request or for other NAS procedures.

Proposal 2: RAN2 to confirm if RA-SR is also prevented during Cell DTX non-active period so that the NW can identify RACH triggered during Cell DTX non-active period is only for emergency call.

Proposal 3: RAN2 to consider if any specification change is needed.

# 3	Conclusion

This document has made the following observations and proposals:

Observation 1: No issue with RACH triggered by BFR since NW can differentiate that RACH triggered during Cell DTX non-active period is for BFR or service request depending on whether if there is a NAS message from the UE.

Observation 2: when there is dedicated SR, the NW can identify RACH triggered during Cell DTX non-active period is for emergency call since for other services it would use SR which will be sent at next active period.

Proposal 1: RAN2 to confirm that at least for the case when there is dedicated SR configuration, the NW can identify RACH triggered during Cell DTX non-active period is for emergency call if there is NAS request.

Proposal 2: RAN2 to confirm if RA-SR is also prevented during Cell DTX non-active period so that the NW can identify RACH triggered during Cell DTX non-active period is only for emergency call.

Proposal 3: RAN2 to consider if any specification change is needed.