3GPP TSG-RAN WG2 Meeting #127	R2-2407061

Maastricht, Netherlands, 19 – 23 August 2024

Agenda item:	7.5.3.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Remaining issues on setting DRX SFN counter during handover

WID/SID:	NR_XR_enh-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

RAN2 #126 meeting made the following agreement for MAC corrections in terms of DRX_SFN_COUNTER initialization in the DRX formula:

RRC controls DRX operation by configuring the following parameters: - drx-onDurationTimer: the duration at the beginning of a DRX cycle; - drx-SlotOffset: the delay before starting the drx-onDurationTimer; … (Text omitted) - drx-TimeReferenceSFN (optional): the configuration to indicate how UE initializes DRX_SFN_COUNTER. The following UE variable is used for the DRX operation if drx-NonIntegerLongCycleStartOffset is configured: - DRX_SFN_COUNTER: the counter that increments when SFN changes to 0. The maximum value of this counter is at least 65535. (Text omitted) 1> if the drx-NonIntegerLongCycleStartOffset is configured: 2> increment DRX_SFN_COUNTER by 1 in the first symbol of a slot in which SFN changes to 0; 2> if DRX is (re-)configured by RRC: 3> if drx-TimeReferenceSFN is included in the RRC (re-)configuration which is received during the first half of a hyper frame (i.e., SFN is between 0 and 511): 4> set DRX_SFN_COUNTER to 1. 3> else: 4> set DRX_SFN_COUNTER to 0.

However, there are still open issues whether source cell, target cell and UE will be synchronized to apply the same drx-TimeReferenceSFN to initialize their DRX_SFN_COUNTER when UE receives RRC (re-)configuration that includes drx-TimeReferenceSFN during handover procedures. In this contribution, we mainly discuss the potential issues caused by handover procedures from different mobility mechanisms.

# 2	Discussion

## 2.1	DRX configuration exchange during handover

HO procedures for UEs in RRC_CONNECTED are discussed in TS38.300 clause 9.2.3 as follows.

With DRX config and drx-TimeReferenceSFN set according to target SFN

Note: source and target gNB might have different SFN numbering for asynchronies system. SFN timing offset is exchanged between gNBs.

Including the HO command from target

the UE set the counter according to reception time of HO command of source SFN (RRC reconfig in step 3)

Figure 1 Issue with SFN reference for HO case

The RRC reconfiguration with HO command from source gNB includes the drx-TimeReferenceSFN set from the target gNB and the SFN timing offset is exchanged between the gNBs, the target gNB might be able to take the SFN difference into account when setting the drx-TimeReferenceSFN value.

#### 9.2.2.75 SFN Offset This IE contains the time offset between an absolute time reference and the SFN0 start. The IE is calculated assuming that the SFN transmission started at the absolute time reference. The absolute time reference chosen is 1980-01-06 T00:00:19 International Atomic Time (TAI). IE/Group Name; Presence; Range; IE type and reference; Semantics description. SFN Time Offset; M; BIT STRING (SIZE(24)); Time offset in microseconds between the absolute time reference "1980-01-06 T00:00:19 International Atomic Time (TAI)” and the SFN0 start. The maximum usable value is (1024*10^4-1). Values higher than the maximum are discarded..

If it is assumed the HO procedure can be completed within 10s, it seems to be no issue with current procedure even based on the reception timing of RRC reconfiguration from the source gNB.

Observation 1: no issue with asynchronized system if HO is completed in 10s as the target gNB could take the SFN time offset into account.

However, in case of CHO/LTM, the actual HO execution when the CHO/LTM condition is met might happen much later than the reception of the RRC reconfiguration message with CHO/LTM configuration, thus the drx-TimeReferenceSFN and the receiving time of the HO command would become irrelevant for DRX_SFN_COUNTER settting.

Observation 2: HO execution might be done much later than RRC reconfiguration with CHO/LTM, thus the drx-TimeReferenceSFN and the receiving time of the HO command would become out-dated.

Besides, another difference is the configurations are applied upon reception of RRC reconfiguration message for non-HO case, while for HO case, the configuration of target cell only happens after the HO is completed, i.e. after RRC reconfiguration complete is sent.

Observation 3: DRX (re)configuration for non-HO case is applied upon reception of the RRC reconfiguration message (after the RRC processing) while DRX (re)configuration for HO case is applied upon transmission of RRC reconfiguration complete message to target cell.

It could make sense to refer to RRC complete message timing for SFN COUNTER initiation for HO case since it is a synchronised procedure with the target cell knowing when it schedules the first UL grant for the UE moving there. While for normal RRC reconfiguration without HO, there is ambiguity period for normal RRC reconfiguration for the NW to know when the UE sends to RRC reconfiguration complete message as it can be any UL grant after the RRC reconfiguration depending on the UE processing time.

Observation 4: there is ambiguity period for normal RRC reconfiguration without HO, while it is always synchronised procedure for HO.

Possible options to solve the CHO/LTM issue:

Option 1: initiate the SFN COUNTER upon HO completion for all HO case;

Option 2: initiate the SFN COUNTER upon CHO/LTM completion without changing other cases.

Option 1 might be a bit difficult to specify in MAC generally for HO since we do not have definition HO. Besides, it is less of an issue for normal HO anyway since T304 would have been expired if HO is not completed within a SFN space of 10s if we assume the target gNB would take the SFN offset into account when setting the drx-TimeReferenceSFN. Thus, it could be enough to just have special handling for CHO/LTM with option 2, which is also easy to specify from MAC point of view to refer to conditionalReconfiguration and ltm-Config.

Proposal: For CHO/LTM, DRX_SFN_COUNTER is initiated to 0 upon HO complete regardless of the drx-TimeReferenceSFN.

# 3	Conclusion

The following observations and proposal are made in this contribution:

Observation 1: no issue with asynchronized system if HO is completed in 10s as the target gNB could take the SFN time offset into account.

Observation 2: HO execution might be done much later than RRC reconfiguration with CHO/LTM, thus the drx-TimeReferenceSFN and the receiving time of the HO command would become out-dated.

Observation 3: DRX (re)configuration for non-HO case is applied upon reception of the RRC reconfiguration message (after the RRC processing) while DRX (re)configuration for HO case is applied upon transmission of RRC reconfiguration complete message to target cell.

Observation 4: there is ambiguity period for normal RRC reconfiguration without HO while it is always synchronised procedure for HO.

Proposal: For CHO/LTM, DRX_SFN_COUNTER is initiated to 0 upon HO complete regardless of the drx-TimeReferenceSFN.

# TP to MAC

1> if the drx-NonIntegerLongCycleStartOffset is configured: 2> increment DRX_SFN_COUNTER by 1 in the first symbol of a slot in which SFN changes to 0; 2> if DRX is (re-)configured by RRC without conditionalReconfiguration nor ltm-Config: 3> if drx-TimeReferenceSFN is included in the RRC (re-)configuration which is received during the first half of a hyper frame (i.e., SFN is between 0 and 511): 4> set DRX_SFN_COUNTER to 1. 3> else: 4> set DRX_SFN_COUNTER to 0. 2> else: 3> set DRX_SFN_COUNTER to 0 upon transmission of RRCReconfigurationcomplete message.