3GPP TSG-RAN WG2 Meeting #127	R2-2407062

Maastricht, Netherlands, 19 – 23 August 2024

Agenda item:	8.7.4

Source:	Nokia, Nokia Shanghai Bell

Title:	Scheduling enhancements for XR

WID/SID:	NR_XR_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

The following agreements were made in RAN2 #126:

LCP enhancements – LCH prioritization - Delay-aware LCP enhancement to resolve the issue of data with low remaining time being delayed due to data from other LCHs with no delay critical data is supported in Rel-19 XR. - The solution should consider impact on UE complexity (as already indicated in SI objective description) - For delay-aware LCP enhancement, RAN2 considers the following option to override/adjust the priority of LCH based on delay/deadline information as a baseline: - Use additional priority configured to LCHs in case of these LCHs with delay-critical data - FFS whether the priority only applies to delay-critical data within the LCH or for the whole LCH LCP enhancements – LCH restrictions - We try to avoid RAN1 impacts. - RAN2 assumes no dynamic indications are needed for triggering the delay-aware LCP mechanism. RAN2 assumes this mechanism is configured in a semi-static way. - For LCP restrictions based solutions, RAN2 will not discuss solutions requiring RAN1 work. FFS whether other LCP restrictions based approaches are needed/beneficial - The solutions should not disallow non-delay critical data from using an UL grant. LCP enhancements – granularity - LCP prioritization within a logical channel will not be considered in RAN2 discussions - FFS whether a separate remaining time threshold can be configured for delay aware LCP (i.e. different from the one used for DSR). DSR enhancements – multiple thresholds and grouping of data - Enhance DSR to report with multiple pairs of remaining time and buffer size for the LCG. - FFS whether DSR triggering is impacted - FFS whether PDU set importance needs to be included

In this contribution we discuss some of the FFSs related to delay-aware LCP and DSR enhancements.

# 2	Discussion

## 2.1	Delay-aware LCP

In WG meeting #126, RAN2 agreed to use additional priority configured to LCHs in case of LCHs with delay-critical data. The following FFSs on LCH prioritization were identified:

- whether the priority only applies to delay-critical data within the LCH or for the whole LCH

- whether a separate remaining time threshold can be configured for delay aware LCP (i.e. different from the one used for DSR).

On the first FFS, the agreement that “LCP prioritization within a logical channel will not be considered in RAN2 discussions” in practices rules out the possibility to only apply the LCH priority to delay-critical data within the LCH.

Proposal 1: the additional priority configured to LCHs in case of LCHs with delay-critical data is applied to the whole LCH.

On the second FFS, DSR is likely triggered before an UL grant is issued and LCP is performed. Therefore, we think that there could be benefits if the remaining time threshold(s) used for delay-aware LCP is different (e.g., smaller) than the remaining time threshold used for triggering the DSR. It should also be noticed that a UE supporting delay-aware LCP may not necessarily support DSR, which would anyway require at least two separate thresholds for DSR triggering and for delay-aware LCP.

Proposal 2: support configuration of separate remaining time threshold(s) for delay-aware LCP (i.e. different from the one used for DSR triggering).

Related to configuration of the additional LCH priority for LCHs with delay-critical data, in our views, it could be beneficial to define multiple remaining time thresholds, and corresponding LCH priorities. It would assist the UE to manage the LCHs priorities as LCHs will be sorted based on their remaining time values.

Proposal 3: Support configuration of multiple remaining time thresholds and corresponding LCH priority(ies) per LCH for delay-aware LCP.

For the FFS on LCP restriction, as agreed in the previous meeting, any enhancement should avoid 1) RAN1 impact; 2) artificially disallow an UL grant to be used for non-delay critical data and 3) intra-LCH prioritization for delay critical data and non-delay critical data. Considering these agreements while still allowing delay-aware LCP restriction enhancements, the option of preventing a LCH without delay-critical data from being allocated resources in one of the steps of the LCP procedure could be considered if needed. Alternatively, RAN2 could agree not to support any LCP restriction based solution.

Proposal 4: Preventing a LCH with no delay-critical data from being allocated resources in one step of the LCP procedure could be considered if LCH restriction enhancement is to be supported.

## 2.2	DSR enhancements

In WG meeting #126, RAN2 agreed to enhance DSR to report multiple pairs of remaining time and buffer size for one LCG. The following FFSs were identified:

- whether DSR triggering is impacted

- whether PDU set importance needs to be included

On the first FFS, we think that a DSR carrying information on multiple pairs of remaining time and buffer size for a LCG should not have any impact on the DSR triggering. The purpose of reporting multiple pairs per LCG is to provide more granular information to assist the gNB in radio resource management. Therefore, we think that the threshold for DSR trigger (i.e., remainingTimeThreshold) and the remaining time threshold(s) for reporting multiple pairs of buffer size and remaining time per LCG should be separately configured. Of course, one of the reporting remaining time threshold(s) could have the same value as the remaining time threshold for DSR trigger. The reporting remaining time threshold(s) could even be configured relatively to the triggering threshold. But still, triggering threshold and reporting threshold(s) should be separately configured.

Proposal 5: remaining time threshold(s) for DSR triggering and reporting in case of multiple pairs of remaining time and buffer size are separately configured.

Proposal 6: no change to the legacy DSR trigger based on single threshold.

On the second FFS, we think that one main limitation of current DSR when used in combination with PSI-based SDU discard is the NW being unaware of low importance data that may be ahead of the queue of high importance delay critical data. If there is low-importance data ahead of delay-critical high importance data in the transmission buffer, and the gNB schedules resources based on what reported in the buffer size field of the DSR, then the allocated resources may not be sufficient for the transmission of delay-critical data. As data belonging to different PDU sets with different importance levels can arrive in the UE buffer interlaced, separately signaling the buffer size and remaining time of high importance and low importance data in DSR may not be sufficient for the gNB to determine how many resources are needed for a UE to be able to transmit high-importance data. This issue is illustrated in Figure 1.

Observation: having separate signaling based on PDU set importance (e.g., only by including an explicit indication of the PDU set importance in the DSR) is not sufficient to provide the gNB with the information to determine the resources needed for the transmission of high-importance data.

Figure 1: DSR with low-importance and high importance data

On the other hand, we think that including low importance data (i.e., data with no running discardTimer) that, in the UE transmission buffer, is ahead of delay critical data (i.e., data with running discardTimer which is reported in the DSR) in the buffer size calculation for DSR could be a good direction to solve the issue to provide more information to the NW.

Proposal 7: Include low-importance data ahead of delay critical data in the buffer size calculation for DSR .

# 3	Conclusion

This document has made the following proposals:

Proposal 1: the additional priority configured to LCHs in case of LCHs with delay-critical data is applied to the whole LCH.

Proposal 2: support configuration of separate remaining time threshold(s) for delay-aware LCP (i.e. different from the one used for DSR triggering).

Proposal 3: Support configuration of multiple remaining time thresholds and corresponding LCH priority(ies) per LCH for delay-aware LCP.

Proposal 4: Preventing a LCH with no delay-critical data from being allocated resources in one step of the LCP procedure could be considered if LCH restriction enhancement is to be supported.

Proposal 5: remaining time threshold(s) for DSR triggering and reporting in case of multiple pairs of remaining time and buffer size are separately configured.

Proposal 6: no change to the legacy DSR trigger based on single threshold.

Proposal 7: Include low-importance data ahead of delay critical data in the buffer size calculation for DSR.