3GPP TSG-RAN WG2 #127	R2-2407063

Maastricht, Netherlands, 19 – 23 August 2024

Source:	NTT DOCOMO, INC.

Title:	Discussion on security aspects for Ambient IoT

Document for:	Discussion and decision

Agenda Item:	8.2.2

- Introduction

This contribution is to discuss remaining and potential issues on security aspects in Ambient IoT.

- Discussion

  - Temporary device ID

In the last meeting, RAN2 made agreements regarding the study scope of A-IoT paging message [1]:

Agreements 1 RAN2 will study the following cases for AIoT paging message: - a message containing an ID of a single A-IoT device. - a message containing a group ID that maps to multiple A-IoT devices. - a message that does not contain an ID, i.e., addressed for all devices that can receive the AIoT message. - a message containing multiple IDs of A-IoT devices. Need to confirm the need for this use case based on SA2 discussion. What device ID and group ID and scenarios is depending on SA2 discussion. 2 AIoT paging message indicate information from which the device can determine resources to be used for response (D2R message). FFS how (e.g. implicit/explicit/configured/preconfigured) and what resources (dedicated and/or shared) are provided to the device taking into account RAN1 discussion. 3 From RAN2 perspective, we assume the device can receive as long as there is enough energy. We will wait for RAN1 further progress on device monitoring details.

We would like to emphasize that the A-IoT paging message can include an ID of A-IoT device or of a group of A-IoT devices, as highlighted in yellow.

On the other hand, since RAN2 already agreed to assume not to support AS security, the A-IoT paging message cannot be security protected.

We have a strong concern to include permanent Device ID(s) in the A-IoT paging message. Using unprotected permanent Device IDs in air interface allows malicious users to intercept permanent Device IDs and enables to physically track certain Ambient IoT Devices (and potentially gives clues to other cheatings).

From above perspectives, we propose to introduce temporary device IDs. The temporary device IDs should be at least used for the indication of targeted devices in the A-IoT paging messages.

Considering that the Ambient IoT system does not support concept of serving cells, it seems impossible to introduce a temporary ID controlled by Ambient IoT Reader (like RNTI), thus the temporary Device ID should be generated and controlled by a CN node (like TMSI).

Observation 1.	Ambient IoT Reader indicates ID(s) of Ambient IoT Device(s) in the A-IoT paging message while the message cannot be security protected.

Proposal 1.	RAN2 assumes that temporary Device ID controlled by a CN node is introduced.

  - (Potential issue) Establishment of security context

In ambient IoT protocol, it is assumed that AS security is not supported. On the other hand, RAN2 is asking SA2/SA3 whether security protection in higher layer (e.g. NAS) is supported in the LS [2]. In this section, we would like to discuss based on assumption that the higher layer security is supported.

One potential issue is how RAN2 procedure takes into account an upper layer procedure for security context establishment (i.e., AIoT device registration). In the last meeting, RAN2 made agreements on three types of A-IoT random access procedure: 4-step RA (could be 3-step RA), 2-step CB RA, and contention-free access [1].

Agreements on “4 step” RA

1	A-IoT Msg1: the device sends an ID to the reader.  ID is a random ID generated by device (FFS how it is generated, e.g. randomly generated or generated based on Device ID).  FFS on ID size.  This doesn’t preclude any other RAN1 agreed information

2	A-IoT Msg2: the reader echos the ID received in Msg1. Further information may be included in mgs2 based on RAN1 agreements

3	A-IoT Msg3: device sends Device ID and/or any other upper layer data (depending on upper layer request)

- The device considers the contention resolution as successful, if the Msg2 including the same random ID in Msg1 is received. RAN2 assumes the size of random ID in Msg1 should be sufficient for contention resolution purpose.

- “Msg4” (i.e. the subsequent R2D transmission after D2R transmission) does not need to be always sent in random access. “Msg4” can be considered to handle the Msg3 transmission failure (due to various reasons). “Msg4” usage/presence can be further discussed.

RAN2 will not use “Msg4” term for further discussion of the random access.

Agreements on 2 step CB RA

- A-IoT Msg1: The device sends Device ID and/or any other upper layer data (depending on upper layer request). FFS what device ID is and whether an additional random ID is needed.  This doesn’t preclude any other RAN1 agreed information

- A-IoT Msg2: the reader may echo some information from Msg1.  FFS what some information is.  “Msg2” usage/presence can be further discussed

Agreement

-	From reader perspective, contention-free access procedure we will study single and multi-device case (depending on RAN1 discussion).

In 4-step RA, upper layer data starts being transmitted in A-IoT Msg3. On the other hand, in 2-step CB RA, upper layer data is transmitted in A-IoT Msg1. If the Ambient IoT Device already has security context for upper layers, the first upper layer messages, i.e., Msg3 for 4-step RA and Msg1 for 2-step CB RA, are successfully received by the CN.

However, if the device does not hold security context, for example because the device is paged for the first time, or because the device has lost the security context for some reason (this issue is discussed in the next section), the first upper layer messages, i.e., Msg3 for 4-step RA and Msg1 for 2-step CB RA, cannot be straightforwardly decoded by the CN.

// Contention-free access would behave similarly to 2-step CBRA although we did not touch in above description since the procedure is not as clear as 4-step RA and 2-step CBRA.

We think that in this case the Ambient IoT device would start with sending registration request message in higher layer instead of Msg3/Msg3 to let the CN trigger registration procedure, but this is what SA3 will clarify. RAN2 should start discussion then impacts on procedures in air interface.

Observation 2.	If SA2/SA3 agree to support upper layer security, the establishment of security contexts might affect RAN procedure, e.g., A-IoT random access procedure.

Proposal 2.	After receiving reply LS from SA2/SA3, RAN2 considers RAN2 impacts of upper layer procedures.

  - (Potential issue) Impacts of energy harvesting

In RAN#104, potential functionalities of Ambient IoT Device for energy harvesting were discussed. Following proposal is provided in the discussion although nothing was concluded [3].

Proposal (from draft folder) - RAN WGs are directed to study energy harvesting impacts in the following way: - Device 1 is assumed to have two states: ON, OFF - Device 2a/2b is assumed to have three states: ON, OFF, SLEEP - Identify function(s) of the device that can be assumed supported and assumed not supported in each of the above device states, subject to: - ON state supports at least: transmission, reception for communication - OFF state does not support at least: transmission, reception for communication - OFF state supports at least: energy harvesting - SLEEP state supports at least: - maintaining a memory content from ON state - Maintaining a timer (RAN1 to discuss purpose(s) of timer) - SLEEP state does not support at least: transmission - No additional physical layer signals/channels specific to support of SLEEP are introduced - Identify reader knowledge / control of the above states - Approximately identify durations of the above device states, by company reports. RAN1 is not required to establish a consensus model. - Companies can structure their report(s) of durations in their preferred way - No modeling of device/component (dis)charging characteristics - This part is aimed to be completed in RAN1#118, with RAN#105 to review progress and finalize if necessary - RAN1 & RAN2 co-lead

For co-existence of energy harvesting and RAN protocol, one possible way is to introduce states of Ambient IoT devices for energy harvesting. Above proposal provides an example: ON/OFF/SLEEP states.

Each behavior intended by the proponent is captured in above proposal, but we would like to emphasize how the memory works. As highlighted in blue, the memory is proposed to be maintained in SLEEP state. According to RAN1 agreement below, Ambient IoT device is equipped with Non-Volatile Memory (NVM) and volatile memory.

Agreement Study at least the following blocks for device 1 architecture. - (omit) - Memory can include two types of memory: 1) Non-Volatile Memory (NVM) such as EEPROM for permanently storing device ID, etc, and 2) registers for temporarily keeping any information required for its operation only while energy is available in energy storage. - (omit)

The blue part in above proposal mentions to the volatile memory, i.e., the memory is maintained by keeping energy in storage available. On the other hand, it seems that the device in OFF state loses its energy thus the volatile memory should be flushed.

Observation 3.	It is proposed that volatile memory in Ambient IoT device is flushed due to energy harvesting behavior.

Therefore, we should keep in mind that any buffered data in volatile memory is flushed due to energy harvesting behaviors. Note that permanent parameters e.g. device ID are stored in NVM, while temporary parameters are stored in volatile memory.

This behavior of memory would affect multiple functionalities, and in our understanding one is the security protection. Once the Ambient IoT device accomplish the registration procedure, it gets a security key and stores it in volatile memory (because security key is a temporary parameter that the network provides every time the security context is established). Although the security protection is not AS functionality, the impacts of memory flushing on RAN functionalities should be investigated.

Observation 4.	Since temporary parameters are stored in volatile memory, flushing of volatile memory could affect multiple functionalities for CN or RAN.

Proposal 3.	After device behaviors for energy harvesting is concluded, discuss whither and how the memory flushing due to energy harvesting affects RAN functionalities.

- Summary and proposal

Observation 1.	Ambient IoT Reader indicates ID(s) of Ambient IoT Device(s) in the A-IoT paging message while the message cannot be security protected.

Proposal 1.	RAN2 assumes that temporary Device ID controlled by a CN node is introduced.

Observation 2.	If SA2/SA3 agree to support upper layer security, the establishment of security contexts might affect RAN procedure, e.g., A-IoT random access procedure.

Proposal 2.	After receiving reply LS from SA2/SA3, RAN2 considers RAN2 impacts of upper layer procedures.

Observation 3.	It is proposed that volatile memory in Ambient IoT device is flushed due to energy harvesting behavior.

Observation 4.	Since temporary parameters are stored in volatile memory, flushing of volatile memory could affect multiple functionalities for CN or RAN.

Proposal 3.	After device behaviors for energy harvesting is concluded, discuss whither and how the memory flushing due to energy harvesting affects RAN functionalities.

- References

[1]	Chair notes, RAN2#126.

[2]	R2-2406150, “LS on RAN2 agreements and assumptions for Ambient IoT,” RAN2#126.

[3]	RP-241688, “Moderator's summary for offline discussion about Ambient IoT,” RAN#104.