3GPP TSG-RAN WG2 Meeting #127	R2-2407064

Maastricht, Netherlands, 19th – 23th Aug, 2024

Agenda Item:	8.10.2

Source: 	China Unicom

Title:  	Discussion on MRO enhancement for LTM

Document for:	Agreement

Introduction

During the previous meeting, RAN2#126 and RAN3#124 reached some agreements on MRO enhancement for LTM as below [1] [2].

If available, log the L1 measurements for serving cell, target cell and other LTM candidate cells in RLF report, upon RLF or mobility failure. Reuse the existing approach of using timeConnFailure and the reconnectCellId in RLF-report also for LTM failures, details TBD. Log the LTM cell ID upon performing recovery an LTM candidate cell, details TBD e.g. which field. Extend lastHO-Type in RLF-Report to indicate the LTM cell switch as last executed mobility procedure.

RAN3 to prioritize MCG LTM over SCG LTM. RAN3 takes RAN2#125-bis agreement on MRO for LTM scenarios as baseline for further study. For failures due to wrong selection of candidate LTM cell, CU is in charge of root cause analysis and performs optimization. RAN3 to discuss enhancements related to the information of RACH-less access and RACH-based access from UE related to MRO for LTM.

This document proposes further insights into the MRO involved L3 measurements, timing information discussion, and analysis.

Discussion

- 2.1	MRO enhancement for LTM

Timing information

In Rel-18 LTM procedures, UE is required to record specific timing information essential for the seamless assessment of both the CP and UP, as well as ensuring the continuity of network services. Compared to legacy L3 handover and CHO functionalities, certain timing information are eligible for reusing, while the introduction of novel timing information is proposed to facilitate performance enhancement. A comprehensive summary of these perspectives, along with their respective definitions, is delineated in both Figure 1 and Table 1.

Figure 1: Description of timing information for SON reports

Table 1: Definition of timing information for SON reports

| Record Time | Definition (Time between…) | Note |
| --- | --- | --- |
| T1 | UE receives LTM command → HOF/RLF detection | T1 is already agreed as timeConnFailure |
| T2 | UE receives LTM candidate configuration → switches to the candidate cell |  |
| T3 | UE obtains the target cell TA →UE receives the cell switch command | T3 is for UE measured based TA acquisition |
| T3’ | UE performs early TA acquisition→UE receives the cell switch command | T3’ is for PDCCH order based TA acquisition, UE performs early TA acquisition means UE sends preamble to the candidate cell according to PDDCH order |
| T4 | UE receives LTM candidate configuration → connection failure | T4 = T7 + T1 |
| T5 | RLF detection in source/target cell→ … | Reuse timeUntilReconnection |
| T6 | UE reports L1 measurement results → HOF/RLF detection |  |
| T7 | UE receives LTM candidate configuration → … | Reuse timeSinceCHO-Reconfig |
| T8 | UE reports L1 measurement results → UE receives the cell switch command |  |
| T9 | UE receives LTM command → UE completes the first DL/UL reception/ transmission on the indicated beam of the target cell | Interruption time of UP |

For the reused timing information, RAN2#127 has reached a consensus on the reusing of the timeConnFailure (T1) within MRO for LTM. This timing information means the elapsed time since the last LTM cell swich execution until connection failure. Similar to timeConnFailure, the IE timeUntilReconnection (T5) is also proposed for reuse, capturing the elapsed time from the connection failure is experienced in failedPCelId to when UE enters RRC_CONNECTED state. The utilization of timeUntilReconnection provides the network with insights into the UE's occupation in a failure connection state during the LTM cell switch procedure.

Proposal 1: Reuse the existing approach of using timeUntilReconnection in RLF-report also for LTM failure scenarios.

T7 can also be defined for MRO by reusing the meaning of timeSinceCHO-Reconfig. In instances of failure (RLF/HOF), as well as near failure, the IE timeSinceCHO-Reconfig serves distinct purposes as documented in the RLF report and SHR as per TS 38.331. Here lists the IE definitions as below:

RLF-Report field descriptions

timeSinceCHO-Reconfig In case of handover failure, this field is used to indicate the time elapsed between the initiation of the last handover execution towards the target cell and the reception of the latest conditional reconfiguration. In case of radio link failure, this field is used to indicate the time elapsed between the radio link failure and the reception of the latest conditional reconfiguration while connected to the source PCell. Actual value = field value * 100ms. The maximum value 1023 means 102.3s or longer.

SuccessHO-Report field descriptions

timeSinceCHO-Reconfig This field is used to indicate the time elapsed between the initiation of the last conditional reconfiguration execution towards the target cell and the reception of the latest conditional reconfiguration for this target cell. Actual value = field value * 100ms. The maximum value 1023 means 102.3s or longer.

Similar with timeSinceCHO-Reconfig, a novel IE designated as timeSinceLTM-Reconfig (T7) is proposed for definition within both RLF report and SHR. This IE will facilitate the network’s ability to know the duration time before UE initiating the last LTM cell switch configuration execution.

As illustrated in the aforementioned table, the relationship T4 = T7 + T1 is established; hence, it is recommended to explicitly define the new T7 parameter in accordance with timeSinceLTM-Reconfig, rather than fabricating a composite T4.

Proposal 2: Define and log timeSinceLTM-Reconfig (like timeSinceCHO-Reconfig) within RLF-report and SHR in LTM failure and near failure cases.

Subsequently, we proceed to examine additional timing information. For instance, T2, analogous to T4, encompasses the entire duration from the UE's reception of LTM configuration to the ultimate completion of the cell switch. However, it’s not clear how network optimization is handled even the network knows the LTM is successful with longer time (T2) than usual.

Regarding T3 and T3', distinctions are identified. T3 is applicable to UE-measured-based early TA acquisition, whereas T3' is exclusive to PDCCH-order-based TA acquisition. T3 serves to discern the elapsed time after the UE has independently obtained TA. Conversely, T3' captures the duration time the network requires to compute the TA and transmit it to the UE. Regarding T3, if the network detects that the UE's self-measured TA procedure is excessively prolonged, potentially leading to RLF/HOF, and thus rendering the TA invalid, the network may opt not to configure future UE-measured early TA acquisition configurations for the UE. For T3', the network may optimize the timing of TA transmission via the MAC CE to synchronize with the reporting of L1measurements, ensuring proper alignment between the TA and the L1 measurements.

Proposal 3: RAN2 to discuss whether to log the following timing information in SON reports (e.g. RLF report) for LTM:

T3: the lapsed time between UE obtains the target cell TA measured by itself and the moment UE receives the cell switch command

T3’: the elapsed time between UE sends preamble according to PDDCH order and the moment UE receives the cell switch command

For T6 and T8, the commencement of the timing period is marked by the L1 measurements reporting procedure; However, the reasons behind the network's requirement for this timing information to facilitate optimization remains unclear, and it is also unclear which point in time report is used as the starting point for the periodically reported L1 measurements.

Regarding T9, as per TS 38.300, the interruption time is shown in below Figures G-2 and G-3, encompassing both RACH-based and RACH-less LTM scenarios. means the elapsed time between UE receives cell switch command and UE completes the first DL/UL reception/ transmission on the indicated beam of the target cell. The logging of T9 within SON reports is instrumental in enabling the network to ascertain the UP interruption time, particularly when providing time-sensitive services, such as XR."

Proposal 4: RAN2 to discuss whether to log interruption time in SON reports (e.g. SHR) for LTM. The interruption time is defined as the elapsed time between UE receives cell switch command and the moment UE completes the first DL/UL reception/ transmission on the indicated beam of the target cell.

Figure G-2: Mobility Latency for RACH-based LTM Figure G-3: Mobility Latency for RACH-less LTM

Other information

In reference to MRO enhancement for CHO procedures, a 1-bit IE choCandidate is specified in RRC specs to indicates whether the associated neighbour cell is a candidate target cell for conditional handover or conditional PSCell change or addition. This field may be included only in the SuccessHO-Report or SuccessPSCell-Report within UEInformationResponse message. Given that RAN2 has consented to log L1 measurements for LTM candidate cells, it is similarly essential to designate which neighbour cells are LTM candidate cells. This designation enables the network to perform optimization and to revise the configured LTM candidate cell list accordingly, for instance, by incorporating neighboring cells that are not currently candidate cells into the LTM candidate cell list.

Proposal 5: Define and log ltmCandidate (similar like choCandidate) in SHR to indicate whether a neighbour cell is an LTM candidate cell or not.

Upon generation of RLF or HOF reports for LTM, the gNB is restricted to obtaining L1 beam measurements as per RAN2 agreements, thereby precluding the CU from offering refined LTM candidate cell configurations due to the absence of L3 measurements at the target node. In the scenarios outlined below, the network is incapable of executing any optimization with L1 measurements reporting alone:

Scenario 1: Superior beam 1 or inferior beam 2 at an inferior cell A while the cell switch command is being received by UE

Scenario 2: Inferior beam at a superior cell B while the cell switch command is being received by UE

Thus, it is proposed to also log the reporting of L3 measurements. This would allow the CU to collaborate with the DU to ascertain whether Scenario 1 occurs and results in RLF/HOF, prompting the CU to remove the inferior cell from the candidate cell list. Conversely, if Scenario 2 occurs and results in RLF/HOF, the DU would reconfigure the UE with a superior beam within the same candidate cell.

Proposal 6: If available, log L3 measurements for serving cell, target cell and other LTM candidate cells in RLF report, upon RLF or mobility failure.

Conclusion

RAN2 is kindly asked to discuss the following proposals:

Proposal 1: Reuse the existing approach of using timeUntilReconnection in RLF-report also for LTM failure scenarios.

Proposal 2: Define and log timeSinceLTM-Reconfig (like timeSinceCHO-Reconfig) within RLF-report and SHR in LTM failure and near failure cases.

Proposal 3: RAN2 to discuss whether to log the following timing information in SON reports (e.g. RLF report) for LTM:

T3: the lapsed time between UE obtains the target cell TA measured by itself and the moment UE receives the cell switch command

T3’: the elapsed time between UE sends preamble according to PDDCH order and the moment UE receives the cell switch command

Proposal 4: RAN2 to discuss whether to log interruption time in SON reports (e.g. SHR) for LTM. The interruption time is defined as the elapsed time between UE receives cell switch command and the moment UE completes the first DL/UL reception/ transmission on the indicated beam of the target cell.

Proposal 5: Define and log ltmCandidate (similar like choCandidate) in SHR to indicate whether a neighbour cell is an LTM candidate cell or not.

Proposal 6: If available, log L3 measurements for serving cell, target cell and other LTM candidate cells in RLF report, upon RLF or mobility failure.

References

- R2-126_Maint_eRedCap_SON-MDT_(Mattias)_20240524_Final

- RAN3_124_agenda_20240524_eom