3GPP TSG-RAN WG2 Meeting #127	R2-2407065

Maastricht, Netherlands, 19th – 23th Aug, 2024

Agenda Item:	8.10.2

Source: 	China Unicom

Title:  	Discussion on MRO enhancement for CHO with candidate SCGs

Document for:	Agreement

Introduction

During the previous meeting, RAN2#126 and RAN3#124 has made some agreements on MRO enhancement for CHO with candidate SCGs as below [1] [2].

RAN2 to enhance the RLF report with additional information regarding the state of the two execution conditions. We see later if we also can enhance the SCGFailureInformation report.

MRO for CHO with candidate SCG failure and near failure cases Do not distinguish between fast MCG recovery/no-fast MCG recovery for now (to simplify use cases). RAN3 will start with the failure scenarios with UEs configured with CHO with candidate SCGs.

This document proposes further insights in the MRO involved MCG with SCG conditions discussion and analysis.

Discussion

- 2.1	MRO enhancement for CHO with candidate SCGs

As RAN2 has agreed to enhance the RLF report with additional information regarding the state of the two execution conditions. Here are different failure cases at the case of CHO with candidate SCGs.

Case 1: CHO condition is not fulfilled, CPAC condition is fulfilled or not fulfilled.

Case 2: CHO condition is fulfilled, yet an MCG failure ensues rapidly prior to the fulfillment of the CPAC condition.

Case 3: Both conditions are satisfied, but an MCG failure occurs prior to the successful completion of the handover.

For above cases, if MCG failure happens, the network requires discerning whether the failure is attributable solely to the unmet CHO condition or to the non-fulfillment of both CHO and CPAC conditions. Thus, it’s beneficial to log CHO condition and CPAC condition fulfillment state in the RLF report. This enables the network to assess the appropriateness of the configured thresholds and to execute corresponding optimizations.

As depicted in the subsequent Figure 1, T2 represents the elapsed time between the fulfillment of the CHO condition and the occurrence of an RLF/HOF. With T2, the network can deduce potential misconfigurations in the CHO execution condition.

Regarding Case 3, even both conditions are fulfilled, but the protracted elapsed time between the fulfillment of the CHO condition and the CPAC condition (T1) may result in a change in the network's state conditions, leading to ultimate failure before the handover is successfully executed. Hence, it’s needed to log T1 in the RLF report. Furthermore, for the network to execute optimizations with precision, it is essential to ascertain the order of condition fulfillments. This will enable the network to identify which of the most recently fulfilled conditions is responsible for the MCG failure.

Figure 1: Failure cases for CHO with SCGs

Proposal 1: RAN2 to agree to log the following information in RLF report:

-	Indication of the fulfillment state of the CHO and CPAC conditions

-	The elapsed time between CHO condition is fulfilled and when the MCG failure occurs

-	The elapsed time between CHO condition is fulfilled and when CPAC condition is fulfilled

-	The order of fulfillment of CHO and CPAC conditions

In instances where the MCG link is established but an SCG failure occurs, e.g., when both CHO and CPAC conditions are met and the MCG handover is successful, yet an SCG failure occurs — it is referred to as a sub-optimal handover. The analysis of the node liable for the SCG failure is as follows:

Case 1: CHO condition fulfillment is after CPAC condition fulfillment, and the late fulfillment of the CHO condition leads to the CPAC condition no longer being met, this can result in an SCG failure after handover.

Case 2: CPAC condition fulfillment is after CHO condition fulfillment, but the CPAC condition is not properly configured, this can also result in an SCG failure after handover.

For case 1, the MN is responsible for addressing the SCG failure issue, it is essential for the UE to log the elapsed time between CHO condition is fulfilled and when CPAC condition is fulfilled, as well as the order of fulfillment of CHO and CPAC conditions in the SHR. This information will assist in determining whether SCG failure is caused by a too late CHO conditions fulfillment.

For case 2, the SN is responsible for addressing the SCG failure issue, it is necessary for the UE to log the elapsed time from the fulfillment of the CPAC condition to the SCG failure, and the order of fulfillment of CHO and CPAC conditions in SHR. This data will assist in determining whether the CPAC condition is improperly configured. Consequently, it is proposed:

Observation 1: Sub-optimal case refers to a scenario where both CHO and CPAC conditions are met, and the MCG handover successfully while an SCG failure is observed.

Proposal 2: RAN2 to discuss the sub-optimal case, to log the following information in SHR:

-	The elapsed time between CHO condition is fulfilled and when CPAC condition is fulfilled

-	The order of fulfillment of CHO and CPAC conditions

Proposal 3: RAN2 to discuss the sub-optimal case, to log the following information in SCGFailureInformation report:

-	the elapsed time between CPAC condition is fulfilled and when the SCG failure happens

-	The order of fulfillment of CHO and CPAC conditions

In the event of a near failure case, where both CHO and CPAC conditions are satisfied and the handover is initially successful, followed by an MCG failure, the network can ascertain whether the condition fulfillment procedure duration is excessive and contributed to the near failure by analysing the elapsed time between the fulfillment of the CHO condition and the CPAC condition. Moreover, for the network to pinpoint precise optimization measures, it should also log the order of fulfilled conditions in the SHR.

Proposal 4: RAN2 to discuss the near failure case, and log the following information in SHR

-	The elapsed time between CHO condition is fulfilled and when CPAC condition is fulfilled

-	The order of fulfillment of CHO and CPAC conditions

Conclusion

RAN2 is kindly asked to discuss the following observations and proposals:

Observation 1: Sub-optimal case refers to a scenario where both CHO and CPAC conditions are met, and the MCG handover successfully while an SCG failure is observed.

Proposal 1: RAN2 to agree to log the following information in RLF report:

-	Indication of the fulfillment state of the CHO and CPAC conditions

-	The elapsed time between CHO condition is fulfilled and when the MCG failure occurs

-	The elapsed time between CHO condition is fulfilled and when CPAC condition is fulfilled

-	The order of fulfillment of CHO and CPAC conditions

Proposal 2: RAN2 to discuss the sub-optimal case, to log the following information in SHR:

-	The elapsed time between CHO condition is fulfilled and when CPAC condition is fulfilled

-	The order of fulfillment of CHO and CPAC conditions

Proposal 3: RAN2 to discuss the sub-optimal case, to log the following information in SCGFailureInformation report:

-	the elapsed time between CPAC condition is fulfilled and when the SCG failure happens

-	The order of fulfillment of CHO and CPAC conditions

Proposal 4: RAN2 to discuss the near failure case, and log the following information in SHR

-	The elapsed time between CHO condition is fulfilled and when CPAC condition is fulfilled

-	The order of fulfillment of CHO and CPAC conditions

References

- R2-126_Maint_eRedCap_SON-MDT_(Mattias)_20240524_Final

- RAN3_124_agenda_20240524_eom