3GPP TSG-RAN WG2 Meeting #127	R2-2407066

Maastricht, Netherlands, 19 – 23 August 2024

Agenda item:	8.1.2.2

Source:	Nokia

Title:	LCM for UE-sided model for Beam Management

WID/SID:	NR_AIML_Air-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

. RAN2#126 Agreements

1	RAN2 will support functionality activation/deactivation after inference configuration.   FFS initial state of configuration and how activation/deactivation is achieved.   FFS what Deactivation refers to:  examples discussed: 1) fallback to legacy 2) switching, etc.

2	We will work offline on the definitions for functionality types and define what is availability.

3	The UE will indicate the gNB/LMF whether the AI/ML functionality is available/applicable.   For a functionality to be applicable at least there should at least one model available within it.   FFS other details on what is applicability/non-applicability.

4	For NW-side additional conditions, RAN2 assumes that RRC signaling from gNB to UE can be designed for consistency between inference and training.  RAN2 will wait for RAN1 input for further details.   FFS if the same applies to positioning

5	For BM use case, As a baseline the UE determines whether a functionality is applicable.  Existing UAI framework is used at least for proactive reporting of applicable functionality.  FFS reactive

# 2	Discussion

In this paper, we first review the definitions of functionality and additional condition and then discuss the procedural impacts for reporting applicable functionalities for controlling functionalities, e.g., activating, deactivating, switching, or falling back from functionalities.

## 2.1	Functionality

TR 38.843 defines functionality as referring “to an AI/ML-enabled Feature/FG enabled by configuration(s), where configuration(s) is(are) supported based on conditions indicated by UE capability. Correspondingly, functionality-based LCM operates based on, at least, one configuration of AI/ML-enabled Feature/FG or specific configurations of an AI/ML-enabled Feature/FG.” As significant aspect, a certain configuration(s) of AI/ML enabled Feature/FG need to be identified during inference phase for the beam prediction use case.

To be more specific, it can be identified by being attributed to a corresponding AI/ML-enabled CSI report configuration. Simply put, a functionality is an AI/ML-enabled configuration constructed using conditions indicated by UE capabilities. In the beam management use case, a functionality is represented by a CSI-ReportConfig.

Observation 1: A functionality in functionality-based LCM is a configuration constructed using conditions indicated by UE capabilities.

Proposal 1: In the beam management use case, configuration of a functionality is signalled by CSI-ReportConfig.

## 2.2	Additional conditions

### 2.2.1	Role

Additional conditions can be interpreted as the main reason causing changes on a functionality applicability at the UE in which occurs either due to configuration changes at the NW side (known as NW side additional conditions) or due to changes on UE related conditions (known as UE-sided additional conditions). In other words, additional conditions determine if a functionality is applicable. These aspects triggered discussions with respect to consistency between what conditions the UE has been trained and what additional conditions causes the UE/NW to be considered during inference UE side in RAN1. The need for any consistency considering UE-sided and NW-sided additional conditions were discussed in RAN1 in which the relevant agreement is discussed in the following section. RAN2 is expected to also consider consistency related discussions in RAN1 as the main role/driver/baseline for defining functionality applicability, evaluation of applicability and reporting scenarios of an applicable functionality in beam management use case.

### 2.2.2	NW-side Additional Conditions

Additional conditions are training assumptions of an AI/ML model. NW-side additional conditions refer to training assumptions that are associated with NW implementation/aspects (e.g., NW-side beam pattern) that were used when collecting data (e.g., mapping relationship of Set A and Set B, consistency of downlink spatial domain transmission filters, antenna height, beam shape, etc..). These additional conditions are not signalled to the UE, but rather they could be represented by an ID, such as the associated ID being discussed in RAN1. More specifically, following agreement was made in RAN1 representing options to meet consistencies between training and inference:

Agreement (RAN1#117) Further study, for the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2, where the NW-side additional condition may at least impact UE assumption on beams of Set A/Set B: - Opt1: Based on associated ID (Referring to AI 9.1.3.3) - FFS on what can be assumed by UE with the same associated ID across training and inference FFS on how associated ID is introduced, e.g., within CSI framework, or outside of CSI framework

Observation 2:  NW-side additional conditions could be signalled through associated ID.

### 2.2.3	UE-side Additional Conditions

Additional conditions are training assumptions of an AI/ML model. UE-side additional conditions may refer to UE implementation assumptions that were used when collecting data (e.g., UE antenna config, Rx beams, etc..). These additional conditions are not signalled to the NW, but rather they are used by UE to evaluate the applicability of AI/ML model to enable a functionality.

Proposal 2 : There is no RAN2 signaling impact due from UE-side additional conditions.

## 2.3	 Functionality Applicability

TR 38.843 states that “after functionality identification, necessity and mechanisms for UE to report updates on applicable functionality(es) among functionality(es) are studied, where the reported applicable functionalities may be a subset of all functionalities. Applicable functionalities can be reported by the UE.”

### 2.3.1	Definition

Applicability of a functionality is determined by a UE  by the presence of an AI/ML model in the UE whose data collection assumptions are consistent with the NW-side additional conditions of connected cell. The mere existence of an AI/ML model which is available for download, or which is being downloaded (without considering data collection assumptions), alone cannot be considered as a criterion in the determination of applicability. Because functionalities are defined by configurations allowed based on UE capabilities, a functionality cannot be configured by the gNB prior to the capability exchange procedure. Once the UE’s capabilities are known to the gNB, the gNB can configure one or more functionalities, each of which may or may not be applicable to the UE.

### 2.3.2	Evaluation of Applicability

The applicability of a functionality can be determined based on essential three factors.

- The existence of at least one AI/ML model on a UE which supports the configuration, e.g., Set B beams dimension beam prediction for a sub use case, Set A beams dimension, predicted CRI, predicted RSRP, observation window size, prediction window size, etc.

- Of the AI/ML models on the UE which support the functionality configuration, at least one AI/ML model is compatible with the NW-side additional conditions, e.g., an NR-CGI and/or one or more associated IDs reported by the NW match an NR-CGI and/or one or more associated IDs associated with one or more AI/ML model.

- One or more of the AI/ML models identified in the previous step are compatible with the UE-side  additional conditions, which could be UE implementation assumptions that were used when collecting data (e.g., UE antenna config, Rx beams, etc..) or UE internal restrictions (e.g., battery life, memory utilization, or processor utilization, or additional conditions such as geographic area or UE velocity), respectively.

If all three factors are satisfied for a functionality, the functionality is applicable, otherwise it is non-applicable. The evaluation of whether a UE-side AI/ML model is applicable is performed by the UE.

Alternatively, the applicability of one or more functionality could be determined based on performance assessment (via an UE side performance monitoring procedure), according to the opt2 in the following RAN1 agreement. If using performance assessment (via a performance monitoring-based approach), only factors 1 and 3 and 4 are required in addition to satisfactory performance determined by the performance assessment.

Agreement (RAN1#117) Further study, for the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2, where the NW-side additional condition may at least impact UE assumption on beams of Set A/Set B: - Opt 2: Performance monitoring based - FFS details

Observation 3 : An AI/ML functionality is assumed to be applicable if three conditions are satisfied: the UE contains one or more AI/ML models which support the configuration provided by the gNB; and, of those AI/ML models, one or more AI/ML models are compatible with the NW-side additional conditions; and, of those AI/ML models, one or more AI/ML models satisfy the UE-side additional conditions.

Proposal 3:  Determining the applicability of an AI/ML functionality is up to UE implementation.

### 2.3.3	Reporting Scenarios

First applicability report, immediately after initial configuration (Reactive Reporting)

The first opportunity for the UE to evaluate and report applicability is after the first RRCReconfiguration message containing one or more configuration for beam management, e.g., CSI-ReportConfig(s). Prior to the evaluation of applicability, it is assumed that a UE cannot be configured with an AI/ML functionality. But after the UE evaluates applicability, it can report which of the one or more configured CSI-ReportConfig(s) are applicable.

In an example, the following steps in Figure 1 can be considered to interpret the applicability reporting after initial configuration scenario:

Figure 1 An example of applicability reporting used for immediately after initial configuration adapted from [1]

Step 3: gNB sends RRCReconfiguration message including functionality configurations possibly to enable UAI to be using for functionality applicability reporting. Functionality configurations may are represented by provided CSI-ReportConfigs which may contains corresponding NW-side additional conditions (e.g.,associated IDs).

Step 4: UE evaluates the applicability of provided CSI-ReportConfigs considering both UE-side and NW-side additional conditions.

Step 5: UE sends applicable CSI-ReportConfigId(s).

Step 6: gNB selects a CSI-ReportConfigId as the activated functionality.

Step 7: Activated CSI-ReportConfigId is sent to the UE using appropriate mechanism depending on report type (e.g., periodic, aperiodic or semi-persistent) of activated CSI-ReportConfig.

Proposal 4: Adopt Figure 1 to illustrate applicability reporting immediately after an initial configuration.

Follow-up applicability reports, when UE-side/NW-side additional conditions change (Proactive Reporting)

The second and following opportunities for the UE to evaluate and report applicability are any time after the first applicable functionality report. During this time, the UE’s internal conditions could have changed, the performance of AI/ML model(s) enabling an AI/ML functionality could have dropped to an unacceptable level, or the NW-side additional condition(s), which could be represented by an associated ID, could have changed. If any change in the UE or NW affects the applicability of one or more AI/ML functionalities, then an applicable functionality report could be triggered.

In an example, the following steps in Figure 2 can be considered to interpret the scenario in which functionality applicability reporting after the first applicable functionality report (based on initial configuration) are reported:

Figure 2 An example of applicability reporting used after changes due to NW-side/UE-side additional conditions

Reporting applicability scenario depicted in Figure 2 is a follow up scenario after initial configuration and activation procedure (Step 7 in Figure 2) described above.  Having said that, the following steps (as illustrated in Figure 2) can be considered while reporting follow-up applicability:

Step 7 : UE is activated to perform an applicable CSI-ReportConfig after initial configuration.

Step 8: UE evaluates the applicability of provided CSI-ReportConfig(s) with respect to both UE-side and NW-side additional conditions and determines a change in the applicability of one or more CSI-ReportConfig(s) and

Step 9: UE sends applicable CSI-ReportConfigId(s) which meets changes on UE-side and NW-side additional conditions.

Step 10: gNB selects optionally the CSI-ReportConfigId reported in Step 2 as activated configuration.

Step 11: activated CSI-ReportConfigId can be optionally sent to the UE using appropriate mechanism depending on report type (e.g., periodic, aperiodic or semi-persistant) of activated CSI-ReportConfig.

Proposal 5: Adopt Figure 2 to illustrate applicability reporting following the initial applicability report.

### 2.3.4 Applicable Functionality Report

In both cases, i.e., once applicable functionalities are initially indicated or after applicable functionalities are updated, the gNB will be able to activate one or more of the applicable functionalities.

Further study concerning applicable reporting of AI/ML functionalities can be addressed by defining an applicable functionality report considering:

- Updates on applicability: updates to applicability could arise from performance decrease or increase, loss of a model, acquisition of a model (e.g., a fully new model, an update to a model, etc), or change in consistency with UE-side and NW-side additional conditions.

- Content of report: applicability and/or non-applicability of functionalities and any reasons therefore.

Proposal 6: RAN2 to agree that an applicable functionality report will at least allow the UE to report applicable functionality related updates and reason (or cause) pertaining to the updated information (e.g., model available, model updated, etc.,)

UAI is used to signal temporary reductions to UE capabilities in the form of “overheating assistance” and is used to signal updates such as an indication that a new flight path is available (for UAV operation). Given that UAI can be used to signal restrictions to previously signalled capabilities, it is a good starting point for follow-up applicable functionality reporting and updates thereto.

Proposal 7: Support the use of UAI to signal updated applicable functionality reports and possibly the initial applicable functionality report following a configuration/reconfiguration.

Proposal 8: Support the use of RRCReconfigurationComplete to signal the initial applicable functionality report following a configuration/reconfiguration.

In either of two explained reporting scenarios, a UE is not expected to perform inference operation for any of the configured functionalities until the completion of the first applicable functionality report, which for BM would be triggered in response to the first RRCReconfiguration, which contains one or more AI/ML-enabled CSI-ReportConfigs.

Proposal 9: UE does not perform inference operation for any of the configured functionalities until the completion of the first applicable functionality report.

As mentioned above, when the UE is performing inference (associated with a functionality), it may report a second applicable functionality report (e.g., triggered in response to changes in the UE internal conditions, information or indications associated with the NW-additional conditions, or performance/assessments of functionalities). For beam prediction, the applicability can be reported corresponding to the configuration IDs of AI/ML-enabled CSI-ReportConfig(s). In [1], the Rapporteur provided an open question on whether the UE can include non-applicable functionalities in Step 4 [1] after the basic procedure of applicable functionality reporting. In our view, reporting of non-applicability of a functionality in the follow-up reporting scenario may assist the NW on performance monitoring or not reconfiguring the same UE with the non-applicable CSI-ReportConfig.

Proposal 10: RAN2 to support reporting the non-applicability of a functionality implicitly through omission or explicitly through a flag indicating applicable or non-applicable.

Other aspects on applicability reporting scenarios:

In [1], the Rapporteur provided an open question based on companies’ contributions on whether neighbouring cell’s applicability needs to be considered in applicable functionality reporting to serving cell or not. Highlighting our views, UAI framework can only report UE’s condition/information changes on serving cell (i.e. the UE is only required to monitor condition changes in serving cell in existing UAI framework). As such, existing UAI procedure cannot work for proactive reporting of neighbour cells’ applicable functionalities reporting which requires the UE to monitor condition change in neighbour cells. As such, it is worth discussing the feasibility of existing UAI on reporting applicable functionalities of neighbour cells in RAN2.

Proposal 11: RAN2 to discuss whether applicable functionality reporting is supported for handover should be discussed in RAN2 during Release 19.

Apart from identifying basic framework on applicable and available functionality is between the network and the UE, changes on UE mode (e.g., moving from CONNECTED to IDLE or INACTIVE) may impact the procedure for applicable and available functionality reporting. As an example, in the existing specifications it is not clear if the UE is mandated to perform the applicability reporting procedure following the first applicability reporting (as illustrated in Figure1) when the UE moves to IDLE mode and comes back to CONNECTED mode.

Proposal 12: RAN2 to discuss whether applicable functionalities are stored prior to transitioning to RRC_INACTIVE or RRC_IDLE and whether applicable functionality reporting is supported while UE transitions from RRC_INACTIVE or RRC_IDLE to RRC_CONNECTED.

## 2.4 Functionality-Based LCM

### 2.4.1	Definitions

An activated functionality is an AI/ML functionality configured to perform inference and report inference results to the NW.

As described in the previous section, a UE must be provided one or more configurations of AI/ML functionalities for evaluation of applicability of functionalities. After the evaluation, the applicability of the AI/ML functionalities is reported by the UE to the NW, and then the NW can select one or more of the AI/ML functionalities to activate. Which of the applicable functionalities to activate is left up to NW implementation.

An AI/ML functionality activation/deactivation is characterized at different LCM phases (e.g., during performance monitoring) in which NW may decide to deactivate a certain AI/ML functionality. Once a certain AI/ML functionality is identified (via associating to a corresponding ML-enabled CSI-ReportConfig), the same or similar procedures may be used at least for activation/deactivation of the functionality.

Activation of a certain AI/ML functionality can be enabled by the signalling associated with a CSI report configuration (e.g., CSI-ReportConfig activating periodic CSI reports, MAC-CE activating semi-persistent CSI reports, CSI-AssociatedReportConfigInfo for aperiodic CSI reports). Upon the CSI report being active/inactive, the AI/ML functionality implicitly identified for the CSI report which is considered to be activated/deactivated.

Proposal 13: An activated functionality is an AI/ML functionality for which the UE has received a radio configuration to perform inference (e.g., CSI-ReportConfig) and reporting (e.g., CSI-ReportConfigId).

Proposal 14: How applicable functionalities, reported by a UE, are activated is left to network implementation.

A deactivated functionality is an AI/ML functionality for which the UE has received a configuration, but which is not configured to perform inference.

Proposal 13: A deactivated functionality is an AI/ML functionality for which the UE has received a radio configuration, e.g., a CSI-ReportConfig, but which is not configured to perform inference.

Functionality fallback is the procedure of simultaneously deactivating one functionality and activating a non-AI/ML feature.

Proposal 14: Functionality fallback means deactivating one AI/ML-enabled functionality and activating a non-AI/ML feature.

Functionality switching is the procedure of simultaneously deactivating one functionality and activating another.

Proposal 15: Functionality switching means simultaneously deactivating one AI/ML-enabled functionality and activating another.

### 2.4.2	Procedure

Sometime after a functionality has been activated, it might become inapplicable because it no longer satisfies the criteria defined in section 2.3.2. The change in applicability would have triggered a new applicable functionality report from the UE to the gNB, on which the gNB would need to make a decision. Depending on whether the functionality is crucial for NW operation, the gNB could take one of a few different actions:

- Deactivate an activated AI/ML functionality.

- Deactivate an activated AI/ML functionality, simultaneously activating a different applicable AI/ML functionality.

- Deactivate an activated AI/ML functionality, simultaneously activating a non-AI/ML feature.

As written in our RAN1 contribution [2], CSI reports for both BM-Case1 and BM-Case2 based on configurations for aperiodic CSI reporting (e.g., CSI-AssociatedReportConfigInfo ) can already be triggered on-demand using a DCI, and CSI reports based on configurations for semi-persistent CSI reporting can already be triggered on-demand using a MAC-CE. In both cases, the functionality is activated through a DCI or MAC-CE, the functionality is deactivated implicitly at the end of reporting, and the switching or fallback of functionalities is implemented implicitly through the choice of which report to activate.

Observation 4: The specifications already support functionality activation, deactivation, switching, and fallback for aperiodic and semi-persistent CSI reporting using DCI and MAC-CE, respectively.

Periodic CSI reporting configurations can be activated or deactivated through RRC Reconfiguration, e.g., by modifying or removing a CSI-ReportConfig. The typical RRC Reconfiguration procedure has a processing delay of up to 10ms and requires the transmission of two messages: RRCReconfiguration and RRCReconfigurationComplete, while for the aperiodic and semi-persistent, only one message, a DCI or MAC-CE, which can be transmitted alongside normal data traffic, is required.

Observation 5: The specifications support functionality activation, deactivation, switching, and fallback for periodic CSI reporting using RRC Reconfiguration of CSI-MeasConfig by adding, modifying, and/or releasing CSI-ReportConfigs.

Proposal 16: Legacy procedures for activating, deactivating, and switching between aperiodic, semi-persistent, and periodic CSI Reports using DCI, MAC-CE, and RRC Reconfiguration (csi-ReportConfigToAddModList, csi-ReportConfigToReleaseList) can be reused to implement functionality activation, deactivation, switching, and fallback.

### 2.4.3	Fallback

As written in the previous section, functionality fallback means deactivating one AI/ML-enabled functionality and activating a non-AI/ML feature. The purpose of fallback is to mitigate performance losses by reverting to a non-ML method for beam management. However, the NW may fall into unnecessary ping-pong between non-ML and ML-enabled beam management configurations, which could lead to signalling overhead to disabling/enabling ML functionality on one hand and imposing unnecessary activation delays on the other hand.

Because the performance of the AI/ML functionality was satisfactory prior to the decision to fall back, it could be reasonably assumed that the AI/ML functionality will perform well once again.

Observation 6: Because the performance of the AI/ML functionality was satisfactory prior to the decision to fall back, it could be reasonably assumed that the AI/ML functionality will perform well once again.

Therefore, monitoring an AI/ML functionality during fallback could be useful to help the NW determine when to exit fallback and resume the use of the AI/ML functionality to make beam management decisions.

Proposal 17: Discuss signalling to enable fallback without deactivating an AI/ML beam management functionality such that performance monitoring of the AI/ML functionality can continue during fallback.

## 2.5	Performance Monitoring

For the UE-sided model performance monitoring, two types based on which entity (configures (e.g. gNB) or indicate/request (eg. UE)) monitors the performance, were introduced during Release 18 SI. Type 1 focuses on LCM decisions based on performance monitoring metrics at the NW-side (Option 1) and LCM decisions at the NW based on performance monitoring metrics calculated and reported by the UE (known as UE-assisted performance monitoring or Option 2). Type 2 performance monitoring considers LCM decisions at the UE based performance monitoring metrics calculated at the UE. Type 2 does not require any RAN2 related signalling/triggering enhancements as it is mainly applicable for model-level monitoring. However, Type 1 performance monitoring including both Option 1 and Option 2 may require triggering/signalling enhancements as either network calculates performance metrics or relevant KPIs (Option1) or UE uses the output of the AI/ML model and calculate the relevant metric (Alt 1 or Alt 4 as were among the most attractive KPIs during Release 18 SI). However, any discussions on the triggering/signalling of the calculated metric in RAN2 should be discussed after RAN1 progress on deciding the performance monitoring metric.

Proposal 18: RAN2 agrees that signalling is only needed to support Type 1 performance monitoring metrics. There is no impact to RAN2 for Type 2 performance monitoring metrics.

## 2.6 	LS response to RAN3

RAN2 informs RAN3 that RAN2 will consider the definition of AI/ML definitions when RAN2 discussion AI/ML terminology for Release 19.

Proposal 19: RAN2 to consider response to LS R2-2406224 in Appendix.

# 3	Conclusion

In this document, the following observations were made:

Observation 1: A functionality in functionality-based LCM is a configuration constructed using conditions indicated by UE capabilities.

Observation 2:  NW-side additional conditions could be signalled through associated ID.

Observation 3 : An AI/ML functionality is assumed to be applicable if three conditions are satisfied: the UE contains one or more AI/ML models which support the configuration provided by the gNB; and, of those AI/ML models, one or more AI/ML models are compatible with the NW-side additional conditions; and, of those AI/ML models, one or more AI/ML models satisfy the UE-side additional conditions.

Observation 4: The specifications already support functionality activation, deactivation, switching, and fallback for aperiodic and semi-persistent CSI reporting using DCI and MAC-CE, respectively.

Observation 5: The specifications support functionality activation, deactivation, switching, and fallback for periodic CSI reporting using RRC Reconfiguration of CSI-MeasConfig by adding, modifying, and/or releasing CSI-ReportConfigs.

Observation 6: Because the performance of the AI/ML functionality was satisfactory prior to the decision to fall back, it could be reasonably assumed that the AI/ML functionality will perform well once again.

And the following proposals were made:

Proposal 1: In the beam management use case, configuration of a functionality is signalled by CSI-ReportConfig.

Proposal 2 : There is no RAN2 signaling impact due from UE-side additional conditions.

Proposal 3:  Determining the applicability of an AI/ML functionality is up to UE implementation.

Proposal 4: Adopt Figure 1 to illustrate applicability reporting immediately after an initial configuration.

Proposal 5: Adopt Figure 2 to illustrate applicability reporting following the initial applicability report.

Proposal 6: RAN2 to agree that an applicable functionality report will at least allow the UE to report applicable functionality related updates and reason (or cause) pertaining to the updated information (e.g., model available, model updated, etc.,)

Proposal 7: Support the use of UAI to signal updated applicable functionality reports and possibly the initial applicable functionality report following a configuration/reconfiguration.

Proposal 8: Support the use of RRCReconfigurationComplete to signal the initial applicable functionality report following a configuration/reconfiguration.

Proposal 9: UE does not perform inference operation for any of the configured functionalities until the completion of the first applicable functionality report.

Proposal 10: RAN2 to support reporting the non-applicability of a functionality implicitly through omission or explicitly through a flag indicating applicable or non-applicable.

Proposal 11: RAN2 to discuss whether applicable functionality reporting is supported for handover should be discussed in RAN2 during Release 19.

Proposal 12: RAN2 to discuss whether applicable functionalities are stored prior to transitioning to RRC_INACTIVE or RRC_IDLE and whether applicable functionality reporting is supported while UE transitions from RRC_INACTIVE or RRC_IDLE to RRC_CONNECTED.

Proposal 13: An activated functionality is an AI/ML functionality for which the UE has received a radio configuration to perform inference (e.g., CSI-ReportConfig) and reporting (e.g., CSI-ReportConfigId).

Proposal 14: How applicable functionalities, reported by a UE, are activated is left to network implementation.

Proposal 15: Functionality switching means simultaneously deactivating one AI/ML-enabled functionality and activating another.

Proposal 16: Legacy procedures for activating, deactivating, and switching between aperiodic, semi-persistent, and periodic CSI Reports using DCI, MAC-CE, and RRC Reconfiguration (csi-ReportConfigToAddModList, csi-ReportConfigToReleaseList) can be reused to implement functionality activation, deactivation, switching, and fallback.

Proposal 17: Discuss signalling to enable fallback without deactivating an AI/ML beam management functionality such that performance monitoring of the AI/ML functionality can continue during fallback.

Proposal 18: RAN2 agrees that signalling is only needed to support Type 1 performance monitoring metrics. There is no impact to RAN2 for Type 2 performance monitoring metrics.

Proposal 19: RAN2 to consider response to LS R2-2406224 in Appendix.

# 4 References

[1] R2-2406381– Report of [POST126][032][AI/ML PHY] LCM (Intel/Samsung)_Phase 2.

[2] R2-2404906 – AI/ML for Beam Management, 3GPP TSG RAN WG1 #117 – May 20th – May 24th, 2024.

# 5 Appendix

3GPP TSG-RAN WG2 Meeting #127	R2-240xxxx

Maastricht, Netherlands, 19 – 23 August 2024

Title:	[DRAFT] Reply LS on terminology definitions for AI-ML in NG-RAN

Response to:	R3-243969 – R2-2406224

Release:	Release 18

Work Item:	NR_AIML_air-Core

Source:	Nokia [to be RAN2]

To:	RAN3

Cc:	RAN1, RAN, SA

Contact Person:

#### Name:	Sakira Hassan

E-mail Address:	sakira.hassan@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN2 thanks RAN3 for their LS regarding defining AI/ML terminology by referencing TS 28.105. RAN2 agrees with the approach of RAN3 to define the terminologies in clause 16.20 in TS 38.300, which is related to the RAN3 AI/ML for NG-RAN. RAN2 will take into account the proposed AI/ML definitions when they discuss AI/ML terminology for Release 19.

2. Actions:

To RAN3.

ACTION: 	RAN2 respectfully asks RAN3 to take the above into account.

3. Date of Next TSG-RAN WG2 Meeting:

RAN2#127bis	from 2024-10-14	to 2024-10-18		China, Hefei

RAN2#128	from 2024-11-18	to 2024-11-22		Orlando, USA