3GPP TSG-RAN WG2 Meeting #127	R2-2407067

Maastricht, Netherlands, 19 – 23 August 2024

Agenda item:	8.1.3

Source:	Nokia

Title:	Data Collection for Training of NW-side models

WID/SID:	NR_AIML_Air-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

This contribution elaborates on data collection for NW-side AI/ML use cases with particular insight into UE configuration  and transfer of the necessary UE-generated data.

Considerations address two identified use cases for the normative phase: Beam Management and Positioning and lead to concluding proposals on principles that should yield Release 19 enhancements.

# 2	Discussion

## 2.1	Beam Management

Prerequisites

Training of NW-side AI/ML models for beam management, where inference happens on gNBs, will rely on measurements of CSI-RS, which are configured by the gNB, and transmitted by the UE to the gNB. In addition to UE-generated data like measurements on CSI-RS, the configurations and other NW-relevant information should be well understood to be accessible and available to gNB for NW-side data collection.

Observation 1: Training of NW-side AI/ML models for beam management will rely on measurements of CSI-RS, which are configured by the gNB, and transmitted by the UE to the gNB.

Proposal 1: Any network-relevant information for training of NW-side AI/ML models for beam management is assumed to be accessible to gNB.

Measurements configuration

Training of NW-side models for beam management will rely on the UE that receives a CSI reporting configuration containing the measurement and reporting configuration for the set of RS resources (e.g. NZP-CSI-RS resources or SSB resources). UE performs measurements on these RS resources specified in CSI-MeasConfig and reports CRI/RSRP or SSBRI/RSRP. For NW-side model, in particular, L1-RSRP will be used as training data. Though pending on the addition of any new measurement quantities that may be defined, the legacy CSI Reporting Framework is sufficient to configure the UE to measure, collect and report beam related measurements for the training of AI/ML NW-side models for beam management.

Proposal 2: For data collection for training of NW-side models for beam management, the legacy CSI measurement configuration and reporting framework is sufficient. CSI-MeasConfig can be used as a baseline to configure CSI-RS measurements. Detailed configuration parameters applicability or extensions is FFS.

Measurements reports configuration

RAN2#126 agreed the following for NW-side data collection:

Agreements for beam management

1. 	For gNB centric and OAM centric (for RRC signaling between UE and gNB), reporting multiple instances of logged L1 measurement result from UE to gNB via a RRC message as configured by gNB is an optional feature.  FFS how to handle case when single RRC message is not sufficient.   FFS if there will be any further enhancement needed pending RAN1 agreement.

2.	Immediate MDT is the baseline framework for OAM-centric data collection for the training of a network-sided model

3.	Enhance the immediate MDT framework to support periodical reporting.   FFS whether and what event-based reporting is supported and FFS on network request reporting

In order to enable data collection by UE, UE may provide its capabilities to log L1 measurements and report the logged L1 measurements from UE to network via RRC message. If UE provides logging capability, gNB can configure UE to log and then send the measurement reports which can be used for data collection for training of network-side model in beam management. In general, Immediate MDT uses an RRCReconfiguration message for activating RRM-related data collection. On the other hand, for beam management, the RRCReconfiguration message already conveys the IE CSI-ReportConfig. Therefore, the legacy CSI-ReportConfig within RRCReconfiguration message serves a good reference and baseline. It is possible to reuse the CSI-ReportConfig to configure the UE to log beam-level L1 measurements. In addition to this, CSI-ReportConfig may need to be enhanced to allow necessary trigger to enable storing these measurements within a report and transmission of this report over L3 layer.

Observation 2: The logging capability of a UE is to log L1 measurements and report via RRC message(s) configured by gNB.

Proposal 3: For data collection, Immediate MDT activation procedure, i.e., RRC Reconfiguration is used to configure the UE to log beam-level L1-RSRP measurements when logging capability is available in the UE.

Proposal 4: Enhance CSI-ReportConfig with necessary information to trigger measurements report storing and measurements delivery over DCCH (L3) considering UE logging availability. Detailed logging and reporting criteria are FFS.

Allowing logging of L1 measurements could mean that transmission of the reports does not need to be in real-time. Additionally, from RAN1 reply LS [R1-2310681], we note that the data collection for offline model training considers a relaxed latency. On the other hand, reportConfigType  included in CSI-ReportConfig IE are periodic, semi-persistent and aperiodic [TS 38.331 V18.2.0] and regardless of report configuration types, the transmission of the measurement report happens in real-time. In many cases, if report transmission is done in real-time for both training data collection and usual network operations, both network performance and data collection can be affected. Thus, to introduce a relaxed reporting in CSI reporting framework, we propose

Proposal 5: Enhance CSI-ReportConfig to enable relaxed latency during reporting of logged beam-level L1-RSRP measurements considering UE logging availability. Detailed report transmission criteria FFS.

RRC messages

Logging of L1 measurements again allows UE to store a large volume of measurements which could be reported by, for example, periodical reporting. Thus, it could be that one RRC message might not be sufficient. However, it is yet to be clarified what the details are for logging and reporting conditions for data collection.

Observation 3: For RRC Connected UE with periodical reporting agreed for data collection overcomes the scenario when single RRC message (9kB) is not sufficient.

Proposal 6: Until detailed logging and reporting conditions are clarified for data collection, RAN2 assumes that single RRC message (9kB) is sufficient for logged CSI-Reports transmission.

## 2.2	Positioning

RAN2 has made good progress and come up with different options for data collection framework for training UE-side model. In our companion paper on “Data Collection for Training of UE-side models” we propose that the data collection framework for training UE-side model for the positioning use case adopts the LMF as the first termination entity and continue to study how to transfer data from the LMF to the server for data collection for UE-side model training for positioning. For positioning use case even for training of the NW-side model, either in gNB or LMF, we can adopt the LMF as the first termination entity for data collection. This allows the data from gNB or LMF, as data generation entities, to be collected at the LMF. It is also possible that the UE can be a data generation entity for training of NW-side model when the model resides in the LMF. When we resolve the issue of what is the final termination entity for the collected data, we will be able to address the issue of how data is transferred from the final termination entity to the place where the model is used for positioning use case (in gNB or LMF).

Proposal 7: The LMF is considered as the first termination entity for data collection for training of NW-side models for AI/ML positioning.

Proposal 8: The LPP and NRPPa protocols are considered as protocols for data transfer, depending on the data generation entity, for training of NW-side models for AI/ML positioning. It is proposed to check with RAN3 on the feasibility for use of NRPPa protocol for data collection for training of NW-side models for AI/ML positioning.

Proposal 9: The LPP and NRPPa protocols are considered as protocols for configuration of the data generating entity to generate the data used for training of NW-side models for AI/ML positioning. It is proposed to check with RAN3 on the feasibility for use of NRPPa protocol for configuration of data generating entity for data collection for training of NW-side models for AI/ML positioning.

# 3	Conclusion

This document has made the following observations and proposals:

Beam Management

Observation 1: Training of NW-side AI/ML models for beam management will rely on measurements of CSI-RS, which are configured by the gNB, and transmitted by the UE to the gNB.

Observation 2: The logging capability of a UE is to log L1 measurements and report via RRC message(s) configured by gNB.

Observation 3: For RRC Connected UE with periodical reporting agreed for data collection overcomes the scenario when single RRC message (9kB) is not sufficient.

Proposal 1: Any network-relevant information for training of NW-side AI/ML models for beam management is assumed to be accessible to gNB.

Proposal 2: For data collection for training of NW-side models for beam management, the legacy CSI measurement configuration and reporting framework is sufficient. CSI-MeasConfig can be used as a baseline to configure CSI-RS measurements. Detailed configuration parameters applicability or extensions is FFS.

Proposal 3: For data collection, Immediate MDT activation procedure, i.e., RRCReconfiguration is used to configure the UE to log beam-level L1-RSRP measurements when logging capability is available in the UE.

Proposal 4: Enhance CSI-ReportConfig with necessary information to trigger measurements report storing and measurements delivery over DCCH (L3) considering UE logging availability. Detailed logging and reporting criteria are FFS.

Proposal 5: Enhance CSI-ReportConfig to enable relaxed latency during reporting of logged beam-level L1-RSRP measurements considering UE logging availability. Detailed report transmission criteria FFS.

Proposal 6: Until detailed logging and reporting conditions are clarified for data collection, RAN2 assumes that single RRC message (9kB) is sufficient for logged CSI-Reports transmission.

Positioning

Proposal 7: The LMF is considered as the first termination entity for data collection for training of NW-side models for AI/ML positioning.

Proposal 8: The LPP and NRPPa protocols are considered as protocols for data transfer, depending on the data generation entity, for training of NW-side models for AI/ML positioning. It is proposed to check with RAN3 on the feasibility for use of NRPPa protocol for data collection for training of NW-side models for AI/ML positioning.

Proposal 9: The LPP and NRPPa protocols are considered as protocols for configuration of the data generating entity to generate the data used for training of NW-side models for AI/ML positioning. It is proposed to check with RAN3 on the feasibility for use of NRPPa protocol for configuration of data generating entity for data collection for training of NW-side models for AI/ML positioning.