3GPP TSG-RAN WG2 Meeting #127	R2-2407068

Maastricht, Netherlands, 19 – 23 August 2024

Agenda item:	8.1.4

Source:	Nokia

Title:	Data Collection for Training of UE-side models

WID/SID:	NR_AIML_Air-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

Release 19 Work Item on AI/ML for NR Air Interface [1] defined the following objective:

Study objectives with corresponding checkpoints in RAN#105 (Sept ’24): - CN/OAM/OTT collection of UE-sided model training data [RAN2/RAN1]: - For the FS_NR_AIML_Air study use cases, identify the corresponding contents of UE data collection - Analyse the UE data collection mechanisms identified during the FS_NR_AIML_Air (TR 38.843 section 7.2.1.3.2) study along with the implications and limitations of each of the methods

RAN2#126 have identified four frameworks that could serve the purpose of data collection of the UE-sided model (Option 1/1b/2/3) and discussed details on the studied solutions in [2] and [3].

This contribution elaborates further on the applicability of studied data collection frameworks for AI/ML Data Collection for offline training of UE-side models. Considerations address Release 18 identified use cases for the normative phase and we propose agreeing on principles that should yield Release 19 enhancements.

# 2	UE-sided model training data transfer

This section addresses the following FFS items discussed in RAN2#126 meeting:

##### 7.2.1.3.2 Data collection for UE-side model training The following options were discussed in RAN2: 1a. UE collects and directly transfers training data to the data collection entity outside the MNO (e.g. Over-The-Top (OTT) server) for UE-side model training. No 3GPP specification impact is expected. 1b. UE collects training data and transfers it to the server for data collection for UE-side model training (inside the MNO) and then optionally from the server for data collection for UE-side model training to the OTT server (outside the MNO). FFS whether according to RAN2 there might be 3GPP specification impact. 2. UE collects training data and transfers it to Core Network. Core Network transfers the training data to the server for data collection for UE-side model training/OTT server. FFS whether according to RAN2 there might be 3GPP specification impact. 3. UE collects training data and transfers it to OAM. OAM transfers the training data to the server for data collection for UE-side model training/OTT server. FFS whether according to RAN2 there might be 3GPP specification impact. […]

Table 7.2.1.3.2-1. Analysis of different data collection options for UE-side model training. Option Aspect; Option 1a); Option 1b); Option 2; Option 3. First termination entity; Training entity (e.g., Over-The-Top (OTT) server); Server for data collection for UE-side model training; Inside the CN; Inside OAM domain. AI/ML-specific Data Transfer Path; UE to OTT server via either 3GPP or non-3GPP network; UE ->Server for data collection for UE-side model training/OTT server (Note 4); UE-> CN -> Server for data collection for UE-side model training/OTT server (Note 4); UE-> gNB->OAM -> Server for data collection for UE-side model training/OTT server (Note 4). UP/CP tunnel; UP tunnel (for the case of data transfer from UE to OTT server via 3GPP network); UP tunnel; CP tunnel (provided that the data volume remains within the NAS signalling capacity) FFS: UP tunnel FFS: LPP; CP tunnel (provided that the data volume remains within the RRC signalling capacity) FFS: UP tunnel. Protocol layer for data transfer; Application layer; Application layer; NAS layer for CP tunnel FFS: the protocol layer for UP tunnel; RRC layer for CP tunnel FFS: the protocol layer for UP tunnel. Controllability of MNO on data transfer; No AI/ML specific controllability; Controllability can be achieved. FFS: level of controllability; Full controllability (Note 1); Full controllability (Note 1). Solution for network controllability; N/A (the OTT server can directly request data from the UE); Example: per PDU sessions; Via NAS procedure FFS: impact to other layers; Via RRC procedure. Possible Options for Visibility of data content in MNO and Data format (Note 2, Note 3); No standardized visibility; FFS; Opt A) Full visibility for standardized data content. FFS: Opt B) Partial visibility for partially standardized data content. FFS Opt C) No standardized visibility. FFS: meaning of ‘partial/partially’ and how to achieve different levels of visibility.; Opt A) Full visibility for standardized data content. FFS: Opt B) Partial visibility for partially standardized data content. FFS Opt C) No standardized visibility. FFS: meaning of ‘partial/partially’ and how to achieve different levels of visibility. Involved WGs; N/A; SA2, SA3, RAN2, RAN3, CT1; SA2, SA3, RAN3, RAN2, CT1 and CT3; RAN2, RAN3, SA3, SA5, FFS SA2. - Note 1: Full controllability: The MNO has the capability to manage data transfer to the server for UE-side data collection. This includes initiating, terminating, and fully managing data transfer. FFS if further refinements/modifications to this definition are needed. - Note 2: Visibility of data content signifies the capability of the MNO to, at least, be aware of, access, and comprehend the data during transfer. FFS if further refinements/modifications to this definition are needed (e.g. on the capability of the MNO to modify the collected data). - Note 3: The following options are identified to realize the different levels of data content visibility to the MNO: - Full visibility for standardized data content. - Partial visibility for partially standardized data content. - No standardized visibility. - Note 4: The potential involvement of NF or other higher layers entities/functionalities should be discussed in other WGs. Impact on the OTT server is not in the scope of RAN2 discussion..

## 2.1	Analysis on UE-side data collection options

The studied options in 3GPP RAN2  to collect data from UEs for offline training of UE-side models considered the Options listed above.

The dependency on application layer (protocol and OTT server) and data transfer path that does not consider Control Plane risk feasibility of the full controllability by RAN for transfer of the collected data. As the Option 1b is supposed to use only UP tunnel and application layer for data transfer, full controllability over data collection transfer cannot be achieved.

Observation 1: Full controllability over the transfer of collected training data can be achieved in RAN only by Options 2 and 3. Full controllability over transfer of collected data cannot be achieved in RAN for Option 1b due to use of only of UP tunnel and application layer transfer mechanisms.

In addition, Option 1b fails to consider the requirement that the RAN node involvement is needed in some cases to support configurations for UE training data collection for the current WI (Rel-19) use cases and the potential use cases in the future. The studied solutions for the transfer of collected (offline) training data for UE-side models need to become eventually applicable to at least two identified use cases for the Rel-19 normative phase so far, i.e., Beam Management and Positioning. Thus, a target solution for data collection transfer need to involve the gNB in the data collection pathway too.

Although the RAN2 analysis focused on the process of transferring the collected training data from the UE to a server for UE data collection (i.e. after data collection has been activated beforehand), the gNB and LMF play a central point for activating data collection for all use cases Rel-19 normative phase.

The lack of a clear link to RAN node for UE configurations and involvement of RAN node challenges data collection framework suitability for RAN use cases and its future proofness on top of the concern on preserving privacy requirements.

We note, the study item phase is tied to a checkpoint in RAN#105 (Sept ’24) [1]. Therefore, we propose to recommend to RAN#105 a way forward for training data collection for UE-side models that considers only solutions with full controllability by the MNO, CP tunnel availability and RAN reachability.

Proposal 1: RAN2 recommendation is to develop data collection mechanism for UE-side models offline training with: a) full controllability by the MNO, b) CP tunnel availability and c) RAN reachability.

## 2.2	Way forward for UE-side data collection

3GPP impact:

In the following, we discuss the potential impact on 3GPP specifications for the identified Options 1b, 2 and 3.

Option 1b - The involvement of 3GPP standardization clearly differentiates this option with Option 1a (where no 3GPP specification impact is involved). Note that, the first termination entity, in this case, is the server for data collection for UE-side model training which is inside the MNO. Moreover, configuration of UE and transfer of data may involve WGs, such as SA2, SA3, RAN2, RAN3, CT1. Thus, from RAN2’s perspective, 3GPP standardization is expected. However, the involvement of NF and other higher level functionalities will be discussed in the respective WGs, which may not be RAN2. The involvement of RAN2 for this option has been discussed in Section 2.1.

Option 2 – In this case, the first termination entity is inside CN domain and the data being transferred to the server for data collection for UE-side model training via CN. This means that there is 3GPP standardization impact which could involve SA2, SA3, RAN3, RAN2, CT1 and CT3. Moreover, full controllability and full visibility may require specification impact at least over NAS protocol layer.

Option 3 – Since the first termination entity is inside OAM domain in this option, the impact on RRC procedure is obvious. Therefore, 3GPP specification impact at least from RAN2, RAN3, SA3, SA5 WGs can be foreseen.

Based on the above justification, we propose

Proposal 2: To replace “FFS whether according to RAN2 there might be 3GPP specification impact” from Option 1b, Option 2, and Option 3 with “RAN2 assumes that 3GPP specification impact is expected.”

Involved WGs

The list of identified involvement of other WGs, includes TSG SA2 WG for all Options. SA2 WG has started SID [5] with one of the work tasks, WT1.1 on “Study whether and how to support UE data collection to meet requirements for RAN AI support for air interface operation (for RAN) for UE-side model training […]” This includes “identifying the potential impacts on the 5G framework, including potential enhancements to policy control and OAM”. Therefore, the clarifications and removal of the some of the FFS items from the Table 7.2.1.3.2-1 in TR 38.843 shall be implemented to allow the SA2 study progress. Similarly, the involvement of all other WGs is eased.

Observation 2: The work progress in the other involved WGs identified in Table 7.2.1.3.2-1 in TR 38.843 strongly depends on the clarity of the information provided by RAN2 in Table 7.2.1.3.2-1.

Mapping with Release 19 use cases

Options 2 and 3 provide full controllability inside the MNO, CP tunnel availability and RAN reachability, but imply two different existing (CP) signalling frameworks:

- Option 2: CN-based collection of data for offline training of UE-side models relies on NAS protocol, and

- Option 3: OAM-based collection of data for offline training of UE-side models relies on RRC protocol.

The approaches have been considered due to matching legacy enablers and potential reuse of existing frameworks. Corresponding LPP and MDT data collection frameworks can be reused (and enhanced if necessary) to meet the requirements for specific data types’ collection in Release 19 (RAN1 use cases identified in [1]). While the Option 2 is suitable for Positioning use case, the Option 3 becomes easily applicable e.g. for Beam Management use case or the CI compression use case.. The suitability of Option 2 and Option 3 for other, future, use cases should not be precluded.

For completeness of the analysis, the applicability of the studied options cannot disconnect the AI/ML-specific Data Transfer Path and associated protocols, from the corresponding data collection activation procedures, which can guarantee fully controllability. For that reason, we observe that the Option 2 and 3 are enabled by more granular control signalling :

- Option 2: CN-based collection of data for offline training of UE-side models relies on LPP (conveyed by NAS) /NRPPa signalling, and

- Option 3: OAM-based collection of data for offline training of UE-side models relies on RRC/Trace signalling.

Proposal 3: RAN2 includes in the TR38.843 the applicability of the studied Options 2 (CN based option) and 3 (OAM based option) to at least Positioning and Beam Management use cases, respectively.

Impact in RAN2

Regardless of the first termination entity selection for data collection, the process will have to rely on gNB or LMF involvement and RRC and LPP protocols’ support. For Rel-19 radio use cases data transfer for offline training of UE-side models shouldn’t essentially differentiate in RAN (from the UE to gNB) in case the first termination entity (as mentioned in the Table 7.2.1.3.2-1) is elsewhere, beyond RAN (e.g. OAM/CN/other NW entity). Thus, to facilitate relevant CN and/or OAM support for Beam Management and Positioning use cases, we believe RAN2 should work based on the assumption that:

- Data collection and transfer for offline training of UE-side models can be supported by LPP procedures for Positioning use case, preserving extensions to support data transfer onwards (pending RAN3/SA2/SA5 decision)

- Data collection and transfer for offline training of UE-side models can be supported by RRC procedures for Beam Management use case, preserving extensions to support data transfer onwards (pending RAN3/SA5 support).

Observation 3: From RAN2 perspective, Option 2 (CN based option) relies on LPP enhancement while Option 3 would imply RRC related enhancements.

Proposal 4: For data collection, RAN2 to adopt data transfer for offline training of UE-side models by:

- LPP procedures for Positioning use,

- RRC procedures for Beam Management use case, and

- Identification of necessary resulting requirements for RAN3, SA2 and SA5.

Proposal 5: RAN2 to consider enhancement on LPP protocol and RRC related enhancements for UE side data collection.

Updates in TR 38.843

Proposal 6:  RAN2 captures in the TR 38.843 that collection of data for offline training of UE-side models relies on LPP (conveyed by NAS)/NRPPa signalling (managed by RAN3) for Option 2, and on RRC enhancement  for Option 3. Trace/MDT signaling enhancements can be envisioned for Option 3 which is managed by SA5.

Proposal 7: Agree TP in Annex that removes OTT server reference and FFS words in Options 1b/2/3, and incorporates Proposal 3 and Proposal 6.

Challenge in a unified framework

The differences in suitable protocols and use cases applicability among the studied options, introduce a challenge to select one unified data collection framework for UE-sided models. The potential aim to identify a centralized data collection training entity may introduce requirements on extensions to network interfaces, which would require support from RAN3, SA5 and SA2.

In our understanding, Option 3 (OAM based training data collection) is supporting configurations for vendor-specific data collection towards gNB, including non-standardized data with minimum security and privacy risks. However, adopting such mechanism to RAN specific data would require further coordination among RAN3, SA5, SA2, CT1 and SA3 (due to captured security concerns [4]).

Observation 4: The OAM based training data collection is the only option supporting configurations for data collection including non-standardized data with minimum security and privacy risks and most capability for overhead reduction.

UE selection

Even though in principle existing data collection procedures can be reused, some enhancements may be necessary in order to make data collection process more efficient. For example, for offline training of different available UE-side models it would save resources both at the network and at the UE side if only UEs from the specific UE vendors or UE types, relating to the corresponding UE-side models to be trained, are selected for the data collection process. In current methods, core network could initiate data collection towards a specific UE. However, initiating data collection requests on a per UE basis is not scalable when data from a large number of UEs need to be collected. In RAN, it is possible that gNB selects UEs in a certain area for measurements. However, when it comes to RRC procedures, a gNB may not be aware of the UE vendor or UE type of the UE selected to provide the measurements.

Proposal 8: Enable data collection for UE-side Model Training from specific UE vendors or UE Types.

# 3	Training data collection configuration for UE-sided model

## 3.1	General aspects

To facilitate data collection from the UE, the UE will need to be configured to take measurements to collect training data for UE-side models regardless of the destination of said measurements.  The configuration of signals for measurement, e.g., CSI-RS and PRS for the training of UE-side models, would imply a pre-requisite configuration to initiate data collection for training of UE-side models which could eventually apply to any (RAN, CN or OAM) -based data collection solution. The UE needs to be configured to take measurements to collect training data for UE-side models regardless of the destination of measurements, e.g., UE, RAN, CN, OAM. Therefore, in the following sections we propose to identify the configurations for measurement signals, e.g., CSI-RS and PRS, which would have come from the gNB or the LMF, depending on the use case to enable data collection.

Proposal 9: To collect training data for UE-side models regardless of the destination of measurements, the UE is configured with reference signals CSI-RS or PRS, by the gNB or LMF.

## 3.2	Beam Management

CSI-RS Configuration

Considering data collection, when configuring a UE with an AI/ML-enabled beam management, for UE-side BM-Case 1 and BM-Case 2, it is desirable to adopt the legacy beam measurement configuration to support the configuration of UE-side AI/ML model. The use case specific configurations developed and agreed under generic LCM procedures should become applicable, without a need to specify a data collection specific purpose:

Proposal 10: To configure data collection for training UE-side models for BM-Case 1 and BM-Case 2, the gNB initiates the configuration through the configuration of resources for measurement for beam management.  FFS detailed identification of applicable configuration parameters (e.g., NZP-CSI-RS-Resource, NZP-CSI-RS-ResourceId, and NZP-CSI-RS-ResourceSets, NZP-CSI-RS-ResourceSetId).

## 3.3	Positioning

The following agreements pertaining to data collection for training UE-side AI/ML positioning models were reached in the previous two RAN1 meetings. In this section, we will describe some ways forward to enable data collection for training UE-side AI/ML positioning models using the LMF and the LPP protocol.

Agreements from RAN1#116bis: Agreement For training data collection of AI/ML based positioning, the collected data sample can include the following components: Part A: - channel measurement - quality indicator of channel measurement - time stamp of channel measurement Part B: - ground truth label (or its approximation) - quality indicator of label - time stamp of label Note: “Part A” and “Part B” terminologies are only for RAN1 discussion purpose, and may not be used in specification. Note: contents in Part A and Part B may or may not be generated by different entities. Note: Part A and/or Part B, and their contents may or may not apply for each case FFS: detailed definition of channel measurement Agreements from RAN1#117 Agreement For training data collection of AI/ML based positioning, if a training data sample contains both Part A and Part B, RAN1 assumes that Part A and Part B in one training data sample are: - for a same UE (PRU or Non-PRU UE), and - for a same location associated with Part B. Note: the association can be discussed

LPP through Release 18 has enabled the configuration of measurement quantities such as PRS, indication of the quality of a location or timing estimate, and the signaling of a timestamp related to measurement data. For LMF-based positioning, the LMF has been able to configure the UE to make measurements and report them to the NW. We propose to evaluate LPP for any possibilities of reusing existing LPP procedures for AI/ML data collection. Some data available for collection in existing positioning methods is exemplified below.

Part A data, pertaining to measurements and their characteristics, described in the previous RAN1 agreements, has been demonstrated to be available through existing mechanisms through the following observation.

Observation 5: Positioning methods such as NR DL TDOA [6] support the reporting of the following, which match the agreement from RAN1#116bis.

- Channel Measurement – NR-DL-TDOA-MeasElement-r16 -> nr-Dl-PRS-RSRP-Result-r16

- Quality Indicator – NR-DL-TDOA-MeasElement-r16 -> nr-TimingQuality

- Timestamp – NR-DL-TDOA-MeasElement-16 -> nr-TimeStamp-416

Proposal 11: The acquisition of Part A data, channel measurement, quality indicator, and timestamp, reusing LPP procedures has been demonstrated through existing NR positioning methods, i.e., through the positioning method-specific LPP Provide Location Information messages, and can be extended to support new AI/ML positioning methods.

Part B data, pertaining to ground truth, described in the previous RAN1 agreements, has been demonstrated to be available through existing LPP procedures through the following observation.

Observation 6: Through the execution of existing positioning methods, the LMF and LPP support the acquisition of UE and PRU locations, which can be used as ground truth associated with collected data.

Proposal 12: The acquisition of Part B data namely, ground truth label, quality indicator, and timestamp, can be realized through LPP by reusing existing LPP procedures.

The following proposal from [7] would set the following requirement on data collection for UE-side models.

Proposal 1: The network should have full control of the data collection process for the training of a UE-sided model, including the initiation, termination and management of the data transfer.

Based on the following observation, we believe that the LMF is a suitable entity to enable full control of the data collection process for the training of a UE-sided model for AI/ML positioning.

Observation 7: The LMF and the LPP protocol satisfy the conditions of the network having full control of the data collection process for the training of a UE-sided model, including initiation, termination, and management of the data transfer.

- Initiation – The LMF does not trigger the initiation of positioning sessions, but the LMF initiates the data collection procedure in the context of LPP by issuing configuration(s) through LPP Request Location Information (reporting configuration) and LPP Provide Assistance Data (PRS configuration). FFS: How the LMF is triggered to initiate data collection procedures.

- Termination – The first termination point after a UE makes a measurement for AI/ML positioning is the LMF. The message that carries the measurement is an LPP Provide Location Information message. FFS: How to transfer data from the LMF to the server for data collection for UE-side model training for positioning.

- Management – The LMF is in charge of choosing PRS configurations, even when the UE requests specific configurations. Management is further achieved because the LMF initiates and terminates the data collection.

Proposal 13: The LMF is considered as the first termination entity for Option 2 for AI/ML positioning.

Proposal 14: The LPP protocol is considered as a protocol for data transfer for Option 2 for AI/ML positioning.

# 4	Conclusion

This document has proposed the following observations:

Observation 1: Full controllability over the transfer of collected training data can be achieved in RAN only by Options 2 and 3. Full controllability over transfer of collected data cannot be achieved in RAN for Option 1b due to use of only of UP tunnel and application layer transfer mechanisms.

Observation 2: The work progress in the other involved WGs identified in Table 7.2.1.3.2-1 in TR 38.843 strongly depends on the clarity of the information provided by RAN2 in Table 7.2.1.3.2-1.

Observation 3: From RAN2 perspective, Option 2 (CN based option) relies on LPP enhancement while Option 3 would imply RRC related enhancements.

Observation 4: The OAM based training data collection is the only option supporting configurations for data collection including non-standardized data with minimum security and privacy risks and most capability for overhead reduction.

Observation 5: Positioning methods such as NR DL TDOA [6] support the reporting of the following, which match the agreement from RAN1#116bis.

•	Channel Measurement – NR-DL-TDOA-MeasElement-r16 -> nr-Dl-PRS-RSRP-Result-r16

•	Quality Indicator – NR-DL-TDOA-MeasElement-r16 -> nr-TimingQuality

•	Timestamp – NR-DL-TDOA-MeasElement-16 -> nr-TimeStamp-416

Observation 6: Through the execution of existing positioning methods, the LMF and LPP support the acquisition of UE and PRU locations, which can be used as ground truth associated with collected data.

Observation 7: The LMF and the LPP protocol satisfy the conditions of the network having full control of the data collection process for the training of a UE-sided model, including initiation, termination, and management of the data transfer.

•	Initiation – The LMF does not trigger the initiation of positioning sessions, but the LMF initiates the data collection procedure in the context of LPP by issuing configuration(s) through LPP Request Location Information (reporting configuration) and LPP Provide Assistance Data (PRS configuration). FFS: How the LMF is triggered to initiate data collection procedures.

•	Termination – The first termination point after a UE makes a measurement for AI/ML positioning is the LMF. The message that carries the measurement is an LPP Provide Location Information message. FFS: How to transfer data from the LMF to the server for data collection for UE-side model training for positioning.

•	Management – The LMF is in charge of choosing PRS configurations, even when the UE requests specific configurations. Management is further achieved because the LMF initiates and terminates the data collection.

And proposals:

Proposal 1: RAN2 recommendation is to develop data collection mechanism for UE-side models offline training with: a) full controllability by the MNO, b) CP tunnel availability and c) RAN reachability.

Proposal 2: To replace “FFS whether according to RAN2 there might be 3GPP specification impact” from Option 1b, Option 2, and Option 3 with “RAN2 assumes that 3GPP specification impact is expected.”

Proposal 3: RAN2 includes in the TR 38.843 the applicability of the studied Options 2 (CN based option) and 3 (OAM based option) to at least Positioning and Beam Management use cases, respectively.

Proposal 4: For data collection, RAN2 to adopt data transfer for offline training of UE-side models by:

-	LPP procedures for Positioning use,

-	RRC procedures for Beam Management use case, and

-	Identification of necessary resulting requirements for RAN3, SA2 and SA5.

Proposal 5: RAN2 to consider enhancement on LPP protocol and RRC related enhancements for UE side data collection.

Proposal 6:  RAN2 captures in the TR 38.843 that collection of data for offline training of UE-side models relies on LPP (conveyed by NAS)/NRPPa signalling (managed by RAN3) for Option 2, and on RRC enhancement  for Option 3. Trace/MDT signaling enhancements can be envisioned for Option 3 which is managed by SA5.

Proposal 7: Agree TP in Annex that removes OTT server reference and FFS words in Options 1b/2/3, and incorporates Proposal 3 and Proposal 6.

Proposal 8: Enable data collection for UE-side Model Training from specific UE vendors or UE Types.

Proposal 9: To collect training data for UE-side models regardless of the destination of measurements, the UE is configured with reference signals CSI-RS or PRS, by the gNB or LMF.

Proposal 10: To configure data collection for training UE-side models for BM-Case 1 and BM-Case 2, the gNB initiates the configuration through the configuration of resources for measurement for beam management.  FFS detailed identification of applicable configuration parameters (e.g., NZP-CSI-RS-Resource, NZP-CSI-RS-ResourceId, and NZP-CSI-RS-ResourceSets, NZP-CSI-RS-ResourceSetId).

Proposal 11: The acquisition of Part A data, channel measurement, quality indicator, and timestamp, has been demonstrated through existing NR positioning methods, i.e., through the positioning method-specific LPP Provide Location Information messages, and can be extended to support new AI/ML positioning methods.

Proposal 12: The acquisition of Part B data namely, ground truth label, quality indicator, and timestamp, can be realized through LPP by reusing existing positioning methods.

Proposal 13: The LMF is considered as the first termination entity for Option 2 for AI/ML positioning.

Proposal 14: The LPP protocol is considered as a protocol for data transfer for Option 2 for AI/ML positioning.

# 5	References

- RP-234039, AI/ML for NR Air Interface, Rel-19 WID

- R2-2405931, Report of [POST125bis][020][AI/ML PHY] UE side data collection, MediaTek Inc.

- R2-2406000, Report of [AT126][020][AI/ML PHY] UE side data collections, MediaTek Inc.

- R2-2407209, RAN2 inputs to TR 38.843, Ericsson

- SP-240134, “Study on Core Network Enhanced Support for Artificial Intelligence (AI)/Machine Learning (ML) - FS_AIML_CN”, TSG SA Meeting #103, 19-22 March 2024, Maastricht, Netherlands.

- 3GPP TS37.355, “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; LTE positioning Protocol (LPP) (Release 18), V18.1.0, 2024-03

- R2-2407413, Data collection for the training of a UE-sided model, InterDigital Inc., Ericsson, Nokia, T-Mobile USA Inc., BT PLC., Deutsche Telekom, Verizon, NTT Docomo, Charter Communications Inc.

# Annex TP to TR38.843

##### 7.2.1.3.2	Data collection for UE-side model training

The following options were discussed in RAN2:

1a.	UE collects and directly transfers training data to the data collection entity outside the MNO (e.g. Over-The-Top (OTT) server) which may perform the UE-side model training. No 3GPP specification involvement is expected.

1b.	UE collects training data and transfers it to the server for data collection for UE-side model training. 3GPP specification involvement is expected.

2.	UE collects training data and transfers it to Core Network. Core Network transfers the training data to the server for data collection for UE-side model training. 3GPP specification involvement is expected.

3.	UE collects training data and transfers it to OAM. OAM transfers the training data to the server for data collection for UE-side model training. 3GPP specification involvement is expected.

The options listed above were analysed to understand potential specification impact. The analysis included aspects such as termination entities, data transfer path, whether control plane (CP) or user plane (UP) should be used to transfer the data, protocol layers involved, MNO controllability and visibility over the collected data, etc… The result of this analysis can be found below in Table 7.2.1.3.2-1.

It is worth noting that for the different options, the data content visibility was discussed. The different levels of data content visibility are captured in the Note 3 in the Table 7.2.1.3.2-1.

Note:	In some scenarios the data content visibility could be achieved via SLA. However, the details have been out of the studies scope.

Table 7.2.1.3.2-1. Analysis of different data collection options for UE-side model training.

| Option Aspect | Option 1a) | Option 1b) | Option 2 | Option 3 |
| --- | --- | --- | --- | --- |
| Termination entity | Training entity (e.g., Over-The-Top (OTT) server) | Server for data collection for UE-side model training | Inside the CN | Inside OAM domain |
| AI/ML-specific Data Transfer Path | UE to OTT server via either 3GPP or non-3GPP network | UE-> CN ->Server for data collection for UE-side model training (Note 4) | UE-> CN -> Server for data collection for UE-side model training (Note 4) | UE-> gNB->OAM -> Server for data collection for UE-side model training (Note 4) |
| UP/CP tunnel | UP tunnel | UP tunnel | CP tunnel (provided that the data volume remains within the NAS signalling capacity) FFS: UP tunnel | CP tunnel (provided that the data volume remains within the RRC signalling capacity) FFS: UP tunnel |
| Protocol layer for data transfer | Application layer | Application layer | NAS layer for CP tunnel (LPP conveyed in NAS), NRPPa FFS: the protocol layer for UP tunnel | RRC layer for CP tunnel, Trace/MDT signalling FFS: the protocol layer for UP tunnel |
| Controllability of MNO on data transfer | No AI/ML specific controllability | Controllability cannot be achieved in RAN. FFS: level of controllability | Full controllability (Note 1) | Full controllability (Note 1) |
| Control granularity by NW | N/A (the OTT server can directly request data from the UE) | Example: per PDU sessions | NAS procedure FFS: impact to other layers | RRC procedure |
| Possible Options for Visibility of data content in MNO and Data format (Note 2, Note 3) | No standardized visibility | FFS | Opt A) Full visibility for standardized data content. FFS: Opt B) Partial visibility for partially standardized data content. FFS Opt C) No standardized visibility. FFS: meaning of ‘partial/partially’ and how to achieve different levels of visibility. | Opt A) Full visibility for standardized data content. FFS: Opt B) Partial visibility for partially standardized data content. FFS Opt C) No standardized visibility. FFS: meaning of ‘partial/partially’ and how to achieve different levels of visibility |
| Applicability to ML-enabled RAN use cases | N/A | N/A | Positioning | Beam Management, FFS for Positioning |
| Involved WGs | N/A | SA2, SA3, RAN2, CT1, RAN3, SA4 | SA2, SA3, RAN3, RAN2, CT1 and CT3 | RAN2, RAN3, SA3, SA5, FFS SA2 |
| - Note 1: Full controllability: The MNO has the capability to manage data transfer to the server for UE-side data collection. This includes initiating, terminating, and fully managing data transfer. FFS if further refinements/modifications to this definition are needed. - Note 2: Visibility of data content signifies the capability of the MNO to, at least, be aware of, access, and comprehend the data during transfer. FFS if further refinements/modifications to this definition are needed (e.g. on the capability of the MNO to modify the collected data). - Note 3: For Solution 1b, 2/3, the following options are identified to realize the different levels of data content visibility if different levels of data content visibility to MNO are considered. - Full visibility for standardized data content. - Partial visibility for partially standardized data content. - No visibility for non-standardized data content. - Note 4: The potential involvement of NF or other higher layers entities/functionalities should be discussed in other WGs. |  |  |  |  |

Related to privacy, it has been stressed in RAN2 the importance that any potential mechanism to collect UE side data for model training purposes (including the options 1a, 1b, 2, 3 listed above) must comply with privacy protection regulations, requirements, laws and/or policies. An informative Annex is included at the end of this document capturing examples of privacy concerns for different stakeholders participating in the discussion.