3GPP TSG-RAN WG2 #127	Tdoc R2-2407070

Maastricht, Netherlands, Agust 19 – 23, 2024

Agenda Item:	7.3.2

Source:	Ericsson

Title:	Handling of inter-band SSB-less configuration

Document for:	Discussion, Decision

# 1	Introduction

This contribution discusses the use of inter-band SSB-less configuration and a remaining aspect to be clarified.

# 2	Discussion

In 38.331, the field description for referenceCell states the following:

FrequencyInfoDL information element

-- ASN1START

-- TAG-FREQUENCYINFODL-START

FrequencyInfoDL ::=                 SEQUENCE {

absoluteFrequencySSB                ARFCN-ValueNR                                                   OPTIONAL,   -- Cond SpCellAdd

frequencyBandList                   MultiFrequencyBandListNR,

absoluteFrequencyPointA             ARFCN-ValueNR,

scs-SpecificCarrierList             SEQUENCE (SIZE (1..maxSCSs)) OF SCS-SpecificCarrier,

...,

[[

referenceCell-r18                   ServCellIndex                                                   OPTIONAL    -- Cond SSBlessSCell

]]

}

-- TAG-FREQUENCYINFODL-STOP

-- ASN1STOP

FrequencyInfoDL field descriptions

absoluteFrequencyPointA Absolute frequency position of the reference resource block (Common RB 0). Its lowest subcarrier is also known as Point A (see TS 38.211 [16], clause 4.4.4.2). Note that the lower edge of the actual carrier is not defined by this field but rather in the scs-SpecificCarrierList.

absoluteFrequencySSB Frequency of the SSB to be used for this serving cell. SSB related parameters (e.g. SSB index) provided for a serving cell refer to this SSB frequency unless mentioned otherwise. The CD-SSB of the PCell is always on the sync raster. Frequencies are considered to be on the sync raster if they are also identifiable with a GSCN value (see TS 38.101-1 [15] or TS 38.101-5 [75]). If the field is absent, the SSB related parameters should be absent, e.g. ssb-PositionsInBurst, ssb-periodicityServingCell and subcarrierSpacing in ServingCellConfigCommon IE. If the field is absent, the UE obtains timing reference from the intra-band SpCell or intra-band SCell if applicable as described in TS 38.213 [13], clause 4.1, or from the SpCell or an SCell indicated by referenceCell, or from the "default cell" defined in TS [xx]. This is supported in case the SCell for which the UE obtains the timing reference is in the same or different frequency band as the cell (i.e. the SpCell or the SCell, respectively) from which the UE obtains the timing reference. For cells supporting (e)RedCap, this field corresponds to the CD-SSB.

frequencyBandList List containing only one frequency band to which this carrier(s) belongs. Multiple values are not supported.

referenceCell Indicates the reference cell, i.e. the cell which provides the timing reference and AGC source for the inter-band SSB-less SCell. If the reference cell is an SCell or PSCell, it should be an activated SCell or activated PSCell. If this field is absent a "default cell" is the reference cell as defined in TS 38.133 [14].

scs-SpecificCarrierList A set of carriers for different subcarrier spacings (numerologies). Defined in relation to Point A. The network configures a scs-SpecificCarrier at least for each numerology (SCS) that is used e.g. in a BWP (see TS 38.211 [16], clause 5.3).

| Conditional Presence | Explanation |
| --- | --- |
| SpCellAdd | The field is mandatory present if this FrequencyInfoDL is for SpCell. Otherwise the field is optionally present, Need S. |
| SSBlessSCell | The field is optionally present, Need S, if the inter-band SSB-less SCell is configured for this cell and absoluteFrequencySSB is absent. It is absent otherwise. |

As highlighted above, when referenceCell is absent, the UE should apply a “default cell” behaviour. However, the UE may simply be configured with absoluteFrequencySSB, in which case the referenceCell field will also be absent, and no “default cell” behaviour is applicable. Another case would be a UE configured with both inter-band and intra-band SCells and capable of both the Rel-15 feature (intra-band SSB-less SCell) and the Rel-18 feature (inter-band SSB-less SCell). Such UE may be connected to a network that understands only the Rel-15 feature, which would expect the UE to operate as intra-band SSB-less SCell, even though both inter-band and intra-band SCells are configured.

Absence of the referenceCell field does not always imply on the use of a “default cell” behaviour.

Therefore, it should be clarified that the “default cell” behaviour upon absence of the referenceCell field is only applicable for inter-band SSB-less case. Note that the field condition SSBlessSCell (above) clarifies already in which case referenceCell can be present, but it does not clarify in which case the UE should apply the default behaviour upon absence of the field.

Clarify in 38.331 that the “default cell” behaviour applied upon absence of the referenceCell field is only applicable if both referenceCell and absoluteFrequencySSB are absent for an inter-band SSB-less SCell.

A TP to 38.331 capturing the proposal above is provided in the Annex in section 4, together with an impact analysis.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	Absence of the referenceCell field does not always imply on the use of a “default cell” behaviour.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Clarify in 38.331 that the “default cell” behaviour applied upon absence of the referenceCell field is only applicable if both referenceCell and absoluteFrequencySSB are absent for an inter-band SSB-less SCell.

# 4 Annex: TP to 38.331

Impact Analysis

Impacted 5G architecture options: Standalone, EN-DC, NGEN-DC, NE-DC, NR-DC

Impacted functionality: inter-band SSB-less SCell

Inter-operability: If the network implements the CR and the UE does not, the UE may apply the default cell behaviour in unintended cases.

If the UE implements the CR and the network does not, there is no inter-operability issue, since the UE should apply the default cell behaviour only in case absoluteFrequencySSB is absent for an inter-band SSB-less SCell configuration.

START OF CHANGE

FrequencyInfoDL field descriptions

absoluteFrequencyPointA Absolute frequency position of the reference resource block (Common RB 0). Its lowest subcarrier is also known as Point A (see TS 38.211 [16], clause 4.4.4.2). Note that the lower edge of the actual carrier is not defined by this field but rather in the scs-SpecificCarrierList.

absoluteFrequencySSB Frequency of the SSB to be used for this serving cell. SSB related parameters (e.g. SSB index) provided for a serving cell refer to this SSB frequency unless mentioned otherwise. The CD-SSB of the PCell is always on the sync raster. Frequencies are considered to be on the sync raster if they are also identifiable with a GSCN value (see TS 38.101-1 [15] or TS 38.101-5 [75]). If the field is absent, the SSB related parameters should be absent, e.g. ssb-PositionsInBurst, ssb-periodicityServingCell and subcarrierSpacing in ServingCellConfigCommon IE. If the field is absent, the UE obtains timing reference from the intra-band SpCell or intra-band SCell if applicable as described in TS 38.213 [13], clause 4.1, or from the SpCell or an SCell indicated by referenceCell, or from the "default cell" defined in TS [xx]. This is supported in case the SCell for which the UE obtains the timing reference is in the same or different frequency band as the cell (i.e. the SpCell or the SCell, respectively) from which the UE obtains the timing reference. For cells supporting (e)RedCap, this field corresponds to the CD-SSB.

frequencyBandList List containing only one frequency band to which this carrier(s) belongs. Multiple values are not supported.

referenceCell Indicates the reference cell, i.e. the cell which provides the timing reference and AGC source for the inter-band SSB-less SCell. If the reference cell is an SCell or PSCell, it should be an activated SCell or activated PSCell. If this field is absent and absoluteFrequencySSB is also absent for the inter-band SSB-less SCell, a "default cell" is the reference cell as defined in TS 38.133 [14].

scs-SpecificCarrierList A set of carriers for different subcarrier spacings (numerologies). Defined in relation to Point A. The network configures a scs-SpecificCarrier at least for each numerology (SCS) that is used e.g. in a BWP (see TS 38.211 [16], clause 5.3).

| Conditional Presence | Explanation |
| --- | --- |
| SpCellAdd | The field is mandatory present if this FrequencyInfoDL is for SpCell. Otherwise the field is optionally present, Need S. |
| SSBlessSCell | The field is optionally present, Need S, if the inter-band SSB-less SCell is configured for this cell and absoluteFrequencySSB is absent. It is absent otherwise. |

END OF CHANGE