3GPP TSG-RAN WG2 Meeting #127	R2-2407073

Maastricht, Netherlands, 19th Aug – 23rd Aug 2024

Agenda item:	8.6.2

Source:	Nokia

Title:	On inter-CU aspects for LTM

WID/SID:	NR_Mob_Ph4 - Release 19

Document for:	Discussion and Decision

# Introduction

In this contribution, we discuss the baseline procedure for inter-CU LTM support and identify the related key issues.

# Discussion

## Stage-2 Signaling Procedure for LTM

In Rel. 18, comprehensive discussions on signaling procedure for LTM took place and we would like to keep the Inter-CU signaling procedure close as much as possible for inter-CU case as well. This is also emphasized in the note included in the work item description.

Figure 1: Signalling procedure for Inter-CU LTM

Proposal 1: Rel-18 LTM signaling procedures are reused for inter-CU LTM is considered as basis for Inter-CU LTM for further discussions.

For inter-CU LTM, an important aspect to be treated is the RRC model to be used for inter-CU LTM configuration. We treat this topic in a separate contribution in X. Apart from that, we identify the following key issues to be discussed in RAN2 and describe our preferred way forward for each issue in the sections below.

Issue # 1: Preparation and signaling aspect for LTM-CSI Resource Configuration

Issue # 2: Subsequent LTM

Issue # 3: Security

Issue # 4: DC – LTM

Issue # 5: Support of configured grant-based RACH-less LTM

Issue # 6: Interworking of inter- and intra-CU LTM

## Signalling Procedure during Preparation Phase for LTM Measurements

Compared to Rel-18 Intra CU LTM, the LTM-CSI-Resource configuration preparation and LTM-Report configuration mapping to this resource configuration need to handle following additional issues.

- Exchange of RS information of candidate cells of other CU is needed to construct the LTM-CSI-Resource configuration.

- LTM CSI Resource configurations needs to contain the resource sets relevant for measurements by each candidate cell. In case of Inter-CU it needs to be provided by candidate CU.

We analyze above issues and propose modified signalling procedure for preparation phase for Inter-CU LTM.

Exchange of RS configurations for L1 measurements across CU

In Rel-18 specifications, the L1 measurement resources for LTM are configured using LTM-CSI-Resource-config that is provided as common IE across all LTM candidates.  The LTM-Report-configuration that  links to the measurement resources within this configuration is included in the RRC-configuration of candidate cell as part of CSI-Meas-Config. The LTM config can consist of multiple LTM-CSI-Resource-configs each consists of a LTM-CSI-SSB-ResourceSet comprising a list of SSBs and associated candidate cells. The common LTM-CSI-Resource-configs are generated by CU based on its awareness of SSB configurations of all candidate cells. In case of Inter-CU scenario, the initial source node is not aware of the SSB configurations of candidate cells prior to preparation. So, the information about SSB configuration (number of SSB /Periodicity) needs to be exchanged.

Observation 1:  To prepare L1 measurement resources (LTM-CSI-Resource-config) for Inter-CU scenario, source CU should be aware of the SSB configurations of candidate cells of other CU.

This issue can be solved in the following ways.

O1: UE specific signaling procedure during Handover Preparation step for LTM

- As part of LTM Preparation, source node trigger UE specific signalling procedure towards candidate cells to obtain the SSB configurations of candidate cells and prepare the LTM-CSI-Resource-configs.

- After generation of LTM-CSI-Resource-configs, the LTM handover request containing LTM-CSI-Resource-configs for actual preparation is triggered. In this case, the Target CU provides the LTM candidate configurations including RRC configuration.  The RRC Configuration includes LTM-Report configuration pointing to the LTM-CSI-Resource-configurations provided by source CU for its subsequent LTM.

O2: The SSB configurations can be exchanged across CU using non-UE associated signalling as the SSB configurations and neighbour list relation are static. This information needs not be repeated for every LTM Preparation. However this option requires new non-UE associated signalling for exchange of the SSB information whose size may be larger depending number of serving cells of each CU.

Both the above options require further changes to support CSI-RS based L1 measurements for Inter-CU LTM. In option-1 the candidate nodes need to provide the CSI-RS resources to be considered for LTM measurements in the UE specific signalling procedure. For option 2 the active CSI-RS configurations in all the serving cells of CU needs to be exchanged via non UE associated signalling.  In our view, use of non-UE associated signalling for exchange of RS configuration (SSB and CSI-RS) prior to handover preparation for LTM reduces the signalling exchanges across CU and also reduces the preparation delay for Inter-CU LTM.

Proposal 2: Non UE associated signalling between CU is considered for exchange of RS information for Inter-CU LTM. RAN2 to send LS to RAN3 for the above proposal and its benefits for RAN2 signalling procedure.

Generation of LTM-CSI-Resource configuration for Inter-CU Subsequent LTM

In the last RAN2 meeting, it was agreed that the source CU is responsible for generating LTM-CSI-Resource-configuration that contains of list of LTM-CSI-Resource-config elements based on the SSB configurations received from each candidate CU.  However, source determining the LTM-CSI-Resource-configuration that needs to be measured is not inline with the principle of serving cell determining the measurement and report configuration for subsequent cell changes. It is worth to recall that for L3 based subsequent CPAC in Rel-18 after long discussions RAN2 concluded that the measurement-configuration and events are determined by candidate cells and the event is only configured in the final conditional configuration.

Observation 1:  The current serving cell should determine the LTM-CSI-Resources to be measured and reported for LTM. In case of subsequent LTM it should be the candidate CU which controls the candidate CU. This is inline with the principle of candidate determining the Measurement-configuration for L3 based subsequent CPAC.

Moreover, source CU generation of LTM-CSI-Resource config will lead to restrictive configuration for subsequent LTM. For example, the LTM-CSI-Resource-configuration generated by source CU may only have one resource-set for each candidate cell that consists of all SSB of the candidate cell.  In case of CSI-RS based measurements source CU generate one CSI-RS resource set for each candidate cell. For exemplary scenario where source CU is preparing three candidate cells C1 from CU1 and C2 (CU2) and C3(CU3) following diagram illustrates this scenario.

Observation 2: When source CU generate LTM-CSI-Resource-config based on SSB and CSI-RS configurations of candidate cells it can only generate one resource set containing all the RS of candidates as possible resource configuration.

In this configuration, if the candidate CU wants the UE to measure its multiple neighbor cells it need to configure 2 LTM Report configurations each pointing to resource sets of each candidate cells.  In the above scenario, UE should be configured with two-report configurations for resource sets containing RSs of C1 and C3 within the candidate configuration of C2. Moreover, if the candidate cell (C2) knows that only specific beams of neighboring cells are relevant for measurements it cannot generate resource-configuration limiting the specific beams.

The impact is more significant for CSI-RS resource-configuration where measuring all the CSI-RS of candidate cells is not necessary. Eventhough it is possible for source node also to generate different combinations of resource-sets it may need to generate large number of resource-sets which may not be referred by candidate CU in report-configuration. Because the source node is not aware of the actual target cells /beams that each candidate cells wants to measure when it becomes serving cell.

Observation 3: When source CU generates LTM-CSI-Resource-config based on SSB and CSI-RS configurations of candidate cells, it is not be optimum for candidate CU once UE moves to that CU and doing measurement for subsequent LTM.

For event based L1 measurement reporting the LTM-CSI-Resource configurations to be used as source and target resource configuration needs to be determined by the serving cell. In case of LTM, it should be the candidate cell. For instance if a LTM candidate cell wants to configure reporting event that compares serving cell active beam against a subset of beams of a target cell, these subset of beams of the target cell should be configured as LTM-CSI-Resource configuration for UE to measure and compare.  In this case this resource-set should be generated by the candidate CU and included as part of LTM-CSI-Resource-configuration.

Observation 4: For event based reporting the candidate cell should determine the target resource set (whether all beams or subset of beams).  This is not possible if source CU determines LTM CSI-Resource-configuration only based on SSB or CSI-RS configurations.

If the source CU generate the LTM-CSI Resource-configuration based on RS information, the preparation phase needs one additional step for the candidate RRC configurations to modify their LTM Report configuration pointing to the common LTM-CSI Resource configuration.  With candidate CU generation of CSI-Resource-configuration, the LTM-Report-configuration of candidate cells can be populated in the initial preparation step itself.

Observation 5: Candidate CU generation of LTM-CSI-Resource-configuration eliminate the additional step for LTM-Report-configuration update for the candidate RRC configuration.

Based on the above observations it is important to support generation of LTM-CSI-Resource-configuration from candidate CU that enable selection of suitable LTM-CSI-Resource config based on the LTM measurement/reporting preference of candidate cells.  This also follows the principles of measurements in subsequent mobility where candidate cell is responsible for determining all the configurations related to measurements including measurement resources and reporting configurations.

Proposal 3:  Candidate CU based generation of LTM-CSI-Resource configuration should be supported for Inter-CU LTM.

We propose the following signaling sequence for LTM preparation phase based on the above proposals.

- Exchange of SSB or CSI-RS configurations of candidate cells :  This information exchange is carried out as non-UE associated signaling and each CU is aware of latest SSB and CSI-RS configuration of other candidate cells.

- Source CU receives L3 measurement Report. Source CU triggers LTM Handover preparation procedure with candidate CU.

- Each candidate CU prepare the LTM-CSI-Resource-configuration based on their preference for subsequent LTM. The LTM Report-configuration within candidate configuration will point to the CU generated LTM-CSI-Resource-configuration

- Source CU combines all the collected CSI-RS resource configurations from candidate CU and generate the LTM Configuration and send RRC Reconfiguration containing LTM-Config to UE.

- In next step the source CU provides this information to candidate CU to indicate other candidate configuration such as Early-UL-sync, TCI-State-Information to be referred for subsequent LTM.

The Inter-CU LTM stage 2 signaling procedure with the above modifications is illustrated in the following Figure.

In the above procedure the number of steps needed for LTM Handover preparation is reduced to 2 steps as similar to any handover preparation and the preparation delay is reduced significantly. This is achieved by moving the exchange of RS information via non-UE associated signalling and also CU generated LTM-CSI-Resource config that avoids the need to modify the RRC configuration to include the LTM-Report-configuration which was needed for source CU generated LTM-CSI-Resource-configuration.

Proposal 4: RAN2 to consider the above message sequence as baseline for Inter-CU LTM Preparation Phase.

## Subsequent LTM

In RAN2#125bis the following were agreed with respect to subsequent LTM

Agreements on scenarios: - RAN2 first focus on inter-CU LTM in NR standalone scenario and use it as baseline for supporting inter-CU LTM in NR-DC scenarios. - Rel-19 inter-CU LTM also supports mixture of subsequent inter-CU LTM and subsequent intra-CU LTM after an inter-CU or intra-CU LTM switch. - UE can be configured with a mixture of intra-CU and inter-CU candidate LTM cells and irrespective of how the UE is configured with this mixture, UE measurement and reporting procedures will be the same for both intra-CU and inter-CU candidate LTM cells.

Considering that UE can be configured with a mixture of subsequent intra-CU and subsequent inter-CU LTM, it becomes relevant to analyze the main benefits associated with subsequent inter-CU LTM and the practical scenarios wherein such a feature would be useful. In our view, subsequent inter-CU LTM would exhibit at least the following benefits.

- Avoiding re-configuration of candidate cells if UE would do mobility to that candidate cell again.

- Allowing fast subsequent cell changes due to the early synchronization procedures and the fact that no preparation is needed towards next cell after one cell change.

In our view, the NW would consider configuring inter-CU subsequent LTM in different scenarios, such as factory deployments (similar to S-CPAC), wherein it might not be possible to place all cells under the same gNB due to number of cells to be controlled in a factory. Another setup where subsequent LTM would be desirable is when the user is located at cell edge and a high number of ping-pongs would be experienced. As an example, consider an indoor cellular user, that might have different cells from different gNBs providing coverage on different sides of a building.

Observation 6: Subsequent inter-CU LTM would be beneficial in different setups wherein it is difficult to place all cells under the same gNB such as factory deployments, and cases where a high number of ping-pongs is expected.

- Generic movement, wherein the UE may move between different cells without any pattern. In this case, the user may or may not go back to the same cell. In such cases, preparing multiple candidates would likely lead to inefficient resource usage.

Proposal 5: As a design principle, subsequent inter-CU LTM should aim to minimize signalling without making the preparation procedure too complex.

Despite its notable benefits, subsequent inter-CU LTM also incurs high signaling together with increased resource reservation both on the NW and on the UE side. Moreover, support of subsequent LTM also requires complex specification such as security key changes. From UE perspective, there would be a need to ensure that the LTM configurations are stored until the NW would decide to release them. Thus, we think that it would be beneficial to assess the different scenarios wherein configuring subsequent inter-CU LTM might not be needed.

Observation 7: Despite its notable benefits, subsequent inter-CU LTM incurs high resource signaling and resource reservation both on the NW and on the UE-side.

In our view, examples of such scenarios consist of cases where the trajectory of the UE is such that the UE would never go back to the previous serving cell (e.g., if the UE is moving along a highway), or when the likelihood of the UE moving back to a certain cell is very low. In such cases, it would be beneficial to have the flexibility to configure non-subsequent LTM on the NW side. This would also be beneficial for the UE, as it does not need to store LTM configurations which it will likely never re-use for long amounts of time.

Observation 8: For cases wherein there is a low likelihood that UE would return to a previous serving cell, it would be desirable to have the benefits of lower interruption times associated with the LTM procedure, without incurring the high signaling and resource reservation needed for subsequent cell switches.

Proposal 6: Inter-CU non-subsequent LTM is supported.

## Remaining RAN2 Impacts for Security Solutions

RAN2 has identified 4 options for the solutions for security key changes during Inter-CU LTM and indicated the same to SA3 for further analysis on security impacts of these solutions.  The RAN2 /RAN3 impacts of these solutions were not discussed within RAN2 as part of the above discussion. Following are the brief summary of these options

- Option 1: The NCC value to be used for security key generation is sent directly via MAC CE or the index which contains the NCC values.

- Option 2: The list of Target keys are pre-provisioned at UE similar to SCPAC.

- Option 3A: UE based solution for rule-based increment of NCC values.

- Option 3B: Pre-configuration of keys from CN to UE.

- Option 4: RRC message including only NCC value sent after cell-switch.

The main purpose of solution for security key change for LTM is to avoid RRC Reconfiguration between successive LTM cell switches across Inter-CU candidates. In this regard all the above solutions aim to avoid RRC-Reconfiguration procedure prior to subsequent LTM. Option 4 eventhough uses RRC message it is not RRC-Reconfiguration message that changes the current configuration instead it is RRC message that informs the key to be used in subsequent cell switching. Moreover Option 4 also allow the UE to know the next security key to be used for conditional LTM and LTM recovery where UE may not receive explicit NCC value as in option 1.

Observation 9:  Indication of next NCC value to be used via RRC message proposed in option 4 does not have any additional security impacts.

For all the solution options 1-3 after the Inter-CU cell switch, the LTM switching needs to be delayed until reception of path-switch from AMF and also update of new NCC values to all the candidate CU. This delay in LTM switching is needed to avoid any race condition leading to mismatch of configurations at source and target. RAN2 to discuss whether this additional delay is acceptable for LTM switching.

Proposal 7: RAN2 to confirm that subsequent LTM switching can be triggered from new CU only after completion of path-switch and update of candidate CU for all the proposed solutions.

Provisioning of new keys via MAC CE prior to switching will work for NW triggered Inter-CU LTM scenarios. However the security key change for Inter-CU LTM recovery was not discussed for the above options in the initial analysis.  In addition to the LTM recovery impacts, RAN2 intend to introduce conditional LTM from October meeting after confirmation in RAN-Plenary. The applicability of solutions for conditional LTM and LTM recovery needs to be discussed in RAN2. Based on the discussions if further updates needed to SA3 on the above solutions OR need for multiple solutions can be considered.

Proposal 8: RAN2 to discuss the impact and modifications needed to the proposed options to handle LTM Recovery and conditional LTM scenarios.

## DC-LTM

The WID [1] explicitly lists the following scenarios to be supported:

- Inter-MN PCell change (or inter-CU MCG LTM) with SCG retained or released

- Inter-SN PSCell change (or inter-CU SCG LTM) with MCG unchanged

In RAN2#126, it was already agreed to prevent simultaneous execution of inter-CU MCG and SCG cell change in Rel-19. This may be achieved with MN-SN coordination via RAN3 signalling. Nevertheless, it remains unclear if the following interworking scenarios whose support may be considered implicit are part of the Rel-19 WI scope or not:

- Simultaneous Inter-CU MCG LTM PCell change together with intra-SN PSCell change

Simultanious Inter-CU SCG LTM PSCell change together with intra-MN PCell change

Observation 10: Inter-MN PCell change (or inter-CU MCG LTM) with SCG retained or released and Inter-SN PSCell change (or inter-CU SCG LTM) with MCG unchanged are listed in the WID to be supported.

Proposal 9: RAN2 to discuss and agree the following scenarios are not precluded:

- Simultanious inter-CU MCG LTM PCell change together with intra-SN PSCell change

- Simultaneous inter-CU SCG LTM PSCell change together with intra-MN PCell change

## Support of configured grant-based RACH-less LTM

RAN2#126 Agreement:

Support CG-based RACH-less and DG-based RACH-less procedures for inter-CU LTM.

In RAN2#126, the following agreement was made on the support of the first UL transmission for RACH-less inter-CU LTM

Compared to dynamic grants (DGs), configured grants (CGs) enable faster access to the target cell, as the UE does not need to monitor PDCCH to receive the UL grant and can directly perform its first UL transmission on one of the valid occasions of the provided CG.

Nevertheless, a known issue of the use of CGs in LTM is the potential excessive resource reservation to support a single RACH-less LTM switch. For each UE prepared for RACH-less LTM and for each candidate cell, a CG must be included in the candidate’s RRC configuration provided to the UE during the preparation phase. Therefore, the PUSCH resources of these grants are reserved for a long period of time stretching from the preparation phase to the execution of one or multiple cell switches and they may never be used, if the UE does not switch to the corresponding cell. The issue may be even more pronounced in inter-CU LTM, as it cannot be solved or mitigated with intra-gNB resource scheduling optimizations.

Observation 11: Support of CG-based inter-CU LTM may lead to excessive PUSCH resource reservation.

On the other hand, CG-based RACH-less LTM results in reduced interruption time, which is one of the main objectives of LTM. Therefore, a mechanism to remedy the above issue is needed. To this end, the probabilistic nature of the use of the provided PUSCH resources can be exploited in a similar way as it is done for CFRA resources. PUSCH resources can be shared by multiple UEs, as it is unlikely that they switch to the same candidate cell at the same time. Still, it is possible that UEs sharing the same PUSCH resource will use the same CG occasion for their first UL transmission to the candidate. Therefore, coordination of the use of shared PUSCH resources is required. As cell switch execution is controlled by the source node, shared PUSCH resources can be provided by the candidate node to the source, which in turn can coordinate their use during cell switch.

Proposal 10: To avoid excessive resource reservation overhead for CG-based RACH-less LTM, PUSCH resources can be shared among multiple UEs. PUSCH resource sharing among UEs should be coordinated by the source node.

## Interworking of intra- and inter-CU LTM

In RAN2#126, the introduction of a new cell set ID was proposed by many companies to indicate to the UE that PDCP reestablishment and security key update is required when an inter-CU cell switch is executed.

An indication to the UE to determine whether it must perform PDCP reestablishment and security key update is indeed necessary. One way to provide such an indication is, as proposed during RAN2#126, to introduce a new RRC-configured Rel-19 ID.

Proposal 11: An indication must be provided to the UE to determine whether PDCP reestablishment and security key update is needed during cell switch.

Nevertheless, depending on the agreed RRC model, the introduction of such an ID may be redundant. More specifically, the indication can be implicit using our proposed RRC model. If the source and target cell belong to a different LTM-ConfigGroup, then PDCP reestablishment and security key update is required. Otherwise, i.e., if the source and target cell belong to the same LTM-ConfigGroup, the legacy Rel-18 behavior for L2 reset, based on ltm-NoResetID. Therefore, the form of the indication should be agreed after the RRC model for inter-CU LTM has been decided

Proposal 12: The form of the indication should be agreed after the RRC model for inter-CU LTM has been decided.

# Conclusions

In this paper we analysed the remaining open issues for Inter-CU LTM and made the following observations and proposals.

Proposal 1: Rel-18 LTM signaling procedures are reused for inter-CU LTM is considered as basis for Inter-CU LTM for further discussions.

Signalling Procedure during Preparation Phase for LTM Measurements

RS Information Exchange for Inter-CU

Observation 1:  To prepare L1 measurement resources (LTM-CSI-Resource-config) for Inter-CU scenario, source CU should be aware of the SSB configurations of candidate cells of other CU

Proposal 2: Non-UE associated signalling between CU is considered for exchange of RS information for Inter-CU LTM. RAN2 to send LS to RAN3 for the above proposal and its benefits for RAN2 signalling procedure.

LTM-CSI-RS Resource configuration for Inter-CU

Observation 1:  The current serving cell should determine the LTM-CSI-Resources to be measured and reported for LTM. In case of subsequent LTM it should be the candidate CU which controls the candidate CU. This is inline with the principle of candidate determining the Measurement-configuration for L3 based subsequent CPAC.

Observation 2: When source CU generate LTM-CSI-Resource-config based on SSB and CSI-RS configurations of candidate cells it can only generate one resource set containing all the RS of candidates as possible resource configuration.

Observation 3: When source CU generates LTM-CSI-Resource-config based on SSB and CSI-RS configurations of candidate cells, it is not be optimum for candidate CU once UE moves to that CU and doing measurement for subsequent LTM.

Observation 4: For event based reporting the candidate cell should determine the target resource set (whether all beams or subset of beams).  This is not possible if source CU determines LTM CSI-Resource-configuration only based on SSB or CSI-RS configurations.

Observation 5: Candidate CU generation of LTM-CSI-Resource-configuration eliminate the additional step for LTM-Report-configuration update for the candidate RRC configuration.

Proposal 3:  Candidate CU based generation of LTM-CSI-Resource configuration should be supported for Inter-CU LTM.

Proposal 4: RAN2 to consider the message sequence in Section 2.2 as baseline for Inter-CU LTM Preparation Phase.

Subsequent LTM

Observation 6: Subsequent inter-CU LTM would be beneficial in different setups wherein it is difficult to place all cells under the same gNB such as factory deployments, and cases where a high number of ping-pongs is expected.

Proposal 5: As a design principle, subsequent inter-CU LTM should aim to minimize signalling without making the preparation procedure too complex.

Observation 7: Despite its notable benefits, subsequent inter-CU LTM incurs high resource signaling and resource reservation both on the NW and on the UE-side.

Observation 8: For cases wherein there is a low likelihood that UE would return to a previous serving cell, it would be desirable to have the benefits of lower interruption times associated with the LTM procedure, without incurring the high signaling and resource reservation needed for subsequent cell switches.

Proposal 6: Inter-CU non-subsequent LTM is supported.

Remaining RAN2 Impacts for Security Solutions

Observation 9:  Indication of next NCC value to be used via RRC message proposed in option 4 does not have any additional security impacts.

Proposal 7: RAN2 to confirm that subsequent LTM switching can be triggered from new CU only after completion of path-switch and update of new security keys to candidate CU for all the proposed solutions.

Proposal 8: RAN2 to discuss the impact and modifications needed to the proposed options to handle LTM Recovery and conditional LTM scenarios.

DC-LTM

Observation 10: Inter-MN PCell change (or inter-CU MCG LTM) with SCG retained or released and Inter-SN PSCell change (or inter-CU SCG LTM) with MCG unchanged are listed in the WID to be supported.

Proposal 9: RAN2 to discuss and agree the following scenarios are not precluded:

- Simultanious inter-CU MCG LTM PCell change together with intra-SN PSCell change

- Simultaneous inter-CU SCG LTM PSCell change together with intra-MN PCell change.

Support of configured grant-based RACH-less LTM

Observation 11: Support of CG-based inter-CU LTM may lead to excessive PUSCH resource reservation.

Proposal 10: To avoid excessive resource reservation overhead for CG-based RACH-less LTM, PUSCH resources can be shared among multiple UEs. PUSCH resource sharing among UEs should be coordinated by the source node.

Interworking of intra- and inter-CU LTM

Proposal 11: An indication must be provided to the UE to determine whether PDCP reestablishment and security key update is needed during cell switch.

Proposal 12: The form of the indication should be agreed after the RRC model for inter-CU LTM has been decided.

# Reference

- RP-240299, Revised Work Item: NR mobility enhancements Phase 4, Mar 2024

- R2-2402907, “Important topics for inter-CU LTM”, ‘Apple Inc.’, RAN2#125-bis