3GPP TSG-RAN WG2 Meeting #127	R2-2407074

Maastricht, Netherlands, 19 – 23 August 2024

Agenda item:	8.2.5

Source:	Nokia

Title:	Considerations on topology 2

WID/SID:	FS_Ambient_IoT_solutions - Release 19

Document for:	Discussion and Decision

# 1	Introduction

In the SID, two topologies; topology 1, and topology 2, are included in the general scope as follows:

General Scope The definitions provided in TR 38.848 are taken into this SI, and the following are the exclusive general scope: - The overall objective shall be to study a harmonized air interface design with minimized differences (where necessary) for Ambient IoT to enable the following devices: - ~1 µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither DL nor UL amplification in the device. The device’s UL transmission is backscattered on a carrier wave provided externally. - ≤ a few hundred µW peak power consumption1, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, DL and/or UL amplification in the device. The device’s UL transmission may be generated internally by the device, or be backscattered on a carrier wave provided externally. - X is to be decided in WGs. - Coverage design target: Maximum distance of 10-50 m with device indoors as per TR 38.848: “…a range that WGs can sub-select within”. - For Topologies 1 & 2 (UE as intermediate node under NW control) per TR 38.848, with no RRC states, no mobility (i.e. at least no cell selection/re-selection -like function), no HARQ, no ARQ. NOTE 1: It is to be understood that “≤ a few hundred µW” means WGs are not tasked with setting a particular value, and that it will be for WG discussions to determine if a presented design with corresponding power consumption satisfies the “≤ a few hundred µW” requirement.

During the first part of the study, RAN2 has made agreements common to topology 1 and topology 2 regarding access procedures and more. However, topology 2 brings more complexity as it involves an intermediate UE handling the D2R and R2D communication.

This contribution discuss’ the important topology 2 related topics, especially on the selection of readers

# 2	Discussion

## 2.1	Control Plane Procedures of Topology 2

In topology 2 the reader node of AIoT interface communicate with AIoT-Application function through NR air interface. The communication over AIoT interface is relayed to the AIoT-function using Uu Interface. The radio interface procedures related to relaying the communication via Uu Interface is the responsibility of UE and RAN depending on the relevant triggers.

Figure 1 AIoT deployment scenario

## 2.1.1     Reader Registration

In this architecture the reader UE need to register for AIoT services as reader towards NR core network and get authorised before it starts the AIoT communication relay.  The specific details on reader registration and authorisation is discussed in SA2. RAN2 impacts if any related to registration and authorisation to be analysed after SA2 conclusion on this procedure.

Observation 1: Reader registration and authentication procedure is discussed within SA2.

Proposal 1: RAN2 to wait for SA2 finishing their study on aspects for authorisation and registration procedure for discussion on stage 2 procedure for reader registration.

## 2.1.2     Reader Selection

When multiple readers are registered with AIoT access function, the specific reader to be selected for triggering AIoT services for the AIoT devices at specific location. The reader registration may include additional information that assist the AIoT access function to select specific reader towards AIoT devices in certain location or towards certain group of AIoT devices. When AIoT procedures to be triggered the AIoT access function needs to identify the anchor BS that servers the relevant reader devices to trigger the reader discovery procedure. There could be some RAN2 aspects in reader discovery and providing additional information for either the BS or AIOT-function to select the suitable reader to start the AIoT procedure.

As seen in Figure 1, different readers may be within range of different devices, but likewise some readers may be in range with the same devices. In a simply inventory command, where the AIoT access function wants to discover the IDs and potentially associated upper layer data of nearby devices, controlling a devices’ access to specific readers will mainly reduce resource usage as;

- CN/Readers may configure sufficient resources to the amount of expected UEs

- Devices may refrain from accessing multiple readers in case of multiple parallel pagings in time.

Observation 2: Multiple readers may be able to receive from the same device(s).

Observation 3: Selecting suitable reader(s) for inventory helps with reducing resource usage due to devices being aware to which reader is responded.

Another aspect of the architecture is to discuss whether the AIoT design should include diversity like behaviour, in the sense that multiple readers could be allowed to receive from the same device, on the same paging request. RAN1 already has defined scenarios of having different readers associated to a particular device, i.e. one reader for activating the device and another reader for reading the signal from the device. Along with this, the scenario of independent CW provider is also considered. The different scenario can be seen in the table below.

| Scenario | CW Inside/outside topology | Diagram of the scenario | Description of the scenario | Device 1/2a/2b | CW spectrum | D2R spectrum | R2D spectrum |
| --- | --- | --- | --- | --- | --- | --- | --- |
| D1T1-A1 | CW inside topology |  | - CW node inside topology 1 - ‘CW’ in CW2D and ‘R2’ in D2R are different - ‘CW’ in CW2D and ‘R1’ in R2D are same - ‘R1’ in R2D and ‘R2’ in D2R are different | Device 1, 2a | Case 1-1 (inside topology, DL) Case 1-2 (inside topology, UL) | Same as CW |  |
| D1T1-A2 |  |  | - CW node inside topology 1 - same ‘CW’ and ‘R’ node for CW2D, D2R and R2D |  | Same as D1T1-A1 | Same as CW |  |
| D1T1-B | CW outside topology |  | - CW node outside topology 1 - ‘CW’ in CW2D and ‘R’ in D2R are different - ‘CW’ in CW2D and ‘R’ in R2D are different - ‘R’ in R2D and ‘R’ in D2R are same |  | Case 1-4 (outside topology, UL) | Same as CW |  |
| D1T1-C | No CW |  | - No CW Node. | Device 2b | N/A | UL |  |
| D2T2-A1 | CW inside topology |  | - CW node inside topology 2 - ‘CW’ in CW2D and ‘R2’ in D2R are different - ‘CW’ in CW2D and ‘R1’ in R2D are same - ‘R1’ in R2D and ‘R2’ in D2R are different - BS communicates with R1 and R2 | Device 1, 2a | Case 2-2 (inside topology, UL) | Same as CW |  |
| D2T2-A2 |  |  | - CW node inside topology 2 - same ‘CW’ and ‘R’ node for CW2D, D2R and R2D - BS communicates with R |  | Same as D2T2-A1 | Same as CW |  |
| D2T2-B | CW outside topology |  | - CW node outside topology 2 - ‘CW’ in CW2D and ‘R’ in D2R are different - ‘CW’ in CW2D and ‘R’ in R2D are different - ‘R’ in R2D and ‘R’ in D2R are same - BS communicates with R |  | Case 2-3 (outside topology, DL) Case 2-4 (outside topology, UL) | Same as CW |  |
| D2T2-C | No CW |  | - No CW Node. - BS communicates with R | Device 2b | N/A | FFS |  |
| Note: this table is for the case where D2R is in the same spectrum as CW2D. |  |  |  |  |  |  |  |

This means that it is already expected that reader operation can be dedicated, and thus RAN2 should not preclude a AIoT access function configuring multiple readers for i.e. D2R communication, or even R2D.

Observation 4: Engaging multiple readers in serving the same paging request may provide sufficient diversity to combat limited / interrupted coverage as well as mobility issues.

Proposal 2: Multiple readers can engage in receiving and forwarding responses from multiple AIoT devices to a given paging request (including multiple readers receiving a response from the same AIoT device).

Both in case the AIoT access function use a blind detection of unknown devices or is able to engage multiple readers for the same paging request, it would be beneficial for the paging to carry a unique identifier to ensure the device is aware of which paging the resources are related to, and whether the device has responded to the given reader.

The reader selection and operation may require information on reader availability and its status on top of reader proximity. The readers can be battery operated and may not be always available to provide reading service to the devices. Further, in case readers are UEs, there could be extra conditions relating UE operation for other transactions (such as UE’s own data communication) which may prohibit UE to perform as a reader. Such information should be considered for reader selection and operation before starting AIoT paging session.

Observation 5: If multiple readers engage in communication with a single device, assisting the device to select one or more readers can be beneficial.

Proposal 3: RAN2 to study adding conditions for reader access in the AIoT paging message e.g. number of readers to reply to, RSRP threshold.

Assuming the AIoT access function is aware of the devices which to communicate, when querying inventory or commant to a set of devices AIoT access function must either 1) be aware of the, expected, nearby devices for each reader or 2) query a larger set of readers to attempt communication with devices in parallel, but not necessarily on the same resource.

Similar issue arose in U2N sidelink relay and NR positioning operation, where the LMF/gNB triggers a discovery procedure in order to select the correct gNB or anchor/relay UE for the procedure. One difference with AIoT is that currently, it is out of scope in the SID to consider mobility. However, even in a warehouse with static deployment, changes in the radio environment may be enough for devices to not be readable by the same readers for each inventory/command request. Furthermore, since this study is targeting an overall AIoT architecture, we should at least consider studying forward compatibility within the basic procedures.

Observation 6: Reader selection will be an important consideration in the AIoT study, as it allows for the devices to be read without significant overhead.

Observation 7: A single reader may not have the full picture of the nearby devices at registration due to change in physical environment.

Proposal 4: RAN2 to study both options for reader selection before down selecting in WI:

- Option 1: Readers are selected based on actual radio conditions. FFS whether this includes reader assistance information based on previous device status, keep alive messaging, or discovery procedure.

- Option 2: Readers are selected by core network based on prior topology knowledge without considering actual radio conditions. FFS fallback options.

### 2.1.2	Considerations on RRC connection for Reader

In AIoT, although the amount of data per message is low (~1000 bits) the data rate is also within the area of few, or even fractional kbps. Given a larger set of devices, and multiple messages being transferred within a single procedure, the duration of an AIoT session i.e. from the initial paging being sent till all devices has replied, may comprise several seconds. In this case, the having a UE being in RRC_CONNECTED for the entire duration of the AIoT session seems not practical. Furthermore, Rel-18 brings SDT to both UL and DL, and thus it is possible to communicate with the reader even when it is in RRC_INACTIVE.

Observation 8: Maintaining the Reader UE in RRC-CONNECTED state during the entire AIoT session is not resource efficient.

Observation 9: SDT transmission may be sufficient to start/stop an AIoT session within a reader, and provide reader data back to the AIoT access function.

Proposal 5: For topology 2, RAN2 to study AIoT operation with reader UE in RRC_CONNECTED and RRC_INACTIVE in the time between initial paging till last response. FFS on RRC_IDLE.

Proposal 6: RAN2 to first discuss whether a UE should be paged and transition to RRC_CONNECTED to start an AIoT session or whether other mechanisms, such as SDT, can be used to support RRC_INACTIVE.

2.2 	     Data Forwarding schemes for Topology 2

The AIoT messages can be carried transparently as NAS payload or UP traffic over the interface between reader and GNB. However, if there could be some cases where GNB may need to know some additional information about the AIoT related message transfers that can be used by GNB for reader selection and discovery later. We propose that RAN2 evaluate both the options based on the final signalling procedure identified for AIoT Radio interface.

Proposal 7: RAN2 to evaluate non-RRC and RRC-based AIoT information transfer over Uu Interface.

The signalling procedures for Inventory and command procedures are under discussion in RAN2.  In case of topology 2 there can be two options to handle the complete AIoT transaction via reader. Either the reader can complete the whole procedure and provide the outcome towards AIoT-Function via NR Air-interface. Alternatively, the reader may forward each transaction transparently.  Each approach have different impacts and advantages. We propose that RAN2 to conclude on the AIoT signalling procedure for above use-cases and analyse the changes needed in data forwarding based on the same.

Proposal 8: RAN2 to study the data forwarding schemes and modifications needed in data forwarding over Uu-radio-interface once the AIoT interface signalling procedure is finalised.

# 3	Conclusion

This document has made the following observations and proposals:

Control Plane Procedures of Topology 2

Reader Registration

Observation 1: Reader registration and authentication procedure is discussed within SA2.

Proposal 1: RAN2 to wait for SA2 finishing their study on aspects for authorisation and registration procedure.

Reader Selection

Observation 2: Multiple readers may be able to receive from the same device(s).

Observation 3: Selecting suitable reader(s) for inventory helps with reducing resource usage due to devices being aware to which reader is responded.

Observation 4: Engaging multiple readers in serving the same paging request may provide sufficient diversity to combat limited / interrupted coverage as well as mobility issues.

Proposal 2: Multiple readers can engage in receiving and forwarding responses from multiple AIoT devices to a given paging request (including multiple readers receiving a response from the same AIoT device).

Observation 5: If multiple readers engage in communication with a single device, assisting the device to select one or more readers can be beneficial.

Proposal 3: RAN2 to study adding conditions for reader access in the AIoT paging message e.g. number of readers to reply to, RSRP threshold.

Observation 6: Reader selection will be an important consideration in the AIoT study, as it allows for the devices to be read without significant overhead.

Observation 7: A single reader may not have the full picture of the nearby devices at registration due to change in physical environment.

Proposal 4: RAN2 to study both options for reader selection before down selecting in WI:

- Option 1: Readers are selected based on actual radio conditions. FFS whether this includes reader assistance information based on previous device status, keep alive messaging, or discovery procedure.

- Option 2: Readers are selected by core network based on prior topology knowledge without considering actual radio conditions. FFS fallback options.

Reader RRC Connection

Observation 8: Maintaining the UE in RRC-CONNECTED state during the entire AIoT session is not resource efficient.

Observation 9: SDT transmission may be sufficient to start/stop an AIoT session within a reader, and provide reader data back to the AIoT access function.

Proposal 5: For topology 2, RAN2 to study AIoT operation with reader UE in RRC_CONNECTED and RRC_INACTIVE in the time between initial paging till last response. FFS on RRC_IDLE.

Proposal 6: RAN2 to first discuss whether a UE should be paged and transition to RRC_CONNECTED to start an AIoT session or whether other mechanisms, such as SDT, can be used to support RRC_INACTIVE.

Data Forwarding Schemes for Topology 2

Proposal 7: RAN2 to evaluate non-RRC and RRC-based AIoT information transfer over Uu Interface.

Proposal 8: RAN2 to study the data forwarding schemes and modifications needed in data forwarding once the AIoT interface signalling procedure is finalised.