3GPP TSG-RAN WG2 Meeting #127	R2-2407075

Maastricht, Netherlands, 19th Aug – 23rd Aug 2024

Agenda item:	8.9.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Radio-Interface Impacts for IoT-NTN SF Operations

WID/SID:	IoT_NTN_Ph3 - Release 19

Document for:	Discussion and Decision

# 1	Introduction

One of the objectives for Rel-19 WID for IoT-NTN enhancements (RP-240776) is

Support of  Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore:

- Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2, RAN4]

- At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3]

Following are the agreements in RAN2 on the SF operation for IoT-NTN.

Agreements:

1.	For the uplink/downlink messages transmission for MO, from RAN2 perspective the following steps are taken as baseline for S&F satellite operation (in case only eNB is on the satellite):

1) The UE sends uplink data signalling to eNB when service link is available and the eNB stores it.

2) When feeder link is available, the eNB sends the uplink data/NAS signalling to the CN.

3) The eNB (same or different) receives the downlink data/NAS signalling from the CN and stores it when feeder link is available (and service link is not available).

4) The eNB (same or different) sends the downlink data/signalling to the UE when service link is available again

2.	S&F indication can be provided by SIB (FFS on the details). RAN2 assumes that no NAS indication is needed

3.	RAN2 understands legacy UEs may be barred by legacy cellBarred and cellBarredNTN

# 2	Discussion

## 2.1 Background

In some of the IoT-NTN deployments the NTN nodes may not have connectivity towards UE and the ground station at the same time. Such cases are applicable when the ground station connectivity is available only from central region and the moving NTN nodes need to cover multiple coverage areas.  For such scenarios, storage capability at NTN node is needed to handle the store and forward operation between UE and Ground node whenever they are available.  The scenario is depicted in the following Figure. The Figure illustrates the scenario where NTN node only contain eNB components. The NTN node need to store the uplink packets received from the IoT-NTN UE when it is serving area without coverage of the Ground node, and it need to store the downlink data received from ground station and the device location is not served by NTN node.

For store and forward operation, the NTN nodes may consist of only eNB components or some components of eNB and CN elements.  The radio interface signaling procedures and impacts to NAS procedures may vary depending on the architecture. For example, in case of eNB at NTN node, the NAS level registration and signaling procedures may not be completed in a single procedure if the CN connectivity is not available at the time of NAS procedures initiated by the UE.  In case of eNB+MME component at NTN node the registration procedure can be completed using a single procedure and security aspects will be handled by the MME related components.

## 2.2  RAN signalling procedure changes for MME Deployment Options

SA2 study on SF Architecture concludes on the MME split options for SF operation. Following is text proposal for TS23.401 for the same.

The S&F operation is supported in regenerative architecture, and two deployment options shall be considered: a) MME functionality is split into two parts: the MME-SAT, which is the MME part onboard the satellite and MME-ground, b) with all CN functions including MME, SGW, PGW, HSS, E-SMLC, SMSC etc are on board each satellite, including proxies that are deployed on the satellite and the ground for application traffic, including support of MT traffic, MO traffic, SMS, etc. The implementation of the proxies used for storing and forwarding of signalling and user plane data is out of scope of 3GPP.

For either of these options the RAN component of gNB need not store the NAS signaling message and it will be responsibility of the MME on further action related to storage of NAS messages. In this regard the RAN2 agreements made during last meeting needs to revisit accordingly. For Complete MME on board option GNB need not store the uplink data packets instead it will be stored at the SGW component.

We propose the following steps on the SF operation from RAN2 perspective for MO-Data and MO-Signalling for the MME-Split option

For Split-MME on board option

MO-Data in SF mode of operation

- UE sends uplink data transmission to eNB when service link is available and the eNB stores it if the SGW component is not available in the on-board satellite.

- When feeder link is available the eNB transfer the data to CN/SGW.

- The same eNB or different eNB receives the downlink data from CN and stores it when service link is not available.

- eNB sends the downlink data to UE when service link is available.

MO-Signalling in SF mode of operation.

- For MO signalling the MME component in the satellite handle the received NAS message and RAN component needs not to store these messages for both variants of CN architecture for SF operation.

For Full-MME on board option

- The uplink data and signalling message received from UE are forwarded to MME and SGW on board.

- When feeder link is available the SGW forwards the data to ground node.

- When SGW have Ground network connectivity it receives the packets from Ground network and stores them.

- SGW component may need to inform ENB the details on the UE-Information that are stored.

- ENB trigger the paging message at the appropriate time when it starts serving the specific UE.

Proposal 1: RAN2 to revisit the signallng steps agreed in RAN2-126 to reflect the modified CN architecture options agreed in SA2.  RAN2 to consider the modified steps provide above as starting point for the signaling procedure changes.

With the presence of complete MME component on the satellite the NAS signalling procedure such as attach and tracking area update procedures can be completed transparently in SF operation. If only partial MME component is available the NAS procedure may be modified to allow delayed or step-wise completion of the procedure.  In this case whether UE need to aware of the NW support of this modified signalling via system information or during initial registration / subscription information needs to be further discussed.

Proposal 2: RAN2 to discuss whether further indication is needed in system-information to indicate the MME Split option.

## 2.2 Support for Multi-Satellite Deployment Options

The need for multi satellite operation for SF mode was discussed in RAN2-125bis meeting without specific conclusion. In our view most of the IoT-NTN deployments with SF mode expected to have more than one satellite to allow faster turn-around time for the store and forward operation. The multi-satellite operation is also essential to avoid single point of failure for this SF mode of operation. Hence, we think it is essential to consider multi-satellite operation as basis for RAN2 analysis for SF functionality.

Proposal 3:  RAN2 to confirm multi-satellite operation as basis for further analysis.

## 2.3 Awareness of SF mode operation at UE

In RAN2-126, Indication in system information on whether eNB is operating in SF mode is agreed. However the handing of SF mode transition from SF mode to normal mode and vice-versa were not discussed.

For the IoT-NTN deployment with discontinuous CN connectivity, it is possible that eNB may operate in normal mode in certain coverage areas where the eNB can have CN connectivity. But this connectivity will be lost after certain time due to movement of satellite away from the CN. In such case the mode needs to be switched from normal more to SF mode. It is possible to indicate this change via system information modification via paging. However, all the idle mode UE need to be paged for such modification. Alternatively the NW can avoid system information modification for this mode change and the UE implementation can read the system information prior to triggering access to know the mode. It is also possible to indicate the mode transition information as timer based mechanism to avoid this system information modification.

Proposal 4:  RAN2 to discuss the options for indication of mode transition (from SF to Normal and vice-versa) towards UE.

## 2.3 Access Control and Paging Changes for SF operation

When NTN node is serving new area, it may need to handle the new data transmissions from the coverage area and also complete the data transmission in downlink that was received and stored from CN at the time of earlier connectivity. In such cases the access control and data transmission across these two variants of users needs to be differentiated. Mainly to avoid the delivering the stored data becomes invalid or time-out at UE.  In the same way for the uplink transmissions depending on the storage available at NTN node to receive and buffer the packets the access control and scheduling of transmissions needs to be handled.

Proposal 5:  RAN2 to investigate access control and scheduling related changes for S&F operation.

When the mobile originated IoT data transmission that requires application level ack are sent to eNB operating in store and forward mode the UE may expect the corresponding downlink packets after connecting to the next eNB. In this case UE monitors the next gNB for receiving ACK and new MT packets if any stored in the eNB. The data packet available at this eNB may contain ACK for earlier MO traffic and MT traffic toward the UE. In some other cases, the next eNB may not have any downlink ACK for earlier transmission depending on its CN connection time. In different scenarios based on the network awareness of CN connectivity time and the actual stored downlink data to be delivered towards UE, it is possible to improve the paging monitoring towards UE.

Proposal 6:  RAN2 to consider paging enhancements in SF mode for delivering ACK for MO Traffic and MT Traffic towards IoT-NTN UE.

## 2.4 Security Context Update across Satellites for SF Mode

For user plane solution the security keys (T-KeNB) to be used in next node is based on earlier node. This may require exchange of eNB keys across NTN nodes to allow the new node to handle the security of the UE. In user plane solution the UE is moved to idle-suspended mode when the connection is released from current eNB. When it resumes the connection to next node the Resume operation will make use of new keys based on old key and NCC value. The target eNB is expected to be aware of the new keys to handle this situation. This issue is common for regenerative architecture, but in S&F mode of operation where the NTN node only have CN connectivity, how this information is exchanged across NTN nodes successively serving the UE needs to be analyzed further,

Proposal 7:  RAN2 to investigate the security context exchange across IoT-NTN nodes in CIoT-UP SF operation and the initial analysis on security impacts for this scenario.

# 3	Conclusion

In this paper we analyse the radio signalling procedure impacts for store and forward operation. Following proposals are made based on the analysis.

Proposal 1: RAN2 to revisit the signallng steps agreed in RAN2-126 to reflect the modified CN architecture options agreed in SA2.  RAN2 to consider the modified steps provided in section 2.1 as starting point for the signaling procedure changes.

Proposal 2: RAN2 to discuss whether further indication is needed in system-information to indicate the MME Split option.

Proposal 3:  RAN2 to confirm multi-satellite operation as basis for further analysis.

Proposal 4:  RAN2 to discuss the options for indication of mode transition (from SF to Normal and vice-versa) towards UE.

Proposal 5:  RAN2 to investigate access control and scheduling related changes for S&F operation.

Proposal 6:  RAN2 to consider paging enhancements in SF mode for delivering ACK for MO Traffic and MT Traffic towards IoT-NTN UE.

Proposal 7:  RAN2 to investigate the security context exchange across IoT-NTN nodes in CIoT-UP SF operation and the initial analysis on security impacts for this scenario.

# Reference

[1]	RP-240776 WID for IoT-NTN Enhancements