3GPP TSG-RAN WG2 #127	R2-2407078

Maastricht, Netherlands, 19th – 23th Aug, 2024

Agenda Item:	8.11.2

Source:	Ericsson

Title:	Discussion on subband full duplex (SBFD) RA operation

Document for:	Discussion

# 1	Introduction

In the WID [1], the following objective regarding random access operation has been defined for RAN2

- For subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier:

  - Specify SBFD operation to support random access in SBFD symbols by UEs in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode [RAN1, RAN2]

We discuss the RAN2 aspects for the above objective in this paper. In the below, we use the term SBFD RA operation to stand for random access operation in SBFD symbols unless otherwise stated. We use the term legacy RA operation to stand for random access operation during non SBFD symbols.

# 2	Discussion

## 2.1	RRC state

According to the above objective, RAN2 needs to study SBFD operation to support random access by UEs in RRC_CONNECTED and RRC_IDLE/INACTIVE.

In our view, the mechanism for SBFD RA operation should be designed as common as possible regardless of the RRC state under which RA procedure is triggered. In this way, the design efforts for RAN2/1 can be simplified.

RAN2 to strive for a common SBFD RA design independent of RRC state.

## 2.2	RACH trigger event

As specified in the stage 2 spec, the random access procedure is triggered by a number of events:

-	Initial access from RRC_IDLE;

-	RRC Connection Re-establishment procedure;

-	DL or UL data arrival, during RRC_CONNECTED or during RRC_INACTIVE while SDT procedure (see clause 18.0) is ongoing, when UL synchronisation status is "non-synchronised";

-	UL data arrival, during RRC_CONNECTED or during RRC_INACTIVE while SDT procedure is ongoing, when there are no PUCCH resources for SR available;

-	Handover, except for when RACH-less HO is configured;

-	SR failure;

-	Explicit request by RRC upon synchronous reconfiguration;

-	RRC Connection Resume procedure from RRC_INACTIVE;

-	To establish time alignment for a primary or a secondary TAG;

-	Request for Other SI (see clause 7.3);

-	Beam failure recovery;

-	Consistent UL LBT failure on SpCell;

-	SDT in RRC_INACTIVE (see clause 18);

-	Positioning purpose during RRC_CONNECTED requiring random access procedure, e.g., when timing advance is needed for UE positioning;

-	Early UL synchronization with an LTM candidate cell;

-	RACH-based LTM cell switch.

SBFD operation is supposed to achieve merits for SBFD aware UE including latency reduction. With this in mind, RAN2 may need to check if all above legacy trigger events are applicable for RACH in SBFD symbols. For example, certain trigger events including beam failure recovery, handover, or RACH-based LTM cell switch may be sensitive to latency, therefore, it may be more reasonable to apply SBFD RACH access for those specific trigger events.

Certain RACH trigger events including e.g., beam failure recovery, handover, or RACH-based LTM cell switch may be sensitive to latency.

RAN2 to discuss whether SBFD RA operation is applicable to all legacy RACH trigger events.

## 2.3	RO prioritization and failure handling

At RAN1#117, RAN1 has agreed to support two options to configure RACH configurations for SBFD in a cell, i.e.,

For SBFD-aware UEs in RRC CONNECTED state, both RACH configuration Option 1 with Alt 1-1 (i.e., use one single RACH configuration, and only based on the existing parameters of the single RACH configuration) and RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) are supported. Enabling both options at the same time for a UE is not supported.

- For Option 1 with Alt 1-1, FFS whether/how to reinterpret msg1-FrequencyStart in rach-ConfigCommon, RO validation rules and SSB-RO mapping rules, etc.

- For Option 2, FFS the RO validation rules, SSB-RO mapping rules, whether all the parameters currently in rach-ConfigCommon are necessary to be included in the additional RACH configuration, etc.

UE is not required to support both options.

According to the above agreements, SBFD RACH configuration can be provided by the gNB via either one single RACH configuration (i.e., the legacy RACH configuration with possible enhancement) or separate RACH configuration. In either of these options, a legacy UE can only see legacy ROs (which are in UL or flexible symbol by TDD-DL-UL common). A SBFD aware UE can see both legacy ROs (ROs in non SBFD symbols) and SBFD ROs (ROs in SBFD symbols, i.e., configured as DL by TDD-DL-UL common).

A legacy UE can only see legacy ROs (which are in UL or flexible symbol by TDD-DL-UL common).

A SBFD aware UE can see both legacy ROs (ROs in non SBFD symbols) and SBFD ROs (ROs in SBFD symbols, i.e., configured as DL or FL by TDD-DL-UL common).

An SBFD aware UE can use SBFD ROs and/or legacy ROs. It may be necessary to discuss if the UE can mix ROs of different RO types when performing RA operation. Given the UE may apply different power control parameters/settings for RACH operation in SBFD symbols and non SBFD symbols, it may be not preferred for the UE to mix ROs of different RO types when performing RA operation. In addition, SBFD ROs may provide the UE with opportunities to achieve lower latency than legacy ROs. If the UE is assumed to choose only SBFD ROs or legacy ROs upon triggering a RA procedure at least in normal scenarios, e.g., there is no congestion of RA resources, it may be beneficial to introduce conditions/criteria based on which the UE can select proper ROs among SBFD ROs and legacy ROs. The example criteria for the RO selection may comprise the below but not limited, e.g.,

- Access category or access class of the UE

- Service or traffic type associated with the RA procedure

- The trigger event/purpose of the RA procedure

Further, the UE may apply different power control parameters/settings for RACH operation in SBFD symbols and non SBFD symbols. It may be not preferred for the UE to mix ROs of different RO types when in RA operation.

Not allow the UE to use ROs of different RO types when in RA operation.

RAN2 to discuss if and how the MAC layer is allowed to prioritize between legacy ROs and SBFD ROs when performing RO selection.

It may be also necessary for RAN2 to study whether the SBFD aware UE to use both SBFD ROs and legacy ROs at the same time especially when the UE has detected congestion or RA problem for the selected ROs. In an example, the UE first uses legacy ROs, since the triggered RA procedure is not delay sensitive. However, the UE detects RA failures/problem after a while, then there is risk that the UE cannot fulfil the latency requirement for the RA procedure, it would be helpful if the UE is allowed to fall back to SBFD ROs. It may be also possible that the UE initially selects SBFD ROs, however the UE later experiences RA problem due to interference e.g., CLI introduced by other UEs or gNBs, thus it may be beneficial to allow the UE to fall back to legacy ROs. Nevertheless, RAN2 is suggested to study this issue.

RAN2 to discuss whether the MAC layer is allowed to fall back to another type of ROs under conditions e.g., the UE experiences RA problem for the initially selected ROs.

For a SBFD aware UE operating in a cell supporting SBFD RA operation, there may be RACH occasions available in both SBFD symbols and non SBFD symbols (e.g., legacy UL symbols), in this case, the two issues below need to be addressed concerning RACH failure detection.

Issue 1: how the UE detects RACH problems/failures. The UE may perform RACH transmissions using the ROs in SBFD symbols, and/or non SBFD symbols. In this case, when the UE determines a RACH problem/failure, whether the UE only counts RACH transmissions on ROs in SBFD symbols or the UE counts RACH transmissions on all ROs (in both SBFD symbols and non SBFD symbols) are unclear.

Issue 2: what are the UE actions upon detection of a RACH problem? The UE actions are expected to be different compared to the legacy RACH procedure.

It may be necessary for the SBFD aware UE to monitor RA problems for RA operation in SBFD symbols and RA operation in non SBFD symbols separately. Upon detection of RA problem for either SBFD RA operation or non SBFD RA operation, the MAC layer indicates the problem to upper layers separately.

RAN2 is therefore suggested to study if the UE needs to monitor RA problems separately for SBFD RA operation and legacy RA operation.

RAN2 to discuss if the MAC layer needs to monitor RA problems separately for SBFD RA operation and legacy RA operation.

## 2.4	SBFD aware early indication

In RAN1, there is on-going discussion on the need of SBFD awareness early indication. It was declared by some companies that It may be beneficial for the gNB to know the UE SBFD-awareness capability in an early stage during a random-access procedure so that the gNB can leverage the SBFD random access operation as early as possible (before the UE can provide its capability report to the gNB). Given the solutions which are under discussions may also affect RAN2. It is suggested to RAN2 to also discuss this issue, i.e., whether the UE needs to give an early indication to the gNB, and if needed, what could be the solutions to address the issue.

If RAN2 agrees to address the issue, some options are:

Option 1: PRACH preamble partitioning. In this option, SBFD aware UE and legacy UE would use different PRACH preamble partitions for perform RACH operation. Upon reception of a PRACH preamble from a UE, the gNB can identify whether the UE is SBFD operation aware or not based on whether the detected preamble is split for SBFD operation or non SBFD operation.

Option 2: RO based identification. As one simple solution, SBFD aware UE may only choose SBFD ROs, while the legacy UE would only choose legacy ROs. Upon reception of PRACH preamble from a UE, the gNB can determine whether the UE is SBFD operation aware based on whether the used RO is a SBFD RO or a legacy RO.

Option 3: indication carried by a MAC CE. This option may be applicable in case an SBFD aware UE can choose either SBFD ROs or legacy ROs.

Different options may require/give different design complexity to RAN2. In addition, different options may have pros and cons depending on whether it is RACH configuration option 1 (i.e., single RACH configuration) or RACH configuration option 2 (i.e., separately RACH configurations) is chosen by the gNB. in addition, RAN2 can also discuss if combination of multiple options is needed, especially a single option alone may be insufficient.

RAN2 to discuss whether SBFD aware UE in RRC_IDLE or RRC_INACTIVE needs to send an early indication during a RA procedure to the gNB (that indicates its SBFD support), and, if needed, what are the potential options.

## 2.5	RA type

In RAN1#116 meeting, the following agreement was made.

Agreement

For SBFD aware UEs in RRC CONNECTED state, support Type-1 random access procedure (4-step RACH) in SBFD symbols.

- FFS Type-2 random access procedure (2-step RACH)

RAN1 discussions have been mainly focusing on 4-step RA procedure. Companies in RAN1 cannot achieve convergence on whether 2-step RA also need to be supported, mainly due to the below reason

the justification of 2-step RA is not clear since the 2-step RA is applied in good RSRP condition, 2-step RACH is optional features, PRACH repetitions is not supported in 2-step RACH which means 2-step RACH cannot enhance uplink coverage.

For RAN2, it is sufficient to leave to RAN1 to decide whether 2-step RA can be supported. RAN2 can just focus on 4-step RA, will start to work on 2-step RA only when RAN1 indicates so.

RAN2 to focus on 4-step RA. Whether 2-step RA can be also discussed is pending on RAN1 decision.

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Observation 1	Certain RACH trigger events including e.g., beam failure recovery, handover, or RACH-based LTM cell switch may be sensitive to latency.

Observation 2	A legacy UE can only see legacy ROs (which are in UL or flexible symbol by TDD-DL-UL common).

Observation 3	A SBFD aware UE can see both legacy ROs (ROs in non SBFD symbols) and SBFD ROs (ROs in SBFD symbols, i.e., configured as DL or FL by TDD-DL-UL common).

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to strive for a common SBFD RA design independent of RRC state.

Proposal 2	RAN2 to discuss whether SBFD RA operation is applicable to all legacy RACH trigger events.

Proposal 3	Not allow the UE to use ROs of different RO types when in RA operation.

Proposal 4	RAN2 to discuss if and how the MAC layer is allowed to prioritize between legacy ROs and SBFD ROs when performing RO selection.

Proposal 5	RAN2 to discuss whether the MAC layer is allowed to fall back to another type of ROs under conditions e.g., the UE experiences RA problem for the initially selected ROs.

Proposal 6	RAN2 to discuss if the MAC layer needs to monitor RA problems separately for SBFD RA operation and legacy RA operation.

Proposal 7	RAN2 to discuss whether SBFD aware UE in RRC_IDLE or RRC_INACTIVE needs to send an early indication during a RA procedure to the gNB (that indicates its SBFD support), and, if needed, what are the potential options.

Proposal 8	RAN2 to focus on 4-step RA. Whether 2-step RA can be also discussed is pending on RAN1 decision.

# 4	References

- [1] 	RP-241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei,

- Shanghai, China, June 17-20, 2024, RAN#104.