3GPP TSG-RAN WG2 #127	R2-2407092

Maastricht, The Netherlands, 19th – 23th Aug, 2024

Agenda Item:	8.3.2.1

Source:	Ericsson

Title:	Simulation results for RRM measurement inter-frequency predictions

Document for:	Discussion

# Introduction

A new study item on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR was approved in RAN plenary #102 with the following objectives [1].

Study and evaluate potential benefits and gains of AI/ML aided mobility for network triggered L3-based handover, considering the following aspects: - AI/ML based RRM measurement and event prediction, - Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model) [RAN2] - Inter-cell Beam-level measurement prediction for L3 Mobility (UE sided and NW sided model) [RAN2] - HO failure/RLF prediction (UE sided model) [RAN2] - Measurement events prediction (UE sided model) [RAN2] - Study the need/benefits of any other UE assistance information for the network side model [RAN2]

In last meeting, two study goals and priorities for RRM measurement prediction were discussed to well-proceed the simulation in the next a few meetings.

| Case number | Prioritization | Evaluation scenario combination | Target study goal |
| --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 2nd goal |

In this contribution, we focus on Inter-Frequency RRM prediction, i.e. case number 3. We capture relevant agreements (to frequency) from chairman’s notes of last meeting in the Appendix Agreements from Last meetings.

# 2	Discussion

## 2.1 	System Setup and Definitions

In RAN2#125bis meeting, it was agreed that following cases are considered for cell-level measurement prediction model,

- Case 1: To predict beam level results, then generate cell level results based on the predicted beam results.

- Case 2: To directly predict cell level results based on cell level results.

- Case 3: To directly predict cell level results based on beam level results.

Considering the importance of the cell level measurement predictions for L3 mobility (which is the main focus of the study item) in this contribution we focus on Case 2 and Case 3, wherein the L3 filtered cell-level measurements are directly predicted by AI/ML model.

For AI/ML model, we use cell-level measurement (Point C in Figure 1) as both output and input of model in Case 2 (for different cells), whereas for Case 3, we use beam-level measurement (Point A1 in Figure 1) as input to model, and cell-level (Point C in Figure 1) as output of Case 3. The reason why we use Point A1 and not A for Case 3 input, is because Layer1 filtering can eliminate the impact of fast fading which is beneficial for the following model training and inference.

- For cell-level measurement prediction model, Case 2 and Case 3 are to be further investigated.

Besides, how to perform layer1 filtering has impact on the prediction although it’s not standardized (currently, it is a UE implementation). In this contribution, we use the already existing L3 filtering formula in TS 38.331 section 5.5.3.2 with specific alpha 0.2, i.e., Fn = (1 – a)*Fn-1 + a*Mn, where the parameters are defined in the mentioned TS 38.331. For layer1 filtering, we used the existing L3 filtering mentioned in TS 38.331 section 5.5.3.2 with specific alpha 20%.

Figure 1. Measurement model illustrated in TS 38.300

Hereafter, we describe system setup and definitions that are needed before discussing the insights from numerical results (Note that: The detailed simulation setup and ML parameters are described in Table 1 and Table 2, consequently).

- Carrier Frequencies: For this setup we considered four FR1 Frequencies, i.e., 800 MHz, 1.8 GHz, 2.6 GHz, and 3.5 GHz.

- Cell Indexing Consideration: As shown later (i.e., ‎Observations 4, ‎5,‎ 6, ‎15), the weaker the RSRPs received by the UE (especially as it gets closer to the noise floor, i.e., randomness will increase) the worse the prediction loss of such cells. Therefore, we have considered preprocessing the dataset that changes the indexing of the cells (within each carrier) based on its strength. Figure 1 describes the sorting method and the implication on indexing. The original indexing is shown in black table, where per site we have 3 cells, and 3 sites per systems. The indexing of spatial cells within a single carrier frequency (say 0.8GHz) is from 0 to 8. Then the follow-up inter-frequency cells indexing of those cells belonging to the second carrier frequency (say 1.8GHz) is from 9 to 17. The same indexing pattern goes for carrier 2.6 GHz and carrier 3.5 GHz.

If the UE is associated with cell-0 in site-1, and we attempt to predict a cell in site-3 that is very weak, hence noisy, the prediction performance is much worse than if we predict the closest cell to cell-0. Therefore, we thought of sorting the cells. In the following we describe the sorting with two example, Ex-1 for serving carrier 0.8GHz, and Ex-2 for serving carrier 3.5 GHz, as described in Figure 1.

In Ex-1, the serving carrier is 0.8GHz, assume that the order cells’ RSRP (from strongest to weakest) is (7,4,8,1,3,5,6,0,2) then this is how the new indexing will be ordered (starting from 0 == 7, etc). And the co-located cells of other frequencies (1.8, 2.6, 3.5 GHz) will be shifted similarly to the 0.8 GHz reordered cells.

In Ex-2, the serving carrier is 3.5 GHz, and the order of cells’ RSRP (from strongest to weakest) is (31,35,34,28,30,32,33,27,29) then this is how the new indexing will be ordered (starting from 0 == 31, etc). And the co-located cells of other frequencies (0.8,1.8,2.6 GHz) will be shifted to the new position of the reordered cells in 3.5GHz serving carrier.

Figure 2. Examples of Sorting Preprocessing on the Inter-Frequency and Spatial dataset

- Frequency Distance: When reading the results, we noticed a behavior of the prediction loss given the difference in input and output frequency of the cells. Therefore, we created a measure called Frequency Distance, which is a simple indication about how different the frequency implication on the channel model, e.g., distance attenuation, slow fading, indoor/outdoor/blockage attenuation, fast fading.

In order to further elaborate this concept of frequency distance, we have calculated the correlation matrix (details matrix is in Table 7, whereas abstracted version is in Table 1) across all four carrier frequencies, and for the first three spatial beams (SB-x), the reason for choosing the first three spatial beams is simply to reduce the size of correlation matrix. It is observed that correlation between (same beam SB-1 of) four carrier 0.8 to 1.8 to 2.6 to 3.5 GHz decreases and reach to the minimum (71.69%) between 0.8 and 3.5 GHz ({1}@row-10). Such correlation could be a measurement of frequency distance. The columns within the first rows represent the spatial correlation across beams within a single carrier. Based on these results, if we compare {2} and {3} @row-1 to {4} @row-4, it is quite clear that the collocated cells/beams across carriers (but the same Spatial beam) have higher correlation in compared to cross correlation across spatial beam (if we show more correlation instance across spatial beams this behaviour (i.e., lower correlation across spatial beams/cells) is further emphasized. Another observation is the stacking effect of both Inter-frequency and Spatial randomness, such effect appears when comparing correlation of SB-1 to SB-2 and SB-3 in {2,3} then observe the correlation reduction in compared to {2a,3a}, {2b,3b}, and finally maximum stacking effecting happening in {2c,3c}.

Correlation across carriers could be a measure of frequency distance. The higher correlation, the lower the frequency distance, and the better prediction is expected.

Correlation becomes worse as we go farther in frequency distance and spatial distance, and such behaviour stacks from both frequency and spatial distances.

Table 1. Mini-Correlation matrix across frequencies and some spatial beams (SB).

|  |  |  | Carrier = 0.8GHz |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Spatial Beam per Carrier |  | SB-1 | SB-2 | SB-3 |
| Carrier = 0.8GHz | SB-1 | 1 | 100.00% | 87.35%{2} | 83.60%{3} |
|  | SB-2 | 2 | 87.35% | 100.00% | 88.76% |
|  | SB-3 | 3 | 83.60% | 88.76% | 100.00% |
| Carrier = 1.8GHz | SB-1 | 4 | 88.50%{4} | 81.39%{2a} | 84.45%{3a} |
|  | SB-2 | 5 | 82.06% | 93.09% | 89.16% |
|  | SB-3 | 6 | 82.04% | 88.10% | 95.42% |
| Carrier = 2.6GHz | SB-1 | 7 | 87.29% | 79.70%{2b} | 84.60%{3b} |
|  | SB-2 | 8 | 81.62% | 92.40% | 89.47% |
|  | SB-3 | 9 | 82.39% | 87.58% | 95.15% |
| Carrier = 3.5GHz | SB-1 | 10 | 71.69%{1} | 69.01%{2c} | 70.48%{3c} |
|  | SB-2 | 11 | 61.92% | 74.95% | 68.47% |
|  | SB-3 | 12 | 73.44% | 75.68% | 77.66% |

We have further conducted some processing of the correlation matrix to find the relation between correlation and frequency distance, which is described in Table 2. We verified that the complementary of Correlation (1-Correlation) is monotonically increasing with the suggested distance (in Table 3)

Table 2. Relation between Correlation and Frequency Distance.

| Pair of Carrier Frequency | 1.8<>2.6 | 0.8<>1.8 | 0.8<>2.6 | 1.8<>3.5 | 0.8<>3.5 |
| --- | --- | --- | --- | --- | --- |
| Correlation* | 95.49% | 88.50% | 87.29% | 79.60% | 71.69% |
| 1 - Correlation | 4.51% | 11.50% | 12.71% | 20.40% | 28.31% |
| Order (distance) | 1 | 1.5 | 2 | 2.5 | 3 |

*Copied from Table 9

- ML-Loss (or prediction loss): is defined as the average of the difference between measured L3 filtered Cell-RSRP (ground truth) and the predicted L3 filtered Cell-RSRP.

- Dataset preparation: In order to make sure that our error (prediction loss) represents a specific output cells/carrier (cluster or individual) prediction based on a specific input cells’/carrier (cluster or individual) measurement, we have generated multiple datasets in a way such that we don’t allow the UE to HO to another carrier (i.e., Fixed serving carrier). In this way, when we calculate the average loss, it is averaged over the same cell(s)/carrier(s) to cell(s)/carrier(s), without the potential of changes in inference or training sets.

- Cluster of Cells: this term is used either at input or output of ML model. It is used in input, when we have multiple cells (either collocated with frequency difference, or spatially difference, as described in the indexing) at the input, or used at the output, when we have multiple to be predicted cells.

- Baseline: for inter-frequency and spatial prediction, we calculate the baseline by averaging the input cells’ RSRPs.

- BL-Loss (or baseline loss): is defined as the average of the difference between measured L3 filtered Cell-RSRP (ground truth) and the Baseline RSRP.

- Gain: is defined as the difference between prediction and baseline loss divided by baseline loss, i.e., (ML-Loss - BL-Loss) / BL-Loss.

- Scenarios: We have experimented with different scenarios that have impacted the ML-Loss in different ways. Though, we will explain the numerical impact of those scenarios in the next section 2.3, hence, we herein mention those scenarios:

  - Speed: We have experimented with different UE speed and their impact on the ML-Loss performance, including, 30, 60, 90, 120 km/hr.

  - Processing of cell indices (Sort): We have experimented with Sorted and UnSorted dataset:

    - Sorted (0: lowest index), from strongest to weakest (8: higher number per carrier, as we have 3 cells per site, and 3 sites per system),

    - UnSorted, where the cell’s index is irrelevant to its RSRP strength.

  - Fixed Serving Carrier: We experiment with four fixed serving carrier that are forced on the system on both training and inference phases, i.e., 0.8, 1.8, 2.6, 3.5 GHz.

  - Spatial and Frequency belonging of In/Out cells: We have experimented with Input/Output cells belonging to different Frequency-Carrier, and Spatial Sector and Site.

    - Spatial indexing, which varies from 0 to 8, i.e., 3 cells per site, and 3 sites per system

    - Frequency indexing, which adds on top of spatial indexing, i.e., cells indices in first (serving) frequency ranges [0,8], cells indices in second frequency ranges in [9,17], cells indices in third frequency ranges in [18,26], cells indices in fourth frequency ranges in [27,35].

  - Beam vs Cell as Input: We experimented with two types of inputs,

    - First, L3 filtered Cells (Point C in Figure 1), either as singular or a cluster of cells.

    - Second, L1 filtered Beams (Point A1 in Figure 1) of associated cells (singular or clusters of cells)

## 2.2 	General Insights from Simulation

In this section we evaluate the model level performance under different scenarios.

### Best performance evaluation

In this subsection, we show and discuss results to minimum bound of prediction’s loss in all different single-cell to single-cell in different frequencies, i.e., ML-Loss and BL-Loss. Table 1 describes the results of Cell-0 at several frequency carriers when dataset (per time instance) is sorted. Best performance prediction is shown when 2.6GHz is used as input to predict collocated Cell-0 at carrier 1.8GHz. We have also evaluated this performance when the fixed serving carrier was 2.6 GHz and all other three carriers (3.5, 1.8, 0.8 GHz), which resulted in the same performance. Comparing the performance of the first three rows, which are also almost the same, we show that if we allow the UE to measure on all four cells, while having any fixed cells, prediction of an output frequency given an input frequency will always yield to the same results. Also, we show in row 4 compared to rows 1-3, that given a higher frequency (e.g., 2.6 GHz) as input, to predict a certain frequency (e.g., 1.8 GHz) will results in a better frequency than having lower frequency as input (e.g., 1.8 GHz), and higher frequency to be predicted (e.g., 2.6 GHz). We also found that the worst performance occurs when we attempted to predict the furthest away frequency in-compared to input, e.g., given 0.8 GHz predict 3.5 GHz.

Table 3. Generally Best Performance Results.

|  | Fixed Serving Carrier | Input Cell | Output Cell | Frequency Distance | ML-Loss (dB) | BL-Loss (dB) | Gain (%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 3.5GHz | Cell0@2.6GHz | Cell0@1.8GHz | 1 | 0.486083 | 3.437059 | 85.858 |
| 2 | 2.6GHz | Cell0@2.6GHz | Cell0@1.8GHz | 1 | 0.486688 | 3.437572 | 85.842 |
| 3 | 1.8GHz | Cell0@2.6GHz | Cell0@1.8GHz | 1 | 0.492864 | 3.460096 | 85.75578 |
| 4 | 1.8GHz | Cell0@1.8GHz | Cell0@2.6GHz | 1 | 0.770947 | 3.399404 | 77.3211 |
| 5 | 3.5GHz | Cell0@3.5GHz | Cell0@2.6GHz | 1.5 | 1.0302 | 5.823883 | 82.31077 |
| 6 | 0.8GHz | Cell0@0.8GHz | Cell0@2.6GHz | 2 | 1.368217 | 6.468918 | 78.84937 |
| 7 | 1.8GHz | Cell0@1.8GHz | Cell0@3.5Ghz | 2.5 | 1.772391 | 9.40383 | 81.15246 |
| 8 | 0.8GHz | Cell0@0.8GHz | Cell0@3.5GHz | 3 | 2.346084 | 12.27671 | 80.88996 |

Best Inter-Frequency ML-loss is obtained when predicting 1.8GHz cell from 2.6GHz cells, mainly strongest cells and across carriers that has highest correlation.

The minimum achievable loss is ~ 0.48 dB, with corresponding gain of 85.9% in compared to baseline.

Using higher frequency (e.g., 2.6 GHz) as input, to predict a certain lower frequency (e.g., 1.8 GHz) will results in a better performance compared to having lower frequency as input (e.g., 1.8 GHz), and predict higher frequency (e.g., 2.6 GHz).

Further investigate if higher frequency predicting lower frequency always outperform lower frequency predicting higher one.

### Variable Carrier Distance performance evaluation

Additionally, Table 1 shows performance evaluation when we change the frequency distance (FD). We assume five distances, between 2.6 – 1.8 GHz (FD=1), 2.6-3.5 GHz (FD=1.5), 0.8-2.6 GHz (FD=2), 1.8-3.5 GHz (FD=2.5), and 0.8 – 3.5 GHz (FD = 3). We observe that the higher the frequency distance between carriers the worse the ML and BL loss performance. This aligns with the result from Table 2, where the lower the correlation (higher complementary correlation) the larger the frequency distance.

The larger the distance between carriers the worse the ML-Loss.

Further investigate the impact of carrier distance on frequency prediction performance.

### Impact of dataset Preprocessing (e.g., Sorting)

In this subsection, we discuss the impact of sorting (input and output) cells on ML-Loss performance. Table 2 shows a spectrum of different scenarios that shows the impact of data processing on ML-Loss. Starting with single-Cell to single-Cell prediction, we observe (from rows 1-2, 3-4,5-6, and 7-8) that even if frequency distance is low or high the sorting of cells is always beneficial. However, the impact of sorting on prediction from lower to higher frequency versus higher to lower frequencies is not yet clear, but it could be that sorting improve more when from lower to higher in compared to higher to lower frequency prediction, as show when comparing rows 3-4 to 1-2. Also, the impact of sorting on frequency distance is not completely clear. Those behaviour are reasonable since sorting dataset act as kind of automatic reduction of the spatial differences between cells, hence improve the prediction. Another insight from Table 2 is cluster-to-single cell prediction (as shown in rows 9-10) and single-to-cluster cell prediction (as shown in rows 11-12), both of those cases show good impact of data processing (sorting) on ML-Loss.

Sorting of the dataset (Input/output cells) improves the prediction accuracy compared to not sorted cells.

Sorting has larger improvement impact when applied on lower-to-higher frequency than higher-to-lower frequency and when applied on higher frequency distance scenarios.

Improvement by the sorting input is also observed for different prediction scenarios Single-Cell to Single-Cell and Single-Cell to Cluster-Cells and Cluster-Cells to Single-Cell.

Further investigate of data processing on higher vs lower frequency distance, and on lower-to-higher vs higher-to-lower frequency prediction.

Study the impact of training on unsorted dataset, while predicting the RSRP value of strongest cell.

Table 4. Performance evaluation for Sorted vs Unsorted dataset.

|  | Fixed Serving Carrier | Input Cell | Output Cell | Dataset cell processing | ML-Loss (dB) | BL-Loss (dB) | Gain % |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2.6GHz | Cell0@2.6GHz | Cell0@1.8GHz | Sorted | 0.49 | 3.44 | 85.84 |
| 2 | 2.6GHz | Cell0@2.6GHz | Cell0@1.8GHz | UnSorted | 0.52 | 3.286 | 84.27 |
| 3 | 1_8GHz | Cell0@1.8GHz | Cell0@2.6GHz | Sorted | 0.77 | 3.40 | 77.32 |
| 4 | 1_8GHz | Cell0@1.8GHz | Cell0@2.6GHz | UnSorted | 1.02 | 3.29 | 69.06 |
| 5 | 3.5GHz | Cell0@3.5GHz | Cell0@2.6GHz | Sorted | 1.03 | 5.824 | 82.31 |
| 6 | 3.5GHz | Cell0@3.5GHz | Cell0@2.6GHz | Unsorted | 2.11 | 7.265 | 70.96 |
| 7 | 0.8GHz | Cell0@0.8GHz | Cell0@2.6GHz | Sorted | 1.43 | 6.30 | 77.24 |
| 8 | 0.8GHz | Cell0@0.8GHz | Cell0@2.6GHz | Unsorted | 1.70 | 6.16 | 72.35 |
| 9 | 0.8GHz | Cluster of Cell0 @[0.8,1.8,2.6] GHz | Cell0@3.5GHz | Sorted | 1.33 | 9.22 | 85.57 |
| 10 | 0.8GHz | Cluster of Cell0 @[0.8,1.8,2.6] GHz | Cell0@3.5GHz | Unsorted | 2.09 | 10.37 | 79.81 |
| 11 | 3.5GHz | Cell0@3.5GHz | Cluster of Cells [0 ,1,2]@2.6GHz | Sorted | 3.87 | 7.131 | 45.67 |
| 12 | 3.5GHz | Cell0@3.5GHz | Cluster of Cells [0 ,1,2]@2.6GHz | UnSorted | 6.385 | 9.979 | 36.01 |

### Impact of Speed

In this subsection, we discuss the impact of speed on the ML-Loss performance, which are described in Table 3. The main insight here is that instantaneous (without temporal factor) inter-frequency prediction is not always impacted by speed. We observe (from green rows 1-3) that increasing the speed (from 30 to 60 to 120 km/h) impacts the ML-Loss and BL-Loss performance, since there is a considerable frequency distance, input@0.8GHz to output@3.5GHz. However, when frequency distance becomes smaller, e.g., prediction of output@1.8GHz from input@2.6GHz (as in red rows 4-6), the difference in ML-Loss becomes unclear. The same observation can be made for Cluster-Cell to Single-Cell prediction, where the impact of speed 30 to 60 km/h is not clear, and only when speed goes up to 120 km/h some difference in performance is observed (as in yellow rows 7-8).

For Single-to-Single Cell and Cluster-to-Single Cell prediction scenarios, large frequency distance highlights the impact of speed on ML-Loss, i.e., higher speed worsens the ML-Loss and BL-Loss.

For Single-to-Single Cell and Cluster-to-Single Cell prediction scenarios, if frequency distance is not large, the speed impact on ML-Loss and BL-Loss is not observable, i.e., the ML-Loss and BL-Loss do not change while changing the speed from 30, to 60, to 120 km/h.

Further investigate the impact of speed on both frequency and spatial prediction.

Further investigate ML-Loss performance when training (either single-to-single cells or cluster-to-cluster) a model given a single speed but predict on a different speed.

Table 5. Evaluation the impact of speed (30, 60, 120 km/h) and Cluster-vs-Single prediction on ML/BL Loss performance.

|  | Speed (km/h) | Fixed Serving Carrier | Input Cell | Output Cell | ML-Loss (dB) | BL-Loss (dB) | Gain (%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 30 | 0.8GHz | Cell0@0.8GHz | Cell0@3.5GHz | 2.3 | 12.3 | 80.89 |
| 2 | 60 | 0.8GHz | Cell0@0.8GHz | Cell0@3.5GHz | 2.5 | 12.6 | 79.87 |
| 3 | 120 | 0.8GHz | Cell0@0.8GHz | Cell0@3.5GHz | 2.7 | 12.9 | 79.08 |
| 4 | 30 | 2.6GHz | Cell0@2.6GHz | Cell0@1.8GHz | 0.5 | 3.4 | 84.59 |
| 5 | 60 | 2.6GHz | Cell0@2.6GHz | Cell0@1.8GHz | 0.5 | 3.4 | 85.05 |
| 6 | 120 | 2.6GHz | Cell0@2.6GHz | Cell0@1.8GHz | 0.5 | 3.4 | 85.84 |
| 7 | 30 | 0.8GHz | Cluster of Cell0 @[0.8,1.8,2.6] GHz | Cell0@3.5GHz | 1.33 | 9.22 | 85.57 |
| 8 | 60 | 0.8GHz | Cluster of Cell0 @[0.8,1.8,2.6] GHz | Cell0@3.5GHz | 1.39 | 9.45 | 85.32 |
| 9 | 120 | 0.8GHz | Cluster of Cell0 @[0.8,1.8,2.6] GHz | Cell0@3.5GHz | 1.52 | 9.72 | 84.39 |

### Impact of Cluster-vs-Single Prediction

Table 5 describes a comparison between single-to-single Cells prediction (in green rows 1-3) versus cluster-to-single cells prediction (in yellow rows 7-9). The performance shows that cluster-to-single cells prediction outperform that of single-to-single cell prediction.

Result shows that (in some cases) cluster-to-single cells prediction outperform that of single-to-single cell prediction.

Further investigation is needed to understand the impact of single- versus cluster-based prediction for different scenarios.

### Impact of Strongest vs Weakest cell prediction

In this subsection, we evaluate the performance of predicting strongest cell vs weakest cells given two 0.8GHz and 2.6GHz carriers as input to predict different carriers (0.8, 1.8, 2.6, and 3.5 GHz) using three scenarios of single-to-single, cluster-to-single, and cluster-to-cluster cells prediction, as illustrated in Table 5. It is observed that predicting strongest cell at other frequency in single-to-single prediction outperform all other scenarios (compare rows 1-2 vs 3-10). For instance, as in rows 1-2, prediction of single strongest cell (@ 2.6GHz or @3.5GHz) outperforms prediction of weakest single cell (at all frequencies @0.8GHz, @1.8GHz, @2.6GHz, and @3.5GHz, as in rows 3-6). Additionally, cluster prediction of first three strongest cells @3.5GHz from 3 strongest cells in 2.6GHz (in row 7) outperforms all other scenarios of cluster-to-single and cluster-to-cluster weakest cell predictions (rows 8-10).

Prediction of single collocated (strongest) cell outperforms other scenarios, including cluster-to-cluster.

Prediction of the strongest cells outperforms prediction of weakest cells.

Including cluster of cells in output or input of model means that spatially random channels are to be learnt by the model, hence it worsens the performance in compared to single-to-single cell prediction. This might lead to a better or worse prediction depending on the correlation between the input and output cluster, it also depends on how ML model can learn such relation.

Further investigation of cluster-to-cluster cells prediction performance in compared to single-to-single cell prediction.

Consider spatial related assistance information to improve performance, such as, location information..

Table 6. Strongest vs Weakest cells prediction, for cluster and singular scenarios.

|  | Fixed Serving Carrier | Input Cell | Output Cell | ML-Loss (dB) | BL-Loss (dB) | Gain (%) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0.8GHz | Cell0@0.8GHz | Cell0@2.6GHz | 1.435 | 6.304 | 77.24 |
| 2 | 0.8GHz | Cell0@0.8GHz | Cell0@3.5GHz | 2.346 | 12.277 | 80.89 |
| 3 | 0.8GHz | Cell0@0.8GHz | Cell8@0.8GHz | 8.138 | 29.392 | 72.31 |
| 4 | 0.8GHz | Cell0@0.8GHz | Cell8@1.8GHz | 8.766 | 32.473 | 73.00 |
| 5 | 0.8GHz | Cell0@0.8GHz | Cell8@2.6GHz | 9.419 | 35.791 | 73.68 |
| 6 | 0.8GHz | Cell0@0.8GHz | Cell8@3.5GHz | 10.570 | 42.531 | 75.15 |
| 7 | 2.6GHz | Cluster of Cells [0,1,2] @2.6GHz | Cluster of Cells [0,1,2] @ 3.5GHz | 3.97 | 8.71 | 54.47 |
| 8 | 2.6GHz | Cluster of Cells [0,1,2] @2.6GHz | Cell8@2.6GHz | 4.63 | 21.53 | 78.51 |
| 9 | 2.6GHz | Cluster of Cells [0,1,2] @2.6GHz | Cluster of Cells [6,7,8]@2.6GHz | 5.09 | 18.97 | 73.14 |
| 10 | 2.6GHz | Cluster of Cells [0,1,2] @2.6GHz | Cluster of Cells [6,7,8]@3.5GHz | 5.69 | 25.67 | 77.85 |

### Beam-to-Cell vs Cell-to-Cell

Table 6 evaluates Beam-to-Cell versus Cell-to-Cell performance. It is clear that Beam-to-Cell has worst performance than Cell-to-Cell, which might be caused by many reasons. One reason might be because we considered all SSB beams and not single consolidated one. Those SSB beam are directed in different angle hence confusing the ML model. Therefore, we propose to further investigate this case after considering consolidated as input (given that if we considered consolidated-beam as input, it is almost similar to the cell as input scenario).

Table 7. Beam2Cell versus Cell2Cell performance.

| Scenario | Serving Carrier | Input Cell | Output Cell | ML-Loss (dB) | BL-Loss (dB) | Gain % |
| --- | --- | --- | --- | --- | --- | --- |
| Cell-to-Cell | 3_5GHz | [0] | [27] | 1.03 | 5.82 | 82.311 |
| Beam-to-Cell | 3_5GHz | [0] | [27] | 2.84 | 5.82 | 51.145 |

Given fixed ML model, Cell-to-Cell has outperformed Beam-to-Cell in ML-Loss.

The case of Beam-to-Cell (with consolidated beam only) versus Cell-to-Cell for inter-frequency prediction is FFS.

# 3	Conclusion

Based on the discussion in the previous sections we have the following observations:

Observation 1	Correlation across carriers could be a measure of frequency distance. The higher correlation, the lower the frequency distance, and the better prediction is expected.

Observation 2	Correlation becomes worse as we go farther in frequency distance and spatial distance, and such behaviour stacks from both frequency and spatial distances.

Observation 3	Best Inter-Frequency ML-loss is obtained when predicting 1.8GHz cell from 2.6GHz cells, mainly strongest cells and across carriers that has highest correlation.

Observation 4	The minimum achievable loss is ~ 0.48 dB, with corresponding gain of 85.9% in compared to baseline.

Observation 5	Using higher frequency (e.g., 2.6 GHz) as input, to predict a certain lower frequency (e.g., 1.8 GHz) will results in a better performance compared to having lower frequency as input (e.g., 1.8 GHz), and predict higher frequency (e.g., 2.6 GHz).

Observation 6	The larger the distance between carriers the worse the ML-Loss.

Observation 7	Sorting of the dataset (Input/output cells) improves the prediction accuracy compared to not sorted cells.

Observation 8	Sorting has larger improvement impact when applied on lower-to-higher frequency than higher-to-lower frequency and when applied on higher frequency distance scenarios.

Observation 9	Improvement by the sorting input is also observed for different prediction scenarios Single-Cell to Single-Cell and Single-Cell to Cluster-Cells and Cluster-Cells to Single-Cell.

Observation 10	For Single-to-Single Cell and Cluster-to-Single Cell prediction scenarios, large frequency distance highlights the impact of speed on ML-Loss, i.e., higher speed worsens the ML-Loss and BL-Loss.

Observation 11	For Single-to-Single Cell and Cluster-to-Single Cell prediction scenarios, if frequency distance is not large, the speed impact on ML-Loss and BL-Loss is not observable, i.e., the ML-Loss and BL-Loss do not change while changing the speed from 30, to 60, to 120 km/h.

Observation 12	Result shows that (in some cases) cluster-to-single cells prediction outperform that of single-to-single cell prediction.

Observation 13	Prediction of single collocated (strongest) cell outperforms other scenarios, including cluster-to-cluster.

Observation 14	Prediction of the strongest cells outperforms prediction of weakest cells.

Observation 15	Including cluster of cells in output or input of model means that spatially random channels are to be learnt by the model, hence it worsens the performance in compared to single-to-single cell prediction. This might lead to a better or worse prediction depending on the correlation between the input and output cluster, it also depends on how ML model can learn such relation.

Observation 16	Given fixed ML model, Cell-to-Cell has outperformed Beam-to-Cell in ML-Loss.

Based on the discussion in the previous sections we propose the following:

Proposal 1	For cell-level measurement prediction model, Case 2 and Case 3 are to be further investigated.

Proposal 2	Further investigate if higher frequency predicting lower frequency always outperform lower frequency predicting higher one.

Proposal 3	Further investigate the impact of carrier distance on frequency prediction performance.

Proposal 4	Further investigate of data processing on higher vs lower frequency distance, and on lower-to-higher vs higher-to-lower frequency prediction.

Proposal 5	Study the impact of training on unsorted dataset, while predicting the RSRP value of strongest cell.

Proposal 6	Further investigate the impact of speed on both frequency and spatial prediction.

Proposal 7	Further investigate ML-Loss performance when training (either single-to-single cells or cluster-to-cluster) a model given a single speed but predict on a different speed.

Proposal 8	Further investigation is needed to understand the impact of single- versus cluster-based prediction for different scenarios.

Proposal 9	Further investigation of cluster-to-cluster cells prediction performance in compared to single-to-single cell prediction.

Proposal 10	Consider spatial related assistance information to improve performance, such as, location information..

Proposal 11	The case of Beam-to-Cell (with consolidated beam only) versus Cell-to-Cell for inter-frequency prediction is FFS.

# 4	References

RP-234055,	SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR

TR38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

R2-2403255, Discussion on AI/ML based measurement events prediction.

# Appendix A

## A.1	Simulation Parameters

Table 8. Simulation Parameters

| Report parameters |  | Inter-Frequency |  |  |
| --- | --- | --- | --- | --- |
| Reported simulation assumptions | UE trajectory option (option 1,2,3 in [4]) | Option 3 |  |  |
|  | UE trajectory boundary processing option (option 1,2,3 in [4]) | Option 1 |  |  |
|  | UE speed (30,60,90,120 Km/h) | 30Km/h, 60, 120Km/h |  |  |
|  | Inter-frequency correlation assumption in general (yes or no)(Note 1) | Yes |  |  |
|  | Inter-frequency shadow fading correction (e.g. full, partial, no)(Note 1) | Not involved |  |  |
|  | Whether LOSsoft is modeled or not | Modelled |  |  |
|  | spatial consistency option (A or B) | Option B |  |  |
|  | Number of TX beams | 4 for FR1 UMa, |  |  |
|  | Number of RX beams | 1 for FR1 |  |  |
| Data Size (Number of Samples) | Training/validity | ~36656 |  |  |
|  | Testing | ~9164 |  |  |
| AI/ML model input/output | Model input (Note 5) | Cell-level results | Beam-level results |  |
|  | Model output(Note 6) | Cell-level results |  |  |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer …) |  |  |  |
|  | Model complexity in a number of parameters(M) |  |  |  |
|  | Model complexity in model size (e.g. Mbyte) |  |  |  |
|  | Computational complexity [FLOPs] |  |  |  |
| Metrics | Average L3 cell level RSRP difference (dB) |  |  |
|  | Other optional KPIs (e.g., L1 beam level RSRP difference and FFS on L3 beam level RSRP difference) |  |  |  |

## A.2	Agreements from Last Meetings

Agreements on simulation assumptions (Meeting 126) 3 Prediction accuracy metric for RRM measurement cell level prediction is defined as “RSRP difference between predicted L3 cell level measurement result and actual L3 cell level measurement result of the same cell” for all RRM sub cases 6 For RRM sub case 1 and 3, it is up to company’s implementation whether L1 filtering is applied for input L1 beam level measurement. Companies are expected to report what L1 filtering they use in their simulation. 11 For both Intra-frequency and inter-frequency inter-cell prediction, the measurement on cell for measurement should not be reduced in both temporal and spatial domain 12 For Inter_F_C (inter-frequency inter-cell), RAN2 start evaluation from co-located scenario 13 for Inter_F_C(inter-frequency inter-cell), RAN2 should focus on the case where cell for measurement and cell for prediction are in the same sector. 14 FR1 to FR1 inter-frequency inter-cell prediction is applicable for all RRM sub cases. And it is up to company to report applied RRM sub cases together with their simulation result. 16 Intra_F_Inter_C (intra-frequency inter-cell) prediction will not be evaluated at least in early stage. Intra_F_Inter_C(Intra-frequency inter-cell) refers to both co-located and non-colocated neighbouring cell prediction. 17 No traffic model is simulated 18 During simulation UE is dropped 100% outdoor 19 It is up to company’s implementation to select how to distribute the UE 20 Fast fading is necessary for RRM sub case 1 and 3. FFS case 2 22 LOSsoft is optionally modelled in the channel modelling 24 For FR1, following parameters are agreed: 1, Table 6.2.1-1 template is taken as starting point ( to be updated by individual proposal later on) 3, to set up 2-tier model (7 sites, 3 sectors/cells per site) (20/20) 4, 500m as ISD 5, channel modelling is Uma with distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901 6, 20MHz as bandwidth 7, The recommendated value in yellow in table 2.3.3-1 ( by removing wording “At least for BM-Case1,”) 25 Section 7.6.5 in 38.901 is taken as baseline for inter-frequency correlation model. Whether inter-frequency correlation model is used is optional and companies can report what they use. FFS on the understanding shadowing correlation in inter-freq. For now companies should report what assumption they have made 26 Following RRC parameters need be aligned as simulation parameters: (17/20) - RRC parameters for measurement consolidation - RRC parameters for L3 filtering (filter coefficient, measurement period) 27 We apply same L1 sampling period for both intra and inter (i.e. we don’t simulate existence of measurement gap as starting point) 28 Simulation assumptions discussed in section 2.3.1~2.3.3 is taken as baseline also for use cases other than RRM measurement prediction. Any update is subject to further discussion on other use cases. 29 The sample period(s) are aligned among companies for intra-frequency intra-cell temporal domain prediction. We can start with 20ms for FR2 and 40ms for FR1. Measurement period: FFS – suggestion from rapporteur is to start with 480ms for FR2 and 200ms for FR1 30 Simulation parameters in table 2.3.4-1 (by removing Table A.2.1-7 and Table 2.1-10) are taken as starting point for both UE sided model and network sided model for FR2. The number of beams could be left for company to report. FFS Use 100Mhz for channel BW? 31 For FR1, following parameters are agreed: 2, {4GHz,30KHz} as frequency for intra-frequency scenario and {2GHz, 15KHz(FDD)/30GHz(TDD)} as another frequency for inter-frequency scenario

## A.3	Correlation Matrix Across Frequency Carriers

Table 9. Correlation Matrix across carriers and (first three) spatial beams

| Carrier |  |  | 0.8GHz |  |  | 1.8GHz |  |  | 2.6GHz |  |  | 3.5GHz |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Space Beam per Carrier |  | SB-1 | SB-2 | SB-3 | SB-1 | SB-2 | SB-3 | SB-1 | SB-2 | SB-3 | SB-1 | SB-2 | SB-3 |
| 0.8GHz | SB-1 | 1 | 100.00% | 87.35% | 83.60% | 88.50% | 82.06% | 82.04% | 87.29% | 81.62% | 82.39% | 71.69% | 61.92% | 73.44% |
|  | SB-2 | 2 | 87.35% | 100.00% | 88.76% | 81.39% | 93.09% | 88.10% | 79.70% | 92.40% | 87.58% | 69.01% | 74.95% | 75.68% |
|  | SB-3 | 3 | 83.60% | 88.76% | 100.00% | 84.45% | 89.16% | 95.42% | 84.60% | 89.47% | 95.15% | 70.48% | 68.47% | 77.66% |
| 1.8GHz | SB-1 | 4 | 88.50% | 81.39% | 84.45% | 100.00% | 88.94% | 86.45% | 95.49% | 86.19% | 85.49% | 79.60% | 66.97% | 78.37% |
|  | SB-2 | 5 | 82.06% | 93.09% | 89.16% | 88.94% | 100.00% | 90.86% | 85.83% | 96.05% | 89.51% | 75.06% | 78.87% | 79.22% |
|  | SB-3 | 6 | 82.04% | 88.10% | 95.42% | 86.45% | 90.86% | 100.00% | 85.31% | 90.70% | 96.86% | 72.46% | 70.69% | 82.20% |
| 2.6GHz | SB-1 | 7 | 87.29% | 79.70% | 84.60% | 95.49% | 85.83% | 85.31% | 100.00% | 88.72% | 87.13% | 81.43% | 67.75% | 79.35% |
|  | SB-2 | 8 | 81.62% | 92.40% | 89.47% | 86.19% | 96.05% | 90.70% | 88.72% | 100.00% | 90.87% | 76.28% | 79.77% | 79.84% |
|  | SB-3 | 9 | 82.39% | 87.58% | 95.15% | 85.49% | 89.51% | 96.86% | 87.13% | 90.87% | 100.00% | 73.15% | 70.30% | 83.06% |
| 3.5GHz | SB-1 | 10 | 71.69% | 69.01% | 70.48% | 79.60% | 75.06% | 72.46% | 81.43% | 76.28% | 73.15% | 100.00% | 87.36% | 87.61% |
|  | SB-2 | 11 | 61.92% | 74.95% | 68.47% | 66.97% | 78.87% | 70.69% | 67.75% | 79.77% | 70.30% | 87.36% | 100.00% | 83.60% |
|  | SB-3 | 12 | 73.44% | 75.68% | 77.66% | 78.37% | 79.22% | 82.20% | 79.35% | 79.84% | 83.06% | 87.61% | 83.60% | 100.00% |