3GPP TSG-RAN WG2 #127	R2-2407093

Maastricht, The Netherlands, 20th – 24th May 2024

Agenda Item:	8.3.4.1 	Simulation assumptions and evaluation methodology for RLF failure prediction

Source:	Ericsson

Title:	AI/ML based RLF predictions

Document for:	Discussion

# 1	Introduction

A new study item on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR was approved in RAN plenary #102 with the following objectives [1].

- The evaluation of the AI/ML aided mobility benefits should consider HO performance KPIs (e.g., Ping-pong HO, HOF/RLF, Time of stay, Handover interruption, prediction accuracy, and measurement reduction) etc.) and complexity tradeoffs [RAN2]

In RAN2#126 meeting, for RLF/HOF prediction in this study item, it has been agreed [2]

Agreements - Study Indirect: RLF prediction based on the temporal domain serving cell measurement predictions (e.g. SINR). - Study Direct: Directly RLF prediction by AI/ML models. - FR2 study will be prioritized for RLF prediction. - The study should focus on RLF due to T310 expiry (i.e. in-synch/out-of-synch case) as the representative RLF case for direct and indirect prediction. - HOF prediction is downprioritized in our study. NO simulations/evaluations should be done/submitted - RLF prediction result is the RLF probability within a time window or at time instance, at least for direct case. FFS on expected RLF time and indirect case. - No evaluation/simulations are expected for August meeting for RLF - Simulation assumption specific to RLF will be discussed in August. The assumption is that we will reuse RRM simulation assumptions (where possible).

In this contribution, the methodologies for evaluation of the AI/ML models for RLF predictions is discussed.

# 2	Discussion

## 2.1 	Direct RLF predictions

As shown above, it was agreed in RAN2#126 to study RLF predictions due to T310 expiry. The model input that could be useful has not been discussed yet.

Some examples of input are measured and/or predicted SINR of the cell, running values of T310, N310 and N311. Additional input may be useful as well and could be further studied. There are also simulation settings that need to be agreed upon, such as UE speed and prediction time window. Such settings are discussed in the simulation assumptions chapter below.

Measured and/or predicted SINR of the cell and running values of T310, N310 and N311 can be used as input to direct RLF prediction model.

Additional input to the direct RLF prediction model is FFS.

The output of the model is agreed to be the RLF probability within a time window. There is an FFS on whether a specific time is needed as output. A specific time could be seen as a subcase of a time window, where only one time instance is reported. There doesn’t seem to be any reason to preclude that specific case.

Expected time of RLF is one output from direct RLF predictions.

## 2.2 	Indirect RLF predictions

Another way of predicting RLF is to replace the RLM resource measurements SINR SSB beams, CSI-RS beams in the current RLF detection mechanism with the predicted RLM resource measurements. This solution boils down to predicting the radio link quality of one cell and the results are fed into the RLF detection algorithm. The input to the prediction is thus the predicted RLM measurements, e.g. predicted SINR. This solution is schematically shown in the Figure 4.

- Prediction of RLF due to T310 expiry

Figure 4. RLF detection based on the monitoring the predicted measurement of RLM resources.

Indirect prediction of RLF means feeding the RLF detection mechanism with predicted radio link quality measurements instead of actual measurements and observing N310 OOS indications which are not followed by N311 IS indications before T310 expiry.

Unlike direct prediction, indirect prediction reuses the existing mechanism of RLF detection directly and therefore reduces the algorithm complexity at the UE side, which is helpful for cost reduction and energy savings.

3GPP agreed to study both direct and indirect prediction of RLF. The output of the direct predictions was agreed to be the RLF probability within a time window, but for indirect prediction the output is still FFS. Feeding the RLF detection mechanism with predicted RLM measurements will result in a specific time for the predicted RLF, after a specific number of OOS indications. Therefore, the output of the indirect RLF predictions can be the predicted time of the RLF. Also the RLF probability can be derived based on the RRM prediction measurement accuracy.

The output of indirect RLF predictions is the predicted time of the RLF and the RLF probability.

## 2.3 	Simulation assumptions

RLF is related to radio link quality of one cell, and one simple possibility to get RLF scenarios is to let the UE move across cell edge (e.g. option 1 and option 2 in Figure 5) and relax the handover margin so that the radio link quality of the serving cell becomes low enough low enough for consecutive Qout indications. To evaluate the performance of the RLF prediction, the UE distribution for cell-edge UEs and evenly distributed UEs can be set to (100%, 0%) or (80%, 20%).

For RLF prediction evaluation, the UE distribution for cell-edge UEs and evenly distributed UEs can be set to (100%, 0%) or (80%, 20%).

(a) UE moving option 1                      (b) UE moving option 2

Figure 5. UE moving options in RLF prediction performance evaluation for cell-edge UEs

RLF due to T310 expiry is related to the parameter configuration of t310, n310 and n311, so settings of them need to be defined for the direct approach. A suggestion is to use 100ms for t310, 2 for both n310 and n311.

For direct RLF prediction evaluation, the configuration for timers and counters is 100ms for t310, 2 for both n310 and n311.

For indirect RLF predictions, predicted RLM measurements are used as input. In order to save simulation efforts, the same simulation assumptions as for prediction of RRM measurements can be reused.

Reuse the same simulation assumptions for indirect RLF predictions as for RRM predictions.

The ground truth for performance analysis of RLF prediction can be the actual RLF report time, for example RLF due to timer T310 expiry, obtained via the existing RLF mechanism.

Ground truth for performance evaluation can be RLF report time obtained by existing solution.

Regarding performance KPI, the time difference between the predicted RLF time (the median time in case of a time window) and the actual RLF time can be used as a simple performance metrics. Additionally, fault RLF prediction percentage and missed RLF prediction percentage can be provided as performance KPI.

The time difference between the predicted RLF (median time is used in case a time window was predicted) and the actual RLF is used as a performance KPI.

Fault RLF prediction percentage and missed RLF prediction percentage are used as KPIs for false detection.

# Conclusion

In the previous sections we made the following observations:

Observation 1	Indirect prediction of RLF means feeding the RLF detection mechanism with predicted radio link quality measurements instead of actual measurements and observing N310 OOS indications which are not followed by N311 IS indications before T310 expiry.

Based on the discussion in the previous section we propose the following:

Proposal 1	Measured and/or predicted SINR of the cell and running values of T310, N310 and N311 can be used as input to direct RLF prediction model.

Proposal 2	Additional input to the direct RLF prediction model is FFS.

Proposal 3	Expected time of RLF is one output from direct RLF predictions.

Proposal 4	The output of indirect RLF predictions is the predicted time of the RLF and the RLF probability.

Proposal 5	For RLF prediction evaluation, the UE distribution for cell-edge UEs and evenly distributed UEs can be set to (100%, 0%) or (80%, 20%).

Proposal 6	For direct RLF prediction evaluation, the configuration for timers and counters is 100ms for t310, 2 for both n310 and n311.

Proposal 7	Reuse the same simulation assumptions for indirect RLF predictions as for RRM predictions.

Proposal 8	Ground truth for performance evaluation can be RLF report time obtained by existing solution.

Proposal 9	The time difference between the predicted RLF (median time is used in case a time window was predicted) and the actual RLF is used as a performance KPI.

Proposal 10	Fault RLF prediction percentage and missed RLF prediction percentage are used as KPIs for false detection.

# References

RP-234055,	SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR