3GPP TSG-RAN WG2 #127	R2-2407094

Maastricht, The Netherlands, Aug 19th – 23rd, 2024

Source:		NEC

Agenda item:		8.10.2	MRO enhancements for Rel-18 mobility features

Title:	MRO enhancements for LTM

Document for:		Discussion and decision

- Introduction

During last RAN2 meeting, the following agreements was made for MRO enhancements of LTM[1]:

If available, log the L1 measurements for serving cell, target cell and other LTM candidate cells in RLF report, upon RLF or mobility failure.

Reuse the existing approach of using timeConnFailure and the reconnectCellId in RLF-report also for LTM failures, details TBD.

Log the LTM cell ID upon performing recovery an LTM candidate cell, details TBD e.g. which field.

Extend lastHO-Type in RLF-Report to indicate the LTM cell switch as last executed mobility procedure.

In this paper, we provide our further consideration on the MRO for LTM.

- Discussion

RACH-less based LTM can be supported to further fasten the latency of LTM procedure. Currently, RA information included in RLF-report if a handover failure happens. For RACH-less LTM, since RA procedure is not performed by the UE during the LTM procedure, the RA information in RLF-report should be omitted. If the RA information is omitted in the RLF-report, the network can identify that the failed LTM procedure is RACH-less.

Proposal 1. Omit RA information in RLF-report in case of RACH-less LTM failure.

For RACH-less LTM, there are two ways for UE to determine the TA of the target cell, i.e., by the TA value carried in LTM cell switch command, or by UE-based TA measurement. If TA command is provided in LTM cell switch command, the UE accesses to the target cell based on the TA command without performing RA procedure. If TA command is not provided in the LTM cell switch command, and if the UE is configured with TA measurement and measured TA is available, the UE accesses to the target cell based on the measured TA without performing RA procedure. To access to the target cell without RA procedure, both dynamic grant and configured grant can be used. And the UE shall apply the TCI state indicated in the LTM cell switch command for dynamic grant reception or configured grant resource selection. To analysis the failure of RACH-less based LTM, it is beneficial to report the related information to the network, and RAN2 consider to include information related to RACH-less LTM in RLF-report. Since the failure of RACH-less LTM may be caused by unproper TA value or TCI state information, the UE can report the TCI state ID and TA value in the LTM cell switch command, and TA value measured by the UE.

Proposal 2: Additional information related to RACH-less LTM can be included in the RLF-report:

- TCI state ID in LTM cell switch command

- TA command value in LTM cell switch command

- TA value measured by the UE

For RACH-based LTM, both CFRA and CBRA can be performed. And CFRA can be based on either CFRA configuration in RRC configuration, or CFRA resource in LTM cell switch command. During RAN2 #125bis meeting, it was agreed that RAN2 considers RA report for MCG-LTM. However, it is not clear yet, whether the RA-report can be supported for a RA procedure for early sync purpose.

To support RACH-less based LTM, early synchronization can be performed, during which the UE can be triggered by PDCCH order from the network to perform RA to the LTM candidate cell. More specifically, UE would transmit preamble to the LTM candidate cell, and the RA procedure is considered as completed. No RAR would be received by the UE , instead, the TA information would be transmitted from the candidate DU to the source DU via CU, and the TA value is sent together in the LTM cell switch command to the UE by the source DU. The UE is not aware of whether the RA procedure for early UL sync is successful or not, and the retransmission of the preamble is indicated by the source cell. The RA resource is allocated per candidate cell per gNB-DU by the candidate DU, so it is beneficial to record RA information for better usage of RA resource for early sync, however, whether there is the worth to work on the related function, and whether this is done by the UE or network needs further discussion.

Proposal 3: For LTM MRO, RA-report for RA procedure during execution of LTM is considered as baseline, and RAN2 discusses whether to support RA-report enhancement for early UL sync procedure.

# 3. Conclusion

In this paper, we discuss on further consideration on the MRO for LTM, and have the following proposals:

Proposal 1. Omit RA information in RLF-report in case of RACH-less LTM failure.

Proposal 2: Additional information related to RACH-less LTM can be included in the RLF-report:

- TCI state ID in LTM cell switch command

- TA command value in LTM cell switch command

- TA value measured by the UE

Proposal 3: For LTM MRO, RA-report for RA during execution of LTM is considered as baseline, and RAN2 discusses whether to support RA-report enhancement for early UL sync procedure.

# Reference

[1] Meeting report of RAN2 #126