3GPP TSG-RAN WG2 #127	R2-2407095

Maastricht, The Netherlands, Aug 19th – 23rd, 2024

Source:		NEC

Agenda item:		8.10.2	MRO enhancements for Rel-18 mobility features

Title:	MRO for CHO with candidate SCG(s)

Document for:		Discussion and decision

- Introduction

During last RAN2 meeting, and achieved the following agreement was made for MRO enhancements for CHO with candidate SCG(s) [1]:

RAN2 to enhance the RLF report with additional information regarding the state of the two execution conditions. We see later if we also can enhance the SCGFailureInformation report.

In this paper, we provide our further consideration on MRO for the CHO with candidate SCG(s).

- Discussion

A CHO with candidate SCG(s) is defined as a PCell change with PSCell addition/change that is executed by the UE when the execution conditions for both candidate PCell and the associated candidate PSCell are met. A failure of CHO with candidate SCG(s) may be caused by timing mismatch between the fulfilment of the execution condition(s) of the candidate PCell and the fulfilment of the execution condition(s) of the candidate PSCell.

In MRO enhancements for CHO/CPAC, UE records the first triggered execution condition for candidate PCell/PSCell, and the time gap between the two execution conditions for the candidate PCell/PSCell if both fulfilled. We can record the similar information for CHO with candidate SCG(s), to record the which of the execution condition(s) of the candidate PCell or execution condition(s) of the candidate PSCell, is fulfilled first, and the time gap between the fulfilment of execution condition(s) of the candidate PCell and fulfilment of execution condition(s) of the candidate PSCell.

Proposal 1: UE stores information indicating which of the execution condition(s) of the candidate PCell or execution condition(s) of the candidate PSCell, is fulfilled first.

Proposal 2: UE stores time gap between the fulfilment of execution condition(s) of the candidate PCell and fulfilment of execution condition(s) of the candidate PSCell.

As said above, the UE can store information related to execution condition(s) for PCell in RLF-report, i.e., the first fulfilled execution condition, and the time gap between the fulfilment of the two execution conditions for candidate PCell. In this way, the network can further optimize the two execution conditions associated with the PCell. For CHO with candidate SCG(s), it is beneficial that the UE can also collect the information related to the two execution conditions for the candidate PSCell, such that the network can optimize the execution conditions of the candidate PSCell in case of CHO with candidate SCG(s).

Proposal 3. UE stores in RLF-report, information related to the execution conditions for the candidate PSCell, which comprises

- the first fulfilled execution condition associate with the candidate PSCell

- the time gap between the fulfilment of the two execution conditions for candidate PSCell

Currently, the UE can store information related to execution condition(s) for PSCell in SCGFailureInformation, i.e., the first fulfilled execution condition, and the time gap between the fulfilment of the two execution conditions for candidate PSCell. For failure of CHO with candidate SCG(s), it is possible that SCG failure happens before the triggering of CHO with candidate SCG(s), or CPAC failure happens during execution of CHO with candidate SCG(s).  In addition to the information of execution conditions associated with the candidate PSCell, it is benefical to also include in the SCGFailureInformation message the information of execution conditions of PCell, i.e., the first fulfilled execution condition associate with the candidate PCell, and the time gap between the fulfilment of the two execution conditions for candidate PCell.

Proposal 4. UE sores, in SCGFailureInformation, information related to the execution conditions for the candidate PCell.

For near failure of CHO with candidate SCG(s), in previous meeting, whether to introduce “time gap between fulfilment of the execution conditions being larger than a threshold” as an additional triggering condition for SHR or SPR. In our understanding, near failure means that the mobility procedure is successful, however the performance of the UE is not good during the mobility procedure, e.g., out of sync is triggered (corresponding to T310 trigger), the service interruption is too long (corresponding to T304 trigger). For the case “time gap between fulfilment of the execution conditions of PCell and PSCell being larger than a threshold”, the performance of the UE may be not impacted. Just like the legacy CHO/CPAC, the time gap between the fulfilment of the two execution conditions of the candidate cell being larger than a threshold is not one triggering condition for SHR and SPR. Therefore, we do not think “time gap between fulfilment of the execution conditions for PCell and PSCell being larger than a threshold” is a useful triggering condition for SHR or SPR.

Proposal 5. Do not introduce “time gap between fulfilment of the execution conditions being larger than a threshold” as an additional triggering condition of SHR or SPR.

# 3. Conclusion

In this paper, we discuss on further consideration on the MRO for CHO with candidate SCG(s), and have the following proposals:

Proposal 1: UE stores information indicating which of the execution condition(s) of the candidate PCell or execution condition(s) of the candidate PSCell, is fulfilled first.

Proposal 2: UE stores time gap between the fulfilment of execution condition(s) of the candidate PCell and fulfilment of execution condition(s) of the candidate PSCell.

Proposal 3. UE stores in RLF-report, information related to the execution conditions for the candidate PSCell, which comprises

- the first fulfilled execution condition associate with the candidate PSCell

- the time gap between the fulfilment of the two execution conditions for candidate PSCell

Proposal 4. UE sores, in SCGFailureInformation, information related to the execution conditions for the candidate PCell.

Proposal 5. Do not introduce “time gap between fulfilment of the execution conditions being larger than a threshold” as an additional triggering condition of SHR or SPR.

# Reference

[1] Meeting report of RAN2 #126