3GPP TSG-RAN WG2 Meeting #127	R2-2407098

# Maastricht, Netherlands, Aug 19th – 23rd, 2024

Source:                Qualcomm Incorporated

Title:	RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE

Agenda Item:	8.4.3

Document for:	Discussion and Decision

# Introduction & Background

The following objective is approved for IDLE and Inactive mode LP-WUS, this contribution discusses the related issues.

  - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2)

# Discussion

## 2.1 Serving cell measurement relaxation

### 2.1.1 Serving cell measurement relaxation with total offloading to LP-WUS

In this case, when the UE is in LP-WUS mode, the UE only performs LP-SS measurement and does not perform SSB measurements. This will save UE power because UE does not need to wake up MR. But this mechanism has impact on cell reselection criterion, and Rs criterion needs to introduce LP-SS measurement based definition.

In last RAN2 meeting, RAN2 achieved agreement for serving cell measurement offloading

For serving cell measurement offloading (i.e., serving cell measurement fully offloaded to LR and no serving cell measurement via MR is required), RAN2 should focus on specifying the offloading criterion for serving cell for UEs supporting LP-WUS, and assume that RAN4 will define the measurement offloading requirements for serving cell.

UE can be configured a threshold to determine whether to offload measurement to LR. Considering MR and LR may have different coverage, it is more safe that UE starts offloading measurements to LR. That means when MR measurement is above a threshold and the LR measurement is above a threshold, UE starts offloading measurements to LR. There could be case that MR measurement is above the threshold but the LR measurement is below a threshold, it can be left to UE implementation to determine when to start LR measurement.

- For MR serving cell totally offloading to LR, UE determines the offloading start and stop as following:

- When the MR measurement is above a threshold and the LR measurement is above a threshold, UE starts totally offloading measurement to LR (i.e, stop MR measurements). It is up to UE implementation to determine when to start LR measurement.

- When the LR measurement is below a threshold, UE switches to MR measurements.

- When the corresponding thresholds are no configured, it is up to UE implementation to determine.

### 2.1.2 Other relaxations

RAN4 is discussing other relaxation scenarios ,e.g. serving cell measurement is partially offloaded to LR, and MR measurement relaxation without offloading to LR. RAN2 should wait for RAN4 progress and then discuss RAN2 impact.

- RAN2 waits for RAN4 progress on other measurement relaxation scenarios, e.g. MR measurement partially offloaded to LR, and MR measurement relaxation without offloading to LR.

### 2.1.4 LP measurement types and metrics

RAN1 is discussing which measurement type, i.e. OFDM-based or OOK-based measurement should be performed. And also which measurement metrics should be measured, e.g. RSRP or RSRQ. It is proposed to wait for RAN1 progress.

- It is proposed to wait for RAN1 progress on LP measurement types (e.g, OFDM-based or OOK-based measurement) or metrics (e.g. RSRP or RSRQ).

## 2.2	Neighbor cell measurement relaxations

RAN2 had achieved the following agreements for neighboring cell measurement relaxation

RAN2 understand that the RRM measurement of the neighboring cell can only be performed by MR. Can discuss again if RAN1 inform us otherwise.

RAN2 will further discuss the neighboring cell measurement relaxation criteria (if the UE is using LR to measure the serving cell), e.g., considering reuse Rel-16 criteria for ‘not at cell edge’ and ‘low mobility’.

Since we already have decent amount of neighbor cell measurement relaxations specified under different conditions, e.g. relaxation based on UE mobility and serving cell coverage, and the maximum measure cycle is up to 1hour. Then RAN2 can confirms to reuse Rel-16 criteria for ‘not at cell edge’ and ‘low mobility’, but the parameter to determine ‘not at cell edge’ and ‘low mobility’ should be LR specific. But the detailed parameters can be discussed during stage 3 CR implementation

- For neighbouring cell measurements relaxation, reuse Rel-16 criteria for ‘not at cell edge’ and ‘low mobility’ with introducing LR specific parameters for coverage and mobility state determination. Detailed parameters can be discussed during CR implementation.

# Conclusion

- For MR serving cell totally offloading to LR, UE determines the offloading start and stop as following:

- When the MR measurement is above a threshold and the LR measurement is above a threshold, UE starts totally offloading measurement to LR (i.e, stop MR measurements). It is up to UE implementation to determine when to start LR measurement.

- When the LR measurement is below a threshold, UE switches to MR measurements.

- When the corresponding thresholds are no configured, it is up to UE implementation to determine.

- RAN2 waits for RAN4 progress on other measurement relaxation scenarios, e.g. MR measurement partially offloaded to LR, and MR measurement relaxation without offloading to LR.

- It is proposed to wait for RAN1 progress on LP measurement types (e.g, OFDM-based or OOK-based measurement) or metrics (e.g. RSRP or RSRQ).

- For neighbouring cell measurements relaxation, reuse Rel-16 criteria for ‘not at cell edge’ and ‘low mobility’ with introducing LR specific parameters for coverage and mobility state determination. Details can be discussed during CR implementation.

# Reference

[1]    RP-234056: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)