3GPP TSG-RAN WG2 Meeting #127	R2-2407099

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Source:                Qualcomm Incorporated

Title:	MRO enhancement for SON and MDT

Agenda Item:	8.10.2

Document for:	Discussion and Decision

# Introduction & Background

- MRO enhancement for R18 mobility mechanisms, including, Lower layer triggered mobility (LTM), CHO with candidate SCGs, subsequent CPAC [RAN3, RAN2]:

- Specification of the inter-node information exchange, including possible enhancements to interfaces [RAN3]

- Identify and specify necessary UE reporting to enhance the mobility parameter tuning [RAN2]

# Discussion

## 2.1 MRO for LTM

In last RAN2 meeting, there were some agreements on the scenarios to be supported for LTM SON/MDT.

If available, log the L1 measurements for serving cell, target cell and other LTM candidate cells in RLF report, upon RLF or mobility failure.

Reuse the existing approach of using timeConnFailure and the reconnectCellId in RLF-report also for LTM failures, details TBD.

Log the LTM cell ID upon performing recovery an LTM candidate cell, details TBD e.g. which field.

Extend lastHO-Type in RLF-Report to indicate the LTM cell switch as last executed mobility procedure.

For too late LTM, the main issue is that an RLF occurs after the UE has stayed for a long period of time in the source cell; the UE attempts to re-establish the radio link connection in a different cell. Then for RLF report, UE should report the following information corresponding to different cases.

For Case 1a, UE should include source cell ID in which RLF is detected and cell ID for re-established cell. Then the network should have the information about the failure situation, e.g. on which cell the RLF is detected, and on which cell the RLF is recovered.

For Case 1b, UE should include source cell ID in which RLF is detected and LTM candidate cell ID in which HOF is detected.

For Case 1c, UE should include source cell ID in which RLF is detected and LTM candidate cell ID in which LTM is executed successfully.

- For too late LTM, UE should report the following information in RLF report.

- For Case 1a, UE should include source cell ID in which RLF is detected and cell ID for re-established cell.

- For Case 1b, UE should include source cell ID in which RLF is detected and LTM candidate cell ID in which HOF is detected.

- For Case 1c, UE should include source cell ID in which RLF is detected and LTM candidate cell ID in which LTM is executed successfully.

For too early LTM or LTM to wrong cell, the main issue is RLF or HOF may happen shortly after a successful LTM, or a failure occurs during the LTM procedure.

- For too early LTM, the following sub-cases are considered but we may down prioritize later (not limiting):

- -	Case 2a: the UE detects HOF/RLF in the LTM target cell and performs reestablishment procedure with the source cell.

- -	Case 2b: the UE detects HOF/RLF in the LTM target cell, selects the source cell which is also an LTM candidate cell, detects HOF with the source cell, and performs reestablishment procedure.

- -	Case 2c: the UE detects HOF/RLF in the LTM target cell, and successfully completes LTM execution with the selected source cell which is also an LTM candidate cell.

For LTM to wrong cell, based on the following agreed cases:

- LTM to wrong cell, the following sub-cases are considered but we may down prioritize later (not limiting):

- -	Case 3a: the UE detects HOF/RLF in the LTM target cell and performs reestablishment procedure with the source cell.

- -	Case 3b: the UE detects HOF/RLF in the LTM target cell, selects an LTM candidate cell which is different from the source or target one, detects HOF with the selected LTM candidate cell, and performs reestablishment procedure.

- -	Case 3c: the UE detects HOF/RLF in the LTM target cell, and successfully completes LTM execution with the selected LTM candidate cell which is different from the source or target one.

For Case 2a/3a, UE should include target cell ID in which HOF or RLF is detected and cell ID for re-established cell. Then the network should have the information about the failure situation, e.g. whether the HOF or RLF is detected in the target LTM cell.

For Case 2b/3b, UE should include target cell ID in which HOF or RLF is detected and HOF in the selected LTM candidate cell and reestablished cell ID.

For Case 2c/3c, UE should include target cell ID in which HOF or RLF is detected and LTM candidate cell ID in which LTM is executed successfully.

Additionally, for case 2b/3b and 2c/3c, it also makes sense to indicate the time between UE receives LTM command and HOF/RLF detection, as shown T2 in following figures, so that network will have the knowledge on how shortly the UE detects RLF after receiving LTM command.

- For too late LTM and LTM to wrong cell, UE should report the following information in RLF report.

- For Case 2a/3a, UE should include target cell ID in which HOF or RLF is detected and cell ID for re-established cell. Then the network should have the information about the failure situation, e.g. whether the HOF or RLF is detected in the target LTM cell.

- For Case 2b/3b, UE should include target cell ID in which HOF or RLF is detected and HOF in the selected LTM candidate cell and reestablished cell ID.

- For Case 2c/3c, UE should include target cell ID in which HOF or RLF is detected and LTM candidate cell ID in which LTM is executed successfully.

- Additionally, UE should include the time between UE receives LTM command and HOF/RLF detection.

RACH information in RLF Report

Currently UE includes RACH information (ra-InformationCommon) in RLF Report if connectionFailureType is hof and if the failed handover is an intra-RAT handover. But LTM can be RACH-based or RACH-less. Therefore, the RACH information should be only included in case of RACH-based LTM and in case of RACH-less HO, UE should omit RACH information in RLF Report during RACH-less LTM and further can add an explicit indicator that the LTM was RACH-less.

- UE can include existing RACH information (ra-InformationCommon) in case RLF Report is collected during RACH-based LTM failure.

- UE should omit RACH information (ra-InformationCommon) in case RLF Report is collected during RACH-less LTM failure. UE can further add an explicit indicator that the LTM was RACH-less.

## 2.2 MRO for Subsequent CPAC (S-CPAC)

A subsequent CPAC is defined as a conditional PSCell addition or change procedure that is executed after a PSCell addition, a PSCell change, a PCell change or an SCG release based on pre-configured subsequent CPAC configuration of candidate PSCell(s) without reconfiguration and re-initiation of CPC/CPA.

The UE keeps the configured subsequent CPAC configuration (unless the network indicates to release it) and evaluates the execution conditions of candidate PSCells after completion of a PSCell addition, a PSCell change, a PCell change or an SCG release.

In last RAN2 meeting, RAN2 agreed that RAN2 will look into failure and near failure scenarios.

RAN2 will look into failure and near failure scenarios for subsequent execution of SCPAC and see if/what enhancements are needed.

The S-CPAC configuration contains

- candidate SCG configuration(s) of candidate PSCell(s)

- execution conditions

- MCG configuration (to be applied when subsequent CPAC execution is triggered)

- reference configuration

- security update configuration

In case a UE configured with S-CPAC configuration encounters an SCG failure, it sends a SCGFailureInformation to MN (including the measurement results) and waits for MN’s subsequent action e.g., whether it wants UE to add a new PSCell or keep it in standalone mode

After the last CPC/CPA execution, SCG Failure occurs as the execution condition for subsequent CPA/CPC does not meet; measurements shows that appropriate cell was not configured for subsequent CPA/CPC. For example (in Figure 5), successful S-CPAC is executed from A0 to A1. While UE is connected to A1, SCG Failure occurs as none of the execution condition for S-CPAC is met; measurements shows that appropriate candidate cell was not configured for subsequent CPA/CPC. measurement report shows PSCell B3 (not configured for subsequent CPA/CPC for PSCell A1) was the strongest cell.

Figure 5

Then it is worthy to discuss the optimization of the S-CPAC configuration (e.g. wrong sequence of candidate cells).

- Existing measurement information in SCGFailureInformation can be used to identify the best cell, so that it can be added into the candidate subsequent PSCell.

- If the new PSCell is an already prepared candidate PSCell whose configuration is provided to the UE as part of S-CPAC configuration, MN should be able to simply indicate the PSCell to add without providing the whole configuration.

- Upon receiving such an indication to activate a PSCell, UE can simply activate the indicated PSCell’s configuration and start evaluating the S-CPAC configuration.

## 2.3 MRO for CHO with candidate SCG

RAN2 agreements:

RAN2 to study failure and near failure scenarios for CHO with candidate SCGs.

RAN2 to enhance the RLF report with additional information regarding the state of the two execution conditions. We see later if we also can enhance the SCGFailureInformation report.

RAN3 agreements:

RAN3 focuses on NR-DC for MRO for CHO with candidate SCG in R19.

RAN3 will start with the failure and near failure scenarios with UEs only configured with CHO with candidate SCGs.

FFS whether to also consider failure and near failure scenarios related with configuration of CHO with candidate SCG(s) and CHO only.

Do not distinguish between fast MCG recovery/no-fast MCG recovery (after the CHO with candidate SCG execution failure)

FFS whether to support concurrent failure cases (RLF + SCG failure). The definition of these cases needs to be further clarified.

In Rel-18, CHO with candidate SCG(s) was specified for NR-DC and the following is captured in 37.340:

- A CHO with candidate SCG(s) is defined as a PCell change with PSCell addition/change that is executed by the UE when the execution conditions for both candidate PCell and the associated candidate PSCell are met.

- The UE starts evaluating the execution conditions for candidate PCell(s) and candidate PSCell(s) simultaneously upon receiving the CHO with candidate SCG(s) configuration

- UE stops evaluating the execution conditions once a PCell change or a PSCell change is triggered.

- The UE does not execute CHO with candidate SCG(s) until the execution conditions for both the candidate PCell and the associated candidate PSCell are met

Further the following is captured in 37.340:

If at least one candidate PCell satisfies the corresponding execution condition and the associated candidate PSCell satisfies the corresponding execution condition, the UE detaches from the source MN, applies the stored corresponding configuration for that selected candidate PCell and the associated candidate PSCell, synchronises to that candidate PCell, and completes the RRC handover procedure by sending RRC reconfiguration complete* message to the target MN.

Else if at least one candidate PCell satisfies the corresponding execution condition and there is no associated execution condition for a candidate PSCell, the UE detaches from the source MN, applies the stored corresponding configuration for that selected candidate PCell and, if included, the associated PSCell, synchronises to that candidate PCell and completes the RRC handover procedure by sending RRC reconfiguration complete* message to the target MN.

Below we consider different scenarios for optimizing CHO with candidate SCG.

Example

- Suppose UE is connected to (PCell A0, PSCell B0)

- UE is configured with the CHO with candidate SCG configurations i.e., (A1, B1), (A1, B2), (A2, B3)

- UE starts evaluating the execution conditions of candidate PCells (A1, A2) and candidate PSCells (B1, B2, B3) simultaneously

Scenario 1: UE encounters an RLF or SCG failure before the CHO with candidate SCG execution and execution conditions of no candidate PCell and no candidate PSCells were met before the RLF or SCG failure

Scenario 2: UE encounters an RLF or SCG failure before the CHO with candidate SCG execution and one execution condition was “blocking” the whole execution e.g.,

- A candidate PCell (A1 in the example below) satisfies the execution condition but none of its associated candidate PSCells (i.e., B1 or B2) satisfy the corresponding execution condition.

- A candidate PSCell (B3 in the example below) satisfies the execution condition but the associated candidate PCell (i.e., A2) doesn’t satisfy the corresponding execution condition.

In scenarios 1 and 2, it might be useful for the network to know why the CHO with candidate SCG execution e.g., it didn’t happen because of one “blocking” execution condition (even though the other execution condition was met) or because none of the execution conditions were met

Scenario 3: UE encounters an RLF or SCG failure before the CHO with candidate SCG execution and non-associated PCells and PSCells met the execution condition

In the example below, Execution condition of candidate PCell A1 and candidate PSCell B3 is met but the CHO with candidate SCG execution didn’t happen because there is no (A1, B3) associated configuration.

It would be beneficial if the network can know what execution conditions were met (i.e., A1 and B3) so that the network can fine-tune the CHO with candidate SCG configuration eg., by including a (A1, B3) configuration or (A2, B3) confguration to avoid RLF or SCG failures in the future.

Considering the above example scenarios, we propose the following:

- RAN2 considers how to optimize the following cases when UE receives a CHO with candidate SCG config but encounters an RLF or SCG failure before the CHO with candidate SCG execution

- Neither any candidate PCell met the CHO execution condition nor any candidate PSCell met the CPC execution condition

- Only one type of execution condition was met (i.e., either the CHO execution condition or CPC execution condition) thereby blocking the complete execution

- A candidate PCell met the CHO execution condition and a candidate PSCell met the CPC execution condition, but they were not associated PCell and PSCell pairs

- Enhance RLF Report and SCGFailureInformation with:

- The type of fulfilled execution condition(s) (i.e., CHO execution condition or CPC execution condition or none)

- Identifier of candidate PCell(s) which met the configured CHO execution conditions before the RLF is encountered

- Identifier of candidate PSCell(s) which met the configured CPC execution conditions before the RLF is encountered

- Time elapsed between the fulfilled execution and RLF

# Conclusion

MRO for LTM,

- For too late LTM, UE should report the following information in RLF report.

- For Case 1a, UE should include source cell ID in which RLF is detected and cell ID for re-established cell.

- For Case 1b, UE should include source cell ID in which RLF is detected and LTM candidate cell ID in which HOF is detected.

- For Case 1c, UE should include source cell ID in which RLF is detected and LTM candidate cell ID in which LTM is executed successfully.

- For too late LTM and LTM to wrong cell, UE should report the following information in RLF report.

- For Case 2a/3a, UE should include target cell ID in which HOF or RLF is detected and cell ID for re-established cell. Then the network should have the information about the failure situation, e.g. whether the HOF or RLF is detected in the target LTM cell.

- For Case 2b/3b, UE should include target cell ID in which HOF or RLF is detected and HOF in the selected LTM candidate cell and reestablished cell ID.

- For Case 2c/3c, UE should include target cell ID in which HOF or RLF is detected and LTM candidate cell ID in which LTM is executed successfully.

- Additionally, UE should include the time between UE receives LTM command and HOF/RLF detection.

- UE can include existing RACH information (ra-InformationCommon) in case RLF Report is collected during RACH-based LTM failure.

- UE should omit RACH information (ra-InformationCommon) in case RLF Report is collected during RACH-less LTM failure. UE can further add an explicit indicator that the LTM was RACH-less.

MRO for Subsequent CPAC (S-CPAC),

- Existing measurement information in SCGFailureInformation can be used to identify the best cell, so that it can be added into the candidate subsequent PSCell.

- If the new PSCell is an already prepared candidate PSCell whose configuration is provided to the UE as part of S-CPAC configuration, MN should be able to simply indicate the PSCell to add without providing the whole configuration.

- Upon receiving such an indication to activate a PSCell, UE can simply activate the indicated PSCell’s configuration and start evaluating the S-CPAC configuration.

MRO for CHO with candidate SCG,

- RAN2 considers how to optimize the following cases when UE receives a CHO with candidate SCG config but encounters an RLF or SCG failure before the CHO with candidate SCG execution

    - Neither any candidate PCell met the CHO execution condition nor any candidate PSCell met the CPC execution condition

    - Only one type of execution condition was met (i.e., either the CHO execution condition or CPC execution condition) thereby blocking the complete execution

    - A candidate PCell met the CHO execution condition and a candidate PSCell met the CPC execution condition, but they were not associated PCell and PSCell pairs

- Enhance RLF Report and SCGFailureInformation with:

    - The type of fulfilled execution condition(s) (i.e., CHO execution condition or CPC execution condition or none)

    - Identifier of candidate PCell(s) which met the configured CHO execution conditions before the RLF is encountered

    - Identifier of candidate PSCell(s) which met the configured CPC execution conditions before the RLF is encountered

    - Time elapsed between the fulfilled execution and RLF

# Reference