3GPP TSG-RAN WG2 Meeting #127	R2-2407100

# Maastricht, Netherlands, Aug 19th – 23rd, 2024

Source:                Qualcomm Incorporated

Title:	SON and MDT Rel-18 leftover issues

Agenda Item:	8.10.5

Document for:	Discussion and Decision

# Introduction & Background

Rel-19 SON/MDT agreed to continue Rel-18 leftovers as following, and this contribution discuss the related issues.

- Support of the leftovers in Rel-18 SON/MDT [RAN3, RAN2]:

•	RACH optimization for SDT

•	MHI Enhancement for SCG Deactivation/Activation

•	MRO for MR-DC SCG failure

# Discussion

## 2.1 RACH optimization for SDT

In the last RAN2 meeting, RAN2 had some discussion on RACH optimization for SDT and achieved the following agreement.

- For the purpose of SON enhancements for SDT, include RSRP/data volume related information

- Downlink RSRP value and buffered uplink data volume at the time when the UE evaluates if it should perform SDT.

- When SDT failure happens, the UE can indicate the failure cause of SDT to the network, e.g. T319a expiration. Details are TBD, e.g. if RSRP and data volume can also be included in such report.

For the failure cause, currently there are some failure causes defined for the SDT failure, including e.g. the maximum number of retransmissions has been; if random access problem indication is received from MCG MAC while SDT procedure is ongoing; or cg-SDT-TimeAlignmentTimer or the configuredGrantTimer expired; integrity check failure indication, or T319a expires. It would be beneficial to indicate the failure case to the gNB and gNB can optimize SDT configuration.

- RAN2 discusses the following cause failure included in RACH report for SDT.

•	Maximum number of retransmissions has been attempted;

•	If random access problem indication is received from MCG MAC while SDT procedure is ongoing; or

•	Integrity check failure indication, or

•	T319a expires.

The failure cause of cg-SDT-TimeAlignmentTimer or configuredGrantTimer is used for CG based SDT, it is also beneficial to report to network when the timers expire. But this is not related to RACH optimization for SDT.

- RAN2 discusses whether to optimize CG based SDT via e.g. reporting the timer cg-SDT-TimeAlignmentTimer or configuredGrantTimer expires.

## 2.2 MHI Enhancement for SCG Deactivation/Activation

MHI was introduced so that according to the information i.e., the visited cell id and time spent in the cell in MobilityHistoryReport, the network could have more knowledge about handover experience and optimize handover related parameters, e.g., A3-offset, hysteresis, timeToTrigger.

SCG deactivation/activation was introduced to save UE power with fast enable of SCG configuration when DC is configured. While the SCG is deactivated, there is no transmission via SCG RLC bearers. Only the NR SCG can be deactivated, and all SCG SCell(s) are in deactivated state while the SCG is deactivated. Upon SCG deactivation and while the SCG is deactivated, the network ensures that there is no uplink control PDU transmission to the deactivated SCG (e.g. the network releases statusReportRequired from PDCP entities of SCG bearers if configured, the network does not perform QoS flow remapping from a DRB associated to the deactivated SCG to another DRB).

In DC, SCG MHI is introduced in Rel-17, and PSCell ID and time spent in the current PSCell are included in the visitedPSCellInfoList of MHI. Based on the MHI of SCG, the MN/SN could have more knowledge about the SN, optimize SN configuration and SCG mobility configuration.

On top of existing mechanism, MHI can be further enchanted to report SCG activation/deactivation information. The detailed information could be the time or percentage of time spent during SCG activation or deactivation. This information can help MN know whether DC is needed or whether SCG should be activated in most time.

Observation 1: percentage of time spent in the PSCell in activated state or deactivated state is beneficial.

There could be two ways to collect percentage of time spent in the PSCell in activated state or deactivated state. One is network based solution, another is UE based solution.

Network based solution: MN should know whether SCG is deactivated or not, then MN can collect the time percentage information.

UE based solution: UE can collect percentage of time spent in the PSCell in activated state or deactivated state.

If network based solution can be used, then UE based solution is not needed. So it is proposed to check with RAN3 whether network based solution can work.

- Send LS to RAN3 to confirm the feasibility of network based solution for collecting percentage of time spent in the PSCell in activated state or deactivated state.

# Conclusion

- RAN2 discusses the following cause failure included in RACH report for SDT.

•	Maximum number of retransmissions has been attempted;

•	If random access problem indication is received from MCG MAC while SDT procedure is ongoing; or

•	Integrity check failure indication, or

•	T319a expires.

- RAN2 discusses whether to optimize CG based SDT via e.g. reporting the timer cg-SDT-TimeAlignmentTimer or configuredGrantTimer expires.

- Send LS to RAN3 to confirm the feasibility of network based solution for collecting percentage of time spent in the PSCell in activated state or deactivated state.

# Reference