3GPP TSG-RAN WG2 Meeting #127	R2-2407101

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Source:	Qualcomm Incorporated

Title:	Discovery and Relay (re)selection for multi-hop U2N relay

Agenda Item:	8.13.2

Document for:	Discussion and Decision

# Introduction & Background

In the WID [1], multi-hop U2N relay discovery and Relay (re)selection needs to discussed. This contribution discuss the related aspects.

- Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays.

  - Relay discovery and (re)selection [RAN2]

  - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3]

  - Impact on SRAP and QoS handling for multi-hop [RAN2]

  - Control plane procedures [RAN2, RAN3]

# Discussion

## 2.1 Terminologies

The multi-hop U2N relay has multiple UE roles in multi-hop U2N relay as shown in the following figure, it is necessary to unify the termnology to avoid the confusion during the discussion.

In the WID, the terminology of first relay UE and second relay UE, and last relay UE is used. It could bring confusion during the discussion, and especially when implementing stage-3 CR.

- Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays.

SA2 has already discussed mulit-hop U2N relay and they use Remote UE, Intermediate Relay UE, U2N Relay UE as the terminology,it is better to have the terminologies aligned with SA2 for potential cross WG discussion. Additionally, RAN2 will firstly discuss the scenario of one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). The terminologies of one additional hop and two additional hops are also hard to be descripted. Then it is proposed to use the following terminologies during RAN2 discussion.

- UE role: U2N Remote UE, Intermediate Relay UE, U2N Relay UE

- Two-hop U2N relay: two Relay UEs, i.e. Remote UE <-> Intermediate Relay UE<->U2N Relay UE<->NW

- Three-hop U2N relay: three relay UEs, Remote UE <-> Intermediate Relay UE 1<-> Intermediate Relay UE 2<->U2N Relay UE<->NW.

- It is proposed to use the following terminologies during RAN2 multi-hop U2N relay discussion.

- UE role: U2N Remote UE, Intermediate Relay UE1/2, U2N Relay UE

-Two-hop U2N relay: one additional hop, two Relay UEs, i.e. Remote UE <-> Intermediate Relay UE<->U2N Relay UE<->NW

- Three-hop U2N relay: three relay UEs, Remote UE <-> Intermediate Relay UE 1<-> Intermediate Relay UE 2<->U2N Relay UE<->NW

## 2.2 Assumptions

For the coverage requirements, as existing U2N relay, Remote UE can be in-coverage or out-of-coverage; U2N Relay UE has to be in-coverage. For Intermediate Relay UE, it is also possible the Intermediate Relay UE is in-coverage or out-of-coverage.

- It is proposed to agree the following coverage requirements

- U2N Remote UE can be in-coverage or out-of-coverage

- U2N Relay UE is in-coverage

- Intermediate Relay UE can be in-coverage or out-of-coverage

For RRC state, Remote UE can be IDLE/Inactive/Connected state as today; U2N Relay UE and Intermediate Relay UEs can be IDLE/Inactive/Connected state; when Remote UE is in CONNECTED state, U2N Relay UE is in CONNECTED state. Whether Intermediate Relay UE is in CONNECTED state depends on whether the intermediate Relay UE needs to be controlled by the gNB and, and this can be further discussed during L2 multi-hop relay discussion.

- It is proposed to agree the following RRC state assumptions

- U2N Remote UE can be in IDLE/Inactive/Connected state

- U2N Relay UE can be IDLE/Inactive/Connected state

- Intermediate Relay UE can be IDLE/Inactive/Connected state

- When the U2N Remote UE is in Connected state, the U2N Relay UE has to be CONNECTED state; It is FFS whether Intermediate Relay UE is in CONNECTED state.

## 2.3 Discovery

Discovery AS layer are well defined in Rel-17 U2N relay and Rel-18 U2U relay, it should be reasonable to reuse the existing discovery AS mechanism as much on aspects of e.g. discovery AS protocol, SL-SRB, resource pools, resource allocation mode, configuration.

- Taking the existing discovery mechanisms as baseline for Multi-hop U2N Discovery message

- Reuse existing AS discovery protocol to transmit Multi-hop U2N Discovery message

- Reuse SL-SRB4 transmit Multi-hop U2N Discovery message.

- Reuse existing resource pools defined for discovery message transmission and reception

- Both of resource allocation mode 1 or mode 2 can be supported as today.

- Configuration can be provided by SIB/dedicated message or pre-configured as today

In Rel-17 U2N, gNB can configure the Uu threshold which is used by Remote UE/U2N Relay to determines whether to transmit Discovery message. Similarly, in Multi-hop U2N relay, the U2N Relay is also needed to determine discovery transmission based on Uu link quality. Then it is proposed that gNB can configure the Uu threshold for U2N relay UE to determine discovery message transmission. And It is FFS whether existing Rel-17 parameter or new parameter is needed.

- As Rel-17, gNB can configure the Uu threshold used by U2N Relay UE to determines whether to transmit Discovery message, and It is FFS whether existing Rel-17 parameter or new parameter is needed.

- As Rel-17, gNB can configure the Uu threshold used by U2N Remote UE to determines whether to transmit Discovery message, and It is FFS whether existing Rel-17 parameter or new parameter is needed.

For discovery transmission in Intermediate Relay UE, the existing Rel-18 U2U discovery transmission mechanism can be taken as baseline, i.e. the Intermediate Relay should only forward the Model A discovery or Model B Solicitation message if the PC5 link quality of the receiving link satisfies the AS layer criteria, e.g. a threshold.

- For discovery transmission in Intermediate Relay UE, takes Rel-18 U2U mechanism as baseline, i.e. the Intermediate Relay should only forward the Model A discovery or Model B Solicitation message if the PC5 link quality of the receiving link satisfies the AS layer criteria, e.g. a threshold.

For the AS container included in discovery message, it is assumed that the NCGI and RRC Container defined in Rel-17 are also supported in the multi-hop Relay discovery message.

- It is assumed that the NCGI and RRC Container defined in Rel-17 are also supported in the multi-hop Relay discovery message.

## 2.4 Relay (Re)selection

In exsiting U2N or U2U relay, the Remote UE (re)selects candidate Relay UE considering the hop link quality (SD-RSRP) between the Remote UE and the Relay UE, similar mechanism can be reused for Remote UE to (re)select the candidate Intermediate Relay UEs. And the Remote UE considers an Intermediate Relay UE for which SD-RSRP exceeds a threshold as candidate Intermediate Relay UE.

- From AS layer, Remote UE (re)selects candidate Intermediate Relay UEs considering the hop link quality (SD-RSRP) between the Remote UE and the Intermediate Relay UE.

- From AS layer, Remote UE considers an Intermediate Relay UE for which SD-RSRP between the Remote UE and the Intermediate Relay UE exceeds a threshold as candidate Intermediate Relay UE.

For Intermediate Relay UE, it can select the candidate U2N Relay UE considering the hop link quality between the Intermediate Relay UE and the next Relay UE (U2N Relay UE for two-hops relay or Intermediate Relay UE for three-hops relay). The Intermediate Relay UE considers an Intermediate Relay UE for which SD-RSRP between the Remote UE and the Intermediate Relay UE exceeds a threshold as candidate Intermediate Relay UE.

- From AS layer, Intermediate Relay UE can select the candidate U2N Relay UE considering the hop link quality between the Intermediate Relay UE and the U2N Relay UE.

- From AS layer, the Intermediate Relay UE considers an Relay UE for which SD-RSRP between the Intermediate Relay UE and the U2N Relay UE exceeds a threshold as candidate U2N Relay UE.

Similar with existing U2N relay, the Remote UE can trigger to reselect a candidate Intermediate Relay UE when PC5 link quality between the Remote UE and the current Intermediate Relay UE is below a threshold; or the current PC5 link between the Remote UE and the Intermediate Relay is released due to e.g. RLF, released by upper layer etc.

- The Remote UE can trigger to reselect a candidate Intermediate Relay UE in the following conditions:

-	PC5 link quality towards the current Intermediate Relay UE is below a threshold

-	The current PC5 link is released due to e.g. RLF, release by upper layer, etc.

The Intermediate Relay UE can trigger to reselect the next Relay UE when PC5 link quality between the Intermediate Relay UE and the next  Relay UE is below a threshold; or the current PC5 link between the Intermediate Relay UE and the next Relay is released due to e.g. RLF, released by upper layer etc.

- When RLF is detected or the current link quality is below a threshold between the Intermediate Relay UE and the U2N Relay UE, the Intermediate Relay U2N triggers to reselect a candidate U2N relay UE.

# Conclusion

Terminologies,

- It is proposed to use the following terminologies during RAN2 multi-hop U2N relay discussion.

- UE role: U2N Remote UE, Intermediate Relay UE, U2N Relay UE

-Two-hop U2N relay: one additional hop, two Relay UEs, i.e. Remote UE <-> Intermediate Relay UE<->U2N Relay UE<->NW

- Three-hop U2N relay: three relay UEs, Remote UE <-> Intermediate Relay UE 1<-> Intermediate Relay UE 2<->U2N Relay UE<->NW

Assumptions

- It is proposed to agree the following coverage requirements

- U2N Remote UE can be in-coverage or out-of-coverage

- U2N Relay UE is in-coverage

- Intermediate Relay UE can be in-coverage or out-of-coverage

- It is proposed to agree the following RRC state assumptions

- U2N Remote UE can be in IDLE/Inactive/Connected state

- U2N Relay UE can be IDLE/Inactive/Connected state

- Intermediate Relay UE can be IDLE/Inactive/Connected state

- When the U2N Remote UE is in Connected state, the U2N Relay UE has to be CONNECTED state; It is FFS whether Intermediate Relay UE is in CONNECTED state.

Discovery

- Taking the existing discovery mechanisms as baseline for Multi-hop U2N Discovery message

- Reuse existing AS discovery protocol to transmit Multi-hop U2N Discovery message

- Reuse SL-SRB4 transmit Multi-hop U2N Discovery message.

- Reuse existing resource pools defined for discovery message transmission and reception

- Both of resource allocation mode 1 or mode 2 can be supported as today.

- Configuration can be provided by SIB/dedicated message or pre-configured as today

- As Rel-17, gNB can configure the Uu threshold used by U2N Relay UE to determines whether to transmit Discovery message, and It is FFS whether existing Rel-17 parameter or new parameter is needed.

- As Rel-17, gNB can configure the Uu threshold used by U2N Remote UE to determines whether to transmit Discovery message, and It is FFS whether existing Rel-17 parameter or new parameter is needed.

- For discovery transmission in Intermediate Relay UE, takes Rel-18 U2U mechanism as baseline, i.e. the Intermediate Relay should only forward the Model A discovery or Model B Solicitation message if the PC5 link quality of the receiving link satisfies the AS layer criteria, e.g. a threshold.

- It is assumed that the NCGI and RRC Container defined in Rel-17 are also supported in the multi-hop Relay discovery message.

Relay UE (re)selection

- From AS layer, Remote UE (re)selects candidate Intermediate Relay UEs considering the hop link quality (SD-RSRP) between the Remote UE and the Intermediate Relay UE.

- From AS layer, Remote UE considers an Intermediate Relay UE for which SD-RSRP between the Remote UE and the Intermediate Relay UE exceeds a threshold as candidate Intermediate Relay UE.

- From AS layer, Intermediate Relay UE can select the candidate U2N Relay UE considering the hop link quality between the Intermediate Relay UE and the U2N Relay UE.

- From AS layer, the Intermediate Relay UE considers an Relay UE for which SD-RSRP between the Intermediate Relay UE and the U2N Relay UE exceeds a threshold as candidate U2N Relay UE.

- The Remote UE can trigger to reselect a candidate Intermediate Relay UE in the following conditions:

-	PC5 link quality towards the current Intermediate Relay UE is below a threshold

-	The current PC5 link is released due to e.g. RLF, release by upper layer, etc.

- When RLF is detected or the current link quality is below a threshold between the Intermediate Relay UE and the U2N Relay UE, the Intermediate Relay U2N triggers to reselect a candidate U2N relay UE.

# Reference

[1]. RP-241609    New WID on NR sidelink multi-hop relay    LG Electronics, InterDigital, FirstNet